* Initialization Of Telebit T3000 For 9600 bps ADLC Workstation Use *

1. With modem unplugged from interface, power OFF modem by placing the rocker

switch (on back panel) in the 0 position.

2. Holding the front panel T/D button IN, power modem ON. Continue holding

button until MR, OH, CD, HS, EC all flash continually; then release T/D

button (the lights remain ON). Press and hold the T/D button once more

until LEDs flash again, then release. The MR and CTS lights should remain

ON, all other LEDs should be OFF

3. Execute configuration batch file by typing:

WSI96TBT COMx

where x is the number of the COM port used by the modem.

4. Verify that modem Initialization String in workstation setup is AT&F and

modem Configuration String is ATZ.