[technical specifications]

eicon

technology

corporation

# EiconCard C31

- Hardware Features

- PCMCIA Type II Release 2.1

- Motorola 68302 @ 15.36MHz

- RAM: 512K

- FLASH memory: 512K

- Interrupt level and I/O address: automatically assigned by Plug-and-Play

- V.24 serial interface port or ISDN Basic Rate Interface port

- Serial Port

- DB-25 female connector (EIA RS-232-C) using external cable assembly

- Line speed up to 64 Kbps

- Internal or external clocking

- Protocol support: X.25, Frame Relay, PPP, SDLC, HDLC

- Throughput:

- X.25: 300 pps

- Frame Relay: 350 pps

- ISDN Port

- S/T interface using external cable assembly (RJ45)

- U interface using Eicon ISDN NT1

- Line speed up to 128 Kbps

- Channel support:

- B-channel circuit switched

- B-channel packet switched (X.31 case A, B)

- D-channel circuit switched (X.31 case B)

- B-channel protocol support: X.25, Frame Relay, PPP, Multilink PPP, SDLC, HDLC

- D-channel protocol support: X.25

- Maximum Power Requirements

- 200 mA at +5 Volts

- Environment Requirements

- Operating temperature: 0°C to 50°C

- Operating humidity: 0 to 90% (non-condensing)

- Barometric operating pressure: 86 to 106 kPascals

- Maximum tolerance in power supply variation: +5% to -5%

- Certification

- FCC Class B, Part 15

- CE marking

- Industry Canada (formerly DOC)

- National ISDN-1 (NI-1)

- AT&T 5ESS Custom

- Euro-ISDN (including German, French and Swiss extensions)

- JATE (Japan)

- Warranty

- 5-year

- Dimensions

- 2.918" x 2.015" (7.41cm x 5.12cm)

- Supported OS platforms

- DOS

- Packaging Contents

- EiconCard C31

- EiconCard C31 Installation Guide

- Hardware Diagnostic diskette

- V.24 cable

- with ISDN S/T interface cable

- ISDN S/T Interface cable

- RJ45 cable

- Terminating Resistor (except Europe and Japan)

- with ISDN NT1

- Eicon ISDN NT1 unit and associated cable

- power supply for NT1

- Eicon ISDN NT1 installation guide

- RJ45 cable

| - Ordering Information |  |
| --- | --- |
| - Product Name | - Product Code |
| - EiconCard C31 | - 310-152 |
| - EiconCard C31 with ISDN S/T cable | - 310-116 |
| - EiconCard C31 with ISDN NT1 unit | - 310-159 |
| - Related Accessories |  |
| - V.24 PCMCIA modem cable | - 310-084 |
| - ISDN S/T2 cable - North America | - 310-091 |
| - ISDN S/T2 cable - Europe, Japan | - 310-093 |
| - ISDN NT1 unit | - 310-095 |
| - ISDN N/T2 cable - North America only | - 310-098 |

Eicon Technology Corporation

Tel.:	(514) 745-5500

Fax:	(514) 745-5588

In the United States, Canada, 
Latin America:

Tel.:	1-800-80-EICON

(214) 239-3270

Fax:	(214) 239-3304

In Europe, Middle East, Africa:

Tel.:	(44) 181 967-8000

Fax:	(44) 181 967-8050

In Australia, New Zealand, Asia:

Tel.:	(61) 2 9919-7200

Fax:	(61) 2 9929-6300

Internet:

North America: sales@eicon.com

Europe: sales.europe@eicon.com

Asia Pacific: sales.asiapac@eicon.com

Worldwide Web: http://www.eicon.com/

© 1996 Eicon Technology Corporation.  All rights reserved.