[technical specifications]

eicon

technology

corporation

# EiconCard PacketBlaster

- Hardware Features

- ISA 16-bit

- Hitachi HD64570 @ 10 MHz

- RAM: 64 KB (static)

- Interrupt levels selectable via software (2, 3, 4, 5, 6, 7, 10, 11, 12, 14, 15)

- I/O address selectable via on-board switch (16 options)

- Two VHSI interface ports

- Serial Ports

- Auto-detection of interface types:

- V.24 and X.21bis (EIA RS-232-C)

- V.35

- X.21 with V.11 (X.27) signaling

- V.36/RS-449

- EIA-530

- 36-pin high-density D-type female connectors, supporting all interfaces with appropriate cable

- Line speed up to 2Mbps per port

- Internal or external clocking (DTE or DCE) or split (transmit internal, receive external)

- Protocol support: software dependent

- Maximum Power Requirements

- 1.5 A at +5 Volts

- 155 mA at +12 Volts

- 45 mA at -12 Volts

- Environment Requirements

- Operating temperature: 0°C to 50°C

- Operating humidity: 0 to 90% (non-condensing)

- Barometric operating pressure: 86 to 106 kPascals

- Maximum tolerance in power supply variation: +5% to -5%

- Certification

- FCC Class B, Part 15

- CE marking

- Warranty

- 5-year

- Dimensions

- 6.905" x 4.2" (17.54cm x 10.67cm)

- Supported OS platforms

- NetWare

- Packaging Contents

- EiconCard PacketBlaster

- EiconCard PacketBlaster Installation Guide

- NetWare drivers diskette

- NetWare drivers installation guide

| - Ordering Information |  |
| --- | --- |
| - Product Name | - Product Code |
| - EiconCard PacketBlaster for NetWare | - 305-674 |
| - Related Accessories |  |
| - V.24 VHSI modem cable | - 310-077 |
| - V.24 VHSI null modem cable | - 310-078 |
| - V.35 VHSI modem cable | - 310-076 |
| - V.35 VHSI modem cable (France) | - 310-083 |
| - X.21 VHSI modem cable | - 310-081 |
| - V.36/RS-449 VHSI cable | - 310-079 |
| - EIA-530 VHSI cable | - 310-080 |
| - VHSI-to-VHSI null modem cable | - 310-075 |

Eicon Technology Corporation

Tel.:	(514) 745-5500

Fax:	(514) 745-5588

In the United States, Canada, 
Latin America:

Tel.:	1-800-80-EICON

(214) 239-3270

Fax:	(214) 239-3304

In Europe, Middle East, Africa:

Tel.:	(44) 181 967-8000

Fax:	(44) 181 967-8050

In Australia, New Zealand, Asia:

Tel.:	(61) 2 9919-7200

Fax:	(61) 2 9929-6300

Internet:

North America: sales@eicon.com

Europe: sales.europe@eicon.com

Asia Pacific: sales.asiapac@eicon.com

Worldwide Web: http://www.eicon.com/

© 1996 Eicon Technology Corporation.  All rights reserved.