[technical specifications]

eicon

technology

corporation

# EiconCard EC/PC

- Hardware Features

- ISA 8-bit

- Motorola 68008 @ 10MHz

- RAM: 1 MB

- Interrupt levels selectable via software (2, 3, 4, 5, 6, 7)

- I/O address selectable via on-board switch (14 options)

- 4K shared memory access with a software selectable starting address (32 options between A000-DFFF)

- V.24 serial interface port

- Serial Port

- DB-25 female connector (EIA RS-232-C)

- Line speed up to 64 Kbps

- Internal or external clocking

- Protocol support: X.25, Frame Relay, PPP, SDLC, HDLC

- Throughput:

- X.25: 50 pps

- Frame Relay: 80 pps

- Maximum Power Requirements

- 1.0 A at +5 Volts

- 45 mA at +12 Volts

- 45 mA at -12 Volts

- Environment Requirements

- Operating temperature: 0°C to 50°C

- Operating humidity: 0 to 90% (non-condensing)

- Certification

- FCC Class B, Part 15

- CE marking

- Warranty

- 5-year

- Dimensions

- 7.8" x 4.2" (19.81cm x 10.67cm)

- Supported OS platforms

- DOS

- NetWare

- OS/2

- UNIX

- Windows 3.x

- Windows NT

- Packaging Contents

- EiconCard EC/PC

- EiconCard EC/PC Installation Guide

- Hardware Diagnostic diskette

| - Ordering Information |  |
| --- | --- |
| - Product Name | - Product Code |
| - EiconCard EC/PC | - 310-053 |
| - Related Accessories |  |
| - V.24 RS-232 modem cable | - 310-007 |
| - V.24 RS-232 null modem cable | - 310-022 |
| - V.24 RS-232 gender-changed null modem cable | - 310-048 |

Eicon Technology Corporation

Tel.:	(514) 745-5500

Fax:	(514) 745-5588

In the United States, Canada, 
Latin America:

Tel.:	1-800-80-EICON

(214) 239-3270

Fax:	(214) 239-3304

In Europe, Middle East, Africa:

Tel.:	(44) 181 967-8000

Fax:	(44) 181 967-8050

In Australia, New Zealand, Asia:

Tel.:	(61) 2 9919-7200

Fax:	(61) 2 9929-6300

Internet:

North America: sales@eicon.com

Europe: sales.europe@eicon.com

Asia Pacific: sales.asiapac@eicon.com

Worldwide Web: http://www.eicon.com/

© 1996 Eicon Technology Corporation.  All rights reserved.