[technical specifications]

eicon

technology

corporation

# DIVA T/A

- Hardware Features

- PCMCIA Type II Release 2.1

- 16550 COM port (COM1 through COM4)

- FLASH memory for firmware upgrade

- ISDN Basic Rate Interface port

- System compatibility

- PC

- Macintosh Powerbook

- Kasten Chase RAS

- ISDN Port

- S/T interface using external cable assembly (RJ45)

- U interface using external integrated-NT1 cable assembly

- Line speeds of 56/64Kbps per B-channel (128Kbps with MLPPP)

- Rate Adaptation: V.120, V.110¹

- B-channel protocol support: async PPP, async/sync PPP, MLPPP1

- D-channel protocol support: X.251

- Auto-detection of ISDN switch type

- PAP/CHAP authentication

- PPP compression (Microsoft, Bay Networks and Cisco compatible) 1

- Environmental Requirements

- Operating temperature: 0°C to 50°C

- Operating humidity: 0 to 90% (non-condensing)

- Barometric operating pressure: 86 to 106 kPascals

- Maximum tolerance in power supply variation: +5% to -5%

- Maximum Power Requirements

- 200 mA at +5 Volts

- Certification

- FCC Class B, Part 15

- IC (formerly DOC) approved

- CE marking

- National ISDN (NI-1)

- AT&T 5ESS Custom

- Euro-ISDN (including German and French extensions)

- TPH (Australia)

- JATE (Japan)

- Warranty

- Five years

- Packaging Contents

- DIVA T/A

- S/T or U interface cable

- Cabling (RJ45)

- User Manual

- Release Note

- ISDN Line Terminator

- IDI (ISDN Direct Interface)

| - Ordering Information |  |
| --- | --- |
| - Product Name | - Product Code |
| - DIVA T/A PC Card - North American (S/T Interface) | - 310-166 |
| - DIVA T/A PC Card - North American (U-Interface)) | - 310-167 |
| - DIVA T/A PC Card - International | - 310-174 |
| - DIVA T/A PC Card - Australia | - 310-180 |
| - Related [items] |  |
| - DIVA T/A ISDN S/T interface cable | - 300-104 |
| - DIVA T/A S/T Interface for Australia | - 300-105 |
| - DIVA T/A U-Interface cable (integrated NT-1) | - 300-106 |

Eicon Technology Corporation

Tel.:	(514) 745-5500

Fax:	(514) 745-5588

In the United States, Canada, 
Latin America:

Tel.:	1-800-80-EICON

(214) 239-3270

Fax:	(214) 239-3304

In Europe, Middle East, Africa:

Tel.:	(44) 181 967-8000

Fax:	(44) 181 967-8050

In Australia, New Zealand, Asia:

Tel.:	(61) 2 9919-7200

Fax:	(61) 2 9929-6300

Internet:

North America: sales@eicon.com

Europe: sales.europe@eicon.com

Asia Pacific: sales.asiapac@eicon.com

Worldwide Web: http://www.eicon.com/

© 1996 Eicon Technology Corporation.  All rights reserved.

Via firmware upgrade