INTRODUCTION:

This package contains three things:

1. A library of routines to interface to an Ampex Megastore PC SCSI

host adapter, written in Turbo C 1.5.

2. A generic MSDOS device driver, written in C and a little assembler.

3. An actual device driver for SCSI disks that uses 1 and 2 above.

Feel free to distribute this however you like.

THE AMPEX MEGASTORE HOST ADAPTOR:

The Ampex Megastore SCSI host adapter is a relatively "dumb" adapter.

Its virtue is that you can get one for $15 or so from Halted Specialities,

in Santa Clara, CA.  It also has supporting software (what you have here)

that will let you use it to talk to SCSI disks, tapes, etc., hooked up to

your PC.  I use one to communicate with two hard disks and a tape drive on

my AT&T PC6300.

This board comes with no documentation.  Here is what I have learned:

The board uses I/O ports 0x320 to 0x323.  These addresses clash with

the standard PC hard disk adaptor addresses.  However, you can set

a switch or two on the board to relocate these addresses to 0X620 to

0x623.  You will have to change a #define in scsi.c if you do this.

Unfortunately, I forget which switches on the board change the address.

Try them and find out.

The board also uses DMA channel 3, and an interrupt line.  However,

the included code does not use the interrupt, or allow the board to

generate it.  There are jumpers on the board to change the DMA or interrupt

numbers, if necessary.  If the DMA channel is changed, code in scsi.c

will need a little bit of surgery.

Remove the EPROM on the board!  The code in it seems to require a special

non-standard SCSI controller to work.

Here is a description of what the I/O ports do, as deduced by me:

Address		Read			Write

----------------------------------------------------------------------

0x320:  	Data			Data

0x321:		SCSI Control Lines	Activate SCSI RESET line

0x322:		Clear SCSI SEL, RESET	Activate SCSI SEL line

0x323:		XXXXX			Enable DMA

Here is more info on reading port 321:

Bit	Signal

-----------------------------

0	A switch on the board (I forget which)

1	Another switch

2	???

3	SCSI MSG line

4	SCSI BSY line

5	SCSI C/D line

6	SCSI R/W line

7	SCSI REQ line

Writing any value to ports 321 and 322 seems to trigger the desired function.

See the code in scsi.c for how these signals are used.  The functionality

provided in scsi.c should complete enough so that you do not have to worry

about the above information.

THE SCSI CODE:

Scsi.c contains two routines:

void scsiop(struct scsireq *req);

void scsiopv(struct scsivreq *req);

The first one is passed a pointer to a structure of data describing

a SCSI operation, and does it.  The second one does the same thing,

but allows you to specify multiple data buffers in separate segments.

This allows you to send or receive more than 64K of data in one operation.

These is the structure that is passed to scsiop():

struct scsireq {

char far *dptr;

char far *cptr;

unsigned dlen;

char busid;

int error;

int timeout;

};

Dptr points to your data buffer.  If the command you are executing

reads or writes no data, this can be NULL.

Cptr points to  your command bytes for the SCSI operation.

The code will read as many command bytes from here as the SCSI

device asks for.

Dlen is the length of the data buffer.  If you have more than 64K-1

bytes of data, use scsiopv() instead.  The code will not read or write

past the end of the buffer.  If the SCSI device tries to, an overrun

error will be generated.

Busid is the SCSI bus ID that the command will be sent to.

Upon return, error will be filled in with an error code.

Timeout is the number of .1 second units that the code should wait

before giving up and generating a timeout error.  Specifying zero

will give a default five-second timeout.  This timing depends on

loops in the code, and will have to be adjusted for extra fast or

slow PCs.  It is correct for a 8 MHz V30.

Here is the meaning of the error word:

The low 8 bits contain the SCSI Status byte generated by the

SCSI device at the end of the transaction.  The upper 8 bits

are as follows:

/* scsireq error bits */

#define S_BUSERROR		0x8000

#define S_BADSTATUS		0x4000

#define S_BADMESSAGE		0x2000

#define S_NOCONNECT		0x1000

#define S_TIMEOUT		0x0800

#define S_OVERRUN		0x0400

#define S_BADTRANS		0x0200

Note that these bits represent hardware errors or errors in the SCSI

protocol itself.  Errors in the device, such as a bad block,

will cause a non-zero Status byte to be generated.  The SCSI

Get Sense command will have to be used to find out more.

These are the arguments to scsiopv():

struct scsivreq {

struct scsibuf far *bufptr;

char far *cptr;

char busid;

int error;

int timeout;

};

struct scsibuf {

char far *dptr;

unsigned dlen;

};

Scsiopv() works the same as scsiop(), except that instead of taking

a single pointer to a data buffer, it takes a pointer to a list

of data buffers of varying lengths.  The last buffer in the list

should have a length of zero and a NULL address.   These buffers

will be read or written in order.

THE MSDOS DEVICE DRIVER CODE:

The makefile included describes how to make the SCSI disk device driver

in this package.  The tricky code is generic, and by changing a couple of,

files, you can make your own device driver.

The files driasm.s, driver.h, and null.c, and they way the makefile

compiles and links them, should not be changed.  The file dheader.s

should have to be modified only if you need to make a character device

instead of a block device.  The file driver3.c should be modified

to do what you need your driver to do.  The only procedure called externally

in this file is dointr().  It  is passed a pointer to the request header

that describes what should be done.  The current version basically

vectors to a seperate procedure for each request type.  You should probably

keep this arrangement, and change as little code as possible.  The file

scsi.c is part of this driver, but would of course be omitted if you

were not using a SCSI device.  Note that making "dtest" with the makefile

should produce error-free compiles and link if everything is written correctly.

The file dtest.c can be added to to make a program that will test your driver

without having to load it at boot time.

All of the above assumes you know enough about how device drivers are

supposed to be written.  I recommend the book "Advanced MS-DOS" by Microsoft

Press as a good reference for learning this.

Good Luck,

Doug Braun

7976 W. Zayante Rd.

Felton, CA 95018