1

:

$**$ MARGIN=.6

RUNNING MTS GRAPHICS PROGRAMS ON THE IBM PC

1. Introduction

This  document  describes how graphics programs (e.g. *PLOTSEE)

that produce graphics on a TEKTRONIX 4010 series terminal  may  be

run on an IBM PC with output on an attached graphics monitor. This

is achieved by use of TEKTRONIX 4010 terminal  emulation  software

(TEKEMUL)  running  on  the PC. This program communicates with MTS

and causes the TEKTRONIX ASCII codes from MTS  to  be  interpreted

and displayed on the monitor. Thus a user needs a graphics monitor

in order to use the system. TEKEMUL is written in complied  BASIC.

The structured nature (as much as BASIC permits) of the program is

intended to allow it to be adapted to other micro-computers in the

future.

2. Using TEKEMUL to Output MTS Graphics on the PC

Here  we  assume  that  the  user  has  some  familiarity  with

operating the IBM PC. The  emulation  program  displays  graphical

information  in the high resolution mode of the computer. Thus the

IBM colour/graphics monitor card and a graphics  monitor  must  be

attached  to  the  IBM  PC  in order that the facility can work as

intended.

Upon inserting the disk containing TEKEMUL into the disk  drive

one  merely  types  in  "TEKEMUL".  This  command loads and begins

executing the emulator program.

The program is of an interactive nature making it quite easy to

use. The following describes the use of the TEKEMUL program:

a)  A general instruction menu is displayed. This explains some

of the operational details on how the program is  to  be  used.

Pressing any key allows one to continue on.

b)  A  communication set-up menu is then displayed. The default

parameter values for baud rate, parity, data size,  and  number

of stop bits is displayed in bold face. It is up to the user to

ensure that the communication  parameters  are  correct  before

continuing.  For  example,  he should insure that the baud rate

given to the program is the same as the baud rate of  the  line

that communicates with MTS.

Changing  the  communication  parameters  is a simple matter of

entering the new value of the parameter in  place  of  the  old

value.  If  all of the parameters are correct the user enters a

"Y" to indicate that the parameters displayed are correct.  The

message  "Time  to  flick  your  data  set now..." is displayed

momentarily and then the screen is cleared. It is at this point

that the user initiates the connection with MTS.

1

c)  The  standard  MTS header should be displayed at this time.

When prompted for terminal-id, you should respond with "T4010".

This  signifies  to  MTS  that you have a Tektronix 4010 series

terminal. You can now sign onto MTS in the usual manner.

d) TEKEMUL at this point in time is emulating a  simple  "dumb"

terminal;  it  is  not  emulating  the Tektronix 4010. Thus any

attempt to run a graphics program will yield erroneous results.

The Tektronix emulation phase is initiated by  pressing  F9  on

the keyboard.

e)  Pressing F9 yields another menu which asks you what type of

display monitor(s) you have. Enter "a", "b", or "c".  And  asks

whether  you want to use *IG or *PLOTSEE. Enter "c" or "d". And

finally it prompts you f�r the object module that will generate

�     the  graphics.  You  may press the enter key if you are running

*PLOTSEE.

f) The program then switches to the color monitor (if  you  are

not already on it) and issues the MTS RUN command in the form:

$RUN -LOAD#+*IG

or

$RUN *PLOTSEE

depending  on  which  of  the two facilities you selected. When

your  graphics  program  has  completed  running  you  will  be

returned to MTS.

3. Running More Than One Graphics Program

Once  you  have  been<eke>  returned to MTS you can run another

graphics program in one of two ways. You can (a) issue an MTS $RUN

command directly or (b) press F9 once again.

The  MTS  $RUN  command will work directly since you previously

pressed F9. Remember that F9 places  you  in  Tektronix  emulation

mode  and onto the color monitor. Thus any graphics codes intended

for a Tektronix 4010 will be interpreted and executed by TEKEMUL.

You may also press F9 again to continue having graphical  programs

run.

4. Exiting the Emulator and the Color Monitor

Pressing  F8  on  the  keyboard  will  transfer you back to the

monochrome monitor ( if your system has one ). It will  also  take

you out of Tektronix emulation mode but will leave you on MTS.

1

5. Exit to DOS

Pressing  the  F10  function  key  will  cause  the  program to

transfer control back to the monochrome monitor ( if  your  system

has  one  )  and will end the TEKEMUL program, leaving you back in

DOS. It will not explictly sign you off MTS. You must either  sign

off yourself or disconnect the line.

6. System Overview

TEKEMUL  accepts  Tektronix  4010 codes and plots the graphical

information represented by these on the color  monitor.  Figure  1

illustrates how the system works.

--------------------

| Graphics Program |

MTS  |        |         |

|  Tektronix 4010  |

|       codes      |

--------------------

|

Communication

line

|

------------------------------

|           TEKEMUL          |

|              |             |          -----------

IBM  |   ----------------------   |          |  Color  |

PC   |   |                     |  |----------| Monitor |

|  Communication  Tektronix  |          -----------

|     Program        Code    |

|                Interpreter |

------------------------------

Figure 1 - System Diagram

To get MTS graphics on the PC color monitor, we  have  to  have

two  programs running; one is the graphics program on the MTS side

and the other is TEKEMUL in the PC. The latter allows the user  to

connect  and sign on to MTS. At this stage no graphics is done yet

and the PC simply acts as a regular  terminal  with  the  help  of

TEKEMUL.  Once  signed on the user can run the graphics program on

MTS (such as *IG or *PLOTSEE) that generates  the  Tektronix  4010

codes.

As MTS sends out the Tektronix 4010 codes, the emulator program

on the IBM PC interprets the information and produces a picture on

the monitor.

1

6.1 General System Information

The  information  passed  to the Tektronix emulation program is

the same as that which would be passed to a Tektronix 4010  series

terminal.

Plotting  of graphical information on the IBM PC is done in the

high resolution mode of the BASIC compiler. Thus a  color  monitor

hooked up to the IBM color monitor adapter must be used.

The  resolution  of  high  resolution mode on the IBM PC is 640

horizontal pixels by 200 vertical. Thus TEKEMUL performs a mapping

from  the  Tektronix  coordinate  space (1024 x 780) to the IBM PC

coordinate space.

7. TEKEMUL Details

The code for this  program  has  been  largely  separated  into

subroutines  and  function  key  and  communication trap routines.

Figure 2 shows the program flow.

Define and initialize

global variables

|

Go to subroutine

Set up screen  border

|

Print out general

instruction menu

|

Wait for keyboard input

|

Go to subroutine:

clear out body of menu

|

Print out communication

parameter  menu

|

Set  up  communications.

Initialize communications

and F8 F9 and F10  trap  routines.

|

|

------>-----------|

|                 |

|                 |

|                 |

------<--------Keyboard input ?

|                 |

|     No          |    Yes

|                 |

-------<------- Send input to MTS

Figure 2 - TEKEMUL program flow

1

The program follows the sequence of events outlined  in  Figure

2.  The last loop is one that the program constantly executes when

it has nothing to do: reading input from the keyboard and  if  any

input exists, transmitting it to the host.

All  other  functions  of  the program are accomplished through

trap  (interrupt)  routines.  When  a  trap  routine  is  finished

executing,  control is once again returned to the keyboard reading

loop.

7.1 Subroutine and Trap Routines

There are three trap routines in this program. One of  them  is

for  the  communications buffer; the other three for function keys

F8, F9 and F10.

Any information received from  the  host  causes  an  interrupt

situation and control is passed to the communication trap routine.

The structure diagram for this routine is in Figure 3.

Communication        YES

---------------> buffer empty? ----------------> Return

|                      |

|                      |

|                      |  NO

|                      |

|               Get a character

|                      |

|               Are we in Tektronix

|               emulation mode?

|                   |  |

|                   |  |

|       -----<---------------->-------

|       |                            |

|       |                            |

|   Go to Tektronix            perform ordinary

|   simulation                 terminal functions based

|   subroutines                on characters received

|       |                            |

|       ----->-----------------<------

|                      |

|                      |

-----------<------------

Figure 3 - Communication Trap Routine

1

The communication trap routine  processes  characters  received

from  MTS one at a time until the IBM PC's communication buffer is

empty. If the program is in  Tektronix  emulation  mode  then  the

character is passed to the subroutines that actually simulates the

Tektronix 4010.

If the program is not in  Tektronix  emulation  mode  then  the

character  is  merely  printed  on  the  screen.  The routine then

carries on to deal with the next character.

The other three trap routines are executed depending on whether

F8  F9  or  F10  is  pressed  on the keyboard. The F8 trap routine

allows the user to stay signed onto MTS, with the  output  of  the

screen  going to the monochrome screen ( if one is attached ). The

advantage is that most people prefer to use the monochrome monitor

for  viewing  text  as  opposed to the colour monitor. The F9 trap

routine begins the Tektronix emulation mode, switches the user  to

the  color  monitor and issues various instruction menus. Figure 4

shows the program flow for this routine.

Set up instruction menu

|

Prompt for monitor types

|

|                       NO

Does user have  colour  monitor  -------->stop

|

YES

|

Prompt for graphics software

to use (*IG or  *PLOTSEE)

|

Prompt  for  user's  object module

|

Switch  to  colour monitor

|

Enable Tektronix emulation mode

|

Send $RUN command to MTS

|

Return

Figure 4 - F9 Trap Routine

1

The F10 trap routine ends Tektronix emulation mode and switches

the  user  back  to  the monochrome monitor (if there is one). The

diagram for this routine is in Figure 5.

Disable Tektronix emulation mode

|

Switch to monochrome monitor if

there is one

|

Exit to DOS

Figure 5 - F10 Trap Routine

The  subroutines  that actually simulate the Tektronix 4010 are

comprised of TEKSIM, ESCAPE-TRUE and ESCAPE-FALSE.

TEKSIM is called from the communications trap routine. Here the

current character from the communications buffer is placed in BYTE

as an ASCII decimal value. If the previous charactor was an escape

character ESCAPE-TRUE is called; else, ESCAPE-FALSE is called.

ESCAPE-TRUE  simulates  the  various  functions  of  the escape

sequences for the Tektronix 4010.

ESCAPE-FALSE checks to  see  if  the  current  character  is  a

control  character  or  not. If it is a control character then the

appropriate control function for the Tektronix 4010 is simulated.

Characters other than control characters are decoded  into  X,Y

coordinates.  When a complete (X,Y) address is decoded then a move

or  a  draw  is  made  to  that  coordinate  address.  A  draw  is

accomplished  by using the BASIC subroutine LINE to draw a line on

the screen.

7.2 Global Variables

Global variables  important  to  the  simulation  part  of  the

program are described below.

TEKAMUL:  a  flag  indicating  whether the program is in Tektronix

emulation mode (TEKEMUL=1) or isn't (TEKEMUL=0).

MODE: indicates if we are in graph mode  (MODE=0)  or  alpha  mode

(MODE=1) of the Tektronix 4010 terminal.

DARK:  this flag, when set to TRUE, indicates that the next vector

drawn is to be a dark vector. DARK  is  set  to  TRUE  when  a  GS

control character is encountered.

1

7.3 Adaptability to other Micros

TEKEMUL has been structured so that it may easily be adapted to

other micro  computers.  Should  the  communications  facility  on

another  micro  be different than the IBM PC's then one can merely

use the simulator routines TEKSIM, ESCAPE-TRUE,  and  ESCAPE-FALSE

from the TEKEMUL program.

All  that  these routines require to function by themselves are

the global variables defined in the start of  the  program  and  a

character given to the subroutine TEKSIM.

8. History of the Emulator Program

The original Tektronix emulation project began with a desire to

preview an MTS graphic Plot Description File (PDF) on the IBM  PC.

We  decided  to write a program for the IBM PC which would be much

like *PLOTSEE on MTS, except it would run in the PC.  However,  we

encountered  some  practical  problems  with this scheme. The most

serious of these problems was the amount of time required to  plot

a PDF.

This  time  factor  was  due to the fact that the PDF had to be

transferred over to the IBM PC in order to be plotted. A  PDF  can

be  on  the  order  of  800,000  bytes.  Given  this  size  it was

calculated that many plot files would require several  minutes  to

be transferred using the file transfer programs.

Another  problem complicating the design approach was that Plot

description files are in binary format. The file transfer programs

transfer   binary   information   much   slower   than   character

information.

One solution to this was to represent vector coordinates  in  a

more  compact form. This solution led to the Tektronix 4010 method

of transferring coordinates as efficiently as possible.

We considered  converting  a  plot  description  file's  vector

coordinates  into  Tektronix  format vector coordinates. This much

smaller file could then be transferred over  to  the  IBM  PC  (as

character data) and then plotted.

But  there  was  no  sense  writing  a  conversion  program  to

translate a PDF into Tektronix  format  vector  coordinates  since

Integrated  Graphics already has a device driver for the Tektronix

4010 that generated what was needed. It was desirable to eliminate

the  file  transfer  step  and have the Tektronix emulator work in

real time as MTS delivered Tektronix 4010 graphics commands.

The regular Tektronix 4010 device driver can be  used  on  MTS,

and,  it would be easy for the user to operate. In fact the IBM PC

would operate just like a Tektronix 4010 from a logical  point  of

view.

1

#

### to operate. In fact the IBM PC

would operate just like a Tektronix 4010 from a logical  point