/==========\

|  NEW14H  |				(2.7)

\==========/

Marty Ross

c/o Computing Resource Center

10801 National Blvd.,  Third Floor

Los Angeles, CA  90064

Release:	2.7, 10/18/85

By:    		Marty Ross, at the Computing Resource Center

System: 	IBM-PC with standard communications adapter (COM1: or COM2:)

Requirements:	Tested using PC-DOS 2.0/2.1/3.1, 128K or larger PC preferred.

Abstract:	NEW14H is a DOS-extension (resident program) which intercepts

calls made to the BIOS communication application interrupt

(INT 14H), and adds extended functions to the communication

repertoire.  The program maintains a 32K input buffer, and

a 255 byte output buffer, and uses both input and output

hardware interrupts for maximum efficiency.

If none of this makes any sense, then you don't need NEW14H

unless you are using application software which requires it,

sich as TSHARE, the background terminal emulator driver.

NEW14H is of primary interest to communication software

developers only.

Distribution:	I am distributing this preliminery version freely, in hopes to

stimulate interest and feedback.  No fee may be charged for its

distribution, and this document must accompany the programs

documented herein.

Features:	NEW14H provides a more accurate way to read characters from

the communication port.  The application program need not

keep up on a character basis with the incoming data: a 32K

circular input buffer is used and always updated while any

normal PC software executes.  If NEW14H is installed on two

ports, then each port has 32K input buffer so is independant

from the other device.  See extended function calls in .ASM

excerpt for details on extended functions provided by NEW14H.

Usage:		There are three steps necessary to activate NEW14H so that

it may be used to by application software:

1.)  Initialize DOS

2.)  Install the NEW14H communication enhancements

4.)  Initialize the COM: port for desired baud/parity

(NEW14H DOES NOT DO THIS: USE PC-DOS "MODE.COM")

A typical installation sequence (which can be combined into

a .BAT file, such as "AUTOEXEC.BAT"):

+---------------------------------------------------------+

|							  |

(1)	|	<ALT>+<CTL>+<DEL> ...				  |

|							  |

|	IBM Disk Operating System			  |

|	Version x.xx ...				  |

|							  |

(2)	|	A>new14h					  |

|	NEW14H - 2.x - (C) Copyright 1985, by Marty Ross  |

|	Auxiliary COM1: port driver active.		  |

|							  |

(3)	|	A>mode com1:300,n,8				  |

|	  COM1:300,n,8,-				  |

|							  |

|	A>_						  |

|							  |

+---------------------------------------------------------+

Why NEW14H?:	NEW14H was written because the BIOS counterpart is severely

deficient, and does not even use the powerful interrupt cap-

abilities on the communication devices used in the IBM-PC.

NEW14H intercepts the common call made to the BIOS services

for async communication, and although not tested for transp-

arency, was modeled after the old INT 14H so that existing

application software can take advantage of the enhancements

provided without modification.  Additional calls provided by

NEW14H also add a degree of power to the INT 14H system inter-

rupt which software developers can use for convenience, and

power which previously was possible only by taking care of

all the details of communication themselves, detracting from

their overall purpose.

Disclaimer:	Though I've tested NEW14H under varying conditions, I cannot

guarantee that it will work in every case/PC configuration.

Also, I suggest you use caution when using NEW14H in new

situations (on two ports, etc): I will not be liable for any

damage caused by NEW14H, but would like to be informed if any

undesireable interactions occur.  Please write me.

On-line help:	For on-line info, type "NEW14H ?" which will cause the following

display:

+----------------------------------------------------------------------+

|	syntax:							          |

|			new14h [port#]				          |

|	port#:							          |

|			1 - COM1:				          |

|			2 - COM2:				          |

|								          |

|	   You can use the PC-DOS "MODE" command to initialize	          |

|	or change the channel baud rate, and parity settings	          |

|	(e.g. "MODE COM1:300,E,7").				          |

|								          |

|	   The communication channel will occupy approximately	          |

|	33K of memory, and will remain active until CTL-ALT-DEL.          |

|								          |

+----------------------------------------------------------------------+

which indicates that you may install NEW14H on either the COM1: or

the COM2: port by specifying the port number.

NOTE: As indicated above, NEW14H does not automatically initialize

the selected communication port for baud rate or parity set-

tings.  Use the DOS 'MODE' command for this purpose BEFORE

attempting to use the port for communication.

About the Author:

Marty Ross is a consultant for the Computing Resource Center,

a data processing firm in Los Angeles.  CRC uses the IBM VM/SP

operating system to provide its clients with an interactive

method to access popular mainframe data handling facilities,

such as SAS, PL/1, FORTRAN, and the On-Line Data System,

CRC's own database management product.  For microcomputers,

CRC has released a Statistics/Data Analysis and Graphics

package for the IBM-PC called STATA, the Data Tool, which

features standard statistical (regression) analyses, excellent

data handling capabilities, and a wide variety of experimental

data analysis techniques with STATA/Graphics.  If you are

interested in obtaining a DEMO copy of STATA, please call CRC

on their toll-free number:  1-800-782-8272 outside California,

or dial them directly at (213) 470-4341.  Be sure to tell them

that Marty sent you!

Technical specs:  (excerpt from source program):

----------------

TITLE	NEW14H - Enhanced communications driver for COM adapter

SUBTTL	Copyright (C) 1985 by Marty Ross

PAGE	66,132

; last mod 10/21/85 - new function x'90': "enable_ints"

;

; NEW14H - Replacement driver for PC BIOS INT 14 call.

;

;	Defines interrupt driver COMmunication "channel," and intercepts

;	INT 14H for UART INIT, character READ/WRITE, and other added functions.

;

;	NOTE: This version supports only one buffer configuration:

;

;		256 byte OUTPUT buffer

;		32K byte INPUT buffer

;

;	Both lists are treated as circular character lists, and an error

;	return is indicated for output overflow by the TIMEOUT BIT, and

;	and the value EOF in AL.  Application must process incoming data

;	fast enough so as not to allow more than 32K (size of input buffer)

;	of data to remain un-processed (READ via RECV_CODE function call).

;

;   functions intercepted (replaced):

;

;	code(AH)	function		added feature(s)

;

;	0		INITIALIZE UART		Re-enables interrupt system

;	1		SEND CHARACTER		Interrupt driven, buffered.

;	2		RECEIVE CHARACTER	Interrupt driven, buffered.

;						NOTE: will HANG for TIMEOUTCNT

;	3		RETURN MSR & LSR

;

;   as with old NEW14H, B7 of AH returned is timeout bit (as per TIMEOUTCNT),

;   and AH will always return with value of line status (bits 0-6).

;

;   added functions:

;

;	80h		RETURN VERSION NUMBER OF THIS DRIVER IN AH, AL.

;			MAJOR VERSION # WILL NOT EXCEED 9, SO IF AH RETURNS

;			WITH VALUE > 9, THIS PROGRAM IS NOT INSTALLED AND

;			'ADDED' SUB-FUNCTIONS (BELOW) SHOULD NOT BE USED.

;	84h		RETURN SIZE OF COMMUNICATION BUFFER

;	85h		FLUSH COMMUNICATION BUFFER

;	86h		CHECK FOR ANY CHARACTERS IN BUFFER/RETURN Z IF NONE

;			IF CHAR(S) EXIST, FIRST CHAR RETURNED IN AL

;	87h		SEND BREAK SIGNAL OVER SELECTED PORT

;	88h		RETURN 0 in AX if no chars in buffer, else non zero

;	89h		Re-ENABLE interrupts (after initializing baud rate)

;			(since we don't intercept SET BAUD RATE function,

;			must use BIOS' which disables our interrupts!).

;	8AH		This call allows caller to predict whether I/O is

;			currently taking place (at the device level).  If

;			char(s) received since last call, AL will be NZ.

;	8BH		This call sets or resets an ALARM feature which causes

;			the IBM-PC (BIOS) BELL to ring when a control-G ASCII

;			character is read from the communication line.

;			(AL=0 means TURN ALARM OFF, AL!=0 means TURN ALARM ON)

;	8CH		Pass non zero value back in AL if exist characters in

;			the output queue.

;	8DH		Read the value at the UART port offset from base port

;			(offset given in AL, i.e. - COM1: base=3F8), value is

;			returned in DL.

;	8EH		Write the UART port (offset from base port given in AL),

;			value to be written is passed in DL.

;	8FH		Set or reset XON/XOFF protocol.

;			AL=0 Means RESET, non zero AX means SET

;	90H		Force enabling the interrupts, similar to 89H but not

;			as extensive; only does OUT ier,INT_TYPS

;

;   DX must contain PORT # (0,1 valid).

;   I.E. 0=COM1, 1=COM2, etc.

----------------------------

[1]  NEW14H will be destroyed by programs such as BASIC or BASICA which

re-define the communication interrupt driver(s).

------------------------------------------------------------------------------

/==========\

|  NEW14H  |				(2.7)

\==========/

Marty Ross

c/o Computing Resource Center

10801 National Blvd., Third Floor

Los Angeles, CA  90064

Release:	2.7, 10/18/85

Comments?

Can you use NEW14H?

Name:       _________________________________________________________

Address:    _________________________________________________________

_________________________________________________________

_________________________________________________________

---------------------------------------------------------

Comments:   _________________________________________________________

_________________________________________________________

_________________________________________________________

_________________________________________________________

_________________________________________________________

_________________________________________________________

_________________________________________________________

_________________________________________________________

_________________________________________________________

_________________________________________________________

_________________________________________________________

If you feel NEW14H contributes to your effectiveness, then please let

me know by sending a donation of $10.00 or whatever you feel is ap-

propriate.  NEW14H was developed in my spare time, and is now not a

commercial effort of CRC.  I will continue to develop products like

NEW14H for those who show interest - provided I can afford it.  Users

who make contributions will be entitled to a single sided diskette

of other PC-DOS utilities I've developed while at CRC: please include

your name and return address when sending donations!

P.S. -  If it's the $10.00 that's keeping your from filling out &

sending this sheet, forget about the money!  I'd like to

hear your comments!

# (0,1 valid).

;   I.E. 0=COM1, 1=COM2, etc.

;

;   Start of program; definitions:

;

;   First set; potentially user-modifyable definitions:

;

main_version	equ	2	; version number - do NOT exceed 9 !!

sub_version	equ	7	; release number - "  "   "      " !!!

intcom1_name	equ	1	; our name (if we are COM1) is 1

intcom2_name	equ	2	; our name (if we are COM2) is 2

EOF		equ	0FFh	; EOF return value; may wish to use ESC, (1AH)

buffer_size	equ	32767	; Size (chars) of input buffer

xbuffer_size	equ	256	; Size (chars) of output buffer

read_id		equ	80h	; 'added' INT14H calls - IDentify self call.

length_code	equ	84h	; 'added' call - get buffer length

flush_code	equ	85h	; added flush buffer call.

chkbuf_code	equ	86h	; added check buffer call

break_code	equ	87h	; added send break signal over line call

bf_hit		equ	88h	; added 'chars in buffer' ? call

reset_ints	equ	89h	; added 'reset interrupts' call

c_activity	equ	8Ah	; added 'active since last call?' call

alarm_code	equ	8Bh	; added 'set alarm' call

oq_empty	equ	8Ch	; added 'output queue empty ?' call

read_port	equ	8Dh	; added 'read port' call

write_port	equ	8Eh	; added 'write port' call

setxon		equ	8Fh	; added 'set xon/xoff mode' call

enable_ints	equ	90h	; added 'reset ier' call

default_port	equ	0	; default to COM1: when no parameters supplied

MSCOUNT		equ	500	; we want timer to be ~ 500 ms (for BRK signl)

MSSIZE		equ	70	; MSSIZE should be set to produce 1 ms loop.

TIMEOUTCNT	equ	65500	; # of loops before giving