This is an overview of the various modules making up the KA9Q Amateur Radio

Internet Protocol package. Each section first describes the protocol that

is supported, followed by a description of the implementation.

Subnet layer:

A. Standard 10 megabit Ethernet.

The driver provided is for the 3-Com 3C500 (IE) controller for the PC.  Tight

(3 instruction) assembler loops are used instead of DMA for copying data in and

out of the controller's buffer; this seems fast enough.  Only the receiver is

interrupt driven; incoming packets are placed on a queue which is then emptied

at a more leisurely pace by the main loop code.  Since Ethernet is so fast,

the transmitter routine busy-waits for completion. The IE controller has only

a single buffer which is shared between receive and transmit. Packets

occasionally get lost under heavy load, especially when several TCP

connections are active at once and/or the TCP receive window sizes are

larger than the MSS values. This is very hard to fix without going to a

newer, more reasonably designed controller.

B. Serial Line IP (SLIP) - a very simple technique for sending Internet

Datagrams across ordinary asynchronous lines (e.g., modems). Its only function

is to delimit datagrams with framing characters, which are "byte stuffed"

for transparency. No error checking is provided; the checksums that are part

of IP, TCP and UDP are relied on for this.  SLIP is popular on UNIX

systems that support TCP/IP, so this represented the easiest way to get my

package up and talking with other Internet sites.  It also allows the use of

existing AX.25 TNCs without modifying the latter, although this is should

only be done as a stopgap measure until an HDLC link level driver can be

integrated directly into the package.

C. AX.25/KISS TNC - sends and receives IP and ARP datagrams encapsulated in

AX.25 Unnumbered Information (UI) frames. This allows an unlimited number of

simultaneous users on the local radio channel, since there are no

connections at link level.  The special AX.25 Level 3 Protocol ID values of

hex CC (IP) and CD (ARP) are used.

This mode of operation requires that the TNC run special "KISS TNC" code.  A

version developed for the TNC-2 by K3MC is provided in this distribution.

The standard TNC-2 ROM must be replaced with a bootstrap ROM and the KISS

TNC code downloaded into the serial port. Note that this style of operation

is NOT compatible with a "stock" TNC carrying SLIP formatted datagrams in

connected mode.

D. AX.25/PACCOMM PC-100 interface (NOT YET WORKING) - The PC-100 is an

I/O adapter card for the PC containing a Zilog 8530 Serial Communications

Controller and two AMD 7910 World Chip modems.  The encapsulation method

is identical to (and compatible with) the AX.25/KISS TNC interface driver.

E. Address Resolution Protocol (ARP) - used for the automatic mapping of IP

addresses to link or subnet addresses. While originally designed for the

specific task of mapping IP to Ethernet addresses, the protocol is general

enough to be used on any link or subnet that supports a broadcast facility.

ARP is described in ARPA RFC-826.

Both Ethernet and AX.25 format addresses are currently supported.  Instead

of discarding a packet when an ARP request is sent, this implementation

holds it on a queue for a limited time, pending a response to the request.

Internet layer:

F. Internet Protocol (IP) - the universal network-level datagram protocol

of the ARPA Internet. It corresponds roughly to level 3 of the ISO Reference

Model (ISORM). (Actually, IP belongs to the 3B internetwork sublayer if you

follow the amoeboid proliferation of ISO sublayers). Routines are provided

to generate, receive and route (i.e., packet switch) IP datagrams. IP is

specified in ARPA RFC-791 and MIL-STD-1777.

IP datagram fragmentation and reassembly is fully implemented. The IP options

Loose Source Routing, Strict Source Routing and Record Route are supported;

Stream ID, Security and Timestamp are ignored. (Few, if any, IP options are

used extensively in the ARPA Internet).  The IP module includes a routing

(packet switching) facility; currently this consists of a table containing a

list of host-specific destinations along with a "default" entry that may be

used when the desired destination is not found in the table. No protocol is

yet provided to actually manage the table; currently it must be done manually,

either locally or remotely.  NB! "Networks" (in the ARPA Class-A/B/C sense)

are not yet understood, in that all entries in the routing table (except for

the default entry) must be host-specific.  This will be fixed in the future

with a "generalized subnetting" scheme that will allow each entry in the table

to have an arbitrary, variable length "subnet mask."

G. Internet Control Message Protocol (ICMP) - the error reporting adjunct

to IP. It appears as a pseudo-protocol on top of IP, but is actually

considered to be part of IP. The IP routines automatically generate ICMP

messages whenever an error occurs in the processing of IP datagrams;

incoming ICMP messages are passed up to the originating end-to-end protocol

for appropriate action. ICMP is specified in ARPA RFC-792.

This ICMP supports all options except ICMP Timestamps.

Host-Host Layer:

H. Transmission Control Protocol (TCP) - provides a reliable, sequenced

"virtual circuit" or "connection-oriented" service atop IP on an end-to-end

basis. It corresponds to the Transport layer (level 4) of the OSI model.

Since a single TCP module supports multiple connections through the use of

port numbers, it also provides Session (level 5) functionality without

the need for a distinct layer. (Or it makes the session layer unnecessary,

depending on your point of view). TCP is specified in ARPA RFC-793 and

MIL-STD-1778.

The implementation supports an arbitrary number of simultaneous connections,

limited only by memory space for control blocks. An "upcall" mechanism is

available to notify the user of three events: connection state change, receive

data available and transmit buffer space available. The Maximum Segment Size

option is understood and used when it is received, and a global variable is

used for generating MSS in outgoing segments. There is currently no

provision for sending URGent data.  The latest recommendations on "tinygram"

avoidance (Nagle, ARPA RFC-896) are implemented and work quite well.

I. User Datagram Protocol (UDP) - provides only a simple, unguaranteed

"datagram" or "connectionless" service, adding only checksums and port

multiplexing to the raw service provided by IP. As an alternative to TCP,

UDP also sits at level 4 (and 5) of the ISORM. UDP is specified in ARPA

RFC-768.

The implementation supports the creation of queues on local sockets. An upcall

mechanism similar to that used in TCP notifies the user when datagrams have

arrived.

Application Layer:

J. File Transfer Protocol (FTP) - for transfer of binary or text files

between hosts. FTP uses two TCP connections - one for exchanging commands and

responses in the form of ASCII strings, and the other for the actual

data transfers. FTP is defined in ARPA RFC-959.

FTP is implemented in two parts, a server half and a client half. The server

half supports multiple simultaneous remote users, although at the moment

there is no access control; anyone may access, delete or overwrite any file

on the machine.  The client half is invoked by the "ftp" command.

K. Remote login protocol (Telnet) - for logging into remote systems and

negotiating various options, such as remote vs local echoing. Telnet itself

is documented in ARPA RFC-854, with the many options defined in other

RFCs.

The client half supports the ECHO and TRANSMIT-BINARY options; it is invoked

by the "telnet" command. A telnet "server" has been written, but since MS-DOS

isn't a timesharing system it is used for keyboard-to-keyboard conversations

instead. An incoming connect request to the local Telnet port creates a local

Telnet session and notifies the local user. He may then select the new session

and carry out a conversation with the remote initiator.

L. Simple Mail Transfer Protocol (SMTP) - as the name implies, is used for the

transfer of electronic mail between hosts. SMTP commands and responses are

strings of ASCII characters resembling those used by FTP. SMTP uses a

single TCP connection for both control and the actual mail messages.

SMTP is described in ARPA RFC-821; headers in mail messages are described

in RFC-822.

A simple version server (receiver) is implemented; it accepts mail from a

remote sender and appends it to a mailbox. Mail forwarding, which accounts for

much of the complexity in other mail servers, is not implemented. A mail

client ("user agent") for composing and sending messages developed by Bdale

Garbee, N3EUA, is included.  It is called 'BM', and is documented elsewhere.

M. Session control - the user may manage several simultaneous Telnet and

FTP client invocations. Only one client session is "active" at any one time,

but the user may quickly switch between them. Terminal output arriving for

an inactive session is held until that session is again made active.  (Note

that this is in addition to the multiple server sessions which go on

automatically "in the background", i.e., without operator intervention).

The Internet package has been tested with and works under DoubleDos 3.1U

by SoftLogic Solutions, allowing the system to be simultaneously "on the net"

while executing conventional MS-DOS commands.  The net program uses the special

DoubleDos function call 0EEH to relinquish the processor when it isn't needed;

this speeds up the task running in the other partition.  This function is a

"no-op" when DoubleDos isn't active.

Phil Karn, KA9Q

4 Nov 1986

updated 870525 by Bdale, N3EUA