This is	a program to allow the SPERRY PC/IT to recognize internal modems and

ASYNC ports not	recognized by the PC.  It will set the equipment table to

indicate that the system has two serial	ports installed	at the standard	COM1

and COM2 addresses.  The internal async	port corresponding to the installed

device must be disabled	as described in	the system installation	guide.

The following is a listing of the FIXSER.COM program included:

MOV	AX,0040		 ; seg adrs of equipment table

MOV	DS,AX		 ; to DS reg

MOV	AX,03F8		 ; adrs	of COM1

MOV	[0000],AX	 ; to 0040:0000	(equip table)

MOV	AX,02F8		 ; adrs	of COM2

MOV	[0002],AX	 ; to 0040:0002	(equip table)

MOV	AL,[0011]	 ; byte	containing number of com ports installed

AND	AL,FD		 ; set bit 1 to	zero

OR AL,04		 ; and bit 2 to	one

MOV	[0011],AL	 ; put back into equip table 0040:0011

RET			 ; done

I hope this patch will help anyone with	a SPERRY IT who	has trouble with an

internal modem as I did.  I include FIXSER.COM in my AUTOEXEC.BAT file.

Not all	software will have problems without the	patch (CROSSTALK works fine),

but any	program	that checks the	equipment table	will not find certain add on

async devices without the patch.

The fix	was provided to	me by SPERRY customer engineering.

Jeremy S. Nichols  11/15/86

You can find Jeremy and other top-notch programmers and techies

at Terrapin Station BBS -- 2400/1200 - 24 hours a day

612/623-0152 -- a BBS for programmers!

Additional lines available for subscribers.