Generic Terminal

Copyright (c) 1985 by P & M Software Co.

August 18, 1985

GT is short for Generic Terminal,  a communications program for IBM

Personal Computers and compatible units.   P & M Software Co.

reserves the COPYRIGHT to this program and all related materials.

The user is granted a license to use the program and is encouraged

to make a donation if the program is found to be useful.  $20 is

recommended.  Send the donations to:

P & M Software Company

9350 Country Creek #30

Houston, Texas 77036

In addition, the user is granted the right to make unlimited copies

of the program and to distribute these copies as he/she desires,

except that P & M Software Co. reserves the sole right to distribute

the program for profit.  If the user distributes the product, the

user may ask for a duplication charge, not to exceed $5 per copy.

Features:

1. ANSI standard terminal emulation.

2. ANSI.SYS style color graphics.

3. BASIC style PLAY for single voice music.

4. Telephone directory, with up to 999 entries.

5. File capture, limited ONLY by the amount of available

memory.

6. ASCII file transmission, using XON/XOFF protocol.

7. Full control of all communication parameters.

8. 40 macro keys, to store logon, passwords, etc.

9. Printer support, using XON/XOFF flow control.

10. Xmodem CRC protocol, using the CCITT polynomial:

x^16 + x^12 + x^5 + 1.

11. TELINK batch file transfer protocol is supported.

Most of the features of the program are self-documenting, however

for the sake of the novice user, some explanation is necessary.

The program is distributed in .COM, which can be executed

immediately by placing your disk into drive A, changing the

default drive to A, if not already, then typing the name of the

program, GT.

Once the user has successfully executed the program, the copy-

right notice will appear, and the user will be asked if music is

wanted, strike a "y" if you want music, if not, then strike any

other key.  The on-line screen will then appear.  At the bottom

of the online screen appears a reminder of the most frequently

used commands.  These commands may be selected by depressing and

holding the ALT key along with the indicated ALPHA key.  If the

user needs a command that does not appear on the screen, a

complete list of available commands can be obtained by selecting

ALT-H.  Commands may be selected while the HELP screen is still

on screen, so that the user does not need to call up the HELP

screen repetitively.  Here is a brief description of each

command:

ALT-A .... Transmit a file using the ASCII XON/XOFF protocol,

where the DC1 character is used for XON and the DC3

character is used for XOFF.

ALT-B .... Send BREAK signal to host computer.  The serial port

is forced into the spacing state.

ALT-C .... Turn capture mode ON if it is OFF, or OFF if it is ON.

When this mode is turned OFF, either thru the ALT-C

command or with the ALT-T command, the user will be

prompted for a file name wherein to save the contents

of the capture buffer.  During capture mode

operations, a '*' or 'W' will appear in the lower left

corner of the screen.  The '*' reminds the user that

capture mode is ON, the 'W' warns the user that memory

space is short.  Capture mode will make use of all

available memory.  Capture mode is useful for

recording complete sessions for review later, so you

don't read so much on those long distance phone calls.

ALT-D .... Modem dialing and phone directory management.  The

phone directory may have up to 999 entries, if there

is enough disk space.  The directory screen will

automatically disappear whenever carrier is detected,

unless the Forced Carrier config param is "TRUE".

ALT-E .... Turn Half Duplex ON if it is OFF, or OFF if it is on.

If Half Duplex is ON, then a 'H' will appear in the

lower right corner of the screen to remind the user.

ALT-F .... Fix time.  That is, mark the current time, so that you

can measure time used, with the ALT-U command.

ALT-G .... Redial the last number dialed.  This command will loop

continuously until carrier is signaled by the modem,

the Forced Carrier config param is 'TRUE', or the user

presses a key.

ALT-H .... Produces the HELP screen for the user.

ALT-I .... Reconfigure the program defaults, without exiting to

DOS.  The parameters listed below for the .CNF file

can be changed via this command.

ALT-J .... Display version number of GT in use.

ALT-K .... Kill a file.  Delete a file from the disk.

ALT-L .... Display disk directory, under a user supplied mask.

For example, to see all files on the logged drive and

current directory, the user would enter the mask

"*.*", when prompted.   Or see just the files with an

extension of .PAS by entering the mask "*.pas", when

prompted.

ALT-M .... Macro key management.  Allows the user to define the

contents of the function keys F1 thru F10, and all

combinations of the function keys and shift, Ctrl or

Alt.  This gives the user 40 different macro keys.

ALT-N .... Allows the user to change to a new logged drive and/or

new current directory.

ALT-O .... Sort the dialing directory into ascending sequence by

name or number.  Performs a IN MEMORY sort, so there

must be enough memory to hold all the phone directory

entries.

ALT-P .... Print a file.

ALT-Q .... Hang-up the phone line, put the modem "ON HOOK".

Drops the terminal ready signal to the modem for 3

seconds and resets the UART.

ALT-R .... Receive a file using Xmodem protocol.  Communications

parameters are automatically changed to/from 8 bit no

parity.  CRC is used automatically, if the remote

system supports it.  In addition, the TELINK batch

protocol may be used, in order to invoke this proto-

col, the user must first prepare the Host cpu to use

the TELINK protocol, then after the ALT-R key has been

pressed and GT has requested the filename, the user

should respond with:  [d:]*   the [] indicates that

the drive specifier is optional.  Under the TELINK

protocol, the filenames will be supplied by the Host

cpu.  Using TELINK preserves the exact file size and

creation date.

ALT-S .... Switch communication parameters.  The program starts

with these default values: 1200 baud, No Parity, 8

data bits, 1 stop bit.  Permissible values for the

baud rate are: 300, 1200, 2400, 4800, or 9600.

ALT-T .... Transmit a file using Xmodem protocol.  Communications

parameters are automatically changed to/from 8 bit no

parity.  CRC is used, if the remote system requests

it.  Also, the TELINK protocol is used if the remote

system supports it.  Using TELINK, it is possible to

transfer up to 30 files at one time.  If the user

wishes to perform a batch transfer, he must specify

the names of the files to transfer using the DOS

wildcard notation; for example, to transfer all files

with the .PAS extension, when asked for the filename,

respond with "*.PAS".  The first 30 files found

matching the criterion will be transmitted.

ALT-U .... Used, time.  Report the time used since the the last

ALT-F command and the last dial of the phone or the

start of the program, whichever occured last.  Total

elapsed time in hours, minutes and seconds, is

reported.

ALT-V .... View the contents of a file, uses the same style as

MORE, the DOS utility.  Use the "End" key to stop the

command.

ALT-W .... Clear the screen.

ALT-X .... Terminate and return to DOS.  If other windows are

open on the screen, this command will be deferred

until all windows have been closed.

ALT-Y .... Copy a file.  This procedure will copy any unprotected

file.  Filenames must be fully specified.  No

wildcards are allowed, ONLY the current drive and

directory need not be specified.

ALT-Z .... Toggle monitor mode on/off.  In monitor mode, all

codes are shown so that the user may see exactly what

is being sent by the host computer.  Control codes are

shown with the ^ symbol preceding, i.e. ^[ is the ESC

code, ^X is the CAN code, etc.  You must refer to an

ASCII code chart for a complete list.

ALT-9 .... Toggle the beeps ON/OFF.  Go into quiet mode or back

to normal mode.  Does not change the playing of music,

just the bell.

^PrtSc ... Toggle the printer ON/OFF.  Hold the 'Ctrl' key, then

press the 'PrtSc' key.

The dialing management sub-section has a list of commands also:

A = Add an entry to the dialing directory.

C = Change an entry in the dialing directory.

K = Kill an entry in the dialing directory, a sort of the

directory is recommended after this option or the Add

option.

M = Dial a number not in the directory.  The user is prompted

for new communications parameters.

D = Dials a number from the directory, and automatically changes

the communications parameters.

S = Stop the modem, after it has dialed, but before it goes

online.  This allows you to dial another number when a

previous number is busy, without waiting for the modem to

timeout waiting for carrier.

F = Display the next page of the directory.

B = Display the previous page of the directory.

Q = Quit, returns to the online screen.

Note: When editing Phone Directory entries or Macro Key definitions,

the program responds to the following editing keys as

indicated:

Backspace .... Delete character to the left of cursor.

Del .......... Delete character under the cursor.

End .......... Move the cursor to the end of the field.

^End ......... Erase from the current cursor position to the

end of the field.  The ^ symbol indicates that

you must hold the 'Ctrl' key, then press the

'End' key.

Home ......... Move the cursor to the start of the field.

Ins .......... Toggle between insert and overwrite mode.

When in insert mode the cursor becomes fat.

Left Arrow ... Move the cursor to the left 1 position.

Right Arrow .. Move the cursor to the right 1 position.

RETURN ....... Finish the current field and move to the next.

There are several files associated with the system.  Here is a

description of each:

GT12.COM ....... Absolute program.  The one to execute.

GT12.DOC ....... This file.

GT.DIR ......... The phone directory, data file.

GT.KEY ......... Macro key definitions, data file.

GT.CNF ......... The configuration, data file.

In position 1-3 of each record in the configuration file is a

code to identify a parameter.  After the "=", the value of the

parameter appears.  The codes are:

CM=  COM port number, 1 or 2.

ST=  Number of Stopbits.

DA=  Number of Databits.

PA=  Parity. 0=None,1=Even,2=Odd

BA=  Baud Rate. 300,1200,2400,4800, or 9600.

WA=  Number of ticks in 1/3 of a second, at 4.77 MHz clock.

The value should be larger if the clock in your CPU is

faster.  The units for the value is milleseconds, and

it is used for waiting, hence the WA=.  For example,

if your CPU runs a twice the 4.77 MHz rate, then the

value of WA= should be 666.

PR=  Dialing Pre String.

PO=  Dialing Post String, use the | character where a

RETURN is needed.

MI=  Modem initialization string.  The default value, AT

S0=0|, will cause the modem not to answer the phone.

Again, use the | character where a RETURN is needed.

SY=  Modem command string to turn the speaker ON.

SN=  Modem command string to turn the speaker OFF.

RT=  After carrier timeout has occured, this is the time

between redials, in seconds.

FC=  Forced Carrier.  Some modems force carrier even when a

call is not in progress, if this is the case, set this

parameter to TRUE.

CT=  Timeout time for carrier to appear, in seconds.

DP=  Directory PATH.  This variable tells the program where

to look for the phone directory.  This param is

provided mainly for people who wish to put the dialing

directory on a RAM-disk.

XO=  The decimal value of the XON character.  Used to

control data flow.  Starts data flow after XOFF has

been issued.

XF=  The decimal value of the XOFF character.  Used to

control data flow.  Stops data flow when issued.

Helps insure that buffer overflow does not occur.

VT=  If TRUE, turns on ANSI emulation, using ANSI escape

codes.  If ANSI emulation is requested, music will not

be played, and will show on the screen as text.  This

happens because of a conflicit between the full ANSI

definition and the musical notes.

LF=  Auto Linefeed mode.  If TRUE, then a linefeed will be

automatically supplied whenever a Carriage Return is

recieved.

FG=  The default foreground color number.  Any one of the

16 values in the table below.

BG=  The default background color number.  Any one of the

DARK shades, 0-7, from the table below.

CC=  The comment character used with the macro keys.  MX=

Number of Xmodem buffers to allocate. 4 - 255 are the

legal values.  A smaller number should be used on

systems with only 128k, larger values can be used if

more memory is available.  Each buffer takes 128

bytes.

* =  Comment lines.

Note: these entries may either be changed via the ALT-I command,

or with any text editor that produces standard ASCII files,

such as EDLIN.

For Example:   CM=1             |

ST=1             |

DA=8             |

PA=0             |

BA=1200          |

WA=333           |

PR=ATDT          |

PO=|             |

MI=AT S0=0|      |

SY=AT M1|        |

SN=AT M0|        |

RT=60            |     The DEFAULT defaults!

FC=FALSE         |

CT=35            |

DP=A:\           |

XO=17            |

XF=19            |

VT=FALSE         |

LF=FALSE         |

FG=15            |

BG=0             |

CC=;             |

XT=FALSE         |

MX=64            |

Color Number Table:

Dark Colors               Light Colors

-----------               ------------

0 : Black                 8 : Dark Gray

1 : Blue                  9 : Light Blue

2 : Green                10 : Light Green

3 : Cyan                 11 : Light Cyan

4 : Red                  12 : Light Red

5 : Magenta              13 : Light Magenta

6 : Brown                14 : Yellow

7 : Light Gray           15 : White

If the program is used without a modem, using a direct cable

connection, all the dialing and modem control strings in the config

file should be set to empty strings, for example the line with the

MI parameter should read: "MI=".  Also, when using a direct cable

connection, the FC parameter should be set to TRUE.

The program has been written to use Hayes compatible commands with

the modem.  Because the program controls the modem via the DTR

signal, Data Terminal Ready, the cable between the computer and

modem must provide a connection for pin 20 and the modem should have

the internal switches set so that the DTR signal is monitored.

The program uses the Carrier Detect signal from the modem to detect

when a call has been connected and when to stop redialing, for this

reason the modem should have it's switches set so that the Carrier

Detect signal is ONLY sent when present on the phone line.  If this

is not possible, set the FC= parameter, to TRUE.

There are a number of status indicators that can appear in line 25

of the screen:

* .... Capture mode is on.

W .... Capture mode is about to run out of memory.

^ .... Caps Lock is on. The ^ will show as the

Up-Arrow on the screen.

# .... Num Lock is on.

Q .... Beeps are off.

H .... Half-duplex is on.

Received Character Processing - ANSI Emulation Mode.

----------------------------------------------------

Cursor Up .................................. ESC [ Pn A

Cursor Down ................................ ESC [ Pn B

Cursor Forward ............................. ESC [ Pn C

Cursor Backward ............................ ESC [ Pn D

Cursor Position ............................ ESC [ Pl ; Pc H

Cursor Position (Home) ..................... ESC [ H

Horizontal and Vertical Position ........... ESC [ Pl ; Pc f

Horizontal and Vertical Position (Home) .... ESC [ f

Set Scrolling Region ....................... ESC [ Pt ; Pb r

Index ...................................... ESC D

Reverse Index .............................. ESC M

Reverse Linefeed ........................... ESC I

Next Line .................................. ESC E

Save Cursor ................................ ESC 7

Restore Cursor ............................. ESC 8

Cursor Key Mode Set ........................ ESC [ ? 1 h

or ESC =

Cursor Key Mode Reset ...................... ESC [ ? 1 l

or ESC >

Select US Character Set .................... ESC ( B

Select Line Drawing Character Set .......... ESC ( 0

Turn Off Character Attributes .............. ESC [ m

Selects Bold Characters .................... ESC [ 1 m

Selects Underlined Characters .............. ESC [ 4 m

Selects Blinking Characters ................ ESC [ 5 m

Selects Reverse Video Characters ........... ESC [ 7 m

Set Horizontal Tab Stop .................... ESC H

Clear A Horizontal Tab Stop ................ ESC [ g

Clear All Horizontal Tab Stops ............. ESC [ 3 g

Erase From Cursor To End Of Line ........... ESC [ K

Erase From Beginning Of Line To Cursor ..... ESC [ 1 K

Erase Complete Line ........................ ESC [ 2 K

Erase From Cursor To End Of Screen ......... ESC [ J

Erase From Beginning Of Screen To Cursor ... ESC [ 1 J

Erase Complete Display ..................... ESC [ 2 J

Delete Character At Cursor ................. ESC [ Pn P

Insert Line At Cursor ...................... ESC [ Pn L

Delete Line At Cursor ...................... ESC [ Pn M

Select Insert Mode ......................... ESC [ 4 h

Reset Insert Mode .......................... ESC [ 4 l

Turn Printer On ............................ ESC [ ? 5 i

Turn Printer Off ........................... ESC [ ? 4 i

Turn Printer Controller On ................. ESC [ 5 i

Turn Printer Controller Off ................ ESC [ 4 i

Print Screen ............................... ESC [ i

Request Cursor Position Report ............. ESC [ 6 n

Response: Cursor Position Report .... ESC [ Pl ; Pc R

Request ID ................................. ESC [ c

Response: Terminal Type ............. ESC [ ? 6 c

Notes: ESC is used to represent the escape character, decimal 27.

Pn represents a variable numeric parameter, the parameter is

a decimal number transmitted to the program by using ASCII

characters.  Pl represents the LINE, and Pc represents the

COLUMN, these are numeric parameters sent using ASCII

characters.  Pt represents the TOP LINE of a scrolling

region, and Pb represents the BOTTOM LINE of a scrolling

region, these are numeric parameters sent using ASCII

characters.  The spaces shown are for readability, they are

not sent by the Host.

Transmitted Characters - ANSI Emulation Mode.

---------------------------------------------

Function Keys .............................. User Programmable.

End ........................................ ESC O R

Ctrl-End ................................... ESC O S

Del ........................................ DEL

Ins ........................................ ESC O n

Home ....................................... ESC [ H

PgUp ....................................... ESC [ O r

PgDn ....................................... ESC [ O q

Shift-Tab .................................. ESC [ O x

Left Arrow ................Normal........... ESC [ C

Keypad Mode...... ESC O C

Right Arrow ...............Normal........... ESC [ D

Keypad Mode...... ESC O D

Up Arrow ..................Normal........... ESC [ A

Keypad Mode...... ESC O A

Down Arrow ................Normal........... ESC [ B

Keypad Mode...... ESC O B

Others ..................................... Standard Values.

Note: The blanks are included for readability only.