The snow on a color graphics adapter occurrs when you change the values

in screen memory during a raster scan, i.e. while the electron gun is

updating the screen from screen memory.  I'm not sure of the exact mechanism,

but this definitely causes interference with the screen update, and the

end result is that ugly snow.

One way to avoid this is to wait for "vertical retrace", that time period

during which the electron gun shifts its beam from the lower right-hand

corner of the screen to the upper left.  During vertical retrace, bit 3

of port 3DA (hex) is set on, and the test in TURBO can thus be made:

Repeat until ((port[$3DA] and $08) = $08);

{*** perform screen update ***}

{*** note: port $3DA is a read-only register ***}

This is how the IBM ROM-BIOS writes a character to the screen, does

scrolling, etc.  But the problem is that vertical retrace lasts a mere

1.25 miliseconds or so, not enough time to perform complicated and/or

large updates.

The other way, the way Sidekick does it, is to turn off the display, do the

update, and turn the display back on.  This is why, when you bring up a

Sidekick window, you see a momentary "flash" on the screen.  The display

enable/disable function is controlled by port 3D8 (hex), a write-only

register.

One thing you should know about this is that this whole concept applies only

to the IBM color graphics adapter.  Monochrome displays do not get snow,

and the Enhanced graphics adapter will crap out on you if you turn off the

display in this way.  This is why earlier versions of Sidekick bombed on the

Enhanced graphics adapter.  Current versions of Sidekick work just fine on

the EGA.

Use the following, or a similar routine to test what kind of display you

are running on, and then to update the screen:

type

Registertype = Record

case boolean of

true : (AX,BX,CX,DX,BP,SI,DI,DS,ES,flags : integer);

false : (AL,AH,BL,BH,CL,CH,DL,DH : byte)

end;

video_mode_ptr = ^video_mode_type;

video_mode_type = (cga,ega,mono);

function get_video_mode : video_mode_ptr;

var

regs : registertype;

equipment : integer;

ega_info1,

ega_info2 : byte;

temp_return : video_mode_ptr;

begin

intr($11,regs);

equipment := regs.AX;

regs.AH := $12;

regs.BL := $10;

intr($10,regs);

ega_info1 := regs.CL;

ega_info2 := regs.BH;

new(temp_return);

if ((equipment and 52) in [0,16,32]) and (ega_info2=0)

then

temp_return^ := ega

else

if ((equipment and 48) in [16,32])

or (((equipment and 52) = 4) and (ega_info1 in [4,5,10,11]))

then

temp_return^ := cga

else

temp_return^ := mono;

get_video_mode := temp_return

end;

begin {* screen update *}

video_mode := get_video_mode;

if video_mode^<>mono

then

if video_mode^=cga

then

begin

port[$3D8] := $21;

{*** perform screen updates ***}

port[$3D8] := $29

end

else

saved_screen := cscr

else

saved_screen := mscr

end; {* screen update*}

There seems to be a bit of controversy as to whether the "snow" or the

"flash" is more ascetically displeasing, but I prefer the "flash".

Please upload some applications of this.