BATES Associates   -   OPTIM.COM  Fixed Disk Setup Utility

Tests conducted prior to the development of OPTIM indicate access speed

variations of up to 400% on incorrectly formatted fixed disk units!  An

incorrect  interleave setting does not prevent the machine from working, it

only slows down the sequential disk access time.

THIS MEANS THAT YOUR FIXED DISK COULD BE RUNNING AT ONLY A QUARTER OF IT'S

SPEED CAPABILITY!

OPTIM completes a low level format on the specified fixed disk unit.  The

Interleave setting is timed for maximum efficiency before the format is

completed.  On completion, a partition table is written to the disk which

allocates the whole disk for MSDOS use.  This is the setting required by

most users, but it can be altered by the FDISK program if needed.

USAGE :-

From the DOS prompt (usually A:> or C:>) type in

OPTIM  and hit <ENTER>.

An introduction screen will appear, giving details of program operation.

When you have read this, press any key, and you will be warned that data on

the selected drive will be erased by the program.

YOU MUST UNDERSTAND THAT THIS PROGRAM DOES NOT COMPLETE AN ORDINARY FORMAT

OPERATION.  DATA IS COMPLETELY OVERWRITTEN AND CANNOT BE RECOVERED.

Once you have elected to continue, the parameters settings returned from

the disk controller will be displayed for you to verify.  If there are two

fixed disks on your system, you will be asked to select one of them for

formatting.  If the parameters displayed are incorrect, you should answer

'N' and you will be returned to the operating system. Up to this point

nothing has been changed on your fixed disk.  If you accept the parameters

as correct, OPTIM will begin a test format on a small part of your disk to

determine which interleave setting is optimum.  The results of each test

will be displayed as they are completed.  When all 16 tests are done, OPTIM

will calculate the best setting and immediately begin to format THE WHOLE

DISK at that setting.  This may take several minutes.

When the formatting process is complete, you will be asked to enter any bad

track information for the disk.  Bad Tracks are areas of the disk which

cannot be relied upon to accurately maintain data storage.  If there are

any bad tracks for your disk, they will be noted either on a label on the

disk itself, or in the accompanying documentation.  If the details are on

the disk, they should be noted BEFORE you run OPTIM in order to avoid

moving the drive unit while it is running.  Track numbers should be entered

as decimal numbers - these may require conversion from the hexadecimal

numbers that some manufacturers supply.  As each track and head location is

specified, OPTIM will mark that track as bad so that the operating system

does not attempt to use it.  DO NOT RELY UPON THE BAD TRACK SELECTION

CAPABILITIES OF THE NORMAL FORMAT PROGRAM.  BAD TRACKS CAN PRODUCE

INTERMITTENT ERRORS WHICH AN ORDINARY FORMAT PROGRAM MAY NOT NOTICE.

When you have completed entering these details, OPTIM will write a

partition table to Track Zero/Head Zero which allocates the whole of the

disk for MSDOS use.  This is done so that you don't need to use the FDISK

partitioning program.  If you require special partitioning, you'll need to

use FDISK anyway and you should refer to your MSDOS manual for further

details.  Once the partition table has been written, OPTIM will reboot the

machine to bring the newly set up disk on-line.  Ensure that a system disk

is available in the floppy drive when rebooting takes place.

After the machine has restarted, you will need to format the fixed disk in

the usual way (see your MSDOS manual) and you may then transfer your data.

OPTIM has been tested on a number of different controller/drive/machine

combinations without any problems.  It is only suitable for the 8 bit bus

controllers normally fitted to standard IBM PC type machines.  OPTIM will

not work with the 16 bit bus controllers (large slot) used on the AT type

machines.  A later version (OPTIMA) will shortly be available for these

models.

BATES Associates  -  Copyright (1987)

########