I have seen numerous suggestions how to set Prowiter/NEC 8023 printer

switches and they have one thing common: Something is missing and you

are not able to use all printer capabilities.

From experience I can tell that it is waste of time to call Leading

Edge or eg. try to correct the writer of the Creative Computing IBM

Images column.

To settle this for once and for all please make record of the following

switch settings:

SW 1 -->        1,2,3,4,5,8-Open  6,7-Closed

SW 2 -->        2,3,4,5,8-Open  1,6,7-Closed

This settup works also with 123 PrintGraph program. If you want the best

quality of graphics print you need to close SW2-8 to disable bidirectional

printing.

When using Wordstar you will be able to print ASCII, CRAPHIC, GREEK and

Japanese KATAKANA character sets. The only unfortunate side effect is

that backslash (\) from ASCII prints like Yen sign but you still can print

backslash from the graphics set. I have downloaded also WS printer patches

for both version 3.2 and 3.3 . These files are named: WS320NEC.DOC  and

WS330NEC.DOC .

Kauko Laurinolli

404-981-9550