[PRT]int [S]creen [S]mall

-------------------------

Although the printer should be the easiest peripheral to use,

printers seem to cause the greatest headache for the beginner

as well as the expert practitioner.

Oddly enough one request seems to surface time after time, the

ability to print in compressed mode.  Although this can easily

be accomplished at the source code stage, it is often forgotten

or omitted due to its 'hardware dependence'.

Many users who dive into a quick BASIC program to re-set the

printer default soon find either it's too complicated for them

or the software they wish to use in compressed mode promptly

resets the printer prior to execution.  (For example compiled

basic).

Well dispair no longer users, compressed mode at any time is

here!!!

***PRTSM.COM is a program which when run establishes itself as

the always resident print screen interrupt.  Always there when

you need it ***PRTSM.COM resets the designated printer to......

COMPRESSED MODE thats 17CPI if your printer supports it......

12CPI if it does'nt.

I have set up four ***PRTSM's as follows :

OKIPRTSM.COM  -  FOR OKIDATA PRINTERS

EPSPRTSM.COM  -  FOR EPSON   PRINTERS

NECPRTSM.COM  -  FOR NEC     PRINTERS

TOSPRTSM.COM  -  FOR TOSHIBA PRINTERS.

If your printer type does not show here, the process of conversion

for any other printer requiring ESCAPE + CODE format can easily

be made via DOS's DEBUG facility.  (See Appendix)

To use ***PRTSM.COM, either place execution of the program in the

AUTOEXEC.BAT file or simply type in the command when required.

To envoke the compressed mode you are (alas) forced to do a print

screen [SHIFT + PrtSc].  Although this can sometimes be annoying

it's a small price for those who need the facility.

To drop the compressed print mode simply on/off your printer.

Yes I did think to toggle this via ***PRTSM but felt that it

was better to on/off the printer than wait out another print

screen (Oh well, even the best solutions aren't perfect).

Upon on/offing the printer, you will be back into the default

mode until another print screen is issued [ SHIFT + PrtSC ].

***PRTSM.COM has been tested on the above printers under

DOS 2.0, 2.1 and 3.0.  And has worked on a COMPAQ, IBM PC,

IBM XT and even an IBM AT.

The program is offered free of charge as PUBLIC DOMAIN software,

provided that the copyright notice and accompanying documentation

are not altered or omitted.

Source code is in ASSEMBLER and is available upon request.

APPENDIX

--------

***PRTSM.COM has been written to easily facilitate changes in

order to be used by a variety of available printers.

To alter PRTSM you will need a copy of the program, DOS's

DEBUG utility and a manual for the intended printer target.

Follow these instructions and customize PRTSM for your very

own!!!

1.  Locate in your printer manual the ESCAPE code sequence for

setting the printer into the required mode.

2.  Translate the decimal code to it's hex code equivilent.

3.  DEBUG  ***PRTSM.COM  (where *** is the first three characters

of the driver you currently have).

4.  E 0144 XX            (where XX is the hex code setting reqd).

5.  N ***PRTSM.COM       (where *** is the first three characters

in the name of the intended printer)

6.  W

7.  Q

You should now be back in DOS with your newly modified ***PRTSM.COM

ready for action.  Simply follow the above user procedures to use

your new print screen generator.

Ken C. Nickerson

PO Box 600

Ngr. Sq. Station

Buffal

)D

�$ߋ�]ÈD#�#�D#w�

@�����]�PSQRVWU��f#�F#��]_^ZY[X��Ў���W�###���_�E

��%��E#��%#��#��##��##�]

�VW�###���_^�D

%#####D#�E