AT & T 6300 PRODUCT INFORMATION

The  following article contains tips on how to get the most  from

your  AT & T 6300.   If you have a dual floppy system with 128  K

RAM,  then the following information (based on my experience) can

save you significant dollars on essential add-on  equipment.   If

you  are  thinking  of purchasing Lotus 123  V.2,  then  you  can

possibly save some time and frustration.

If you are thinking of buying an AT & T 6300,  then I suggest you

save money from the beginning and purchase a system with 1 floppy

drive and 128 K memory on the motherboard.  You will also want to

be  sure  you  have version 1.21 of the ROM BIOS.   Read  on  for

details.

STEP ONE - UPGRADING TO HARD DISK DRIVE

Anyone  with a PC can acquire a hard disk unit for their  system.

I found a 10 Meg hard disk selling for less than $500;  takes 30-

45 minutes to install; comes with utility software and an on-line

DOS  manual package called ASSIST.   A bon

us that AT &  T  6300's

seem  to have over other systems is that its power supply is more

than sufficient to handle the burden of a hard disk!  To be sure,

verify your unit has a power supply of at least 135 V.

I recommend Express Systems,  (312) 882-7733.   This company  has

been  advertising  in  BYTE for some  time  now.  TECH  JOURNAL's

August, 1985 issue gave the company's products a glowing report.

Express Systems announced recently that it has a 20 Meg Hard Disk

card  for  $595,  a  concentrator  to combine the  card  with  an

existing hard disk for $95.

DIP SWITCH SETTINGS for PC 6300*

Bank 1 Settings with A drive and C hard disk

****  Off 1 2     5   7 8   ****

****   On     3 4   6       ****

STEP TWO - UPGRADING MEMORY (RAM)

Most  PC owners think of upgrading their system's RAM  after  the

first  big  software purchase.   If you want to  use  Lotus,  for

example,  you  definitely  have to have at least 384 K to  create

useful  spreadsheets.   If  you want to use hard  disk  organizer

packages you'll use 50-60 K in RAM to support the memory resident

application.   Upgrading is usually very expensive.   For a 128 K

system  owner,  you  would need to acquire 128 K for  the  mother

board,  an expansion board,  and 384 K of RAM.   Depending on the

vendor this can cost you $300 to $600.

The  AT & T 6300 has a little known secret.   It can  accommodate

640 K on its mother board.   For less than $100 I was able to add

512   K  in  memory  using  18  256k,   150  ns  dynamic   chips.

(specifically,  NEC...256Kx1.D41256C-15) Total installation  time

was less than 30 minutes.

I  recommend  Microprocessors  Unlimited,  (918)  267-4961.  This

company   will   provide  fast  and   dependable   service---with

precautionary and helpful documentation at no additional expense.

DIP SWITCH SETTINGS for PC 6300*

Bank 0 settings with 640 K memory on motherboard (BYTE DEC'85)

**** Off   2     5 6      ****

****  On 1   3 4     7 8  ****

Bank 0 settings with 256 K memory on motherboard

**** Off   2           8  ****

****  On 1   3 4 5 6 7    ****

Bank 0 settings with 128 K memory on motherboard

**** Off 1             8  ****

****  On   2 3 4 5 6 7    ****

STEP THREE - UPGRADING PC 6300 WITH CO-PROCESSOR

I  can't  recommend  any  bargains to you  for  saving  money  on

installing  the  co-processor for the AT & T 6300 (also known  as

the 8087-2 Numeric Data Processor). I can provide some assistance

that will save you time and significant aggravation.

I  wanted  to  co-processor to improve  Lotus  123's  calculating

power.  This information with therefore be relative to installing

the co-processor for use with Lotus 123.  First of all,  contrary

to  the  general  comments being made  about  123---I've  had  no

problems  for the record.   When I called Lotus support for  help

with installation of the co-processor,  their staff was courteous

and  very well versed in the AT & T 6300!   I found this attitude

very comforting --- unfortunately,  the Lotus support staff  knew

more  about the problem I was to face that AT & T support  staff.

(To AT & T's credit,  their staff went out of their way to assist

me---their only problem was their lack of familiarity with 123.)

As far as the potential problem:  Lotus documentation states that

the  program  will  recognize  the math  co-processor  if  it  is

present.  The  co-processor's instructions are extremely easy  to

follow.  (The dip switch setting is provided:  Bank 1, set switch

5  to off.)  You put everything together and the Lotus 123 status

screen indicates "Math co-processor: (None)"!

The solution:  if you come up with the same problem,  you need to

order  1.

#reen indicates "Math co-processor: (None)"!

The solution:  if you come up with the same problem,  you need to

order