Audio-Handler

THIS INFORMATION IS PROVIDED "AS IS"; NO WARRANTIES ARE MADE.

ALL USE IS AT YOUR OWN RISK, AND NO LIABILITY OR

RESPONSIBILITY IS ASSUMED.

The audio-handler is the DOS Interface to the audio hardware and its

associated audio.device. It is opened (for example) by using Open()

with the name

AUD:

The mountlist entry does look like the following:

/* Mountlist for Audio Handler */

AUD:	Handler = L:Audio-Handler

Priority = 5

StackSize = 2000

GlobVec = -1

#

This is a generic device. Any channel will be allocated when the

device is opened, but you will keep the same channel when you feed

more data into AUD: after opening, so that sound output will be

continuous. A different channel might be allocated if the original

channel is stolen by another, higher priority allocation request

by another audio handler task or another program using audio.device.

Once opened, you can feed audio bytes into the handler by using the

AmigaDOS Write() library call. This call will not block if there is

enough space left in the sound buffer. Multiple writes are played

sequentially without glitch. There is no limit in the size of the

write packets.

You will have enough time to supply more data because written data

is immediately processed and the write request is replied immediately

as long there is space in the sound buffers. Guaranteed available time

ranges between number_of_written_bytes/frequency and about

buffer_size/frequency, where buffer_size is at least 2 kByte.

Of course you can also use the dos packet-level interface and feed

the handler dos write packets. This has the advantage that you can

wait for write completion as well as other signals. See [2], page 639

for an example of using asynchronous dos packets to copy a file.

There is no limit in the number of packets you can send simultaneously.

Parameters can be given with the device name, to control the volume,

period and priority of the sound output. The syntax is similar to

console names:

AUD:[/OPTIONS]

For a quick test, feed your favourite sample to the audio-handler:

copy audio-handler L:

mount AUD: from mountlist.aud

copy <your_favourite_sample> AUD:

You can also put the AUD icon into your devs:DosDrivers drawer so

it will be automatically mounted when your Amiga executes the

startup-sequence.

OPTIONS may be one or more of the following:

PERIOD p

sets the sampling period (inverse of frequency) of the waveform

to play. The default is to play the sound at 8 kHz sampling frequency.

The minimum period is 124 because of hardware reasons (custom chip DMA

allocation). The maximum is 65535 (size of the WORD custom chip register).

This value goes into the AUDxPER custom chip register. Computing this

value is explained in [3], pages 140-143. You can also specify the

sampling frequency instead with the option explained below.

FREQENCY f

sets the sampling frequency of the waveform to play.

This is an alternative to specifying PERIOD. The maximum usable

frequency is 28603, the minimum is 55. This value gives the

number of samples (bytes) per second to play.

VOLUME v

sets the overall volume of the sound. Valid numbers range from

0 (silent) to 64 (loudest), which is the default.

PRIORITY pri

is the priority used for precedence managment within the

audio.device. Audio channels with low priority may be 'stolen' by

other applications or different invocations of the audio-handler.

Valid priorities range from -128 (always stoppable) to 127 (unstoppable).

The default value is 0. See [1], page 22 for a suggested value of

priority for different types of sound.

BUFFER size

is the buffer size allocated for sound. As explained above, this

restricts how much time you have to update the buffer (write the

next bunch of samples). The minimum is 4kB to allow for a safe

continuation of play, the maximum is 256kB, allowing for double

buffers of each 128kB, which is the maximum possible for the

Amiga hardware. The buffers use CHIP memory.

DEBUG n

This is an option not normally used which outputs event information

about incoming packets and io requests to the serial port. n is a

number which is added to each information to show which handler

task is doing the output.

The following are special channel requests.

CHANNEL n

is an optional number between 0 and 3 (or maybe up to the number

of supported channels, if ever implemented).

LEFT

The sample is to be played on one of the left channels.

RIGHT

The sample is to be played on one of the right channels.

STEREO

The sample is a stereo sample (alternating bytes) and is to be

played at one left and one right channel simultaneously.

(Not yet implemented)

16BIT

This is set to specify that samples are Words instead of bytes.

This is not yet implemented, anyway, the Amiga Audio hardware

doesn't support it.

Examples:

Playing a sample with a sampling frequency of 4 kHz on a right

channel only:

AUD:FREQUENCY4000/RIGHT

Playing a sample with a smaller buffer (8 KB total, 2 buffers of

4 KB each) and low priority:

AUD:buffer0x2000/priority-100

Technical details:

* Lattice C 5.10 was used to compile the source.

* Stolen channels are not automatically given away, but the lock

protocol is used. When another process wants to steal the channel,

the audio handler frees the channel at the next best time.

This assures that the sample can be played glitch-free, if

the handler can immediately allocate another channel, and it

guarantees no sound is lost before the handler can allocate

a channel again.

* Each invocation of AUD: creates a new handler process, but the

code and data segment is shared between all processes (resident).

TODO:

* Implement stereo samples.

* Implement 16 bit samples. Might be interesting for 16 bit sound

cards if they have a device similar to audio.device.

* If sampling is implemented at some time, the audio handler might

also be used for reading (sampling) from audio sources. This will

require the user process to read data fast enough to keep in pace

with the sampler.device delivering samples. Might as well use

the parallel.device for well known samplers.

* Makefiles for other compilers. Volunteers?

The source and executable may be freely used in projects, but a

notice of its origin must be given. If the program is to be included

on a CDRom or disk being sold, one sample must be given to the author.

Comments, ideas, bug reports please to (e-mail preferred):

Martin Brenner

Kaiserstr. 83

52134 Herzogenrath

martin@ego.oche.de

References:

[1] AMIGA ROM Kernel Reference Manual: Devices, 3rd edition

[2] Ralph Babel: The Amiga Guru Book

[3] AMIGA Hardware Reference Manual, 3rd edition