StarAcc Desk Accessory Program Version 1.2. (9th July 1990).

StarAcc is a program which will allow you to control all  of

the usual functions of the Star LC-10 Multi Font printer from the

ST Desktop.  It also provides buttons for functions that are only

usually available through a word processor eg BOLD ,DOUBLE-HEIGHT

etc.  I wrote StarAcc initially for my own use, so the facilities

it provides are the ones I tend to use most often. There are many

printer control programs in the Public Domain,  but few are aimed

at specific printers so some compromises are usually  inevitable.

As  the  Star LC-10 printer is becoming very  popular  I  thought

others may appreciate a program aimed specifically for  it.  This

version  of  StarAcc  will run in mono or  colour.  To  load  the

program copy the three files ...

STARACC.ACC

STARMONO.RSC

STARCOLO.RSC

...  to your boot disk.  The program will then load whenever  you

boot your ST.  If you forget to copy the .RSC files onto the disk

Staracc will complain about it,  and will reboot your ST  without

loading the program.(The program will then have renamed itself to

STARACC.ACX).  If  you only ever run in one screen mode (mono  or

colour), you will only need one .RSC file (STARMONO or STARCOLO).

Staracc  will look for the correct version of the .RSC file  when

it loads.

Using StarAcc

As  with all Desktop Accessory programs Staracc is  accessed

from  the Desk menu.  The first time the program called during  a

session  it  will  reset the printer to  its  defaults.  This  is

necessary to get the printer into a known state.  If the  printer

goes off-line at any time while using StarAcc,  the program  will

detect it,  and prompt you to take suitable actions. A reset will

then occur before any further commands are sent to the printer.

StarAcc is very simple to use, simply click on the button or

Icon to select the options you require.  Selecting NLQ allows you

you  to choose between any of the 4  nlq fonts available  on  the

Star.  Reset does exactly that, resets the printer to its default

state,  this is useful for resetting top-of-form. LFEED feeds the

paper on line at a time.  Holding the mouse button down on  LFEED

will  cause  multiple linefeeds.  FFEED works the  same  way  for

Formfeeds. Select the OK icon to exit.

StarAcc   v1.2  is  Public  Domain,   and  so  may  be   freely

distributed. Any problems, suggestions etc should be addressed to

Paul Robins,

11 Tregwilym walk,

Rogerstone,

Newport,

Gwent NP1 9DZ