Subject: Fixing the 1040 whistle

Atari has issued a technical note to its dealers about the high-

pitched whine in the 1040.  I hope I am not stepping on anybody's

toes in posting this here.

The whine is at about 16 KHz, which most people who grew up with TV

cannot hear.  I love the SM124 B/W monitor because it's silent (35 KHz

scan rate), but the whine from the computer still bothered me.

I thought it comes from the main power-supply, but the note says it's

from the circuit around the "converter" chip on the mother board (U62).

This chip makes negative voltage from the positive (using an oscillator and

a "multiplier" rectifier arrangement?).  The fix for the whine is to raise

the frequency, by RAISING the values of a resistor, a capacitor and an

inductor.  The latter claim is against all my electronic instincts, but I

tried it and it works.  Here is how you do it:

Obtain a 5 ohm 1/4 W resistor, a 330 pF capacitor, and a small, axial,

220 microH inductor.  Take the top cover off the 1040ST (unscrew the short,

self-tapping screws from the bottom).  Take the metal cover off the

power-supply module (one little screw holds it down) and remove the module

itself (two screws at the two corners on the keyboard side).

Just "north" of the power-supply-module connector you will find U62.

Locate R17 (north of U62, 1 ohm), C28 (third component on the west, 100 pF,

looks like a resistor), and L4 (east, 100 microH, looks like a big resistor).

Replace them with the new components.  That's it.

There is a pair of wires, blue and red, going from the U62 area towards the

RS232 interface area.  The red wire carries +8V or so, relative to ground.

The blue carries the negative, and I measured -7V on it after the fix.

(I expected -12V, but 7 is close to 8 and the computer works fine...).

If you don't get good negative voltage on the blue wire, relative to

ground (not the red wire), you may have stopped the oscillations in U62.

Try a different L4, perhaps?