Publishing Partner Font Editor Documentation

--------------------------------------------

Written by Alan Reeve

Updated by ??????????

Here is the (start) long awaited Publishing Partner (Screen  and

Printer) font editor documentation.  Please read it, add any

information that is not present and upload it to GEnie, and design

a lot of fonts with its information.

The  first part to the Publishing Partner font editor   is  the

Screen  font  editor which allows you to design the  screen  fonts

that you actually see while using Publishing Partner.  Use of this

part is fairly straight forward and easy if you've used other font

editor for ROM based character sets from other/earlier  computers.

The name of this editor is CED.PRG and it also has a resource file

that  goes  along  with it.  Both font  editors only  function  on

Monochrome systems, but allow you to develop Mono and Color fonts.

If,  when loading a font editor,  it doesn't work, try turning off

your  system  (or  hitting reset) as neither font editor will run

on a computer with garbage in memory.

Drop Down Menus

---------------

File:

Load/Save font which allow you to (you guessed it) load and

save screen fonts.

Edit:

Name Font-Allows you to set up the name of the font when it

is loaded into Publishing Partner select font menu.

(e.g. NORMAL.FNT appears as System in the font menu)

Make Medium/High resolution font-Allows you to choose whether

you are making medium or high resolution fonts!

Increase/Decrease Height-Allows you to increase or decrease

your character sets height. A dark black bar will move up and

down your character grid with this option.

Other Commands

--------------

Using  the mouse you can edit your character by  pointing  to

squares in the 16 by 17 dot character grid and clicking the left

mouse button. Select which character you are editing by pointing

to them with the mouse.

Boxes and Arrows-

There are three groups of Boxes with arrows in them which

serve a purpose. The lower right pair allows you to set the

thickness of your characters which is indicated by a number

next to the grid ranging from 100 to 748.

Above  the pair is a group of four,  which allows you

to scroll your character around inside the grid.

To the right of these there are four more which allow

you to choose the baseline of the character,  and the  bottom

of the character.

Note: When adjusting the width or baseline of the character

you  will  see arrows move under and on  the  left  of  the

character grid indicating current locations.

Printer Font Editor

-------------------

Why have a printer font editor? After, all the screen fonts

look pretty good to me...except when they're enlarged...?

The purpose of a printer font editor is to design fonts  that

look nice when they're big as well as small.  The curves look nice

and  smooth  this way regardless of whether the letter 'O'  is  72

point, or 7 point.

The printer font editor is based on objects.  In essence, you

are drawing an outline of each letter, and when the program prints

your  document it fill in these characters at the appropriate  size

so that they look 'really nice'.

Commands

--------

Drop Down Menus

---------------

File:

Load/Save printer font-These commands (right?) they load and

save Printer Fonts.

Print Test Page-Disable/Inactive/Doesn't do much

Configure-Disabled/Inactive/Acts like Print Test Page

Quit-Exit to Desktop (Don't forget to turn the computer off

before reloading a font editor).

Edit-

Change Cutoff Point-?

Edit Thick/Tin Font-?

Copy Thick->Thin/Thin->Thick-?

Character-

Remove  Fill  Points-Fill point are spots that  the  computer

fills when printing. (e.g. When printing an 'O' two circles

make up this character.  A fill point would reside  in  the

out ring so that the computer would know where to put ink.

Mirror/Invert/Slant/Distort-All are similar as they are

disabled and do nothing.

Help (or Help key)-

Brings up Help Dialog Box.

Designing a Character

---------------------

Each character is made up of the following objects:

0) Fill points

1) Start Points

2) Lines

3) Curves

4) End Points

When designing a characters you must first set a start point.

You  can then use lines and curves to design  the  character,  and

finally and endpoint to end the character, or start another part

with a Start point/Lines/Curves/End point. Fill points are used

to tell the computer where ink should be placed.

The Letter 'O' theory-

---------------------

The letter 'O' is a very nice letter, but a pain to design.

Here's how the letter 'O' is designed. Create a circle for the

outer ring, and a circle for the inner ring. Put a fill point

between these two rings so that the computer fills this area, and

wa laa it's done. (SOund Easy...it isn't) Load a sample font and

look at the letters for an example of what i'm babbling about.

Using Commands

--------------

0) Set fill Point

Press 0, followed by a number from 0 to 9 indicating which

fill point you wish to enter/change (A character can have

up  to  ten fill points),  and then point to a position and

click the mouse button. A fill point will be born.

(Use Delete fill points to dispose of them...all)

1) Goto

Press 1, and point to a screen position and click the left

mouse button. A goto or start point will be there.

2) Line To

Press 2, and press the mouse button over a point on the

screen. A line will connect you last point to the current

one.

3) Curve To

The most difficult one to comprehend. Press 3 for starters,

followed by three more points. The first point is the end

point of the curve, the second is (what I call) the first

pull point, and then the second pull point which determine

how your curve will be formed.

4) Close path

End curve. Press 4, followed by a point.

[ and ]-

These allow you to select which character is being edited.

The character you are working on is shown in the upper right

corner of the screen.

j and k (Lowercase only!)-

Select  which  object (Line/Curve/End/Start  point)  you  are

working with/manipulating. To edit an object hold the mouse button

down  over  a square and draw it.  Release the  mouse  button  and

presto.

d (lower case only)-

Delete  object...ever  want  to delete that  line  or  curve,

here's your chance. Press 'd' and it will disappear.

c (lower case only)

Accepts two keystrokes, and I would guess has something to do

with the character boundaries (Thickness, Height, Baseline).

*** ATTENTION ***

We're looking for a few good facts to complete this document.

If you know how to use these commands better than I do, or find an

error,  or  know  addition information...etc  please  update  this

document or leave me (REEVESOFT) mail on GEnie and I'll do it.

Thanks,

Alan Reeve