SECTION 06072

FIRE-RETARDANT TREATMENT OF WOOD

NOTE ** This section is based on products manufactured by Hoover Treated Wood Products, Inc., at the following address:

NOTE **      P.O. Box 746

NOTE **      Thomson, GA  30824.

NOTE **      Telephone:  800-832-9663.

NOTE **      FAX:  706-595-1326.

NOTE **

NOTE ** "Pyro-Guard" is a UL listed third-generation Type "A" (low-hygroscopic) treatment for lumber and plywood used in interior structural and non-structural applications, with strength ratings for use at elevated temperatures that have been reviewed by the National Evaluation Service, Inc.

NOTE **

NOTE ** SECTION 06072 - FIRE-RETARDANT TREATMENT OF WOOD, Copyright 1996, The Architect's Catalog, Inc.

NOTE ** This specification section is for treatment only.  Grades and species of plywood and lumber to be treated must be specified separately.  See Related Sections article below.

PART 1 - GENERAL

1.1	SECTION INCLUDES

NOTE ** Delete items below not required for project.

A.	Fire-retardant treatment for the following:

1.	Wood framing.

2.	Decking.

3.	Sheathing.

4.	Blocking.

5.	Miscellaneous rough carpentry.

6.	Wood siding.

7.	Wood trim and moldings.

8.	Wood frames for doors, windows, and glazed lights.

9.	_________.

1.2	RELATED SECTIONS

NOTE ** Delete any sections below not relevant to this project;  add others as required.

A.	Section 06075 - Preservative Treatment of Wood.

B.	Section 06100 - Rough Carpentry.

C.	Section 06110 - Wood Framing.

D.	Section 06112 - Wood Framing and Sheathing.

E.	Section 06114 - Wood Blocking and Curbing.

F.	Section 06130 - Heavy Timber Construction.

G.	Section 06150 - Wood Decking.

H.	Section 06170 - Prefabricated Structural Wood.

I.	Section 06200 - Finish Carpentry.

J.	Section 06400 - Architectural Woodwork.

1.3	REFERENCES

NOTE ** Delete references from the list below that are not actually required by the text of the edited section.

A.	ASTM E 84 - Standard Test Method for Surface Burning Characteristics of Building Materials.

B.	MIL-L-1914O - Lumber and Plywood, Fire-Retardant Treated.

C.	NFPA 255 - Standard Method of Test of Surface Burning Characteristics of Building Materials; National Fire Protection Association.

D.	UL 723 - Standard Test Method for Surface Burning Characteristics of Building Materials;  Underwriters Laboratories, Inc.

1.4	SUBMITTALS

A.	Submit under provisions of Section 01300.

B.	Product Data:

1.	National Evaluation Report indicating limitation of use, high temperature strength testing, and flame spread, corrosion, and hygroscopic properties.

2.	Manufacturer's product specifications, storage instructions, and installation instructions.

C.	Samples:  Supply samples for finishing where finish samples are required by other sections of the specifications.

D.	Certificates:  Submit wood treater's specifications showing compliance with referenced standards.

1.5	DELIVERY, STORAGE, AND HANDLING

A.	Store products indoors and protect from moisture, construction traffic, and damage.

PART 2 - PRODUCTS

2.1	MANUFACTURER

A.	Provide fire-retardant wood treatment by or licensed by Hoover Treated Wood Products, Inc., P.O. Box 746, Thomson, GA  30824.  Telephone 800-832-9663.  FAX 706-595-1326.

B.	Requests for substitutions will be considered in accordance with provisions of Section 01600.

NOTE ** Delete paragraph above or below;  coordinate with Division 1 requirements.

C.	Substitutions:  Not permitted.

2.2	MATERIALS

A.	"Pyro-Guard" third generation fire-retardant treated wood, complying with MIL-L-19140.

B.	Fire Retardancy:

1.	Flame spread, when tested in accordance with ASTM E 84, NFPA 255, or UL 723:  Not more than 25.

2.	Smoke toxicity: No more than that of untreated wood.

3.	Each piece shall bear the UL Classification Mark certifying flame spread rating.

4.	When test is extended for 20 additional minutes (30 minutes total), the flame front shall not extend more than 10.5 feet from the center line of the burner and there shall be no evidence of significant progressive combustion.

5.	Treatment formulation shall contain no halogens, sulfates, chlorides, ammonium phosphate, oils, or solvents.

C.	Reviewed by the National Evaluation Service, Inc. for structural use at elevated temperatures.

D.	Production and kiln drying after treatment monitored by Timber Products Inspection, Inc.

E.	Kiln dry lumber to 19 percent or less moisture content after treatment.

F.	Kiln dry plywood to 15 percent or less moisture content after treatment.

2.3	FABRICATION

A.	Milling:  Lumber shall be ripped or run to pattern, as required, prior to treatment.

PART 3 - EXECUTION

3.1	EXAMINATION

A.	Discard pieces with defects which might impair quality or work.

3.2	FIELD CUTTING

A.	Lumber:  Do not rip or mill lumber after fire-retardant treatment; end cuts, drilling of holes, and joining cuts are permitted.

B.	Plywood:  Fire-retardant-treated plywood can be cut in any direction.

3.3	ADJUST AND CLEAN

A.	Remove scraps and debris from site, and leave in a broom-clean condition.

END OF SECTION