SECTION 13032

SAUNAS

NOTE ** Baltic Leisure saunas.

NOTE **

NOTE ** This section is based on products manufactured by Baltic Leisure, which is located at the following address:

NOTE **      601 Lincoln Street

NOTE **      P.O. Box 530

NOTE **      Oxford PA 19363

NOTE **      Tel: (800) 441-7147

NOTE **      Tel: (610) 932-5700

NOTE **      Fax: (610) 932-5713

NOTE **

NOTE ** Baltic Leisure offers a complete line of sauna enclosures - factory-cut components for field assembly, and complete saunas for indoor and outdoor installations shipped in modular components for field assembly.

NOTE **

NOTE ** SECTION 13032 - SAUNAS, Copyright 1997, The Architect's Catalog, Inc.

PART 1	GENERAL

1.1	SECTION INCLUDES

NOTE ** Delete items below not required for project.

A.	Factory-fabricated modular demountable sauna enclosure.

B.	Factory-cut sauna enclosure components.

C.	Factory-fabricated modular demountable outdoor sauna enclosure.

D.	Sauna benches.

E.	Sauna heater and control/monitoring equipment.

F.	Sauna accessories.

1.2	RELATED SECTIONS

NOTE ** Delete any sections below not relevant to this project; add others as required.

A.	Section 03300 - Cast-In-Place Concrete.

B.	Section 06100 - Rough Carpentry.

C.	Section 07212 - Board and Batt Insulation.

D.	Section 09300 - Tile.

E.	Section 09650 - Resilient Flooring.

F.	Section 16123 - Building Wire and Cable.

1.3	SUBMITTALS

A.	Submit under provisions of Section 01300.

B.	Product data:  Manufacturer's descriptive literature for sauna enclosure and components specified in this section.

C.	Shop drawings:  Indicate, in plan and elevations, sauna enclosure design; indicate construction details.

D.	Two samples, minimum size 5 inches wide by 8 inches long, of specified materials for wood paneling.

E.	Manufacturer's printed installation instructions for sauna enclosure, benches, heater, and control/monitoring equipment.

F.	Operation and maintenance data:  Sauna enclosure, heater, and control/monitoring equipment.

G.	Warranty documents:  Issued and executed by manufacturer.

1.4	DELIVERY, STORAGE, AND HANDLING

A.	Store products of this section in manufacturer's unopened packaging until installation.

B.	Maintain dry, heated storage area for products of this section until installation of products.

1.5	WARRANTY

A.	Manufacturer's Warranties:

NOTE ** Retain one of the following two sub-paragraphs; 5-year warranty applies to residential installations, 1-year warranty applies to commercial installations.

1.	Sauna manufacturer's 5-year warranty against defects in product workmanship and materials for sauna enclosure.

2.	Sauna manufacturer's 1-year warranty against defects in product workmanship and materials for sauna enclosure.

NOTE ** Retain one of the following two sub-paragraphs; 5-year warranty applies to residential installations, 1-year warranty applies to commercial installations.

3.	Sauna manufacturer's 5-year warranty against defects in product workmanship and materials for sauna heater.

4.	Sauna manufacturer's 1-year warranty against defects in product workmanship and materials for sauna heater.

PART 2	PRODUCTS

2.1	MANUFACTURERS

A.	Acceptable Manufacturer:  Baltic Leisure; 601 Lincoln Street, P.O. Box 530, Oxford, PA 19363. ASD. Tel: (800) 441-7147 or (610) 932-5700, Fax: (610) 932-5713.

NOTE ** Delete one of the two following paragraphs; coordinate with Division 1 requirements.

B.	Requests for substitutions will be considered in accordance with provisions of Section 01600.

C.	Substitutions:  Not permitted.

2.2	COMPONENTS

NOTE ** Retain or edit one of the following sub-paragraphs; coordinate with RELATED SECTIONS Article in PART 1 of this section.

A.	Sub-Floor:  Specified in Section 03300.

B.	Sub-Floor:  Specified in Section 09300.

C.	Sub-Floor:  Specified in Section 09650.

NOTE ** Retain the following paragraph if specifying pre-cut sauna; manufacturer recommends fir framing with maximum moisture content of 15 percent, spaced 16 inches on centers, with bottom of ceiling joists at 85 inches above subfloor.

D.	Wall and Ceiling Framing:  Specified in Section 06100.

NOTE ** Retain the following paragraph if specifying pre-built modular sauna or outdoor sauna.

E.	Wall and Ceiling Framing:  Manufacturer's standard factory-fabricated, panelized, demountable system.

NOTE ** Retain first of the following two sub-paragraphs if specifying pre-cut sauna; retain second paragraph if specifying pre-built or outdoor sauna.

F.	Wall and Ceiling Insulation:  Specified in Section 07212.

G.	Wall and Ceiling Insulation:  Expanded polystyrene foam, minimum 1 inch thick.

H.	Interior Wall and Ceiling Finish:  Board paneling of nominal 1 inch by 6 inches softwood lumber, S4S, with maximum 10 percent moisture content; long edges machined for vee-type tongue-and-groove jointing:

NOTE ** Delete one of the following two sub-paragraphs.

1.	Species:  Redwood, graded All Heart by RIS.

2.	Species:  Western Red Cedar, graded A & Better by WWPA.

3.	Lengths:  As required for single board length runs; end-to-end joints not permitted.

I.	Interior Flooring:  Modular duckboard construction:

1.	Floor boards:  Nominal 1 inch by 4 inch softwood vertical-grain lumber, S4S, same species as interior wall and ceiling finish material, with maximum 10 percent moisture content, spaced 4 inches on centers, fastened to cross-pieces with galvanized fasteners.

2.	Cross-pieces:  Same material and size as floor boards, spaced at 16 inches on centers.

J.	Door and Frame:  Pre-hung unit as follows:

1.	Frame:  Lauan Mahogany lumber, S4S, maximum 10 percent moisture content, machined for integral stop.

2.	Top and bottom hinges:  1 pair 3-1/2 inch by 3-1/2 inch butts, US4 finish.

3.	Intermediate hinge:  1/2 pair 3-1/2 inch by 3-1/2 inch self-closing butt, US4 finish.

4.	Door:

a.	Species of wood components:  Same as interior wall and ceiling finish material.

NOTE ** Delete all but one of the following seven sub-paragraphs.

b.	Lite:  9-inch by 9-inch diamond lite of 3/16 inch thick gray tempered glass.

c.	Lite:  13-inch octagonal lite of 3/16 inch thick tempered glass.

d.	Lite:  10-inch wide by 42-inch high rectangular lite of 3/16 inch thick tempered glass.

e.	Lite:  16-inch wide by 66-inch high rectangular lite of 3/16 inch thick tempered glass.

NOTE ** The following is available for pre-built units only.

f.	Lite:  16-inch wide by 60-inch high rectangular lite of 3/16 inch thick tempered glass.

g.	Lite:  16-inch wide by 60-inch high octagonal lite of 3/16 inch thick tempered glass.

h.	Lite:  16-inch wide by 60-inch high rectangular two-way mirror of 3/16 inch thick tempered glass.

i.	Pulls:  Machined from same material as interior wall and ceiling finish material; provide pull for each side of door.

NOTE ** Retain one of the following sub-paragraphs.  Standard door size for pre-cut units is 2 feet by 6 feet 8 inches; standard door size for pre-built units is 2 feet by 6 feet.  6-foot high door, and 3-foot wide door for handicap access, are options.

j.	Door size:  2 feet wide by 6 feet, 8 inches.

k.	Door size:  2 feet wide by 6 feet.

NOTE ** Delete one of the following two sub-paragraphs.

l.	Door swing, as viewed from outside sauna, outswinging:  Hinged left side.

m.	Door swing, as viewed from outside sauna, outswinging:  Hinged right side.

K.	Windows:

NOTE ** Retain one of the following eight sub-paragraphs to specify lite.

1.	Lite:  10-inch wide by 42-inch high rectangular lite of 3/16 inch thick tempered glass.

2.	Lite:  16-inch wide by 36-inch high rectangular lite of 3/16 inch thick tempered glass.

3.	Lite:  16-inch wide by 66-inch high rectangular lite of 3/16 inch thick tempered glass.

4.	Lite:  16-inch wide by 36-inch high octagonal lite of 3/16 inch thick tempered glass.

5.	Lite:  16-inch wide by 66-inch high octagonal lite of 3/16 inch thick tempered glass.

6.	Lite:  16-inch wide by 66-inch high rectangular two-way mirror of 3/16 inch thick tempered glass.

7.	Lite:  24-inch wide by 36-inch high rectangular lite of 3/16 inch thick tempered glass.

NOTE ** Retain the following sub-paragraph if specifying Riga Style lite.

8.	Lite:  16-inch wide by 36-inch high octagonal window with octagonal window in panel.

9.	Trim:  Same material as interior wall and ceiling finish material.

NOTE ** Delete one of the following paragraphs if exterior finish material is required; delete both if finishing is specified in another section of the specifications.

L.	Exterior Finish  Material:  Board paneling of same material as interior wall and ceiling finish material.

M.	Exterior Finish  Material:  Lauan Mahogany sheet paneling.

N.	Benches:  Constructed of same material as interior wall and ceiling material, S4S, vertical grain, and as follows:

1.	Framing:  Vertical members nominal 2 inches by 2 inches, minimum; horizontal members nominal 1 inch by 4 inches, minimum.

NOTE ** Retain one of the following two sub-paragraphs.

2.	Seating slats:  Nominal 1 inch by 4 inches, with eased edges to minimize splinters, spaced 4 inches on centers.

3.	Seating slats:  Nominal 2 inches by 2 inches, with eased edges to minimize splinters, spaced 2 inches on centers.

4.	Trim: Constructed of same material as seating slats, with eased edges to minimize splinters, and as follows:

a.	Bench fascia:  Nominal 1 inch by 4 inches.

b.	Bench blinders:  Nominal 1 inch by 3 inches.

5.	Fasteners:  Galvanized, concealed from view where possible.

O.	Heater:  Electric heater containing igneous dolerite rocks for heat retention, UL listed, with gravity heat flow; forced-air fan not permitted.

1.	Acceptable product:  TYLO AB, Halmstad, Sweden.

2.	Metal enclosure finished with covering to protect against injury from user contact.

3.	Capacity:  Recommended by manufacturer for sauna interior volume.

NOTE ** Consult manufacturer's product data for electrical requirements of recommended size unit.

4.	Electrical requirements:  ___VAC, ____ watts.

P.	Heater Control/Monitoring Equipment:  Timer-activated switch with 60-minute maximum duration before automatic heater shut-off.

NOTE ** Delete accessory items not required for project; refer to manufacturer's product literature for fuller description of accessories.

Q.	Accessories:  Supply the following:

1.	Wall-mount sauna light; manufacturer's standard, with vapor-proof lens.

2.	Sliding adjustable vent; wood construction, species matching sauna interior.

3.	Corner lampshade; wood construction, species matching sauna interior.

4.	Vent grille.

5.	Diagonal backrest.

6.	Wooden water bucket.

7.	Wooden ladle.

8.	Sauna headrest.

9.	Hygrometer, teardrop shape.

10.	(Hygrometer/Thermometer I - Item discontinued).

11.	Hygrometer/Thermometer FP.

12.	Audio transducer.

13.	Thermometer, teardrop shape.

14.	Skirting panel, horizontal.

15.	Skirting panel, vertical.

16.	Sauna sign.

17.	Clock, with wood housing.

18.	Hygrometer.

19.	Thermometer.

20.	Thermometer BA.

21.	Hygrometer BA.

22.	Thermometer C.

23.	Hygrometer C.

24.	Hygrometer/Thermometer E.

25.	Wall-mount lampshade; wood construction, species matching sauna interior.

26.	Hourglass.

27.	Wooden tub.

28.	Sauna Kleana.

29.	Eucalyptus fragrance.

30.	Copper bucket and ladle.

31.	Towel bar; four-peg.

32.	Towel bar; two-peg.

33.	Brush set; two brushes.

34.	Brush set; four brushes.

35.	Body massage.

2.3	FABRICATION

NOTE ** Delete one of the following two paragraphs; first is for pre-cut sauna, second is for pre-built or outdoor sauna.

A.	Cut wood materials for sauna enclosure to exact sizes required for field assembly without further trimming or adjustment.

B.	Assemble sauna enclosure, to extent practical within shipping limitations, in modular demountable units for field assembly without further trimming or adjustment.

C.	Assemble door and frame as pre-hung unit, complete with hardware.

D.	Package components for shipping.

PART 3	EXECUTION

3.1	EXAMINATION

A.	Verification of Conditions:

1.	Utility services are of correct type and in correct locations.

2.	Construction activities specified in RELATED SECTIONS Article of PART 1 of this section that affect construction activities of this section are correct and complete.

B.	Installer's Examination:

1.	Have installer of this section examine conditions under which construction activities of this section are to be performed, then submit written notification if such conditions are unacceptable.

2.	Transmit two copies of installer's report to Architect within 24 hours of receipt.

3.	Beginning construction activities of this section before unacceptable conditions have been corrected is prohibited.

4.	Beginning construction activities of this section indicates installer's acceptance of conditions.

3.2	INSTALLATION

A.	Install sauna enclosure, heater, and control/monitoring equipment in accordance with shop drawings and manufacturer's printed installation instructions.

B.	Connection of heater, lighting, and control/monitoring equipment, to electrical service is specified in Section 16100.

3.3	ADJUSTING

A.	Operate heater, lighting, and control/operating equipment, through all modes to ensure correct operation.

3.4	CLEANING

A.	Remove soil from surfaces in accordance with manufacturer's instructions.

3.5	PROTECTION OF INSTALLED PRODUCTS

A.	Protect sauna enclosure and other components from damage to finishes and operation until Substantial Completion.

END OF SECTION