Sinclair ZX Spectrum Emulator 'Z80' v2.01  -  3/5/93  -  by G.A. Lunter

Contents:

1.    INTRODUCTION, REGISTRATION, GENERAL INFORMATION

1.1   Some general remarks

1.2   Registering - sounds interesting!

1.3   Other emulators

1.4   Thanks

2.    THE EMULATOR

2.1   Starting the emulator

2.2   Using the emulator

2.3   Emulation of the Keyboard, Screen and Beeper

2.4   Using the Tape

2.5   Using the Microdrive

2.6   Using the RS232 channel

2.7   Joysticks

2.8   Transferring programs

2.9   Converting file formats - the utility CONVERT

2.10  The utilities Z802TAP and TAP2TAPE

3.    THE SAMRAM

3.1   Basic extensions

3.2   NMI software

3.3   The built-in monitor

4.    THE SPECTRUM

4.1   The Spectrum

4.2   The Interface I

4.3   The Spectrum 128

5.    TECHNICAL INFORMATION

5.1   The Spectrum

5.2   The Interface I

5.3   The SamRam

5.4   The Z80 microprocessor

5.5   File formats

1. INTRODUCTION, REGISTRATION, GENERAL INFORMATION

1.1 Some general remarks

This is the documentation for 'Z80', a Sinclair ZX Spectrum 48/128

emulator.  This program turns your PC into a Spectrum.  To make you

read on,...

The program emulates a Spectrum 48K model 2 or 3 or Spectrum 128K,

is very, very compatible with the real machines, includes the

Interface I, supports Microdrives, tape files, the RS232 channel,

'128 sound through internal speaker or Adlib Compatible soundcard,

able to save and load every Spectrum program directly to and from

tape, even able to load speed-saved programs, supporting digital

and analogue PC joysticks and four common Spectrum joysticks, Z80

processor emulation including the R register, inofficial opcodes

and flags, accurate timing of individual instructions, control

over the emulated Spectrum's speed, and all that while requiring

only a PC-XT with 512K with CGA, Hercules, EGA or VGA; including

conversion programs to convert between various emulators' snapshot

formats and to read from Disciple and Plus D diskettes, to create

.PCX and .GIF files of Spectrum screen dumps, an English manual...

There is much to tell and explain in this documentation.  First of all

the emulator itself must run, and uses your PC's resources.  It is not

really a demanding program, but there are some things that need

attention.  These technicalities are dealt with in section 2.1.

Some general things about the emulator are explained in section 2.2. If

you read 2.1 and 2.2, you'll be able to do most of the things you

probably ever want to do.  But to exploit all of its possibilities (and

oh, it can do so much!), you will really have to read it all.

The Spectrum has a number of ways to communicate with the outside world,

like the obvious keyboard and the screen, but also the microdrives, the

tape interface, the beeper, the sound chip of the Spectrum 128, the

Kempston joystick, and the RS232 channel of the Interface I and Spectrum

128.  All these channels can be used to communicate with PC channels in

some way; for instance the keyboard is connected to the PC keyboard

(sounds obvious) and the tape I/O can be routed to a file, as well as to

a physical tape recorder.  All these things are explained in the rest of

chapter 2.  Paragraph 8 of that chapter contains a number of suggestions

how to transfer Spectrum programs to the PC.

For our own Spectrums Johan Muizelaar and I built a piece of hardware we

called the SamRam (which has nothing to do with the SAM coupe, by the

way!).  It contains a monitor program and software to make snapshots of

programs.  It's still very useful and I still use it a lot, although the

physical SamRam doesn't work anymore.  An explanation of its functions

is to be found in chapter 3.

Some things peculiar to the Spectrum, not specific to this program but

useful to know are collected in chapter 4.  It contains for instance a

table of Spectrum keywords and the key combination to get it;

unfortunately this information is not printed on standard PC keyboards!

There are some interesting, rather unknown technical facts about the

Spectrum that I discovered while debugging the emulator.  As much as I

could think of is contained in the final chapter.  You don't need to

read this chapter to use the emulator; if you don't find it interesting

then skip it, but I think programmers will like it.

A remark about copyrights.  The source files are not public domain, and

you may not use them in other PC-based Spectrum emulators. Also, the

information in this documentation file, especially the info in the final

chapter (except for the file-format info in the final section) may not

be used for that purpose.  But you're free to use the info for Spectrum

emulators for other machines, provided that whenever you do so you

should name the source.

For Spectrum software, utilities, other emulators for PC's as well as

other computers, and other Spectrum related software, you can call the

Spectrum Emulator support BBS in Groningen:

Tatort BBS Groningen

050-264840

(+31-50-264840)

v22, v22bis, v32, v32bis, MNP2-5, v42, v42bis (300-14400 baud)

At the time of writing the BBS is open 24 hours a day, but this is

subject to change.  Please try calling between 22:00 and 9:00 local time

first.

If you have access to Internet, you can find several Spectrum emulators

in a directory of for instance wuarchive.wustl.edu (take a look in

/systems/sinclair and -/msdos/emulators) or nic.funet.fi.  And if you

want to get in touch with me, my email address is gerton@rcondw.rug.nl.

1.2  Registering - sounds interesting!

First of all, this program is shareware.  That means that you're

encouraged to give it away to others, but if you do be sure not to alter

the files.  Also please don't add things to the archive file. Shareware

means that you may try the program some time; if you like it you should

register for it.

The shareware version of the emulator consists of the following files:

Z80.EXE      - The emulator

Z80.INI      - Default initialisation file

Z80.DOC      - This documentation file

Z80.TEX      - LaTeX documentation file, by Lars Koeller

Z80.PS       - PostScript documentation file, from the .TEX one

Z80.PIF      - Program Info File to run 'Z80' under Windows 3.1

Z80.ICO      - Windows icon

ROMS.BIN     - Various ROM images

LAYOUT.SCR   - Keyboard lay-out help screen

GETRS.COM    - Utility to receive blocks through RS232 lead

SAVESPEC.Z80 - Utility to send .Z80 files from Spectrum to PC

DIAGRAM.Z80  - Circuit diagram for tape interface, and calibration

Z80FAQ.DOC   - Frequently asked questions - and answers!

NEW.DOC      - The What's New file

The shareware version of the emulator program is not fully functional.

It cannot be slowed down, and it can't load programs from tape.  All

other functions work the same in both versions.  If you register, you

will receive the fully functional emulator together with the following

utilities:

CONVERT  - a general conversion program: can list out BASIC and

tranlate it back, produce .GIF or .PCX files from

screendumps, translate Spectrum ASCII (CR) to PC ASCII

(CR/LF), and some other things.

CONVZ80  - Translates various snapshot and tape formats of other

Spectrum emulators into each other.  Can handle Arnt

Gulbrandsen's (JPP) .SNA format, Pedro Gimeno's

(VGASPEC and SPECTRUM) .SP format and Kevin J. Phairs'

(SPECEM) .PRG format.  It can also handle tape files

of SPECEM and L. Rindt and E. Brukner's emulator ZX.

DISCIPLE - Reads DISCiPLE and Plus D diskettes, both 3.5'' and

5.25''.  It translates the 48K and 128K snapshot files

to .Z80 snapshots, and ordinary files and screen

snapshots to .TAP tape files.

Z802TAP  - Converts a .Z80 snapshot, 48K or 128K, to a .TAP file

which can be loaded into the emulator and saved to tape

by the next utility:

TAP2TAPE - Saves the contents of a .TAP file back to tape, to

load it into an ordinary Spectrum.

Z80DUMP  - Shows the contents of the header of a .Z80 file.

You will also receive the source files of the emulator, the above

utilities and the SamRam, and you'll be kept informed about future

updates.

The registration fee is 20 US$, or 15 British pounds, or 35 German

Marks, or 35 Dutch guilders, or some of your local (hard) currency of

about that amount.  Now there are several way to get the money to me.

In order of preference:

1.  Simply send banknotes.

2.  From Europe, send a Eurocheque of HFL 35,--

3.  Send a postal money-order (Works fine from e.g. Italy and

Spain)

4.  Send a bank cheque.  Please add the equivalent of 20 Dutch

guilders, for that's the amount the banks charge for drawing

foreign cheques.

If you're sending a Eurocheque, make sure you fill it in completely

(don't forget the number at the back!) and fill in 'Groningen' for the

place.  If you don't send Dutch currency, or don't fill it in

completely, or fill in a foreign city, the banks charge me fifteen to

twenty guilders to cash the cheque.

For Dutch users, the fee is HFL 25,--.  In Nederland gaat het betalen

het gemakkelijkst via de giro.  Maak het bedrag over op giro 59.45.263

t.n.v. G.A. Lunter, Groningen.  Zorg er wel voor dat uw naam en adres

vermeld staan! (vooral als u Girotel gebruikt)

Send the money, together with your name and address to:

Gerton Lunter

P.O. Box 2535

NL-9704 CM  Groningen

The Netherlands

You'll get the files on a 3.5'' DD disk by default, but you can also get

in on 5.25 inch disks if you want.

Registrations can also be handled by B G Services in the UK if this is

more convenient.  The cost is the same (15 British pounds).  Payment can

be by cheque or postal order made payable to B G Services.  The address

is:

B G Services

64 Roebuck Road

Chessington

Surrey KT9 1JX

Telephone enquiries on 081 397 0763, Fax 081 391 0744.

For registrations in the Czech Republic, I recommend to contact JIMAZ,

who will provide details on registering in the local currency:

JIMAZ s.r.o.

Hermanova 37

170 00 Praha 7

phone: +42 2 379 498

fax:   +42 2 378 103

1.3  Other emulators

There are several other Spectrum emulators, both for the PC and other

computers.  The list below is partly due to Carlo Delhez (the QL

emulators) and partly copied from Arnt Gulbrandsen's documentation of

his JPP.  I don't think the list is complete, so if you know more

Spectrum emulators, for any computer, please let me know.

For the PC:

o JPP, by Arnt Gulbrandsen (Norway).  Faster than mine (but according

to an OUTLET review slower on some boards), by using a very smart

decoding technique, but requires a 80386 or '486 processor.  Is less

compatible than Z80.  Uses the .SNA snapshot format.  Needs VGA.

Not many extra features.

o VGASPEC, by Alberto Olloqui (Spain).  Needs VGA and 80286.  Quite

slow, and crashes on quite a lot of programs.  Uses the .SP snapshot

format.  Allows ROM pokes.  This program is an illegal pre-release

of SPECTRUM, by Pedro Gimeno.

o SPECTRUM, by Pedro Gimeno (Spain).  Uses another .SP snapshot

format.  Has a tape interface.  Also quite slow.  Allows changing

the rom.

o SP, by J. Swiatek and K. Makowski (Poland).  Cannot load or save

snapshots, but can load programs using LOAD "" via a file called

TAPE_ZX.SPC.  Crashes many programs; even basic behaves weird

sometimes.  Has a built-in monitor, but no documentation.  No

border.

o SPECEM, by Kevin J. Phair (Ireland).  Also allows rom changes.

Displays the registers on screen.  Can save and load directly

from disk using LOAD/SAVE "filename" in BASIC.  Loads .PRG

snapshots, but cannot save them.  Emulates a Multiface I.

o ZX, by L. Rindt and E. Brukner (Czech Republic).  Haven't tested

its compatibility thoroughly, but one of the games supplied didn't

respond well to the keyboard, while it did work on Z80 after

conversion.  Good tape file support including headerless files,

almost identical to the multiple .TAP file mode of Z80.  Somewhat

slower than Z80.  Includes program to load from tape and convert

to tape file.  No documentation at all.

For the Sinclair QL:

o SPECTATOR by Carlo Delhez, The Netherlands; shareware; supports

tape-files, Microdrives, RS232, Z80 snapshots, MBF snapshots and

Disciple (SNP) snapshots; utilities to convert Disciple, Beta and

Opus disks enclosed.

o ZM-1/2/3/4 by Ergon Development, Italy; ZM-1 is shareware, ZM-2/3/4

are commercial. They all support tape-files and Z80 snapshots, some

support Microdrives and RS232; contain a utility to transfer programs

from tape via a Spectrum to the QL.

o ZX by Andew Lavrov, CIS; shareware; supports tape-files, MBF

snapshots en Z80 snapshots; utility to read from Spectrum tapes

(and write them).

Spectator, ZM-1 and ZX are all about as fast (approximately 30 to 40%

on a 16 MHz MC68000 machine). ZM-2/3 are faster, but this at the cost

of compatibility. ZM-4 is not an emulator, but a real-time Z80-compiler:

very fast and seems to be compatible as well.

For the Amiga:

o Spectrum, by Peter McGavin.  Very good, JPP is based to a large

extent on it.  Needs about a 25MHz machine to run at full speed.

Has tape support.

o KGB.  I haven't seen it.  A bit slower than Peter's, and the

version Peter saw wouldn't work on the Amiga 3000.

o An Italian emulator which I don't know the name of.  Excellent

compatibility, rather fast.  May be shareware.

o Several unreleased emulators.  Peter knows more about them.

For the Atari ST/TT:

o One, called Spectrum.  Don't know anything about it, but the doc

file is written in quite the worst English I've seen.  [This is

Arnt speaking --- I've seen worse! GAL] Available by anonymous

ftp from terminator.cc.umich.edu.

o There's another one in the make, to be released very soon as one of

the programmers told me, written by Markus Oberhumer and other(s).

For the Acorn Archimedes:

o A company called Arxe wrote one, intended to be commercial but

never released because Amstrad wouldn't permit Arxe to enclose the

ROM.

o Someone called D. Lawrence wrote another, or maybe the same.

This one is floating around but nobody has any documentation.  I

don't know what its status is.  Runs at about 70% of Spectrum speed on

an ARM2, not quite perfect graphics emulation.

o There is a PC emulator for the Archimedes...

For the Commodore 64:

o The Whitby Software Spectrum simulator is a rewrite of the

Spectrum Basic.  It will not run machine-code programs.  I don't

know whether it's PD, shareware, or commercial.  Quite good.

(Responds nicely to a POKE 23659,0)

All emulators for PC, and some for the Atari, Amiga and QL are available

on the support BBS.

There are also emulators available for the ZX81.  Carlo Delhez, who also

wrote a Spectrum emulator for the QL, wrote the ZX81 emulators XTricator

(for the QL) and XTender (for PC's).  These programs can also be

downloaded from the support BBS.

1.4  Thanks

From the very first beginning in november 1988, when I wrote the first

lines of code, Johan Muizelaar has been a very demanding and critical

user, being only satisfied when it was perfect.  And quite a few things

I would never have started working on if he hadn't insisted that I

would...

Secondly, I have to thank Brian Gaff, who is now handling the UK

registrations for nearly a year, and besides doing lots of PR for my

program there helped me with many things, especially with the DISCiPLE

conversion program.

Finally, I'd like to thank

o   Lars Koeller for transforming this plain ASCII doc to LaTeX,

o   Thomas Franke for translating the entire Dutch v1.45 manual into

German,

o   Carlo Delhez for information on the '128 and several other things,

o   Andre Mostert for some more '128 info and info on EMS memory,

o   Walter Prins for many '128 programs and a nice African chat,

o   Marco Holmer for making the program such a big hit at the HCC dagen,

o   Henk de Groot, for finding and helping me work around a bug in A86

version 3.22,

o   Arnt Gulbrandsen for pointing out to me that it is not necessary to

clear a register when it contains 0,

o   Ruud Zandbergen for his digital joystick interface,

o   Ettore de Simone for finding a noisy bug, and for some very wise

remarks about theological issues,

o   Jan Garnier for providing the chips to reanimate my real Spectrum,

o   Rudy Biesma and Tonnie Stap for providing info on the DISCiPLE disk

formats,

o   Hugh McLenaghan for his very valuable help on the DISCiPLE program,

o   Burkhard Taige for various bug reports on it,

o   Ian Cull for enhancing the DISCiPLE program and fixing two early

bugs,

o   Bert Lenarts for information on the AZERTY keyboard, and

o   Andre Brus for writing the most enthousiastic letter I've ever

read!

2.  THE EMULATOR

2.1  Starting the emulator

The emulator will work on any PC with at least 512K memory, with a VGA,

EGA, Hercules, CGA or Plantronics video adapter.  If available, it will

also use EMS memory, an Adlib compatible soundcard, and an analogue or

digital joystick.

The emulator will first read in the switches that are given in the

Z80.INI file.  You can enter switches there in the same way you would on

the command line.  Lines starting with a % sign will be ignored.

After any switches, you may specify a snapshot file on the command line.

This file will then be loaded and executed directly.  The extension .Z80

is not necessary.  The emulator will also read .SNA files (the snapshot

format of, amongst others, Arnt Gulbrandsen's JPP); you don't have to

convert them to .Z80 files (but you may want to to save disk space).

The emulator tries to figure out what hardware is available, and uses

things as it finds it.  Most of the time this will work without you

having to tell it anything, but if you have to you can override the

defaults by putting switches on the command line.  Switches that you use

often can be put in the Z80.INI file.  If you give a switch a second

time, for instance if it is also in the Z80.INI file, it will disable it

again.

If you are using Hercules, try starting the emulator with -xh on the

command line.  The emulator will use a non-standard Hercules mode to

display a full-screen Spectrum picture.  You may need to calibrate your

monitor to make the image steady.

If you're using Plantronics, try -p and -q to see which gives the best

result.

Some black-and-white VGA monitors only display one of the three RGB

colours (green most of the times), resulting in several Spectrum colours

becoming indistinguishable.  Use -xb to use grey tones instead of

colours.

If you're using a Trident VGA with version 3 BIOS, you may see the

picture compressed at the top of the screen, while the bottom half

contains vertical white lines.  This is due to a bug in the Trident VGA

Bios.  Start the emulator with the switch -xv to get a full picture.

If you haven't got EMS memory, the page swapping of the Spectrum 128

cannot be emulated exactly.  Most programs will work - although quite

slowly because page swapping will take much time without EMS - but some

may crash.  On 386 and 486 machines you can emulate EMS by software,

using EMM386 for instance.  Of all the EMS emulators I've tried (that's

three or four) QEMM was by far the fastest, but the EMM386 supplied with

the new DOS 6 seems to be about as fast.  A slow EMS emulator can

degrade the performance of the '128 emulation significantly!  Some

computers have hardware EMS capabilities, some '286 boards for instance.

Refer to your own documentation for details.

And don't use hard disk based EMS emulators: the Spectrum emulator will

drive your hard disk nuts!

There are a few Spectrum programs that have an odd stack pointer, and

run over the ram/rom boundary, for instance Deep Strike.  This crashed

version 1.45 of the emulator.  The bug has been removed in version 2: if

the emulator tries to read a word at FFFF, the processor generates an

INT 0D interrupt and the emulator will handle it correctly. However,

this won't work when an EMS emulator is installed that puts the 386 or

486 processor in virtual 8086 mode.  You can test all this by typing

CLEAR 65535:POKE 65535,0: RETURN in Basic, and the emulator will lock up

if it runs in virtual mode.  There is no simple solution to this

problem, but luckily it doesn't happen often.  If it does, the easiest

way to to solve it is to change the Spectrum program so that it uses an

even SP --- this is always possible, but not always easy!

A very few programs (the only examples known to me are Fireman and

Thing) are quite sensitive to the relative actual execution speed of

emulated Z80 instructions, and crash if it isn't exactly right. If you

slow down the emulator, these program will run fine, because then

individual instructions are more carefully timed.

The Spectrum 128 has a built-in sound chip.  If you have an Adlib

compatible soundcard installed, the Spectrum 128 sound will be played

through the Adlib card.  If you haven't, the loudest of the three sound

channels will be played through the internal PC speaker.  Sometimes the

effect is quite nice, sometimes it is horrible, but it's all I can do on

a standard PC.  If you don't want to have the Spectrum 128 sound played

through the internal speaker, use the switch -xi.  If you don't want the

Adlib card to be used (for instance to hear the sound through the

internal speaker) use -xa.

If you're using the Pro-Audio Spectrum 16 sound card, do not install the

resident FM.EXE program; it causes problems with the emulator. Do make

sure that MVSOUND.SYS is installed in your CONFIG.SYS file, to make the

Pro-Audio Spectrum 16 Adlib compatible.

The noise channels of the Spectrum 128 sound chip can work on different

frequencies, whereas the FM chips of the Adlib card cannot.  However, if

your Soundblaster is equipped with CMS chips, the noise frequency can be

programmed.  Specify -xc to use the CMS chips.  (These chips are not

available on Soundblaster Pro cards, and neither on most Soundblaster

clones).

If you're living in Belgium or France, you are probably using an AZERTY

keyboard.  Specifying -xz on the command line will make all letter keys

and many punctuation keys work in the right way.

If the emulator erroneously detects an analogue or digital joystick, use

the switch -kk.

It may be annoying to have to press Num-Lock every time you use the

Spectrum 128 (because otherwise you'll have to use Shift with the cursor

keys to move the menu bar).  To make the emulator press shift by default

every time you use the PC cursor keys in '128 mode, use the switch -xs.

If you press Num-Lock now (in '128 mode), the shift-key won't be

pressed.  The 48K modes are not affected by this switch.

The emulator can now also be run under Windows 3.1!  However, you cannot

use the tape interface, Real mode doesn't work anymore, and the keyboard

is not emulated as well as usual, because I have to scan it using BIOS

calls.  Be sure not to set the keyboard repeat rate too low; an initial

delay of 250 ms followed by 10 keys a second will do, but don't make it

slower.  Some key combinations may not work, such as ALT 4 for $.  So if

you want to use the emulator seriously then you shouldn't run it under

Windows, but it's nice to see three Spectrums run simultaneously...  If

you let the emulator run full-screen you may use EGA or VGA, if you want

to run it windowed you'll probably have to use CGA, because the virtual

video display driver of Windows cannot handle the VGA mode I use

(although it's only a standard text mode!).  You'll probably want other

default settings of some parameters (such as the video mode) if you run

the emulator under Windows; the emulator will always use the .INI file

in the directory of the Z80.EXE file so the other switches must be put

on the command line, in a .PIF file.  An example .PIF file (which runs

the emulator in windowed CGA mode) and a .ICO icon file are supplied

Since the execution speed of a program running under Windows heavily

depends on other processes, the emulator doesn't try to measure how fast

it is running under Windows.  It says it runs at 100%, and you can slow

it down in the usual way, but the percentage is now relative to the

maximum speed, and has nothing to do with the actual execution speed.

The emulator will automatically detect whether Windows is running, and

act appropriately.  To run the emulator in Windows compatibility mode in

a normal DOS environment, use -xw.

When running the emulator under Desqview, use -e for EGA mode display.

To run the emulator with a different rom than the standard one, you can

specify a rom image file on the command line.  Use the switch -xr file,

where 'file' is the name of the image file.  This file should be exactly

16384 bytes long.  It will of course not be used in Spectrum 128 or

SamRam mode.

The emulator 'ZX' by Rindt and Bruckner comes with several roms, stored

in their tape format.  You can convert these files to .TAP files, and

then load them in the normal way, but to run the emulator with these

roms you need the bare 16K binary image file.  To extract it from the

rom files, type the following at the DOS prompt:

C:\>debug rom.000       (or other rom file (of 16406 bytes))

-m 115 L 4000,100       (move the rom down, overwrite header)

-rcx                    (new length of exactly 16K bytes)

CX 4016

4000

-n rom000.bin           (or some other name)

-w                      (write it)

Writing 04000 bytes

-q                      (and quit)

These are the most important switches that you have to specify when you

start the emulator.  Most of the other switches are used to select

default values for various things which can also be changed when the

emulator is started.  Some useful things to select are default

directories for .Z80, .TAP and .MDR files; these will be explained

below.

2.2  Using the emulator

When the emulator starts, you'll see the usual Spectrum copyright

message appear on screen.  Pressing F1 will pop up a small help screen

that explains the function of the function keys and various other

special keys.

By pressing F10, you enter the main menu of the emulator.  Most of the

menu options can be chosen directly by pressing another function key.

The only exception is X, Extra functions, for which no function keys

were available anymore.  If you're somewhere deep in the menu structure

of the main menu, pressing ESC will get you one level higher most of the

time.  Pressing F10 will get you back to the main menu.

The 'Select Hardware' menu option sits under function key F9.  There are

five configurations you can choose: a normal Spectrum 48K with or

without Interface I, a Spectrum 128K with or without Interface I, and a

Spectrum with Interface I and SamRam.  Switching to another mode will by

default reset the Spectrum.  If you don't want this to happen, press

CTRL-ENTER instead of ENTER when you've made your choice.  It cannot be

guaranteed however that the Spectrum won't crash or behave weirdly, for

obvious reasons.  Going from a Spectrum 128 to a normal Spectrum will

almost always crash it, except if you enter the SPECTRUM command before

switching.

To use SamRam's monitor on a 128 program, switch the hardware from the

main menu, and generate an NMI (Extra functions - N) before returning to

the emulator.  This will often work, but you can't return to the program

without crashing it.

On a real Spectrum 128, the menu bar of the startup screen is moved

using the cursor keys on the '128 keyboard.  These keys simultaneously

press a normal cursor key (5,6,7 or 8) and shift.  So you can shift the

menu bar with shift-6 and shift-7.  As is already said above, it is

possible to use the PC cursor keys for this; you have to select Cursor

joystick emulation (which is selected by default) and press Num-Lock

once to have the PC-cursor keys press the Spectrum Shift key too.  You

could also specify -xs on the command line (or put it in the Z80.INI

file) to make the PC cursor keys by default press shift for you in '128

mode.

The Save and Load Program options (F2 and F3) will save the whole state

of the Spectrum and some of the emulators' settings to a .Z80 snapshot

file.  It will pack the data somewhat, so that the length of the file

varies.  The amount of memory saved depends on the current hardware

mode; 48K for normal Spectrum, 80K for SamRam, and 128K for Spectrum

128.  The settings that are saved are those that are program dependent,

for instance which joystick emulation is used, and more technical

settings like those of the R register, LDIR and Issue 2 emulation,

double interrupt frequency and video synchronisation.  These are

explained below.

Loading a .Z80 file will cause several settings to be changed. Resetting

the Spectrum will not reset these settings to their default values!

Especially the joystick emulation setting change can be confusing, so

keep track of that.

The Change Settings menu pops up if you press F4.  You can do many

things here, and I won't explain them all here.  The I and O options can

be used to redirect the RS232 output; see paragraph 2.6 for information

on this.  R - R register emulation, and L - LDIR emulation are seldom

needed.  For remarks on these options see chapter 5, and paragraph 2.8.

2 - Issue 2 emulation will turn the emulated Spectrum in an Issue 2

Spectrum.  (This option also works, but is ridiculous, in Spectrum 128

mode).  Some very old programs (Spinads) will not respond to the

keyboard properly on Issue 3 Spectrums, and for these programs this

option was added.  Seldom needed.

F - fast flash makes flashing go twice as fast.  Not very useful.

S - sound enables you to turn off all sound, useful for late-night

playing.

D - double interrupt frequency is useful for slow machines, as some

programs will run faster when this option is on.  If you're typing in a

BASIC program on a slow machine, always turn this on, since the

keyboard, which is polled by an interrupt routine, will respond much

better.  On the other hand, some programs will crash with this option

active.

V - video synchronisation is used to remove the flickering of moving

characters in some programs.  You can choose between Normal, High and

Low.  Normal works well for almost all programs; Ghosts and Goblins and

Zynaps look much better when this is turned to High.  If you see

characters not moving smoothly or flicker, or a background not moving as

a whole, experiment a little bit with this setting, and re-save the

snapshot when you've found the best setting.  (For a slightly more

detailed discussion of this option see section 5.1)

J - joystick emulation specifies which Spectrum joystick the PC cursor

keys (and analogue or digital joystick, if it is available) control. You

can choose from Cursor (default), Kempston, Interface 1 and 2.  As I

already said above, if Cursor joystick is chosen, the Num-Lock key

controls whether Shift is pressed too if the PC cursor keys are pressed.

(Since the shift and number keys are pressed exactly simultaneously, it

is possible that the Spectrum has already read the Shift key, but not

yet the others, when you press both keys down. Sometimes you will

therefore get the number 5,6,7 or 8 instead of a cursor movement.)

Finally, C - Change speed lets you control the speed of the emulator. As

a side effect, slowing down the emulator makes the timing of the various

opcodes correspond more exactly to the actual timing on a real

processor.

That concludes the discussion of the F4-'change settings' menu.  Let's

continue with the other function keys.

F5 generates an NMI.  Only useful if in SamRam mode; otherwise it may

reset the Spectrum or (sometimes) crash a program.  ALT-F5 or CTRL-F5

resets the Spectrum.

F6 turns on Real Mode.  Try this when the emulator is playing a tune and

sounds a little harsh.  This mode is needed when you want to load

speed-saved games from tape; see below for more information.

F7 and F8 activate the tape- and microdrive-menus.  Again, see below for

more information.

Resetting the Spectrum, or generating an NMI can be done from the main

menu too, in the X - Extra Functions menu.  This is useful if you want

to activate the NMI software of the SamRam for instance just after

loading a snapshot file, or just after you changed the hardware mode.

From this menu you can also shell to DOS, and save or load a screen

snapshot: a 6912 byte file with extension .SCR that contains a dump of

the screen information.  This enables you to very easily transfer

screens from one Spectrum program to another.  The .SCR files can be

converted to .GIF or .PCX files by the CONVERT program, available to

registered users.

When you're typing BASIC-programs in 48K mode, you'll probably have to

look up some Spectrum keywords.  Further down in this documentation

there is an alphabetical list of all keywords and their key-combination.

For 'on-line' help, press ALT-F1 to see the Spectrum keyboard layout.

2.3  Emulation of the Keyboard, Screen and Beeper

The keyboard.  Letter keys are mapped to the Spectrum's letter keys. The

ALT and CTRL keys can both be used for Symbol Shift.  Then, there are a

lot of keys on the PC keyboard which don't exist on the Spectrum

keyboard.  Many of them are used, to make things easier:

The function keys have several special functions.  See the previous

paragraph.

CTRL-Break and CTRL-ALT-DEL quit the emulator.

The punctuation keys - = ; ' , .  / and their shifts: _ + : " < > ? have

the effect of pressing Symbol Shift and the corresponding letter key, so

you can use these in the straightforward way.

The ESC key presses Shift-1, EDIT, used as a sort of ESC key in many

Spectrum programs.  The Backspace key presses Shift-0, the Delete of the

Spectrum.  CapsLock presses Shift-2, Spectrum's capslock key.

The PC-cursor keys and the numeric keypad keys 8,4,6 and 2 control the

Cursor, Interface 2 or Kempston joystick.  The TAB key, and 0,5 and

./DEL on the numeric keypad control the fire button.  If the Cursor

joystick is selected, you can select whether Shift should also be

pressed with the NumLock key (but see the discussion above of the -xs

switch).

If you're running the emulator on a slow computer, try selecting double

interrupt frequency.  Most programs poll the keyboard by interrupt, in

any case the ROM does, and doubling the frequency with which this

happens will make the emulated Spectrum react much more quickly on your

keystrokes.

If you've got an AZERTY keyboard, the standard mappings of the keys

won't work at all properly.  Include the switch -xz in your Z80.INI file

in this case; many punctuation keys will now also work properly. There

is no support for other non-US keyboard layouts; sorry!

Now about the screen emulation.  Fifty times an (emulated) second, the

screen is checked for changes.  If anything has changed, the change is

displayed on the PC screen.  It turned out that this was the fastest

method of updating the screen.

I tried to update the screen at about the same time the real Spectrum

shows it on the TV screen, relative to the 50 Hz interrupt.  There is a

problem; the Spectrum takes about 1/100th of a second to generate the

whole picture, while I stop the emulator at some point in the 1/50th-

of-a-second cycle and display the whole screen at once.  Usually this

makes little difference, but with some programs it does: characters may

flicker heavily or disappear entirely (see for instance BC's Quest for

Tires).  By selecting the 'video synchronisation mode', you have some

control over the exact point of the cycle at which the screen is

updated.

In the Hercules, CGA and Plantronics modes, not all colours can be

displayed.  In the EGA mode, all colours can be displayed, but some

colours have the same intensity in bright 1 as in bright 0.  In VGA

mode, all colours closely resemble the original Spectrum colours, and

furthermore in this mode the screen updating is the fastest of all

modes.

The border updated every 1/50th of a second, so you cannot see the

familiar stripes when saving.  However, in real mode the emulator uses

the overscan of EGA to display the border, and you can see some stripes

there, and in VGA mode the border can be shown full-size.  The only

drawback of the border emulation in real mode is that there appears some

'snow' on the screen at each OUT - I don't know a way around this.

Finally, the sound emulation.  The Spectrum beeper is emulated by the

PC's internal beeper.  Because every 1/50th of a second the screen has

to be updated, and this takes a little time even if there are no

changes, the sound is a bit harsh.  If you select real mode, the

emulator won't update the screen anymore and the sound will sound

better.

The sound of the Spectrum 128's sound chip is played through the Adlib

card; if you haven't got such a card some notes are played through the

internal speaker.  That sound will be turned off, however, as soon as

the program makes a sound through the normal speaker of the Spectrum.

Some Spectrum 128 programs use the sound chip and the beeper at the same

time, and this won't work properly without an Adlib card.

2.4  Using the tape

This emulator can load programs that are saved to tape in the usual way,

but also speed-saved programs can be loaded.  Furthermore, you can also

make a disk file act as an 'emulated tape', so that the normal SAVE and

LOAD commands can be used to transfer data to and from disk easily.

Let's first discuss the saving and loading of programs using a tape

recorder - that'll be the first thing you want to do, to transfer your

programs to the PC.  First of all, you need an interface to connect the

tape recorder to the PC.  The parallel printer interface is used for

this.  All you need is a very simple and cheap piece of electronics to

get the input and output signals at the appropriate and safe levels; the

circuit diagram is in the program DIAGRAM.Z80.  The interface has to be

calibrated; a program to help doing this is contained in the snapshot

file.  Adjust the variable resistor so that when the tape recorder is

played at normal volume, the bar points just below 50%. When the tape

recorder is turned off, the bar should go to 0%.

You have to tell the emulator which LPT port you use for tape I/O.  This

can be selected in the tape menu, but it can also be specified on the

command line or in the Z80.INI file with the -b switch; for instance

-b2 selects LPT2.  Default is LPT1.

There are two ways to load programs: in 'real' or normal mode.  In real

mode, the emulator doesn't update the screen or scan the keyboard

anymore, so that the emulated Spectrum program can run smoothly.  The

emulator has to run at about 100%, but then you're able to load

everything a normal Spectrum would load, including speed-saved programs.

The only thing you see on screen are the loading bars in the border (on

EGA or VGA screens).  Real mode is selected by pressing F6. Saving

programs in real mode is a bit useless but it works; enter the SAVE

command, press a key to start saving and quickly press F6 when the

saving starts.  It will continue in real mode.

If your computer is just fast enough, don't slow the emulator down too

much.  Because the IN instruction is relatively slow, the emulator has

to run at about 110% for the best results.  If your computer is really

fast, you can best slow it down to exactly 100%.  If your computer is

just a bit too slow, you can try to make your tape recorder run slower

too (usually you can do this by adjusting a little screw at the back of

the motor), I successfully loaded several speed-saved programs at 90%.

In normal mode, the standard ROM loading and saving routines are

'trapped' (at addresses 04d8 and 056a) when they're about to start

saving or loading.  A routine in the emulator itself then takes over,

and loads or saves a block to tape or a disk file.  By default, this

routine uses the tape instead of a file, and I'll discuss that mode of

operation first.

Using these SAVE and LOAD routines has a great advantage as well as a

disadvantage compared to using the Spectrum's own routines in real mode.

The advantage is that the internal routines work on every machine, no

matter how slow or fast, without having to make the emulator run at

100%.  The disadvantage at using them is that they obviously won't

understand speed-saved files.  For normal use, these internal routines

work much easier, and real mode loading is only necessary for

speed-saved and very well protected programs.

So far for the general information about tape loading.

The emulator uses files with the extension .TAP to hold a piece of

'tape', with several blocks on it.  Each block is usually either a

header or a data block; a normal file thus consists of two blocks. There

are two modes of operation when loading and saving to disk files, single

and multiple .TAP file mode.

In single .TAP file mode, each block saved is appended to the end of the

.TAP file, like would happen if you were actually saving to tape. In the

same way, when loading in single file mode, each time the ROM wants to

load a block, it is presented the next block in the .TAP file. It is

handled as it would if the block was loaded from tape, that is, if the

ROM needs a header and is presented a data block, it will skip it.  The

header will however be considered to be read.  So, entering LOAD

"rubbish" will show all headers in the .TAP file, just as an ordinary

Spectrum would show all headers on the tape if you left the tape

running.

If the last block is loaded, the file pointer is moved to the start

again.  So a .TAP file can be considered to be an infinite tape. Single

.TAP file mode is useful to save whole programs to disk, or for

multi-load games that need to load in levels as you play.

A sort of 'random access' file management would also be useful, for

instance when you're developing a program and need to save several

pieces of data to disk and later load back a specific one.  This can be

done in single .TAP file mode (by positioning the file pointer using the

Browse function), but there's a different mode of operation that makes

things easier: multiple .TAP file mode.  In fact, by default the

emulator is in this mode.

When the emulator is in multiple .TAP file mode, it will read all blocks

from all .TAP files in a specified directory, one after the other.  When

it has finished reading the last one, it will start all over again.

When saving, the emulator will put the two blocks of a normal file, the

header and the data block, in one .TAP file with a unique name made up

of the printable letters of the file name and a two-digit number.  The

name of the .TAP file is irrelevant to the emulator, but to have it

resemble the name of the actual Spectrum file you saved is simply

convenient.  If the Spectrum program saves a data block to tape without

first saving a header, the .TAP file will contain only this data block,

and the DOS file name will be HDRLES, with a two-ditit number appended

to make it unique.  The format of the .TAP files saved in multiple .TAP

file mode is exactly the same as the format used in single .TAP file

mode.

You can easily string together .TAP files; for instance a number of .TAP

files created in multiple .TAP file mode can be put into one big .TAP

file simply by copying them together, e.g.

COPY /B FILE1.TAP + FILE2.TAP ALL.TAP.

(Note: in some versions of DR DOS the /B switch, necessary because

otherwise copying stops after a CTRL-Z character, doesn't work properly;

load your old COMMAND.COM to copy the files).

Now you know what you can do, but how to get the emulator to do it?

That's what the final section is about: the tape menu.

Press F7 to enter the tape menu.  Pressing S will select or de-select

single file mode.  By default, multiple .TAP file mode is selected.  In

this case, there are three other possible choices in this menu.  First

of all, D selects a tape-file directory where the .TAP files will be

saved into and loaded from.  A default directory can be selected by

putting the -xs switch on the command line or in the Z80.INI file; for

example -xs c:\spectrum\taps selects that directory.

The I and O options are used to select the source and destination of the

saving and loading: the LPT port for a physical tape recorder, or 'disk'

for disk files.  By default LPT1 is selected; another LPT port can be

selected with for instance -b2 or by pressing I and O.  Input and output

are directed to disk by default if a default tape file directory is

given by means of a switch on the command line or .INI file.

If Single .TAP file mode is selected, different and more menu options

appear.  With G and P, the input and output tape files can be selected.

They may be the same.  If a specified output file already exists, you

may choose to append to or overwrite this old file.  Saving is always at

the end of the file; loading always starts at the beginning of the .TAP

file.

With the B option - Browse - the position of the file pointer into the

input .TAP file can be changed.  If you, for instance, type LOAD""

instead of LOAD "" CODE, the first header is read, and you would have to

read all other headers before trying to load the file again.  With the

browse option you can conveniently change the file pointer.  Of every

header (that is, every block with flag byte 0 and length exactly 17) the

name and type, and of every data block the length is shown.

The option B can also be used to delete specific blocks from a .TAP

file.  Make sure you do not only delete a data block or a header, or the

ROM may get confused! (Double data blocks will be skipped, but double

headers can generate Tape Loading errors).

As in multiple .TAP file mode, I and O are used to specify the source

and destination for saving and loading.  If you enter a .TAP file name

with G or P, this will automatically be set correctly.  You can then

always reset the input or output back to LPTn again, of course.

Finally, in Single .TAP file mode you can use 'tape mirroring': loading

programs from tape (in normal mode, i.e.  not using Real mode) and at

the same time saving a copy of each block loaded into a .TAP file.  This

.TAP file can later be used to load the program again, might anything go

wrong.  There are two ways of mirroring: normal mirroring and exact

mirroring.  The last one must be used only in exceptional cases; it will

always make a copy of a block, even if it had a tape error (the

corresponding block in the .TAP file will also have a tape error).  This

causes ticks in leader tones to make 0-byte blocks, so the .TAP file may

get messy.  Do not use exact mirroring if you don't really have to; I

think normal mirroring will always work in practice.

If you try to leave the tape menu when for instance tape mirroring is

selected, and no output filename is given, the emulator will warn you

and will insist that the error be corrected.  Yes, it's stubborn!

2.5  Using the microdrive

Compared to the tape, this is really simple.  Cartridges are emulated by

files of 137923 bytes.  These files have the extension .MDR, and can

contain up to 126K of data.  The emulator emulates 8 microdrives, the

maximum amount the Interface I software can handle, and each of these

cartridge files can be inserted in any of the 8 microdrives.  (Do not

insert one file into more than one microdrive; this will cause problems

with the buffering done by the emulator as well as the Interface I, and

might result in data loss).

Press F8 to enter the microdrive menu.  Press 1 to 8 to select a

microdrive, and I to insert a microdrive cartridge.  You can select an

existing one, or type a new name.  If the cartridge file isn't found,

the emulator asks whether it should create it.  When created, you'll

have to format it first; if you don't, you'll get a 'microdrive not

present' error when you try to read it, just as happens with real

unformatted cartridges.  To format a cartridge, type

FORMAT "m";1;"name"

After this the cartridge should have 126K of free space.

The cartridge can be write protected; see the menu option in the F8

menu.  This is a characteristic of the cartridge, and the write protect

tab information is therefore stored in the cartridge file.

As on the real Spectrum, you'll have to be careful with OUT's if a

cartridge is inserted.  Try OUT 239,0 (on a real Spectrum, this turns on

the microdrive motor) and wait a few seconds; most of your data will be

lost!  You can stop the microdrive motor by typing STOP (or, more

generally, generate an error).

The microdrives are emulated at IN/OUT level.  This means that every

utility or program that uses microdrives ought to work on the emulator.

Most utilities use hook codes, and these will certainly work.

The GAP line is emulated; this signal is activated if the interface I

senses a piece of tape with no data on it.  If the checksum of the first

header block of a microdrive header or data block is not correct, that

block is considered to be a GAP.  This will only happen if some utility

writes a bad block to microdrive deliberately, if the file is newly

created and unformatted, or when you type OUT 239,0.

2.6  Using the RS232 channel

This was the only Spectrum i/o channel that could be used in the early

versions of the emulator.  Using .TAP files instead of the RS232 channel

is often easier, but sometimes using the RS232 channel can be very

useful too, for instance if you've got a null-modem lead that connects a

Spectrum with interface I to the PC you can use it to transfer data and

programs easily.  Furthermore, the RS232 channel is the easiest way to

let the emulator communicate with a PC printer.

The Interface I RS232 port is called the "B" or "T" channel.  The first

is the binary channel, the "T" channel won't let all control codes

through and will expand any keyword; useful for LISTing a program but

otherwise annoying.

The Spectrum 128 has its own RS232 port; it is called the "P" channel.

Output to either the Interface I's or Spectrum 128's own RS232 port will

all be processed as 'RS232 output', and input will go to both (that is,

to the one you happen to read from).

The output to the RS232 channel can be routed to an LPT port, to a COM

port or to a file on disk.  Input can come from either a file or a COM

port.

If you want to use the RS232 channel for printing using LPRINT and LLIST

(shorthand for PRINT #3 and LIST #3), be sure to open that channel for

output to RS232; by default it sends its output to the ZX Printer, which

is not supported.  You can open the channel by typing OPEN #3,"B" (or

"T" for listings, or "P" on a Spectrum 128).

Input and output are buffered.  This is important to remember when

you're transferring files using the SAVE and LOAD *"b" commands of the

Interface I.  If the header is missed, for instance if you try to load

the wrong file type, re-sending the file will not directly work because

there will still be bytes in the buffer.  You have to clear the input

buffer before re-sending the file.  When inputting from a disk file, the

file pointer can be reset to point to the start of the file again to

re-read the header.

When inputting or outputting from or to a disk file, the read or write

position is displayed as a byte-count.  An <EOF> sign will appear if an

input file is read completely through to the end.

The RS232 redirection options are in the Change Settings (F4) menu.  The

menu options are pretty obvious if you keep above remarks in mind, so I

won't go into that.

When using a COM port, make sure you have initialised it before starting

the emulator with the Dos MODE command, for instance

MODE com1:96,n,8,1

initialises COM1 to send and receive at 9600 baud, no parity, 8 data

bits and 1 stop bit, the default for the Interface I.

Here is how to transfer programs from a Spectrum to the PC using the

RS232 lead.  First, you need a null-modem lead.  I myself use the

following cable:

Spectrum                                  'AT'      'PC'

(9 pins)                                (9 pins)  (25 pins)

3 TxD   ������������������������� RxD     2         3

4 DSR   ������������������������� DTR     4        20

����� CTS     7         4

����� RTS     8         5

7 GND   ������������������������� GND     5         7

(so CTS and RTS have to be connected!) This is not a full null-modem

lead; you can only send data from the Spectrum to a PC.  Here's how to

transfer: load the program SAVESPEC.Z80 in the emulator and type the

basic program over into the real Spectrum, and run it.  It saves a short

piece of code to tape.

Now load the program you want to transfer, and stop it.  (This may be

tricky!)  Load the code back into memory at address 16384 (the code is

relocatable but this is the safest place):

LOAD "RS232" CODE 16384

Now open channel three for output to RS232; on a Spectrum with Interface

I this would be OPEN #3,"b", on a Spectrum 128 it would be OPEN #3,"p",

and with other interfaces you'll probably know what to do. Select the

right baud rate on the Spectrum (probably FORMAT "b",9600 or something

like that).  Now initialise the appropriate COM port on the PC and type

GETRS /n filename.z80           (n=COM port used)

at the DOS prompt, and then type RANDOMIZE USR 16384 to send the whole

memory over to the PC.  The resulting .Z80 file should now be exactly

49182 bytes long (that is 48K+30 bytes), if not try again or try a lower

baud-rate.  Voila, transferred!

To transfer short blocks of data it's often easier to use the LOAD *"b"

and SAVE *"b" commands of the Interface I.  When the right options have

been selected in the RS232 i/o redirection menu, you should just follow

the instructions of the Interface I user manual and all should work as

expected.

2.7  Joysticks

As was already said in the introduction, the emulated Spectrum joystick

(Cursor, Interface 2 or Kempston) is controlled by the PC cursor keys

and 5/0/.  on the numeric keypad and TAB as fire keys.  The emulated

joystick can also be controlled by a real joystick, both an analogue (PC

standard) or a digital one.

The analogue joystick support is rather straightforward.  If you've got

one, it works - it couldn't be simpler.  The digital joystick support is

less obvious, since PC's don't support these.

To use digital joysticks, Ruud Zandbergen has made a device that uses

the two inputs of a normal analogue joystickinterface to connect a

digital joystick to a PC.  Here's the circuit diagram:

15 pins male  (pc)                           9 pins male (joystick)

1+9 <���������������������������������������������������> 7 (5V)

��         ��          ��           ��

4 x 1 k� � � � Watt  � �          � �           � �

���         ���          ���           ���

3   <������������> 4 (up) �            �             �

�            �             �

6   <������������������������> 3 (dwn) �             �

�             �

13  <�������������������������������������> 1 (rght) �

�

11  <���������������������������������������������������> 2 (lft)

��������������������Ŀ

2   <�������������Ĵ    47 � � Watt     ����������������> 6 (fire)

����������������������

4+5+14 <������������������������������������������������> 8 (0V)

4+5+14 means: connect pins 4, 5 and 14.  The same applies for pins 1 and

9.  Here's the list of ingredients:

1 x 9 pins D plug, male

1 x 15 pins D plug, male

4 x 1k� , � Watt resistors

1 x 47 �, � Watt resistor

piece of 7-wire flatcable

Everything can be fit into the 15-pins plug.  Make sure the resistors

don't touch the other blank connections!  This interface can be used for

all usual digital joysticks, with or without auto fire (that is every

joystick that work with a Kempston joystick interface, or that work on a

Commodore 64/Amiga or Atari).  The joysticks for the Spectrum +2/+3 will

not work, however the pin layout is easy to change.

This joystickinterface needs an analogue PC-joystickinterface on which

you can connect TWO analogue joysticks (on one plug!).  Most cards can

do this, but some multi-I/O cards support only one joystick.  Check the

documentation of your I/O card to see whether your joystickinterface is

suitable.  The soundblaster joystick interface works fine.

A number of PC games will behave strange when the digital joystick

interface is connected; they run very slow or crash.  When this happens,

remove the joystick interface (not only the joystick!).

2.8  Transferring programs

There are a number of ways to transfer programs from the Spectrum to the

PC: loading them directly from tape, using the RS232 lead or

transferring from disks of Spectrum disk interfaces.  And then you might

have snapshot files from other emulators that you want to convert to

.Z80 files.  I'll discuss these cases one after the other.

Converting using the COM port is not so easy most of the times, but if

you've got a null-modem lead waiting to do something you could read

section 2.6.  Luckily, there are easier ways.

First of all, you can use the tape.  If you want to do this, then the

first thing to do is to read section 2.4 carefully - now you know almost

everything you need.  Most programs you have probably use the normal

tape format; you will find that these usually load right away. If the

programs use speed-load, using real mode will probably load most of

these right away too.

But some programs are really cleverly protected, and use obscure

features of the Z80 processor.  To run these programs, turn on LDIR

emulator and R register emulation (see the 'Change Settings' menu, F4).

Note that the emulator will slow down a bit when R register emulation is

selected; if you need to use real mode then make sure you speed the

emulator up again to 100%.  After the program has loaded successfully,

you may try to turn R register emulation off again; I don't know any

program that needs R register emulation after loading.  Read chapter 5

for more technical information about these options.

If you've got Spectrum disks, you will probably be able to convert the

programs on them to a useful format and use them in the emulator.  The

registered package of this emulator contains a program DISCIPLE, that

can read DISCiPLE and Plus D disks and convert the snapshots and other

files on it to .TAP and .Z80 files.  The previous version of this

program could only read 3.5'' Disciple disks, and had several bugs in

the file and snapshot translation routines.  So if you transferred

programs with the old DISCIPLE program and they don't work, don't blame

the emulator but try to transfer them again with the new program.

The current version of the DISCIPLE program reads 3.5'' as well as

5.25'' DISCiPLE disks, will translate 48K, 128K and screen snapshots,

and other normal files.  The previous version used the .SAV file format

for normal files, which could be loaded into the emulator using LOAD

*"b"; this version converts them into .TAP files which can be loaded

simply by using the normal tape LOAD statements (see 2.4).

The DISCiPLE interface modifies the Spectrum system variables in such a

way that LPRINT sends its output to DISCiPLE's own printer interface.

When you transfer a snapshot that uses the printer, you'll have to tell

it to use the Interface I's RS232 printer output instead, by breaking

the program and typing OPEN #3,"b".  If you don't, you'll get strange

results.

If you have got a Beta disk interface, your problem is solved too. J.L.

Bezemer wrote a program called BDDE that reads Beta disks.  The program

can be downloaded from the Spectrum emulator support BBS.

Finally, maybe you were using another Spectrum emulator for the PC

before using this one, and you may have already got a collection of

snapshot or other files.  CONVZ80, another utility for registered users,

can convert between several snapshot formats, namely the .SNA format of

JPP, the .SP formats of VGASPEC and SPECTRUM, the .PRG files of SpecEm,

and the .Z80 format of course.  (It is by the way not necessary to

convert .SNA files, the emulator can read them as they are.)  CONVZ80

can also convert the tape files used by SpecEm and ZX to .TAP files.

CONVZ80 recognizes what it should do by the extension of the files you

enter on the command line; to distinguish between VGASPEC's and

SPECTRUM's .SP formats you can use the switch -o.  If the extension

consists of digits only, it is taken to be a ZX tape file, and if it

contains non-digits and is none of .SP, .Z80, .SNA, .PRG or .TAP it is

regarded as a SpecEm tape file.

SpecEm can load .PRG snapshot files, but cannot save them.  However, it

emulates the Multiface I, which can save snapshots to tape.  SpecEm will

save these blocks as tape files to disk.  If you convert these to a .TAP

file (in the correct order!), you can load them into Z80 and save the

program as a .Z80 file.

2.9  Converting file formats - the utility CONVERT

This section is about the utility CONVERT, which can convert some of the

Spectrum's own format into each other, and also converts some of the

emulator's formats into others.  It is not about converting files from

other emulators; read section 2.8 if you want to know about that.

CONVERT was useful when the emulator could only communicate with

snapshot files and the RS232 link.  It has become less useful now, with

.TAP files, but it still has some useful features.

It can read three types of input files: pure ASCII, pure bytes (for

instance a .SCR screen dump), and files produced by a SAVE *"b" command.

Output is pure bytes, ASCII with either CR (Spectrum standard) or CR/LF

(PC standard) for line breaks, SAVE *"b" files containing a Basic or

code file, a .PCX or a .GIF file.

So what can you do? Main uses are adding LF (10 hex) bytes to a text

file produced by the Spectrum; converting a code block into a SAVE *"b"

to load it into the Spectrum using LOAD *"b" (and the reverse of course:

converting a SAVE *"b" file to pure bytes), and converting a screen dump

to .PCX or .GIF graphics files.

Less useful, but possible: LISTing a program (SAVE *"b" file) to produce

readable ASCII, and the reverse: converting an ASCII listing to

executable Basic again.

If you want to make a .PCX or a .GIF file, input should be a SAVE *"b"

file of a screen (length 6921 bytes exactly) or a bare .SCR screendump

(length 6912 bytes).  You can make screendumps by selecting the X-Extra

functions menu from the main menu.

2.10  The utilities Z802TAP and TAP2TAPE

The SamRam has built in it some snapshot software.  Using this software

you can save any 48K Spectrum program to tape or to a .TAP file, as is

explained in section 3.2 below.  But the SamRam software cannot handle a

128K program.

The utility that can convert a 128K snapshot (and 48K ones for that

matter) to a .TAP file is called Z802TAP.  The .TAP file includes a

basic loader, and a loading screen if you want.  Z802TAP compresses the

blocks it writes (using a better method than used in compressing .Z80

files) to save loading time.  If you don't want it to compress the

blocks, for instance when you want to take a look at the ram pages of

the Spectrum 128, specify -u when you run Z802TAP.  You can load the

converted program simply by executing

Z80 -ti tapefile

and typing LOAD "" (for a 48K program) or changing the hardware mode to

Spectrum 128 and choose 'Tape Loader' in the menu.

The program TAP2TAPE writes .TAP files back to tape.  The program

consists of a batch file TAP2TAPE.BAT, which executes the TAP2TAPE.Z80

file using the emulator.  The .TAP file is written to tape exactly as it

is, so that if a block contains a tape error, it won't load correctly

from tape either.  If the entire .TAP file has been saved the emulator

will start loading from tape.  At that point, press space once to return

to DOS.

3.  THE SAMRAM

3.1  Basic extensions

The SamRam is a hardware device Johan and I built for our Spectrums.  It

consists of a 32K static RAM chip which contains a modified copy of the

normal Basic ROM and a number of other useful routines, like a monitor

and snapshot software.  You can compare it to a Multiface I interface,

but it's more versatile.  Another useful feature was a simple hardware

switch which allowed use of the shadow 32K Ram, present at 8000-FFFF in

most Spectrums, but hardly ever actually used.

For more details on the low-level hardware features of the SamRam read

chapter 5.  In this chapter I'll explain the software features of the

SamRam software, somewhat bombastically called the 'SamRam 32 Software

System' or the 'Sam Operating System'.  By the way, all similarity

between existing computers is in fact purely coincidental and has in no

way been intended.  Really!

The SamRam offers a few new Basic commands, and a lot of useful routines

that are activated by an NMI, i.e.  by pressing F5.  First I'll discuss

the Basic extension.

Select the SamRam by starting the emulator with the -s switch, or by

selecting it from the F9 menu.  Normal Basic functions as usual; the

character set is different from the original one.  There are four new

commands: *RS, *MOVE, *SAVE and *SPECTRUM, and two new functions, DEC

and HEX, which have replaced ASN and ACS.  DEC takes a string argument

containing a hexadecimal number, and returns the decimal value of it.

HEX is the inverse of the DEC function, and yields a four-character

string.

*RS sends its arguments directly to the RS232 channel.  You don't have

to open a "b" or "t" channel first.  You're right, it's of limited use.

Example: *RS 13,10

*MOVE is useful: it moves a block of memory to another place.  Example:

*MOVE 50000,16384,6912 moves a screen-sized block from 50000 to the

start of the screen memory.

*SAVE works like *MOVE, except that it activates the shadow SamRam ROM

before moving.  I used this command to update the shadow ROM, but on the

emulator you can use it to move the shadow ROM to a convenient place in

Ram where you can take a look at it, for instance by executing

*SAVE 0,32768,16384.

*SPECTRUM resets the SamRam Spectrum to a normal one.  You lose all data

in memory.  By resetting the emulator by pressing ALT-F5, the SamRam is

activated again.  Not very useful either.

Then there's the Ramdisk, which is, like the Spectrum 128 ramdisk,

accessed via the SAVE!, LOAD!, CAT!, ERASE! and FORMAT!.  The syntax is

straightforward.  FORMAT! and CAT! need no parameters; ERASE! only needs

a name.  If a file is not found, the SamRam will respond with a 5-End of

File error.  The Ramdisk has a capacity of 25K.

3.2  The NMI software

Select the SamRam (F9-3), and press F5.  A menu with eight icons pops

up.  You can select each icon by moving the arrow to it (using the

cursor keys or the Kempston joystick), and pressing '0' or fire.  The

icons can also be selected by pressing the appropriate letter key.

The eight icons are two arrows with N and E within them, a magnifying

glass with the letters 'mc' in it (activated by pressing D), two screens

(identified by 1 and 2), a printer (P), a cassette (S) and a box saying

'overig'.  The 'D' activates the monitor or disassembler; read section

3.3 for information on this program.

Pressing N or E returns you to the Spectrum.  If you pressed N, the

normal Spectrum rom will be selected when the NMI software returns; if

you press E, the Rom with the Basic extensions will be selected.  Some

games may crash if they see a different rom than the standard Spectrum

one.

Pressing 1 selects the tiny screen editor.  You can move a '+' shaped

cursor about the screen using the cursor keys.  The following commands

are available:

H: Get the current ATTR color from the screen at the cursor's

current position, and store it in memory.  This color will be

used by the next command:

Z: Put the color on the screen

G: Get a character from the screen

P: Put the character on the screen

R: Remove all screen data that is invisible by the ATTR color

L: Take a look at the bitmap below the ATTR color codes

T: Return to the main menu.  You can also return by pressing

EDIT, or ESC in the emulator.

B: Change border color

V: Clear the whole screen

If you press 0, you can edit the current 8x8 character block at pixel

level.  Again you control the cursor with the cursor keys.  Now 0

toggles a pixel.  In this mode there are two commands: C clears the

whole block, and I inverts it.  Pressing EDIT (ESC) returns you to the

big screen again.

The SamRam has two screen buffers.  Buffer 1 is used to hold the screen

which was visible when you pressed NMI, to be able to restore it when

returning.  This is the screen you edit with '1'.  The second screen

buffer can be used to hold a screen for some time; it is not touched by

the NMI software directly, and will not even be destroyed by a Reset. If

you press '2', a menu appears with four Dutch entries:

1: Scherm 1 opslaan        (Store screen 1 into buffer 2)

2: Scherm 2 veranderen     (Edit screen 2)

3: Schermen verwisselen    (Swap screens)

4: Scherm 2 weghalen       (Remove screen 2)

These four functions are rather obvious, I believe.

Pressing 'P' pops up the printer menu.  The screendump program is

written specifically for my printer, a Star SG-10.  It will probably

work on some other printers, but not on most.  The output is sent to the

RS232 channel, so you have to redirect it to an LPT output.

Skipping the most interesting, 'S', for a moment, let's first discuss

the final menu, 'O' for 'Overig', Dutch for miscellaneous.  There are

five menu options, of which three are not useful.  The first gives a

directory of the cartridge currently in Microdrive 1.  The last, 'E',

returns you to Basic if this is anywhere possible: it resets some

crucial system variables and generates a Break into Program.  You can

use this for instance to break in a BEEP, or crack a not-so-very-well-

protected program.  The three other options select normal or speed-

save, and store the current setting in CMOS Ram.  Speed-save won't work

properly on the emulator, because the speed-save routine toggles the

upper 32K ram bank regularly, and this takes too much time on the

emulator.  The setting is not important if you use the internal save

routine (which will be used by default, unless you select Real Mode).

Finally, the 'S' option.  This option allows you to save a snapshot to

tape or microdrive.  I used it a lot on my real Spectrum, and it works

just as well on the emulator.  It is very useful is you want to load a

.Z80 program back into a real Spectrum again.  There are three

'switches' you can toggle.  The active choice is indicated by a bright

green box, inactive boxes are non-bright.  You have to use EGA or VGA to

be able to see it...  The first switch lets you select whether the

SamRam rom should be active if the program loads or not.  This is only

meaningful is you load it back in a SamRam again.  Usually I want the

SamRam rom to be active because I like the character set better.  The

second switch indicates whether the SamRam should save a 'loading

screen', which it takes from screen buffer 2.  If screen buffer 2

contains a screen, this switch will by default be on.  Finally, the last

switch lets you select the output media, tape or cartridge.

If the program is loaded back into the SamRam, the only bytes that have

been corrupted are four bytes down on the stack; this will virtually

never be any problem.  If the program is loaded back to a normal

Spectrum, these four bytes will also be corrupted, and the bottom two

pixel lines of the screen will be filled with data.  (This is

considerably less than any other snapshotter I've seen: for instance the

Multiface I uses more than 35% of the screen!)

The Microdrive BASIC loader needs code in the SamRam rom to start the

program (the RANDOMIZE USR 43 calls it).  It won't be very difficult to

write a standard BASIC loader that doesn't need this code, but I don't

think many people desperately need it...

3.3  The built-in monitor

This is a really very convenient part of the emulator, and I use it a

lot.  It is very MONS-like in its commands and visual appearance.  It

cannot single-step however, but on the positive side it has some

features MONS hasn't.  It is a part of the SamRam, and cannot therefore

be used with Spectrum 128 programs.  If you want to take a look at a

Spectrum 128 program, press F10, then change the hardware to SamRam

without resetting, and finally generate an NMI in the Extra Functions

menu.  You won't probably be able to continue to run the program, but at

least you're able to see what it was doing.

Press F5 for NMI, and D to enter the monitor/disassembler.  The first

eight lines are the first eight instructions, starting at the Memory

Pointer, from here on abbreviated by MP.  At first, MP is zero.  The

disassembler knows all official instructions, and the SLL instruction.

If another inofficial instruction (i.e.  starting with DD, FD or ED) is

encountered, the first byte is displayed on a blank line.  The four

lines below these display the value of PC and SP, the first nine words

on the stack (including AF and the program counter, which have been

pushed during NMI), and three MP-memories.  These can be used for

temporary storage of the MP, for instance when you take a look at the

body of a CALL, and want to return to the main procedure later.

The bottom part of the screen displays 24 bytes around the memory

pointer.

Commands are one letter long; no ENTER needs to be given.  If one or

more operands are needed, a colon will appear.  By default the monitor

accepts hexadecimal input.  A leading $ denotes that the number is to be

regarded as decimal.  If you give the # command, the default will toggle

to decimal, and you need to explicitly put a # in front of a number

which is to be interpreted as a hex number.  Also, after the # command

all addresses on screen will be decimal.  A single character preceded by

the " symbol evaluates to its ASCII code, and the single character M

will evaluate to the current value of the memory pointer.

The monitor commands:

Q: Decrease the memory pointer by one.  You effectively shift one

byte up.

A: Increase the memory pointer, shifting one byte down.

ENTER: Shift one instruction down: the memory pointer is

increased by the length of first instruction displayed on

screen.

M: Change the value of the memory pointer.  For instance, M:M

won't change it.

P: Put.  The word operand supplied will be stored in the first MP

memory, and the others will shift on place to the right.

Usually, you'll want to store the memory pointer by P:M

G: Get.  Typing G:1, G:2 or G:3 moves the value of one of the MP

memories to the MP.

B: Byte.  This command needs a byte operand; it will be poked

into memory, and the memory pointer will move one up.

I: Insert.  The same as B, except that you can poke more than one

byte.  It continues to ask for bytes to poke until you type

Enter on a blank line.

#: Toggles the default number base between hexadecimal and

decimal.

F: Find.  You can enter up to ten bytes, which will be searched

through memory.  Searching will stop at address 0, because

since the search string is stored in shadow Ram, searching

would otherwise not always terminate.  Typing Enter on a blank

line starts the search.  Byte operands are entered as usual,

but:

- If a number bigger than 256 decimal is entered, it is

treated as a word in the standard LSB/MSB format.  So, 1234

will search for 34,12 hex in that order.  Note that 0012

will search for 12, not 12,00.

- A line starting with " decodes into the string of characters

(up to ten) behind it.  Normally this would only be the

first character.  So instead of typing "M "Y "N "A "M "E

(space=enter here) you type "MYNAME.  Note that any

terminating " will also be searched for!

- An x is treated as a wildcard.  So if you search for CD x 80

any call to a subroutine in the block 8000-80FF is a hit.

If you search for x 8000, you'll see every one-byte

instruction that has the address 8000 as operand.

N: Continues the search started by F from the current MP.

$: Displays one page of disassembly on screen.  In this mode,

the following commands are possible:

$: Back to the main screen

7: [Shift 7 also works, cursor up]: Go to the previous page.

The monitor stores the addresses of the previous eight

pages only.

Q: Go back one byte (decrease MP by one)

A: Go one byte forward (increase MP by one)

Z: Dump this screen to the printer, in ASCII format.  Redirect

the RS232 output to a file, and run CONVERT on it to convert

the CR's into CR/LF's before printing (or tell your printer

to do the conversion).

Every other key displays the next page of disassembly.

K: List.  The same mode as with $ is entered, but instead of a

disassembly the bytes with their ASCII characters are

displayed.  Useful to look for text.

C: Clear.  Fills blocks of memory with a specified value.  The

monitor prompts with 'First', 'Last' and 'With'.  The 'Last'

address is inclusive!

D: Dump.  Prompts with 'First' and 'Last', and dumps a

disassembly of the block between these addresses to the

printer.  See remark at $-Z.  The 'Last' address is again

inclusive.

R: Registers.  If you press Enter after R, an overview of the

registers contents is displayed.  If you type one of A,B,C,D,

E,H,L,A',B',C',D',E',H',L',I,R,AF,BC,DE,HL,AF',BC',DE',HL',

IX,IY,SP or PC, you can change the value of it.  Changing the

value of SP also changes the PC and AF values by the way.  You

cannot change the Interrupt mode or IFF.

V: Verplaats.  (Move).  Prompts with 'From', 'To' and 'Length'.

Obvious.

S: Save.  Enter the start of the block you wish to save first.

The monitor then prompts with 'Length'.  The block is saved

without a header, as a normal data block (A, the flagbyte, is

0FF)

L: Load.  Loads a block of data from tape, at the specified

address.  Normal data blocks, headers and blocks with non-

standard flag bytes can be loaded.  The first byte in memory

will contain the flag byte.  If the checksum isn't 0 after

loading, indicating a tape error, you'll hear a beep.

H: Header read.  Loads headers and displays the contents on

screen.

As you're reading this part, I assume you know something of machine

code.  Probably you would be interested in peeking into the software of

the SamRam, the Interface I or the Spectrum 128.  You'll first have to

move these roms in ram to be able to look at them with the monitor.

The Interface I rom can be moved into ram by saving it to microdrive or

to the "b" channel, with SAVE *"m";1;"rom" CODE 0,8192 or SAVE *"b" CODE

0,8192, and loading it back again at 32768 for instance.  You can also

put this small machine code routine at 23296 and run it: F3 21 0C 5B E5

21 00 00 E5 C3 08 00 21 00 00 11 00 80 01 00 20 ED B0 FB C3 00 07.

The two SamRam roms are easy.  The first you don't need to transfer; the

monitor looks at the extended basic rom by default.  The second rom can

be moved to 32768 by typing *SAVE 0,32768,16384.  (The SAVE is not the

keyword SAVE!)

The first '128 rom, the one which is active at reset and contains most

of the new code, is moved up by typing SAVE!"rom"CODE 0,16384, then

LOAD!"rom"CODE 32768.  The other rom is most conveniently moved by

saving it to a .TAP file and loading it back again in ram.  To select

the SamRam type SPECTRUM first, and then switch the hardware without

resetting.

4.  THE SPECTRUM

4.1  The Spectrum

This emulator supports the Interface I and the Spectrum 128.  Many

Spectrum users will have no experience with them, so some comments may

be useful.  On the other hand, I don't think this is the right place to

describe the Spectrum Basic in full detail.  If you want to know it all,

read the official manuals!

If you want to use Spectrum Basic, you will need the keywords.  You

could by the way now also use the Spectrum 128 Basic where you can type

the keywords in by full.

If you press ALT-F1 in the emulator, the Spectrum keyboard layout will

appear.  For completeness I include an alphabetical list of all keywords

and their key-combination.  In the list below, K stands for Keyword

mode, E for E-mode (type Shift-Alt of Shift-Ctrl to select E-mode), S

for Symbol Shift, and SE for Symbol Shifted (Alt/Ctrl) E-mode: select E

mode and type the letter while depressing Symbol Shift.

Keyw.    Code | Keyw.    Code | Keyw.    Code | Keyw.    Code

ABS      E g    DRAW     K w    MERGE    SE t   SAVE     K s

ACS      SE w   ERASE    SE 7   MOVE     SE 6   SCREEN$  SE k

AND      S y    EXP      E x    NEW      K a    SGN      E f

ASN      SE q   FLASH    SE v   NEXT     K n    SIN      E q

AT       S i    FN       SE 2   NOT      S s    SQR      E h

ATN      SE e   FOR      K f    OPEN #   SE 4   STEP     S d

ATTR     SE l   FORMAT   SE 0   OR       S u    STOP     S a

BEEP     SE z   GO SUB   K h    OUT      SE o   STR$     E y

BIN      E b    GO TO    K g    OVER     SE n   TAB      E p

BORDER   K b    IF       K u    PAPER    SE c   TAN      E e

BRIGHT   SE b   IN       SE i   PAUSE    K m    THEN     S g

CAT      SE 9   INK      SE x   PEEK     E o    TO       S f

CHR$     E u    INKEY$   E n    PI       E m    USR      E l

CIRCLE   SE h   INPUT    K i    PLOT     K q    VAL      E j

CLEAR    K x    INT      E r    POINT    SE 8   VAL$     SE j

CLOSE #  SE 5   INVERSE  SE m   POKE     K o    VERIFY   SE r

CLS      K v    LEN      E k    PRINT    K p    <=       S q

CODE     E i    LET      K l    RANDOMIZE K t   >=       S e

CONTINUE K c    LIST     K k    READ     E a    <>       S w

COPY     K z    LINE     SE 3   REM      K e

COS      E w    LLIST    E v    RESTORE  E s    DEC      SE q

DATA     E d    LN       E z    RETURN   K y    HEX      SE w

DEF FN   SE 1   LOAD     K j    RND      E t

DIM      K d    LPRINT   E c    RUN      K r

Character  Spectrum kbrd   On PC keyboard

&          S 6             ALT (or CTRL) 6

'          S 7             ALT 7 or '/"

(          S 8             ALT 8

)          S 9             ALT 9

_          S 0             ALT 0 or SHFT _/-

<          S r             ALT r or SHFT </,

>          S t             ALT t or SHFT >/,

;          S o             ALT o or :/;

"          S p             ALT p or SHFT "/'

^          S h             ALT h

-          S j             ALT j or _/-

+          S k             ALT k or SHFT +/= or GREY +

=          S l             ALT l or +/=

:          S z             ALT z or SFHT :/;

?          S c             ALT c or SHFT ?//

/          S v             ALT v or ?//

*          S b             ALT b or GREY PRTSC/*

,          S n             ALT n or </,

.          S m             ALT m or >/.

4.2   The Interface I

If you want to use the microdrive, you'll need cartridge files.  The

emulator can create an empty cartridge file for you.  You have to format

it before you can use it.  Type

FORMAT "m";1;"name"

to format the cartridge currently in Microdrive 1 giving it the name

'name'.  Next, type CAT 1 to get a catalogue of the files on it (none of

course) and the number of kilobytes free.  You can save a file by typing

for instance

SAVE *"m";1;"screen"SCREEN$

Instead of SCREEN$ you can use all other expressions that are permitted

also when saving to tape, like LINE nnnn or CODE x,y etcetera.  To load

a file back from cartridge, you type (you guessed it)

LOAD *"m";1;"screen"SCREEN$

If the file doesn't exist or is of the wrong type you'll get the

appropriate error message.  To erase a file, type for instance

ERASE "m";1;"screen"

Note that no * is needed (or even permitted), and that only the name

should be given.  There's another way to create a file on a cartridge,

and that is by using a command like OPEN #3;"m";1;"name", and printing

to that stream.  You can use MOVE to move data from stream to stream,

but I'll not go into that --- it's not very much used anyway.

Instead of to the microdrive, you can also 'save to the RS232 link'. For

instance, type SAVE *"b"SCREEN$ (note: there's no name!) to save a

screen.  On the emulator you can send the output to the RS232 channel to

a printer (then SAVE *"b" is useless), to a file (can be useful) or to

the COM port (very useful if you connect a real Spectrum to the PC's COM

port!).  You can load the data back by typing LOAD *"b"SCREEN$ and

making sure the RS232 channel is fed with the right input (from a COM

port or a file).  See also paragraph 2.6.

If you want to use the RS232 channel for printing, open stream 3 for

output to that channel by typing

OPEN #3,"b"

or

OPEN #3,"t"

The first will simply copy everything you send to stream 3 (using for

instance LPRINT or LLIST) to the RS232 channel; the second converts CR's

into CR/LF's, breaks off lines at 80 characters and translates keywords

into character sequences.  "t" is useful for LLISTings, but not for

anything else.

Useful extra commands: CLS #, to clear the screen and reset the

attributes to their reset defaults, and CLEAR # to do a CLS # and close

all currently open streams (discarding all data that may still be

buffered!)

The Interface I uses its own system variables.  At the first error

message you make (or RASP, or flashing question mark) and at the first

Interface I statement you execute, it inserts them automatically.  Some

programs will not run when the Interface I has inserted its system

variables.  So if you load a game from tape, reset the Spectrum first

and don't make an error typing LOAD "".  With a bit of exercise you

should be able to do this.

4.3  The Spectrum 128

The main new features of the Spectrum 128 are its larger memory, that

can be used as a Ram drive in Basic, and music capabilities.

The Ram drive is accessed via the LOAD!, SAVE!, ERASE! and CAT!

commands.  They work as you would expect.  Examples:

SAVE !"name"SCREEN$

CAT!

LOAD !"name"SCREEN$

ERASE !"name"

The 3 channel sound chip of the Spectrum 128 can be used in Basic with

the PLAY command.  Example:

PLAY "cde","efg","gAB"

plays three chords.  You can program complex effects, melodies and

rhythms with the play command; they require many commands in the three

voice strings which I won't explain...  They are explained in the

Spectrum 128's user guide.

5.  TECHNICAL INFORMATION

5.1  The Spectrum

The Spectrum is at the hardware level a very simple machine.  There's

the 16K ROM which occupies the lowest part of the address space, and 48K

of RAM which fills up the rest.  An ULA which reads the lowest 6912

bytes of RAM to display the screen, and contains the logic for just one

I/O port completes the machine, from a software point of view at least.

Every even I/O address will address the ULA, but to avoid problems with

other I/O devices only port FE should be used.  If this port is written

to, bits have the following meaning:

Bit   7   6   5   4   3   2   1   0

�������������������������������Ŀ

�   �   �   � E � M �   Border  �

���������������������������������

The lowest three bits specify the border colour; a zero in bit 3

activates the MIC output, and a one in bit 4 activates the EAR output

(which sounds the internal speaker).  The real Spectrum also activates

the MIC when the ear is written to; the emulator doesn't.  This is no

problem; MIC is only used for saving, and when saving the Spectrum never

sounds the internal speaker.  The upper three bits are unused.

If port FE is read from, the highest eight address lines are important

too.  A zero on one of these lines selects a particular half-row of five

keys:

IN:    Reads keys (bit 0 to bit 4 inclusive)

#FEFE  SHIFT, Z, X, C, V            #EFFE  0, 9, 8, 7, 6

#FDFE  A, S, D, F, G                #DFFE  P, O, I, U, Y

#FBFE  Q, W, E, R, T                #BFFE  ENTER, L, K, J, H

#F7FE  1, 2, 3, 4, 5                #7FFE  SPACE, SYM SHFT, M, N,

A zero in one of the five lowest bits means that the corresponding key

is being pressed.  If more than one address line is made low, the result

is the logical AND of all single inputs, so a zero in a bit means that

at least one of the appropriate keys is pressed.  For example, only if

each of the five lowest bits of the result from reading from port 00FE

(for instance by XOR A/IN A,(FE)) is one, no key is pressed.

A final remark about the keyboard.  It is connected in a matrix-like

fashion, with 8 rows of 5 columns, as is obvious from the above remarks.

Any two keys pressed simultaneously can be uniquely decoded by reading

from the IN ports, however, if more than two keys are pressed decoding

may not be uniquely possible.  For instance, if you press Caps shift, B

and V, the Spectrum will think also the Space key is pressed, and react

by giving the 'Break into Program' report.  This matrix behaviour is

also emulated - without it, Zynaps for instance won't pause when you

press 5,6,7,8 and 0 simultaneously.

Bit 5 (value 64) of IN-port FE is the ear input bit.  When the line is

silent, its value is zero, except in the early Model 2 of the Spectrum,

where it was one.  When there is a signal, this bit toggles.  The

Spectrum loading software is not sensitive to the polarity of this bit

(which it definitely should not be, not only because of this model

difference, but also because you cannot be sure the tape recorder

doesn't change the polarity of the signal recorded!) Some old programs

rely on the fact that bit 5 is always one (for instance Spinads); for

these programs the emulator can mimic a Model 2 Spectrum.

Bits 6 and 7 are always one.

The ULA with the lower 16K of RAM, and the processor with the upper 32K

RAM and 16K ROM are working independently of each other.  The data and

address buses of the Z80 and the ULA are connected by small resistors;

normally, these do effectively decouple the buses.  However, if the Z80

wants to read of write the lower 16K, the ULA halts the processor if it

is busy reading, and after it's finished it lets the processor access

lower memory through the resistors.  A very fast, cheap and neat design

indeed!

If you run a program in the lower 16K of RAM, or read or write in that

memory, the processor is halted sometimes.  This part of memory is

therefore somewhat slower than the upper 32K block.  This is also the

reason that you cannot write a sound- or save-routine in lower memory;

the timing won't be exact, and the music will sound harsh.  Also, INning

from port FE will halt the processor, because the ULA has to supply the

result.  Therefore, INning from port FE is a tiny bit slower on average

than INning from other ports; whilst normally an IN A,(nn) instruction

would take 11 T states, it takes 12.15 T states on average if nn=FE. See

below for more exact information.

If the processor reads from a non-existing IN port, for instance FF, the

ULA won't stop, but nothing will put anything on the data bus.

Therefore, you'll read a mixture of FF's (idle bus), and screen and ATTR

data bytes (the latter being very scarce, by the way).  This will only

happen when the ULA is reading the screen memory, about 60% of the

1/50th second time slice in which a frame is generated.  The other 40%

the ULA is building the border or generating a vertical retrace.  This

behaviour is actually used in some program, for instance by Arkanoid,

and the emulator also emulates this behaviour.

Finally, there is an interesting bug in the ULA which also has to do

with this split bus.  After each instruction fetch cycle of the

processor, the processor puts the I-R register 'pair' (not the 8 bit

internal Instruction Register, but the Interrupt and R registers) on the

address bus.  The lowest 7 bits, the R register, are used for memory

refresh.  However, the ULA gets confused if I is in the range 64-127,

because it thinks the processor wants to read from lower 16K ram very,

very often.  The ULA can't cope with this read-frequency, and regularly

misses a screen byte.  Instead of the actual byte, the byte previously

read is used to build up the video signal.  The screen seems to be

filled with 'snow'; however, the Spectrum won't crash, and program will

continue to run normally.  There's one program I know of that uses this

to generate a nice effect: Vectron.  (which has very nice music too by

the way).  This effect has not been implemented however - it's a bit

useless (but maybe I'll include it in the future).

The processor has three interrupt modes, selected by the instructions IM

0, IM 1 and IM 2.  In mode 1, the processor simply executes a RST #38

instruction if an interrupt is requested.  This is the mode the Spectrum

is normally in.  The other mode that is commonly used is IM 2.  If an

interrupt is requested, the processor first builds a 16 bit address by

combining the I register (as the high byte) with whatever the

interrupting device places on the data bus.  The word at this address is

then called.  Rodnay Zaks in his book 'Programming the Z80' states that

only even bytes are allowed as low index byte, but that isn't true.  The

normal Spectrum contains no hardware to place a byte on the bus, and the

bus will therefore always read FF (because the ULA also doesn't read the

screen if it generates an interrupt), so the resulting index address is

256*I+0FF.  However, some not-so-neat hardware devices put things on the

data bus when they shouldn't, so later programs didn't assume the low

index byte was 0FF.  These programs contain a 257 byte table of equal

bytes starting at 256*I, and the interrupt routine is placed at an

address that is a multiple of 257.  A useful but not so much used trick

is to make the table contain FF's (or use the ROM for this) and put a

byte 18 hex, the opcode for JR, at FFFF.  The first byte of the ROM is a

DI, F3 hex, so the JR will jump to FFF4, where a long JP to the actual

interrupt routine is put.

In interrupt mode 0, the processor executes the instruction that the

interrupting device places on the data bus.  On a standard Spectrum this

will be the byte FF, coincidentally (...) the opcode for RST #38. But

for the same reasons as above, this is not really reliable.

The 50 Hz interrupt is synchronized with the video signal generation by

the ULA; both the interrupt and the video signal are generated by it.

Many programs use the interrupt to synchronize with the frame cycle.

Some use it to generate fantastic effects, such as full-screen

characters, full-screen horizon (Aquaplane) or pixel colour (Uridium for

instance).  Very many modern programs use the fact that the screen is

'written' (or 'fired') to the CRT in a finite time to do as much

time-consuming screen calculations as possible without causing character

flickering:  although the ULA has started displaying the screen for this

frame already, the electron beam will for a moment not 'pass'

this-or-that part of the screen so it's safe to change something there.

So the exact time in the 1/50 second time-slice at which the screen is

updated is very important.  Because the emulator updates the screen at

once, no single best solution can be given, and therefore the user can

select one of three possibilities (low, normal or high video

synchronisation, corresponding to a screen update after 1/200, 2/200 or

3/200 of a (relative) second after a Z80 interrupt) which gives the best

results.  Try for instance Zynaps; with normal video synchronisation the

top four or five lines of the background move out-of-phase with the

rest, and your space-ship flickers in that region.  With low video

synchronisation the background moves smoothly but the sprites flicker in

all parts of the screen.  Only with high video sync everything moves

smoothly and doesn't flicker.

This emulator does not try to emulate the really time-critical border

pattern effects (except when loading, but the width of the loading

stripes are not quite right because also PC video timings come into

play), but maybe I'll include it in the future.  I will need some hard

data on video timings then, and I've figured these out recently.  Here

they are.

Each line takes exactly 224 T states.  After an interrupt occurs, 64

line times pass before the byte 16384 is displayed.  At least the last

48 of these are actual border-lines.  I could not determine whether my

monitor didn't display the others or whether it was in vertical retrace,

but luckily that's not really important.  Then the 192 screen+border

lines are displayed, followed by about 56 border lines again.  56.5

border lines would make up exactly 70000 T states, 1/50th of 3500000.

However, I noticed that the frequency of the 50 Hz interrupt (measured

in 1/T states!) changes very slightly when my Spectrum gets hot (I think

it has something to do with the relative change of the frequencies of

the two crystals in the Spectrum), so the time between interrupts will

probably not be exactly 70000 T states. Anyway, whether the final border

block is of fixed or variable length doesn't concern us either, the

timings of the start and end of the screen, which are the timings of

real interest, are fixed.

Now for the timings of each line itself.  I define a screen line to

start with 256 screen pixels, then border, then horizontal retrace, and

then border again.  All this takes 224 T states.  Every half T state a

pixel is written to the CRT, so if the ULA is reading bytes it does so

each 4 T states (and then it reads two: a screen and an ATTR byte).  The

border is 48 pixels wide at each side.  A video screen line is therefore

timed as follows: 128 T states of screen, 24 T states of right border,

48 T states of horizontal retrace and 24 T states of left border.

When an interrupt occurs, the running instruction has to be completed

first.  So the start of the interrupt is fixed relative to the start of

the frame up to the length of the last instruction in T states.  If the

processor was executing a HALT (which, according to the Z80 books I

read, is effectively many NOPs), the interrupt routine starts at most 3

T states away from the start of the frame.  Of course the processor also

needs some T states to store the program counter on the stack, read the

interrupt vector and jump to the routine, but since I cannot determine

that by only using the Spectrum, it is useless information by that very

reason alone!

Now when to OUT to the border to change it at the place you want? First

of all, you cannot change the border within a 'byte', an 8-pixel chunk.

If we forget about the screen for a moment, if you OUT to port FE after

14326 to 14329 T states (including the OUT) from the start of the IM 2

interrupt routine, the border will change at exactly the position of

byte 16384 of the screen.  The other positions can be computed by

remembering that 8 pixels take 4 T states, and a line takes 224 T

states.  You would think that OUTing after 14322 to 14325 T states, the

border would change at 8 pixels left of the upper left corner of the

screen.  This is right for 14322, 14323 and 14324 T states, but if you

wait 14325 T states the ULA happens to be reading byte 16384 (or 22528,

or both) and will halt the processor for a while, thereby making you

miss the 8 pixels.  This exception happens again after 224 T states, and

again after 448, an so forth.  These 192 exceptions left of the actual

screen rectangle are the only ones; similar things don't happen at the

right edge because the ULA don't need to read things there - it has just

finished!

As noted above, reading or writing in low ram (or OUTing to the ULA)

causes the ULA to halt the processor.  When and how much? The processor

is halted each time you want to access the ULA or low memory and the ULA

is busy reading.  Of the 312.5 'lines' the ULA generates, only 192

contain actual screen pixels, and the ULA will only read bytes during

128 of the 224 T states of each screen line.  But if it does, the

processor is halted for exactly 4 T states.

5.2  The Interface I

The Interface I is quite complicated.  It uses three different I/O

ports, and contains logic to page and unpage an 8K ROM if new commands

are used.  I won't be very detailed here; you could refer to the source

code of the emulator if you want to know some details, or read the

'Spectrum Shadow ROM Disassembly' by Gianlura Carri, published by

Melbourne House - but don't expect the same level of detail as of Ian

Logan and Frank O'Hara in their Rom disassembly book.

The ROM is paged if the processor executes the instruction at ROM

address 0008 or 1708 hexadecimal, the error and close# routines.  It is

inactivated when the Z80 executes the RET at address 0700.

I/O Port E7 is used to send or receive data to and from the microdrive.

Accessing this port will halt the Z80 until the Interface I has

collected 8 bits from the microdrive head; therefore, it the microdrive

motor isn't running, or there is no formatted cartridge in the

microdrive, the Spectrum hangs.  This is the famous 'IN 0 crash'.

Port EF is used for several things:

Bit    7   6    5    4    3    2    1     0

���������������������������������������Ŀ

READ�   �   �    �busy� dtr �gap� sync�write�

�   �   �    �    �     �   �     �prot.�

���������������������������������������Ĵ

WRITE�   �   �wait� cts�erase�r/w�comms�comms�

�   �   �    �    �     �   � clk � data�

�����������������������������������������

Bits DTR and CTS are used by the RS232 interface.  The WAIT bit is used

by the Network to synchronise, GAP, SYNC, WR_PROT, ERASE, R/_W, COMMS

CLK and COMMS DATA are used by the microdrive system.  If the microdrive

is not being used, the COMMS DATA output selects the function of bit 0

of out-port F7:

Bit      7    6   5   4   3   2   1       0

������������������������������������������Ŀ

READ�txdata�   �   �   �   �   �   �    net    �

�      �   �   �   �   �   �   �   input   �

������������������������������������������Ĵ

WRITE�      �   �   �   �   �   �   �net output/�

�      �   �   �   �   �   �   �   rxdata  �

��������������������������������������������

TXDATA and RXDATA are the input and output of the RS232 port.  COMMS

DATA determines whether bit 0 of F7 is output for the RS232 or the

network.

5.3  The SamRam

The SamRam contains a 32K static CMOS Ram chip, and some I/O logic for

port 31.  If this port is read, it returns the position of the joystick,

as a normal Kempston joystickinterface would.  If written to, the port

controls a programmable latch chip (the 74LS259) which contains 8

latches:

Bit    7   6   5   4   3   2   1   0

�������������������������������Ŀ

WRITE�   �   �   �   �  address  �bit�

���������������������������������

The address selects on of the eight latches; bit 0 is the new state of

the latch.  The 16 different possibilities are collected in the diagram

below:

OUT 31,   �  Latch  � Result

�������������������������������������������������������������

0     �    0    � Switch on write protect of CMOS RAM

1     �    "    � Writes to CMOS RAM allowed

2     �    1    � turn on CMOS RAM (see also 6/7)

3     �    "    � turn off CMOS RAM (standard Spec.  ROM)

4     �    2    � -

5     �    "    � Ignore all OUT's to 31 hereafter

6     �    3    � Select CMOS bank 0 (Basic ROM)

7     �    "    � Select CMOS bank 1 (Monitor,...)

8     �    4    � Select interface 1

9     �    "    � Turn off IF 1 (IF1 rom won't be paged)

10     �    5    � Select 32K ram bank 0 (32768-65535)

11     �    "    � Select 32K ram bank 1 (32768-65535)

12     �    6    � Turn off beeper

13     �    "    � Turn on beeper

14     �    7    � -

15     �    "    � -

At reset, all latches are 0.  If an OUT 31,5 is issued, only a reset

will give you control over the latches again.  The write protect latch

is not emulated; you're never able to write the emulated CMOS ram in the

emulator.  Latch 4 will pull up the M1 output of the Z80.  The Interface

I won't page the ROM anymore then.

5.4  The Z80 microprocessor

The Z80 processor is quite straightforward, and contains to my knowledge

no interesting bugs or quirks.  However, it has some undocumented

features.  Some of these are quite useful, and some are not, but since

many programs use the useful ones, and a few programs use the weird

ones, I tried to figure them out and emulate them as best as I could.

There is a Z80 emulator around, intended as a CP/M emulator, which halts

the program if an undocumented opcode is encountered.  I don't think

this makes sense.  ZiLOG doesn't dictate the law, the programs which use

the processor's features do!

Most Z80 opcodes are one byte long, not counting a possible byte or word

operand.  The four opcodes CB, DD, ED and FD are shift opcodes: they

change the meaning of the opcode following them.

There are 248 different CB opcodes.  The block CB 30 to CB 37 is missing

from the official list.  These instructions, usually denoted by the

mnemonic SLL, Shift Left Logical, shift left the operand and make bit 0

always one.  Bounder and Enduro Racer use them.  The SamRam monitor can

disassemble these and uses the mnemonic SLL.  These instructions are

quite commonly used.

The DD and FD opcodes precede instructions using the IX and IY

registers.  If you look at the instructions carefully, you see how they

work:

2A nn           LD HL,(nn)

DD 2A nn        LD IX,(nn)

7E              LD A,(HL)

DD 7E d         LD A,(IX+d)

A DD opcode simply changes the meaning of HL in the next instruction.

If a memory byte is addressed indirectly via HL, as in the second

example, a displacement byte is added.  Otherwise the instruction simply

acts on IX instead of HL.  (A notational awkwardness, that will only

bother assembler and disassembler writers: JP (HL) is not indirect; it

should have been denoted by JP HL) If a DD opcode precedes an

instruction that doesn't use the HL register pair at all, the

instruction is executed as usual.  However, if the instruction uses the

H or L register, it will now use the high or low halves of the IX

register! Example:

44              LD B,H

FD 44           LD B,IYh

These types of inofficial instructions are used by very many programs.

By the way, many DD or FD opcodes after each other will effectively be

NOPs, doing nothing except repeatedly setting the flag 'treat HL as IX'

(or IY) and taking up 4 T states.  (But try to let MONS disassemble such

a block.)

I've never seen a program using inofficial ED instructions, and except

for ED 6B nn, a long version of 2A nn, LD HL,(nn) I don't know any.  I

am pretty sure however that they exist, but I never took the trouble to

test them all.

About the R register.  This is not really an undocumented feature,

although I have never seen any thorough description of it anywhere.  The

R register is a counter that is updated every instruction, where DD, FD,

ED and CB are to be regarded as separate instructions.  So shifted

instruction will increase R by two.  There's an interesting exception:

doubly-shifted opcodes, the DDCB and FDCB ones, increase R by two too.

LDI increases R by two, LDIR increases it by 2 times BC, as does LDDR

etcetera.  The sequence LD R,A / LD A,R increases A by two, except for

the highest bit: this bit of the R register is never changed.  This is

because in the old days everyone used 16 Kbit chips.  Inside the chip

the bits where grouped in a 128x128 matrix, needing a 7 bit refresh

cycle.  Therefore ZiLOG decided to count only the lowest 7 bits. Anyway,

if the R register emulation is switched on the R register will behave as

is does on a real Spectrum; if it is off it will (except for the upper

bit) act as a random generator.

You can easily check that the R register is really crucial to memory

refresh.  Assemble this program:

ORG 32768

DI

LD B,0

L1  XOR A

LD R,A

DEC HL

LD A,H

OR L

JR NZ,L1

DJNZ L1

EI

RET

It will take about three minutes to run.  Look at the upper 32K of

memory, for instance the UDG graphics.  It will have faded.  Only the

first few bytes of each 256 byte block will still contain zeros, because

they were refreshed during the execution of the loop.  The ULA took care

of the refreshing of the lower 16K.  (This example won't work on the

emulator of course!)

Then there's one other dark corner of the Z80 which has its effect on

programs like Sabre Wulf, Ghosts'n Goblins and Speedlock.  The Mystery

of the Undocumented Flags!

Bit 3 and 5 of the F register are not used.  They can contain

information, as you can readily figure out by using PUSH AF and POP AF.

Furthermore, sometimes their values change.  I found the following

empirical rule:

The values of bit 7, 5 and 3 follow the values of the

corresponding bits of the last 8 bit result of an instruction

that changed the usual flags.

For instance, after an ADD A,B those bits will be identical to the bits

of the A register.  (Bit 7 of F is the sign flag, and fits the rule

exactly).  An exception is the CP x instruction (x=register, (HL) or

direct argument).  In that case the bits are copied from the argument.

If the instruction is one that operates on a 16 bit word, the 8 bits of

the rule are the highest 8 bits of the 16 bit result - that was to be

expected since the S flag is extracted from bit 15.

Ghosts'n Goblins use the undocumented flag due to a programming error.

The rhino in Sabre Wulf walks backward or keeps running in little

circles in a corner, if the (in this case undocumented) behaviour of the

sign flag in the BIT instruction isn't right.  I quote:

AD86    DD CB 06 7E        BIT 7,(IX+6)

AD89    F2 8F AD           JP P,#AD8F

An amazing piece of code! Speedlock does so many weird things that all

must be exactly right for it to run.  Finally, the '128 rom uses the AF

register to hold the return address of a subroutine for a while.  To

keep all programs happy, and still have a fast emulator, I had to make a

compromise.  The undocumented flags are not always emulated right, but

they are most of the time.

Finally, a remark about the interrupt flip flops IFF1 and IFF2.  There

seems to be a little confusion about these.  These flip flops are

simultaneously set or reset by the EI and DI instructions.  IFF1

determines whether interrupts are allowed, but its value cannot be read.

The value of IFF2 is copied to the P/V flag by LD A,I and LD A,R.  When

an NMI occurs, IFF1 is reset, thereby disallowing further (maskable)

interrupts, but IFF2 is left unchanged.  This enables the NMI service

routine to check whether the interrupted program had enabled or disabled

maskable interrupts.  So, Spectrum snapshot software can only read IFF2,

but most emulators will emulate both, and then the one that matters most

is IFF1.

Now for the emulated Z80.  I have added eight instructions, to speed up

the RS232 input and output of the Interface I and several things of the

SamRam.  These opcodes, ED F8 to ED FE are of little use to any other

program.  ED FF is a nice one: it returns you to DOS immediately.  I

used it for debugging purposes.

5.5  File formats

This sections describes the formats of the files used by the emulator.

ROMS.BIN:

---------

00000-03fff     Ordinary Spectrum rom

04000-05fff     Interface I rom (8K)

06000-09fff     First SamRam rom (contains BASIC)

0a000-0dfff     Second SamRam rom (contains monitor,...)

0e000-11fff     First Spectrum 128K rom (active at RESET)

12000-15fff     Second Spectrum 128K rom (contains BASIC)

The ordinary rom has not been modified.  The Interface I rom has

undergone some modifications, to speed up the RS232 input/output

routines.  If you don't like this, or want to use another version of the

Interface I, you could put that code at the right place in the ROMS.BIN

file.  The interface I should work properly, although the RS232 will be

slower (always FORMAT the "b" or "t" channel at 19200 baud, by the way,

if you replace the rom code, there's no point in waiting for nothing!)

The microdrive routines have not been modified in any way.  Here are the

changes of the Interface I rom:

Address: Old: New:           Address: Old: New:

0B9E     ED   ED             0D20     FB   00

0B9F     5B   FC             0D2A     37   ED

0BA0     C3   F5             0D2B     F3   FD

0BA1     5C   C3             0D2C     CE   18

0BA2     21   34             0D2D     00   10

0BA3     20   0C             0D4C     FB   00

These changes are not likely to cause problems; there are several

versions of the Interface I rom around, and program developers know

this.  It is also a bit pointless to check whether the Interface I rom

hasn't been modified; who would put his snapshot software in there

anyway, and that's what those people are afraid of.

The first and second SamRam rom have been modified more extensively. The

biggest problem was that switching the upper 32K ram bank is very fast

in reality, but on the PC two blocks of 32K bytes had to be REP

MOVSWded.  But since no programs know of the SamRam code anyway, this

won't cause any more problems it wouldn't already cause either.

The two Spectrum 128 roms have not been modified.

.TAP FILES:

-----------

The .TAP files contain blocks of tape-saved data.  All blocks start with

two bytes specifying how many bytes will follow (not counting the two

length bytes).  Then raw tape data follows, including the flag and

checksum bytes.  The checksum is the bitwise XOR of all bytes including

the flag byte.  For example, when you execute the line SAVE "ROM" CODE

0,2 this will result:

|------ Spectrum-generated data -------|       |---------|

13 00 00 03 52 4f 4d 7x20 02 00 00 00 00 80 f1 04 00 ff f3 af a3

^^^^^       first block is 19 bytes (17 bytes+flag+checksum)

^^    flag byte (A reg, 00 for headers, ff for datablocks)

^^ first byte of header, indicating a code block

filename    ^^^^^^^^^^^^^

header info               ^^^^^^^^^^^^^^^^^

checksum of header                          ^^

length of second block                         ^^^^^

flag byte                                             ^^

first two bytes of rom                                  ^^^^^

checksum (checkbittoggle would be better)                     ^^

The emulator will always start reading bytes at the beginning of a

block.  If less bytes are loaded than are available, the other bytes are

skipped, and the last byte loaded is used as checksum.  If more bytes

are asked for than exist in the block, the loading routine will

terminate with the usual tape-loading-error flags set, leaving the error

handling to the calling Z80 program.

Note that it is possible to join .TAP files by simply stringing them

together, for example COPY /B FILE1.TAP + FILE2.TAP ALL.TAP

For completeness, I'll include the structure of a tape header.  A header

always consists of 17 bytes:

Byte    Length  Description

0       1       Type (0,1,2 or 3)

1       10      Filename (padded with blanks)

11      2       Length of data block

13      2       Parameter 1

15      2       Parameter 2

The type is 0,1,2 or 3 for a Program, Number array, Character array or

Code file.  A screen$ file is regarded as a Code file with start address

16384 and length 6912 decimal.  If the file is a Program file, parameter

1 holds the autostart line number (or a number >=32768 if no LINE

parameter was given) and parameter 2 holds the start of the variable

area relative to the start of the program.  If it's a Code file,

parameter 1 holds the start of the code block when saved, and parameter

2 holds 32768.  For data files finally, the byte at position 14 decimal

holds the variable name.

.MDR FILES:

-----------

The emulator uses a cartridge file format identical to the 'Microdrive

File' format of Carlo Delhez' Spectrum emulator Spectator for the QL.

The following information is adapted from Carlo's documentation.  It can

also be found in the 'Spectrum Microdrive Book', by Ian Logan (co-writer

of the excellent 'Complete Spectrum ROM Disassembly').

A cartridge file contains 254 'sectors' of 543 bytes each, and a final

byte flag which is non-zero is the cartridge is write protected, so the

total length is 137923 bytes.  On the cartridge tape, after a GAP of

some time the Interface I writes 10 zeros and 2 FF bytes (the preamble),

and then a fifteen byte header-block-with-checksum.  After another GAP,

it writes a preamble again, with a 15-byte record-

descriptor-with-checksum (which has a structure very much like the

header block), immediately followed by the data block of 512 bytes, and

a final checksum of those 512 bytes.  The preamble is used by the

Interface I hardware to synchronise, and is not explicitly used by the

software.  The preamble is not saved to the microdrive file:

offset length name    contents

0      1   HDFLAG   Value 1, to indicate header block

1      1   HDNUMB   sector number (values 254 down to 1)

2      2            not used

4     10   HDNAME   microdrive cartridge name (blank padded)

14      1   HDCHK    header checksum (of first 14 bytes)

15      1   RECFLG   - bit 0: always 0 to indicate record block

- bit 1: set for the EOF block

- bit 2: reset for a PRINT file

- bits 3-7: not used (value 0)

16      1   RECNUM   data block sequence number (value starts at 0)

17      2   RECLEN   data block length (<=512, LSB first)

19     10   RECNAM   filename (blank padded)

29      1   DESCHK   record descriptor checksum (of previous 14 bytes)

30    512            data block

542      1   DCHK     data block checksum (of all 512 bytes of data

block, even when not all bytes are used)

---------

254 times

(Actually, this information is 'transparent' to the emulator.  All it

does is store 2 times 254 blocks in the .MDR file as it is OUTed,

alternatingly of length 15 and 528 bytes.  The emulator does check

checksums, see below; the other fields are dealt with by the emulated

Interface I software.)

A used record block is either an EOF block (bit 1 of RECFLG is 1) or

contains 512 bytes of data (RECLEN=512, i.e.  bit 1 of MSB is 1).  An

empty record block has a zero in bit 1 of RECFLG and also RECLEN=0.  An

unusable block (as determined by the FORMAT command) is an EOF block

with RECLEN=0.

The three checksums are calculated by adding all the bytes together

modulo 255; this will never produce a checksum of 255.  Possibly, this

is the value that is read by the Interface I if there's no or bad data

on the tape.

In normal operation, all first-fifteen-byte blocks of each header or

record block will have the right checksum.  If the checksum is not

right, the block will be treated as a GAP.  For instance, if you type

OUT 239,0 on a normal Spectrum with interface I, the microdrive motor

starts running and the cartridge will be erased completely in 7 seconds.

CAT 1 will respond with 'microdrive not ready'.  Try it on the

emulator...

.SCR FILES:

-----------

.SCR files are memory dumps of the first 6912 bytes of the Spectrum

memory.  A coordinate (x,y), x between 0 and 255 and y between 0 and

192, (0,0) being the upper left corner of the screen, corresponds to the

pixel address

16384+INT (x/8)+1792*INT (y/64)-2016*INT (y/8)+256*y

I admit this is not quite the clearest way to explain the organization

of Spectrum's video memory, but with a bit or (hard) thinking you can

extract from above formula all information you need...  The lowest three

bits of x determine which bit of this address corresponds to the pixel

(x,y).  This bit-map constitutes the larger part of the screen memory,

256*192/8=6144 bytes.  The final 768 bytes are attribute bytes.  The

address of the attribute byte corresponding to pixel (x,y) is

22528+INT (x/8)+32*INT (y/8)

The lowest three bits of the attribute byte control the foreground color

(the color of the pixel if the corresponding bit in the bitmap is set),

bits 3-5 control the background color, bit 6 is the bright bit and bit 7

is the flash bit - if it is set, every 16/50th of a second the ULA

effectively flips the foreground and background colours.

.Z80 FILES:

-----------

The old .Z80 snapshot format (for version 1.45 and below) looks like

this:

Byte    Length  Description

0       1       A register

1       1       F register

2       2       BC register pair (LSB, i.e.  C, first)

4       2       HL register pair

6       2       Program counter

8       2       Stack pointer

10      1       Interrupt register

11      1       Refresh register (Bit 7 is not significant!)

12      1       Bit 0  : Bit 7 of the R-register

Bit 1-3: Border colour

Bit 4  : 1=Basic SamRom switched in

Bit 5  : 1=Block of data is compressed

Bit 6-7: No meaning

13      2       DE register pair

15      2       BC' register pair

17      2       DE' register pair

19      2       HL' register pair

21      1       A' register

22      1       F' register

23      2       IY register (Again LSB first)

25      2       IX register

27      1       Interrupt flipflop, 0=DI, otherwise EI

28      1       IFF2 (not particularly important...)

29      1       Bit 0-1: Interrupt mode (0, 1 or 2)

Bit 2  : 1=Issue 2 emulation

Bit 3  : 1=Double interrupt frequency

Bit 4-5: 1=High video synchronisation

3=Low video synchronisation

0,2=Normal

Bit 6-7: 0=Cursor/Protek/AGF joystick

1=Kempston joystick

2=Sinclair 1 joystick

3=Sinclair 2 joystick

Because of compatibility, if byte 12 is 255, it has to be regarded as

being 1.  After this header block of 30 bytes the 48K bytes of Spectrum

memory follows in a compressed format (if bit 5 of byte 12 is one). The

compression method is very simple: it replaces repetitions of at least

five equal bytes by a four-byte code ED ED xx yy, which stands for "byte

yy repeated xx times".  Only sequences of length at least 5 are coded.

The exception is sequences consisting of ED's; if they are encountered,

even two ED's are encoded into ED ED 02 ED.  Finally, every byte

directly following a single ED is not taken into a block, for example

ED 6*00 is not encoded into ED ED ED 06 00 but into ED 00 ED ED 05 00.

The block is terminated by an end marker, 00 ED ED 00.

That's the format of .Z80 files as used by versions up to 1.45.  Since

version 2.01 emulates the Spectrum 128 too, there was a need for a new

format.

The first 30 bytes are almost the same as the old versions' header.  Of

the flag byte, bit 4 and 5 have got no meaning anymore, and the program

counter (bytes 6 and 7) are zero to signal a version 2.01 .Z80 file.  So

loading a new style .Z80 file into an old emulator will cause an error

or a reset at the most.

After the first 30 bytes, an additional header follows:

Byte    Length  Description

30      2       Length of additional header block (contains 23)

32      2       Program counter

34      1       Hardware mode: 0=Spectrum 48K, 1=0+interface I,

2=SamRam, 3=Spectrum 128K, 4=3+interface I.

35      1       If in SamRam mode, bitwise state of 74ls259.

For example, bit 6=1 after an OUT 31,13 (=2*6+1)

If in 128 mode, contains last OUT to 7ffd

36      1       Contains 0FF if Interface I rom paged

37      1       Bit 0: 1 if R register emulation on

Bit 1: 1 if LDIR emulation on

38      1       Last OUT to fffd (soundchip register number)

39      16      Contents of the sound chip registers

Hereafter a number of memory blocks follow, each containing the

compressed data of a 16K block.  The compression is according to the old

scheme, except for the end-marker, which is now absent.  The structure

of a memory block is:

Byte    Length  Description

0       2       Length of data (without this 3-byte header)

2       1       Page number of block

3       [0]     Compressed data

The pages are numbered, depending on the hardware mode, in the following

way:

Page    In '48 mode     In '128 mode    In SamRam mode

0      48K rom         rom (basic)     48K rom

1      Interf. I rom   Interf. I rom   Interf. I rom

2      -               rom (reset)     samram rom (basic)

3      -               page 0          samram rom (monitor,..)

4      8000-bfff       page 1          Normal 8000-bfff

5      c000-ffff       page 2          Normal c000-ffff

6      -               page 3          Shadow 8000-bfff

7      -               page 4          Shadow c000-ffff

8      4000-7fff       page 5          4000-7fff

9      -               page 6          -

10      -               page 7          -

In 48K mode, pages 4,5 and 8 are saved.  In SamRam mode, pages 4 to 8

are saved.  In 128 mode, all pages from 3 to 10 are saved.  This

version saves the pages in numerical order.  There is no end marker.

end marker.