PrintSpec

Freeware Spectrum Picture Printer

for the Okimate 20 Printer

By Greg Wageman

The Spectrum picture format opened up new possibilities for the Atari ST

computer.  By freeing us from the limitation of only 16 colors, Spectrum

made images on the ST look like they had never looked before.  Unfortunately,

we can't take our computers and color monitors everywhere we go.  If

you're like me, you'd been looking for a way to capture all those

colors on hardcopy.

Being a programmer, and impatient, I took matters into my own hands and

wrote PrintSpec.

PrintSpec uses the same halftoning techniques used by magazines and

newspapers to print black-and-white and full-color pictures on your

Okimate 20 printer.  Spectrum pictures may be rendered in full color using

a color ribbon, or in greyscale using a black ribbon.  Pictures may be

printed in two different sizes: 4 by 6 inches, which can be printed

two-to-a-page, and 7 by 9 inches, which fills nearly an entire 8.5 by 11

inch sheet.

HOW TO USE PRINTSPEC

Double-click on printspc.prg to start the program.  When PrintSpec has

finished loading, you will be presented with a title box.  Press return

or click on the button to continue.  The title box is only shown once, no

matter how many pictures you print.  After dismissing the title box, you

will be shown the main dialog box.  The dialog allows you to set the

ribbon color, the picture size and the printer port from within PrintSpec.

PrintSpec reads your system's printer configuration to determine if you

have a color or black ribbon loaded, and which port the printer is

connected to.  It uses this information to determine the defaults.  If

you change the ribbon color or printer port within PrintSpec, however,

the system configuration is NOT updated.

When you have configured PrintSpec for the type of picture you wish to

print, click on "Continue" to exit the dialog.  You will then be presented

with a file selector box.  Select the Spectrum picture you wish to print,

either compressed or uncompressed.  There will be a pause while PrintSpec

reads the picture from the disk (and uncompresses it, if necessary).

Now that the picture has been loaded into memory, PrintSpec displays a

bar graph that indicates the relative portions of printed and unprinted

picture.  The bar is updated after PrintSpec has sent the data for each

complete print line.

PrintSpec translates the Spectrum picture into matrices of dots, then

into characters, and then sends the appropriate codes to the printer.

PrintSpec drives the Okimate in its 24-pin mode, so it takes a considerable

number of bytes to generate one pass of the printhead.  Even so, PrintSpec

drives your printer faster than the printer will accept data (but only

slightly faster).

You may abort a printout once it's begun by pressing the "Undo" key.  If

you are printing in color, printing will stop after the current line has

printed in all three colors.  Buffering in the printer may cause it to

print more than one line after you press "Undo", however, particularly

when printing in black-and-white.

When printing completes, and after an abort, the configuration dialogue box

reappears.  Clicking on "Quit" will terminate the program and return you to

the desktop.

CONSERVATION OF RIBBON

The Okimate 20's firmware causes the printhead to make a full pass across

the page when printing in color, even if the image does not take up the

entire page width.  The printhead must do this in order to advance the ribbon

to the next color.  Consequently, there is no way to reduce ribbon consumption

during color printing.  During black-and-white printing, however, the

printhead only loads and advances the ribbon when actually printing, and

prints only to the rightmost edge of the image.  A 4-by-6 black and white

picture will therefore use the least amount of ribbon.

OF RESOLUTIONS AND PIXELS

The Atari ST displays 320 pixels horizontally in an eight-inch area of the

SC1224 monitor.  This is a horizontal resolution of 40 pixels per inch.  In

the vertical direction, the ST displays 200 pixels in a six-inch area.  This

is a vertical resolution of 33.33 pixels per inch.  The ratio of horizontal

resolution to vertical is therefore 40/33.33, or about 1.2 to 1.

The Okimate 20 has a maximum resolution of 160 dots per inch horizontally,

and 144 dots per inch vertically.  This is a ratio of 1.11 to 1, which doesn't

quite match the Atari screen's.  However, the amount of distortion introduced

is not objectionable, so this high-resolution mode is exploited in the 4-by-6

inch mode of PrintSpec.  A 3-by-3 halftoning matrix is used to render each

pixel of the image, resulting in 960 dots horizontally (not including the

left margin) and 600 dots vertically.

A 3-by-3 halftoning matrix provides 10 intensities.  All ten are not needed

in color (since a Spectrum image only provides 8 intensities each for Red, Green

and Blue), so only eight are used per color, to reproduce the entire Spectrum

palette.  The RGB values of the image are converted to the Cyan, Magenta and

Yellow values of the Okimate for color printing.  When printing in black, the

RGB values of the Atari are converted to a luminance (brightness) value, and

all 10 possible intensity values are used to approximate the color image.

The Okimate 20 can also display 120 dots per inch horizontally at 144 dots

per inch vertically.  This results in an exact 1.2 to 1 ratio when the

picture is rotated 90 degrees.  This mode is used to print the 7-by-9 inch

pictures.  A 4-by-4 halftoning matrix is used to render each pixel, resulting

in 1280 dots vertically and 800 dots horizontally (again, not including

the left margin).

A 4-by-4 halftoning matrix provides 17 intensities.  In color, only a select

8 intensities are used.  All 17 intensities are used when printing 7 by 9

format pictures in black, resulting in a surprisingly faithful halftoned image.

FREEWARE

This program is Freeware.  I have released it to the Atari ST community

in the hope that it will prove useful to other Okimate owners.  If you have

questions or comments I can be reached by electronic mail as:

74016,352 on Compuserve;

cemax!greg@sj.ate.slb.com on the Internet.

You are hereby granted permission to give a copy of this program to anyone

whom you feel may benefit from its use, provided that you don't alter the

program or the resource file, and provided this documentation accompanies

any and all copies you distribute.  All other rights remain with the author.

CREDIT WHERE CREDIT IS DUE

This program was written in the C Programming Language.  It was compiled

with Mark Williams C, version 3.0.6.  The Resource file was constructed

using the excellent Mark Williams ReSource editor.

Documentation on the Spectrum file format was obtained via Compuserve,

courtesy of Boris Tsikanovsky.

Information on halftoning and RGB to CMY conversion was obtained from

"Fundamentals of Interactive Computer Graphics", by Foley and Van Dam,

first edition.

Information on programming the Okimate 20 for color printing was discovered

by the author through experiment, and by reading between the lines of the

woefully inadequate and error-ridden Okimate 20 printer manual.