In order to keep up with the fast paced arcade action built into today's

computer games, it is often necessary to come up with an edge. Sometimes

you need to be able to shoot 30 to 50 aliens a second, or blast tunnels

through solid raster.  With the device presented here, you will be able

to modify your favorite joystick to become an adjustable switch select-

able rapid fire menace!

This project should work on any Atari-style joystick interface using

DB-9s, including Amigas, C-64's and C-128's, Atari ST's etc. and even many

console machines.

The circuit developed to do this has been optimized to a single CMOS

gate package (4011) and a few capacitors and resistors. It generates a

50% duty cycle square wave, which is important for programs which respond

to the highest rates of fire. Imagine the program sampling the port for

fire button presses trying to see a signal that is only on for 10% of the

time. Many of these signals would go unnoticed at high rates of fire.

This circuit does not suffer from a duty cycle problem unlike many 555 IC

based designs.  It should also function well on a wide variety of supply

voltages (+3 and up) although +5v is the recommended voltage.

It uses a 50K to 100K potentiometer to control the rate.  There is

an LED to indicate when the fire button is pressed in normal mode, or at

what rate it is firing in rapid-fire mode. And of course, there is a  switch

to select between normal and rapid-fire modes. The circuit is designed around

a pair of NAND gates wired up as a free running oscillator. An additional

NAND gate is used on the input, to invert the incoming signal from the fire

button on the joystick, and to enable the oscillator. The output from the

oscillator drives a small LED and the final NAND gate,  which reinverts

the signal, making it the right polarity for the computer. Before the output

from the last NAND gate gets to the computer, it is passed through a small

signal diode to keep the signal active only when low.

When constructing the circuit, it is possible to make it small enough

to fit in an existing joystick. This takes some patience and practice, but

the results are well worth the added brainwork & effort. Alternately, it

may be place in a small box in-line between a joystick and the computer.

The only disadvantage with this approach is that the enable/disable switch

is not as convienient to reach.  Take whatever approach you like, it's easy

enough to build several, or remove them if you don't like them.

In most joysticks only up, down, left, right, fire and ground are wired

to the computer, the +5v signal is absent from the cable. In order to power

the circuit, you must get a cable that has all nine wires, or perhaps, add

external power (A 9 volt battery may suffice). If your joystick does not

have the necessary wires, I suggest you go to your local Radio Shack, and

purchase a joystick extension cord. These cords are 10 feet long and pass

all 9 wires!  Cut off the male end & use the rest on your joystick, or

save a few feet of wire and the male end for your next hacker project.

Use the diagram in the included IFF picture to wire up your rapid-fire

module. Be careful when wiring up the 9-pin connector to your computer,

a short circuit could cause permanent damage. The IC used here is CMOS,

and sensitive to static discharge, so take the normal precautions when

handling it. Good Luck!

This text file and picture can be redistributed as long as the credits

remain intact.

-Mark Spankus

(Mark @ Shorty.CS.wisc.edu)