TABLE OF CONTENTS

i2c.library/AllocI2C

i2c.library/FreeI2C

i2c.library/GetI2COpponent

i2c.library/InitI2C

i2c.library/ReceiveI2C

i2c.library/SendI2C

i2c.library/SetDelay

i2c.library/AllocI2C                                     i2c.library/AllocI2C

NAME

AllocI2C -- Allocate port and timer for use with IC-bus hardware.

SYNOPSIS

ret = AllocI2C (Delay_Type, Name);

D0                 D0       A1

BYTE AllocI2C (UBYTE, char *);

FUNCTION

Allocates the parallelport and needed bits to communicate according

to the IC-bus standard specified by Philips.

If Timer-delay is used, this routine will also handle initializing

of the timer needed to keep the clock-frequency (SCL) under 100 KHz

as specified in the IC-bus standard.

INPUTS

Delay_Type      Specifies if a loop or a timer should be used

to handle the SCL-delay. To get the best

performance, it is recommended to use loop-delay

on non-accelerated Amigas, and only use the timer-

delay with accelerated Amigas.

Name            Name of the program using the IC-bus hardware and

thereby blocking the parallelport for other tasks.

RESULTS

ret             Zero if no error. Detailed error-codes can be found

in i2c_library.h

NOTES

Remember to call InitI2C() after AllocI2C() before using SendI2C()

or ReceiveI2C().

SEE ALSO

BUGS

i2c.library/FreeI2C                                       i2c.library/FreeI2C

NAME

FreeI2C -- Deallocate port and bits after use of IC-bus.

SYNOPSIS

FreeI2C();

void FreeI2C(void);

FUNCTION

Deallocates the parallelport and used bits.

If Timer-delay was used, the timer will now be released.

RESULTS

NOTES

This function may only be called, if AllocI2C() returned zero.

SEE ALSO

BUGS

i2c.library/GetI2COpponent                         i2c.library/GetI2COpponent

NAME

GetI2COpponent -- Get name of task using the parallel-port.

SYNOPSIS

ret = GetI2COpponent();

STRPTR GetI2COpponent(void);

FUNCTION

If AllocI2C returns I2C_PORT_BUSY or I2C_BITS_BUSY this function

can be called to get the name of the Task using the port.

RESULTS

STRPTR to task using the parallel-port

NOTES

Only call this function if AllocI2C returns I2C_PORT_BUSY or

I2C_BITS_BUSY.

SEE ALSO

BUGS

i2c.library/InitI2C                                       i2c.library/InitI2C

NAME

InitI2C -- Initializes the port and sets default IC-bus conditions

SYNOPSIS

InitI2C();

void InitI2C(void);

FUNCTION

Sets the needed bits for input/output-operation. After this, the

default IC-bus condition will de set, that is both SDA and SCL

high.

RESULTS

NOTES

This function should be called right after AllocI2C() if AllocI2C()

returns zero.

SEE ALSO

BUGS

i2c.library/ReceiveI2C                                 i2c.library/ReceiveI2C

NAME

ReceiveI2C -- Receive bytes from the IC-bus

SYNOPSIS

error = ReceiveI2C(addr, number, data);

D0                D0    D1      A1

UBYTE ReceiveI2C(UBYTE, int, UBYTE []);

FUNCTION

Receive data from a device connected to the IC-bus hardware.

INPUTS

addr            Address of device to receive data from.

number          Number of bytes to receive from the specified device.

data            Array, where to put the received data.

RESULTS

error           Zero if an error occurred during reception of data

from the specified device.

NOTES

Be sure, that the size of the data-array is the same or larger than

the requested number of bytes to receive.

SEE ALSO

BUGS

i2c.library/SendI2C                                       i2c.library/SendI2C

NAME

SendI2C -- Transmit bytes on the IC-bus

SYNOPSIS

error = SendI2C(addr, number, data);

D0             D0    D1      A1

UBYTE SendI2C(UBYTE, int, UBYTE []);

FUNCTION

Send data to a device connected to the IC-bus hardware.

INPUTS

addr            Address of device to receive the transmitted data.

number          Number of bytes to transmit from the array specified

by data.

data            Array, where the data to be transmitted is placed in.

RESULTS

error           Zero if an error occurred during transmitting the

data to the specified device.

NOTES

SEE ALSO

BUGS

i2c.library/SetDelay                                     i2c.library/SetDelay

NAME

SetDelay -- Set the delay for IC-bus clock-rate

SYNOPSIS

SetDelay(ticks);

D0

void SetDelay(ULONG);

FUNCTION

Set the delay for the IC-bus clock. This is needed on some Amiga's,

since the IC-bus standard specifies the maximum clock-frequency

on SCL to 100 KHz.

If timer-delay is used, the input will be handled like UNIT_MICROHZ

in the delay-routine.

RESULTS

Adjusts speed of operations on the I2C-bus

NOTES

SEE ALSO

BUGS