FastDisk(1)           AMIGA Programmer's Manual            FastDisk(1)

�1mNAME

�0mFastDisk - Disk-Optimizer/Copier

�1mSYNOPSIS

FastDisk �0m[ [FROM] <device> ] [ [TO] <device> ]

[ FORMAT | NOFORMAT ] [ VERIFY | NOVERIFY ]

[ [FILESYSTEM] FFS | SFS | SRC | DEV ]

[ [BUFFERMODE] HUMBLE | FAIR | GREEDY | IGNORANT ]

[ [DISPLAY] CLI | WINDOW ]

�1mDEFAULTS

FastDisk �0mDF0: TO DF1: FORMAT NOVERIFY DEV GREEDY WINDOW

�1mOVERVIEW

FastDisk  �0mis  a  high-performance disk optimizing tool which

decreases access time for files and directories.    This  is

achieved by creating a disk with an optimized sector layout,

including  linear  directory  layout,  .info  and small file

precedence and minimized track-boundary  crossing  for  data

blocks.

�1mFastDisk  �0mwill  work  with  any  �4mAmigaDOS�0m device of any size

provided   that   the   underlying   Exec-device    supports

trackdisk.device    commands    (i.e.   all   block-oriented

Exec-devices).

�1mFastDisk �0msupports blocksizes other than 512 Byte.

�1mFastDisk �0msupports the standard �4mAmigaDOS�0m filesystem, the fast

filing   system   and   automatic   or   forced   filesystem

conversion.

�1mFastDisk  �0mperforms  intelligent  block pre-caching, reducing

execution time to that of a standard �4mDiskCopy�0m or less.

�1mFastDisk �0mhas four different modes of operation, ranging from

a minimum memory mode  without  block  caching  to  reckless

memory eating for maximum performance.

�1mFastDisk  �0mis  able  to  detect  low-memory  situations  (via

�3mASDG-LowMem.library �0m) and will release  allocated  non-vital

memory if these occur.

�1mFastDisk  �0mhas  a  nice  intuition-interface, but can also be

used in a CLI-only mode and can take its parameters from the

commandline,  WorkBench-Tooltypes,  Manx   environment   and

Commodore ENV: environment.

�1mFastDisk  �0mruns  under  KickStart 1.2 / 1.3 and 2.0 and needs

the �3marp.library(V.39+) �0mto run.  The  �3mASDG-LowMem.library  �0mis

recommended for full functionality but not mandatory.

Torsten Stolpmann                -1-                       Version 1.9

FastDisk(1)           AMIGA Programmer's Manual            FastDisk(1)

�1mDESCRIPTION

FastDisk  �0mcan  be  started from either �4mWorkBench�0m or �4mCLI�0m. The

following �4mCLI�0m parameters are possible:

�1mFROM �0m<�1mdevice�0m>

Using the �1mFROM �0mparameter you may specify the  source  device

which  contains  the  �4mAmigaDOS�0m  volume you want to optimize.

The ':' character must be included for the device name to be

recognized.   The  �1mFROM  �0mkeyword  is  optional  and  may  be

omitted.

Default source device is �4mDF0�0m:.

�1mTO �0m<�1mdevice�0m>

Using  the  �1mTO  �0mparameter  you  may  specify the destination

device which contains the disk to be  overwritten  with  the

contents of the source disk.  The �1mTO �0mkeyword is optional and

can  be  omitted  unless you want to specify the destination

device before  the  source  device,  in  which  case  it  is

mandatory.   The  ':'  character  must  be  included for the

device name to be recognized.

Default destination device is �4mDF1�0m:.

Please note that source and destination may  not  be  equal,

'on-disk' optimization will be covered in the next release.

�1mFORMAT

�0mThis   parameter   tells   �1mFastDisk   �0mto  format  the  whole

destination disk.  This is needed for unformatted disks  and

generally useful for every optimization because it wipes out

all  former  contents  of  the  destination  disk, which may

irritate disk-repair utilities such as �4mDiskDoctor�0m or �4mFixDisk�0m

(especially �4mDiskDoctor�0m, it gets confused often).    This  is

the default option.

�1mNOFORMAT

FastDisk  �0mwill  only  write those tracks which contain data,

speeding   up   the    optimization    (with    the    above

disadvantages).   If  �1mFastDisk �0mdetects an unreadable disk in

the destination device this option is automatically switched

to �1mFORMAT�0m.

�1mVERIFY

�0mThe  complete  destination  disk   will   be   scanned   for

read-errors after the optimization.

�1mNOVERIFY

�0mNo verification  of  the  destination  disk.    This  is the

default option.

Torsten Stolpmann                -2-                       Version 1.9

FastDisk(1)           AMIGA Programmer's Manual            FastDisk(1)

�1mFILESYSTEM SFS

�0mThe destination disk will be a standard file-system disk.

The �1mFILESYSTEM �0mkeyword is optional.

�1mFILESYSTEM FFS

�0mThe destination disk will be a fast file-system disk.

The �1mFILESYSTEM �0mkeyword is optional.

�1mFILESYSTEM SRC

�0mThe destination disk will have the same file-system type  as

the source disk.

The �1mFILESYSTEM �0mkeyword is optional.

�1mFILESYSTEM DEV

�0mThe destination disk will have the file-system type which is

supplied  by the �4mMountList�0m entry �4mDosType�0m for the destination

device.  If this entry is not  present  (KickStart  1.3  and

earlier) default is the standard file-system.

The �1mFILESYSTEM �0mkeyword is optional.

This is the default option.

�1mBUFFERMODE HUMBLE

�0mIn  �1mHUMBLE  �0mmode  �1mFastDisk �0mwill allocate as little memory as

possible to do the optimization.  This of course leads to  a

cutdown   in   execution   speed,   due   to   the  lack  of

block-caching.  The advantage of this option is the constant

memory usage after initialization.    In  all  other  modes,

memory will be dynamically allocated when needed.

The �1mBUFFERMODE �0mkeyword is optional.

�1mBUFFERMODE FAIR

�0mWith  this mode selected, all memory currently not used will

be returned to the system immediatly and block-caching  will

be turned on.  When receiving a notification of a Low-Memory

situation (via �4mASDG�0m-�4mLowMem�0m.�4mlibrary�0m if installed) memory used

for  pre-cached  blocks will be given back and �1mFastDisk �0mwill

continue its operation in �1mHUMBLE �0mmode.

The �1mBUFFERMODE �0mkeyword is optional.

�1mBUFFERMODE GREEDY

�0mUnused memory is cached to  avoid  subsequent  �4mAllocMem�0m()  -

�4mFreeMem�0m() calls.   �1mFastDisk �0mwill honor Low-Memory situations

by freeing non-vital memory, but  will  continue  in  �1mGREEDY

�0mmode.

The �1mBUFFERMODE  �0mkeyword  is  optional.   This is the default

option.

Torsten Stolpmann                -3-                       Version 1.9

FastDisk(1)           AMIGA Programmer's Manual            FastDisk(1)

�1mBUFFERMODE IGNORANT

FastDisk �0mwill ignore all Low-Memory situations and will keep

all memory until completion.  �1mFastDisk �0mwill also  raise  its

Task-Priority to 1.

The �1mBUFFERMODE �0mkeyword is optional.

�1mDISPLAY CLI

�0mWith this option �1mFastDisk �0mwill not open its Intuition-Window

and  will  use  the  CLI-Window it has been started from for

output.   All  neccessary  parameters  differing  from   the

default/environment settings must be provided in the command

line.

�1mFastDisk �0mwill ask you to insert the source/destination disks

in the  appropriate  devices  and  to  press <RETURN>.  When

started non-interactive  (for  example  with  �3mRun  FastDisk)

�0m�1mFastDisk �0mwill start immediatly, making it possible to use it

in a script or the startup-sequence.

The �1mDISPLAY �0mkeyword is optional.

�1mDISPLAY WINDOW

�0mWith  this  option  �1mFastDisk  �0mwill open its Intuition-Window

where all of the above options can be selected via Gadgets.

The �1mDISPLAY �0mkeyword is optional.

This is the default option.

All the above parameters can be specified in the environment

variable    �4mFASTDISKOPTS�0m    also    (for    example:     �3mSet

FASTDISKOPTS="VERIFY   BUFFERMODE  FAIR")  �0mto  override  the

default settings.

The   WorkBench-Tooltype   settings   are   of   the    form

�3mKEYWORD=OPTION �0mwhere �3mKEYWORD �0mis one of �1mFROM�0m, �1mTO�0m, �1mFILESYSTEM�0m,

�1mBUFFERMODE�0m,  �1mFORMAT �0mand �1mVERIFY �0mand �3mOPTION �0mone of the options

listed above (the valid options for �1mFORMAT �0mand �1mVERIFY �0mare �1mON

�0mor �1mOFF �0m).  When started from Workbench, �1mFastDisk �0mwill always

open its Intuition-Window.

CLI-Parameters and ToolTypes will override the  default  and

environment settings.

You can  use  �1mFastDisk �0mwith any combination of devices.  The

only condition that must be fulfilled is that  the  data  on

the source device must fit onto the destination device.

Torsten Stolpmann                -4-                       Version 1.9

FastDisk(1)           AMIGA Programmer's Manual            FastDisk(1)

�1mTECHNICAL NOTES

�0mMEMORY REQUIREMENTS

Every  effort  was  taken  to  make  memory-usage  as low as

possible while  retaining   maximum   performance.      Each

file/directory/link   on  the  source  disk  is  represented

internally by a 32-Byte structure  which  contains  all  the

necessary information.  This and a dynamic map of pre-cached

sectors   (only   for   non-humble   modes)   are  the  main

memory-sinks.  By comparison, additional  memory  allocation

is neglegible.    �1mFastDisk  �0mdoes not rely on large contigous

memory-areas like many other optimizers.  Using �1mHUMBLE �0mmode,

it is possible to optimize a 30 MB  Harddisk-Partition  with

512 KB  Amigas.  1 MB Memory should be sufficient for 130 MB

Partitions.  These are  rough  estimates,  as  memory  usage

mainly  depends  on  the number of files, not on the size of

the partition.

OPTIMIZATION METHODS

In this release I have tried to achieve the following  three

optimization-goals.

Goal 1: Fast directory listings

This  is  done  by  placing  all  file-headers  in  the same

physical order on the disk as read  by  the  filing  system,

including  the  ascending  sorting  of  hash-chains which is

required for the correct behaviour of the ExNext()  function

in KickStart  2.0.    In  addition these blocks are centered

around the  RootBlock.     Directory   blocks   are   placed

immediatly  after the root and bitmap blocks for fast access

to deeper nested levels.

Goal 2: Fast read-access for �3mspecial �0mFiles

To obtain minimum access time for small  or  often  accessed

files,  the data blocks of these files are placed immediatly

after the corresponding header blocks.  These �3mspecial  �0mfiles

include  all files which contain only one or two data blocks

or     whose     names      are      'System-Configuration',

'Startup-Sequence', 'MountList' or 'Shell-Startup' or end in

'.info'.    This   has  a  negative  influence  on  Goal  1,

especially with directories containing  many  special  files

(by increasing the number of tracks to be read), but is more

efficient (regarding the advantages).

Goal 3: Minimimum track boundary crossing

The  remaining  sectors  on  the destination disk are filled

with the data-sectors of the non-special  files.    This  is

done  in  such a way, that each file will cross as few track

boundaries as possible, so that the  number  of  track-seeks

and  (for  track-oriented  devices such as trackdisk.device)

the number of complete tracks to be read is minimized.

Torsten Stolpmann                -5-                       Version 1.9

FastDisk(1)           AMIGA Programmer's Manual            FastDisk(1)

�1mBUGS�0m/�1mLIMITATIONS

�0m- �4mSoftlinks�0m are not  supported,  since  Kickstart  2.0  does

currently not  support  them (V37.166).  �1mFastDisk �0mis able to

detect and suppress Softlinks (they will not be  present  on

the destination disk).

-  The  �4mMountList�0m  entries  �4mBootBlocks�0m,  and  �4mInterleave�0m are

ignored in this release.

- �1mFastDisk �0mshould detach itself  if  the  �1mWINDOW  �0moption  is

provided.

�1mLEGAL STUFF �0m/ �1mDISTRIBUTION

FastDisk �0mVersion 1.9 is Copyright 1991 by Torsten Stolpmann,

all rights reserved.

The  author  takes  absolutely  no  responsibility  for  the

functionality or the possible damage done by this  software.

Use this at your own risk.

�1mFastDisk   �0mVersion   1.9   is   freely   distributable   for

non-commercial purposes.

The author hereby grants Fred Fish the right to include this

software in his library of freely distributable software.

Lots of time and sweat were spent to make this an efficient,

reliable and hopefully useful tool.

If you like �1mFastDisk �0mand you can spare some money,  I  won't

object if you send some.

Send  flames,  suggestions, bug reports and lots of money to

the following adress:

�����Torsten Stolpmann

�����Wilhelm-Raabe-Str. 26 / App. 911

�����W 6750 Kaiserslautern

�����Germany

�����Phone: 0631/22781

�����You may also send email to: stolp@informatik.uni-kl.de

�����The adress above is valid upto November 1991.

�����The new adress will then be:

�����Industriestr. 7

�����W 6750 Kaiserslautern

�����(No new phone-number yet)

Torsten Stolpmann                -6-                       Version 1.9

FastDisk(1)           AMIGA Programmer's Manual            FastDisk(1)

�1mACKNOWLEDGEMENTS

�0mThis document was created with �1mnroff 1�0m.�1m5 �0m-  ported  by  Olaf

Seibert (KosmoSoft).

Window,  Requesters  and  Gadgets were created with �1mGadgetEd

2�0m.�1m3 �0mby Jan van den Baard (Jaba Developments).

�1mFixDisk 1�0m.�1m2 �0mby Werner G�nther was an  often  used  tool  for

Disk-Validation during development.

J�rg  Mayer hacked up the fmsdisk.device to emulate disks of

any size.  This way it was possible to test �1mFastDisk �0mon  330

MB Partions with a 30 MB Harddisk.

Additional  thanks  for  suggestions,  beta  testing, manual

proofreading and bug reports to:

J�rg Mayer, Bernd  Reh,  Deon  Ramsey,  Volker  Rudolph  and

Oliver  Wagner  and  all the others who have sent letters or

email ...

Torsten Stolpmann                -7-                       Version 1.9

FastDisk(1)           AMIGA Programmer's Manual            FastDisk(1)

�1mHISTORY

�0mVersion 1.5:

First public release (1987).

Version 1.8:

Second public release (1989) (minor rewrite).

Known Bugs removed:

1.5 can't process files with length 0.

Performance loss if out of memory.

New Features:

Fast memory support (1.5 used only chipmem).

Supports  all  kinds  of   devices   (1.5   only   supported

trackdisk.device).

Manx-Style environment support.

Version 1.9:

Third public release (July 1991) (complete rewrite).

Known Bugs removed:

No bugs reported.

New Features:

Fast-FileSystem support.

Intuition interface.

Low-Mem library support.

New asynchronous I/O routines.

Algorithm  for  minimized  track  boundary  crossing of file

data-blocks.

New memory managment.

Removed necessity to hold Header-Blocks in memory.

Support for different blocksizes.

Verify mode added.

Lots more...

There exists also a program called �3mFastDisk  II  �0mwritten  by

Oliver Wagner  (published  on Fish Disk).  This is mainly an

'enhancement' of Version 1.5  with  an  Intuition-Interface.

This  program  is  completely  useless  due to a serious bug

created by the global optimizer.  Don't blame  me  for  that

version,  it  was  released before I could test or authorize

it.  Due to this incident there will be no more  source-code

releases of �1mFastDisk�0m.

See ya in version 2.0 ...

Torsten Stolpmann                -8-                       Version 1.9