C 4.0  /  Codeview  Application Note                      October 20, 1986

Codeview Compatible Display Adapter Patch

Codeview Sperry/Corona Patch

Codeview/Int 1A Patch

-Codeview Compatible Display Adapter Patch

------------------------------------------------------------------------------

The purpose of this patch is to fix a problem that may appear when

running Codeview in window mode on certain IBM EGA compatible display

adapters.  The major symptom of this problem is unusual colors for the

display when Codeview is started in window mode.  If you are having

problems getting the correct colors using Codeview on a compatible

display adapter follow these steps to patch a BACKUP COPY of Codeview:

1) Put the Fixdisk that you received with this application note into the

A: drive.

2) Copy CV.EXE to the A: drive.

3) Make sure that DEBUG.EXE can be found on your path.

4) Move to the A: drive.

5) Run EGAFIX.BAT

After you have run EGAFIX.BAT you will have a modified CV.EXE on the Fixdisk

as well as a file CVOLD.EXE which is an unmodified version and should be

saved on a disk to be used as a backup.

If after applying this patch, your display does not display the correct colors

there are other hardware incompatibilities and it will be necessary for you

to run Codeview in sequential mode (/T).

-Codeview Sperry/Corona Patch

------------------------------------------------------------------------------

The purpose of this patch is to correct video hardware differences that

exist with certain Sperry computers when running Codeview in window mode.

******Use this patch only if the display of your Sperry computer blanks out

and the machine hangs when you invoke Codeview.  It is not required for all

Sperry computers.  If the display on your Sperry computer does not blank out

but your machine hangs, use the /M switch (disable the mouse) when you

invoke Codeview.

1) Put the Fixdisk that you received with this application note into the

A: drive.

2) Copy CV.EXE to the A: drive.

3) Make sure that DEBUG.EXE can be found on your path.

4) Move to the A: drive.

5) Run SPFIX.BAT

After you have run SPFIX.BAT you will have a modified CV.EXE on the Fixdisk

as well as a file CVOLD.EXE which is an unmodified version and should be

saved on a disk and used as a backup.

If after having applied this patch, you continue to get a blank screen and

the machine hangs, it will be necessary for you to run Codeview in

sequential mode (/T).

Codeview/Int 1A Patch

------------------------------------------------------------------------------

The purpose of this patch is to correct a problem that exists on machines

that do not support interrupt 1A (read the real time clock).  Two machines

that do not support this are the HP 150 and Apricot XEN.  A symptom of this

problem is Codeview hanging when invoking the Execute command from the Run

menu.

If you have an HP150 or an Apricot XEN and are experiencing problems carry

out these instructions:

1) Put the Fixdisk that you received with this application note into the

A: drive.

2) Copy CV.EXE to the A: drive.

3) Make sure that DEBUG.EXE can be found on your path.

4) Move to the A: drive.

3) Run INTFIX.BAT

After you have run INTFIX.BAT you will have a modified CV.EXE on the Fixdisk

as well as a file CVOLD.EXE which is an unmodified version and should be saved

on a disk and used as a backup.

After you perform this patch you will need to invoke CodeView as follows:

cv /D /M /T sieve.exe

This combination of switches tells Codeview to:

a) disable IBM specific instructions (/D)

b) disable the Apricot mouse which is not Microsoft mouse compatible (/M)

c) run in sequential (non IBM compatible) mode (/T).