WESTERN DIGITAL

DIAGNOSTIC PROGRAM FOR

"PLUS"-FAMILY LAN ADAPTER BOARDS

OPERATING INSTRUCTIONS

This document contains the procedures for testing the following

"PLUS"-family LAN adapter boards:

StarLAN:

StarCard PLUS                     (WD8003S)

StarLink PLUS                     (WD8003SH)

Ethernet:

EtherCard PLUS                    (WD8003E)

EtherCard PLUS With 32K RAM And   (WD8003EBT)

Boot ROM Socket

<< DISK FILES >>

DIAG8003.EXE   Diagnostic Program

INSTALL.DOC    This document

RELEASE.DOC    Program version number and known problems

<< OVERVIEW >>

The Diagnostic Program can be used to verify correct configuration

and operation of a LAN Adapter board before installing the network

software.

Before you use the Diagnostic Program, please note that:

o It should be run directly from DOS.  Running the

Diagnostic Program from any network operating system

may give unpredictable results.

o An Ethernet adapter board will not pass any of the

tests unless it is connected to a network that is

properly terminated at each end.

o Use of the External Loopback Test may interfere with

other network software that is running on a network.

<< STARTING THE PROGRAM >>

Type

DIAG8003

and press RETURN.

The list of test options will appear as follows:

1. Configuration

2. Interrupt Test

3. Read 8390 Registers

4. Write 8390 Registers

5. Internal Loopback Test

6. External Loopback Test

Use the ARROW keys to select a test option (1. - 6.) and press

RETURN to bring up the menu for that test.  To return to DOS from

this list of test options, press ESC.

For the purposes of verifying board configuration and operation,

test options 1, 2, 5 and 6 are appropriate.  Test options 3 and 4

are intended for use by the Western Digital factory and service

center and are not discussed here.

The Diagnostic Program is operated by menus that use the following

keys for selection:

ARROWS/TAB  Move the cursor between fields in a menu.

SPACE BAR   Sequences through the list of possible values

for a field.

ESC         Exits the current menu and returns to the

previous menu.

The following sections describe test options 1, 2, 5, and 6.

<< TEST 1. CONFIGURATION >>

Perform this test first.  The screen will display the following

configuration parameters:

Base I/O Address

RAM Base Address

Receive FIFO Depth

IRQ Channel

Node Address Selection

You must first select values in the Base I/O Address and IRQ Channel

fields that match the jumper settings on your LAN adapter board.

Use the SPACE BAR to sequence through the possible values, and the

UP/DOWN ARROW keys to move between fields in the menu.

NOTE:  If the Base I/O Address or IRQ Channel values

are wrong, none of the tests will pass.

The RAM Base Address is determined by the Diagnostic Program, not by

jumpers.  You must select a value that does not conflict with other

adapters (such as EGA, VGA) that may be installed.  D0000 is usually

a good choice, although D4000 or higher may be required if a VGA

adapter is installed.

The Receive FIFO Depth and Node Address Selection fields should not

be changed.

When the correct values are displayed in the fields:

o first reset the LAN controller device by pressing ALT-C;

press RETURN when prompted;

o then install the new parameters and start the configuration

test by pressing ALT-P.

NOTE:  If an Ethernet adapter board is not connected

to a network that has proper 50-Ohm cable

terminators at both ends, none of the tests

will pass.

Possible resulting messages are:

RAM Test Passed     The configuration test is completed

successfully.

Checksum Error      The board is not installed at all or is

not working, or the configuration

parameters (above) do not match the board

jumper settings.

No Response From    The board is not working, or an Ethernet

8390                network is not properly terminated at

both ends.

RAM Error           The RAM buffer memory test failed.

Press ESC when prompted at the completion of the test.  Return to

the main menu by pressing ESC again.

<< TEST 2. INTERRUPT TEST >>

Run this test next to ensure that the interrupt request generated

by the adapter board properly interrupts the PC.

Possible resulting messages are:

Interrupt Test Passed      The interrupt test is completed

successfully.

Interrupt Test Failed      The interrupt test failed.  Confirm

or               the correct jumper setting and

No Response From 8390      Ethernet network termination (if

applicable).

WD8003 Not Initialized     The Configuration Test (Test 1.)

was not completed successfully.

<< TEST 5. INTERNAL LOOPBACK TEST >>

This test verifies that groups of data (called "frames") can be

transmitted and received correctly by the 8390 LAN controller

device on the LAN adapter board.  The frames are transmitted

internally to the same adapter board without using the network.

An Ethernet adapter board must be connected to a properly

terminated network, however, when this test is run.  The RAM

buffer memory is not used.

Use the SPACE BAR to select a value of 1000 in the ITERATIONS

field.  This is the number of times the test will run before

reporting the results.  (ITERATIONS=0 means run continuously

until ALT-P is pressed to stop the test.)

Don't change the values in the PATTERN or FRAME LEN fields.

When the desired values are displayed in the fields:

o first press ALT-C to clear the statistics;

o then press ALT-P to start the test.

The messages "Internal Loopback In Progress/Internal Loopback

Test Completed" indicate a successful test.  The Frames Sent

and Frames Received statistics should be equal.  Press ESC

when prompted at the completion of the test.

You can start the test over by pressing ALT-C and ALT-P again.

ALT-U is only used to cause a snapshot update of the statistics

while the test is running.

<< TEST 6. EXTERNAL LOOPBACK TEST >>

This test allows the adapter board to transmit and receive data

on the network.

NOTE:  this test may interfere with other network

software that is running on the network.

For a simple test, you need two systems on the same network

running the External Loopback Test portion of the Diagnostic

Program.  Select the following values in each system:

System 1   MODE = ECHO

ITERATIONS = 0

FRAME LEN = 1518

CHECK METHOD = NONE

DEST.ADDR = (this value doesn't matter)

System 2   MODE = TX/RX

ITERATIONS = 1000

FRAME LEN = 1024

CHECK METHOD = COMPARE

DEST.ADDR = the value shown in the SOURCE ADDR

field of System 1

Once started, System 1 will operate in echo mode indefinitely,

returning every frame that it receives to the system that sent

it.  System 2 will send 1000 frames to System 1, checking to

see that it receives each one after it is echoed.

Test 6 is operated just like Test 5.  Press ALT-C and then ALT-P

on each system to start the test.  Be sure to start System 1

first; then start System 2.  System 2 will stop by itself after

1000 frames are sent and received.

If you have other systems that you want to test, leave System 1

running in ECHO mode while you add the other systems to the

network running the External Loopback Test configured like

System 2.  When you want to stop System 1, press ALT-P.

The following information is for more advanced testing.  The

MODE field selects one of four operating modes:

TX Only Transmits frames (data) to another adapter board

without receiving anything.  Set your DEST.ADDR

(Destination Address) field value to the address of

the other adapter board, which is displayed in its

SOURCE ADDR field.  CHECK METHOD is ignored.

ITERATIONS is the number of frames to transmit.

TX/RX   Transmits frames and waits for echo.  Set the

DEST.ADDR field of your system to the value shown in

the SOURCE ADDR field of the system you want to send

the frames to.  Use CHECK METHOD = COMPARE.

ITERATIONS is the number of frames to transmit.

RX Only Receives frames from other adapter boards.  The

address of your adapter board is shown in your

SOURCE ADDR field.  DEST.ADDR is ignored.

Use CHECK METHOD = CHECKSUM and ITERATIONS = 0.

Echo    Receives frames from other adapter boards and sends

each frame back to the adapter that transmitted it.

The DEST.ADDR field is ignored.  Set the FRAME LEN

field to the maximum value (1518) to ensure that

it is at least as large as the frame length of any

of the frames being received.  Use ITERATIONS = 0

and CHECK METHOD = NONE.

The SOURCE ADDR field is for reference only.  It displays the

LAN address of your adapter board.

The DEST.ADDR field is used in the TX/RX and TX Only modes to

specify the LAN address of the adapter board where frames are to

be sent.  The special Destination Address "FF FF FF FF FF FF"

is the broadcast address:  frames sent to this address should be

received by all adapter boards on the network except the board

that sent them.  Use the LEFT/RIGHT ARROW keys to edit this

field.

The FRAME LEN field determines the number of bytes of data in

each frame that is sent.

The PATTERN field shows the data (in hexadecimal) that is sent

in each frame.

In some cases, such as the ECHO and RX ONLY modes, it is

convenient to select ITERATIONS = 0 to let the test run

continuously.  When doing this, press ALT-P to alternately start

and stop the test.  Press ALT-U while the test is running to get

a snapshot update of the statistics, which are normally not

updated until the test is stopped.  Press ALT-C if you want to

clear the statistics.

#######################