WESTERN DIGITAL

"PLUS"-FAMILY LAN ADAPTER BOARDS

JUMPER SETTINGS

Each Western Digital LAN adapter board provides several parameters

that are configured by setting appropriate jumpers on the board.

The following tables summarize the jumper configurations for

certain adapter boards.

For more detailed information on any board, or for information on

jumpers for an adapter board that is not listed here, refer to the

Western Digital "User Installation Guide" that comes with the

adapter board.

NOTE:

o Each "PLUS"-family LAN adapter board contains a RAM buffer

of either 8 or 32 Kbytes that occupies PC memory space.

The base address for this RAM memory is set by the software

you use, not by jumpers on the board.

o Network software distributed by Western Digital originally

used a default RAM base address of C4000 (hex); later

software will use a D0000 default base address.

o EGA and VGA adapters also have memory that occupies this

area and could conflict.  D0000 is often a good RAM base

address for users of EGA adapters; D4000 is often good for

users of VGA adapters.

o Other add-on boards may also introduce conflicts with this

memory space.  Other conflicts can occur with the I/O base

address and interrupt request level, which are controlled

by jumpers as described below.

== EtherCard PLUS (WD8003E) ==

And

== EtherCard PLUS With Boot ROM Socket (WD8003EBT) ==

JUMPER  PIN              FUNCTION SELECTED

BLOCK   NUMBERS   NAME   WHEN JUMPER IS INSTALLED

------  --------  ----   ----------------------------

W1       1-2      Zero   Select zero wait states for optimized

Wait   operation in a 6MHz AT.  (Default is

jumper removed, which ensures correct

operation in faster systems.)  This

jumper is ignored in XT bus systems.

I/O Base Address (W1/3-4, 5-6, 7-8, 9-10):

W1       3-4*, 5-6*,     280 (*-the default)

9-10*

5-6, 9-10       2A0

3-4, 9-10       2C0

9-10            2E0

3-4, 5-6, 7-8   300

5-6, 7-8        320

3-4, 7-8        340

W2       1-2      IRQ7   Interrupt request level.

3-4      IRQ6   Only one jumper may be installed at a

5-6      IRQ5   time.

7-8      IRQ4

9-10*    IRQ3   (*-Default is IRQ3)

11-12    IRQ2

W3       1-2*            Ethernet type: pins all jumpered to use

3-4*            J1 BNC connector for thin Ethernet cable

5-6*            (*-the default).  Jumpers all removed to

7-8*            use J2 AUI connector (thick Ethernet

9-10*           cable).

11-12*

W4       1-2     Version Ethernet version: jumper removed for thin

Ethernet and IEEE 802.3/Version 2 thick

Ethernet (the default); jumper installed

for thick Ethernet Version 1.

W5       1-2*    Segment Thin Ethernet segment length:  pins

Length  jumpered for IEEE 802.3 standard 185m

length (*-the default); jumper removed

for 300m extended length.

[* denotes the default jumpers installed at the factory]

== Additional Jumpers On ==

== EtherCard PLUS With Boot ROM Socket Only (WD8003EBT) ==

JUMPER  PIN         FUNCTION SELECTED WHEN

BLOCK   NUMBERS     INDICATED JUMPERS ARE INSTALLED

------  --------    --------------------------------------------

Boot ROM Base Address (W6/1-2, 3-4, 5-6):

W6      1-2*, 3-4*, Boot ROM disabled (*-the default)

5-6*

1-2         C4000 (16K ROMs)

3-4         C8000 (16K, 32K ROMs)

1-2, 3-4    CC000 (16K ROMs)

5-6         D0000 (16K, 32K, 64K ROMs)

1-2, 5-6    D4000 (16K ROMs)

3-4, 5-6    D8000 (16K, 32K ROMs)

Boot ROM Size (W6/8-9, 10-11) (W9/1-2, 3-4) (W10/1-2, 3-4):

W6      Open*       16 Kbytes ROM (*-the default)

W9      3-4*          "

W10     3-4*          "

W6      8-9         32 Kbytes ROM

W9      3-4           "

W10     1-2           "

W6      10-11       64 Kbytes ROM

W9      1-2           "

W10     1-2           "

RAM Buffer Size (W7/1-2, 3-4) (W8/1-2, 3-4) (W11/1-2-3):

W7      3-4*        8 Kbytes RAM (*-the default)

W8      3-4*         "

W11     2-3*         "

(NOTE:  valid 8 Kbyte RAM base addresses

are C0000, C2000, C8000, CA000, D0000,

D2000, D8000, and DA000.)

W7      1-2         32 Kbytes RAM

W8      1-2           "

W11     1-2           "

(NOTE:  valid 32 Kbyte RAM base addresses

are C0000, C8000, D0000 and D8000.)

[* denotes the default jumpers installed at the factory]

== StarCard PLUS (WD8003S) ==

And

== StarLink PLUS (WD8003SH) ==

JUMPER  PIN              FUNCTION SELECTED

BLOCK   NUMBERS   NAME   WHEN JUMPER IS INSTALLED

------  --------  ----   ----------------------------

W2       1-2      Zero   Select zero wait states for optimized

Wait   operation in a 6MHz AT.  (Default is

jumper removed, which ensures correct

operation in faster systems.)  This

jumper is ignored in XT bus systems.

I/O Base Address (W2/3-4, 5-6, 7-8, 9-10):

W2       3-4*, 5-6*,     280 (*-the default)

9-10*

5-6, 9-10       2A0

3-4, 9-10       2C0

9-10            2E0

3-4, 5-6, 7-8   300

5-6, 7-8        320

3-4, 7-8        340

W1       1-2      IRQ7   Interrupt request level.

3-4      IRQ6   Only one jumper may be installed at a

5-6      IRQ5   time.

7-8      IRQ4

9-10*    IRQ3   (*-Default is IRQ3)

11-12    IRQ2

W3       1-2*            StarCard PLUS (WD8003S):

Jumper must be installed

1-2             StarLink PLUS (WD8003SH):

Jumper must be removed

(for compatibility with 802.3 1BASE5

StarLAN standard.)

[* denotes the default jumpers installed at the factory]

== Additional Jumpers On ==

== StarLink PLUS Only (WD8003SH) ==

JUMPER  PIN         FUNCTION SELECTED WHEN

BLOCK   NUMBERS     INDICATED JUMPERS ARE INSTALLED

------  --------    --------------------------------------------

W4      11-12       Boot ROM is enabled when jumper installed

or disabled (the default) when jumper

removed

Boot ROM Base Address (W4/3-4, 5-6, 7-8, 9-10):

W4      5-6, 7-8,   C4000 (16K ROMs)

9-10

3-4*, 7-8*  C8000 (16K, 32K ROMs; *-the default)

9-10*

7-8, 9-10   CC000 (16K ROMs)

3-4, 5-6,   D0000 (16K, 32K, 64K ROMs)

9-10

5-6, 9-10   D4000 (16K ROMs)

3-4, 9-10   D8000 (16K, 32K ROMs)

Boot ROM Size (W5/1-3-5, 2-4-6) (W6/1-3-5, 2-4-6):

W5      1-3*, 2-4*  16 Kbytes ROM (*-the default)

W6      1-3*, 2-4*    "

W5      1-3, 4-6    32 Kbytes ROM

W6      1-3, 4-6      "

W5      3-5, 4-6    64 Kbytes ROM

W6      3-5, 4-6      "

[* denotes the default jumpers installed at the factory]

##########################