WESTERN DIGITAL

NETBIOS/OSI INTERFACE PROGRAM

FOR

"PLUS"-FAMILY StarLAN/ETHERNET PC ADAPTER BOARDS

RELEASE INFORMATION

Version 3.0                1 March 1988

The purpose of this document (file RELEASE.DOC) is to briefly

explain the contents of this diskette.  The file WPLUSNBE.EXE

is the NETBIOS/OSI Interface Program, intended for use with the

following Western Digital LAN adapter boards:

EtherCard PLUS      (WD8003E, WD8003EBT)

StarCard PLUS       (WD8003S)

StarLink PLUS       (WD8003SH)

Refer to the WD8003-NB NETBIOS/OSI Interface Program Installation

Guide (or file INSTALL.DOC) for an explanation of the command line

parameters that can be used with this program.

This version of the WD8003-NB NETBIOS/OSI Interface Program has

been qualified for use with the ViaNet PROFESSIONAL LAN Operating

System.  Final testing has not been completed, but it is also

believed to be fully operational with:

o IBM PC Local Area Network Program

o Microsoft MS-NET

OPERATIONAL CONSIDERATIONS

Please consider the following when using the NETBIOS/OSI

Interface Program:

o The ViaNet 3.06 Network Operating System will not function

with the NETBIOS/OSI Interface Program loaded.

o If your Western Digital adapter card is set to use IRQ3 (the

factory default setting) or IRQ4, you may experience problems

after exiting from BASIC or compiled BASIC programs.  While

executing, some versions of BASIC manipulate interrupt vectors

3 and 4, and do not properly reset these vectors upon exiting.

This problem has apparently been corrected in later versions

of BASIC.  If you plan to use BASIC programs on your computer,

you may wish to set the adapter card's IRQ level to 2, and

enter the following command when loading the NETBIOS/OSI

Interface Program:

WPLUSNBE /IRQ:2

If you are planning to use BASIC programs on your computer and

you find that another device in your computer is already using

IRQ2, set the adapter card's IRQ level to 7, and enter the

following command:

WPLUSNBE /IRQ:7

KNOWN PROBLEMS WITH THIS VERSION

The following problems have been noted with version 3.0 of

the NETBIOS/OSI Interface Program and are being fixed:

1. This version does not operate properly with The Network

Courier mail program by Consumers Software, Inc.

2. A network node running ViaNet PROFESSIONAL will occasionally

crash when ViaNet PROFESSIONAL is attempting to perform

dynamic session allocation.  This only occurs when the

number of nodes using the server exceeds the number of

sessions that were configured at that server.

Solution:  the node must be rebooted.

3. When running with the IBM PC LAN Program v1.2, a NET SEND

command sometimes fails with a "Network Busy" message.

Solution:  retry the NET SEND command.

4. A "Drive Not Ready" message sometimes occurs under very

heavy loading.  (This has not yet been proven to be a

problem with the NETBIOS/OSI Interface Program.)

Solution:  retry.

MODIFICATION HISTORY

<< Version 3.0 -- Current version >>

o This version of the NETBIOS/OSI Interface Program uses an

improved communication protocol that makes it incompatible

with previous versions (2.1 and earlier) of the Program.

Version 3.0 or subsequent versions must now be used with

every adapter board in the network.

o Only 8K of the 32K RAM buffer on WD8003EBT adapter boards

is supported by this version.

<< Version 2.1 -- 17 November 1987 >>

The following improvements were made:

o The Program will reliably receive back-to-back frames from

an AT-class machine running at 6 MHz or higher.

o The Program returns an error to the caller if an invalid

command NCB is provided.

o Error code 0x09 ("No resources available") is returned when

a CALL command fails due to lack of available space in

internal buffers.

o The timeout on internal buffers has been shortened to

reduce the occurrence of error code 0x09 ("No resources

available").

o The "/v" and "/?" parameters function properly to display

the software version number and help information,

respectively, if the Program has already been loaded.

o The number of buffers has been increased to support more

user sessions.

o The Program no longer locks up during its initialization

when there is broadcast traffic on the network.