DropShadow 					Copyright James Mackraz, 1987

Version 2, Revision 0						April 7, 1987

-----	-----------------------------------------   -----

Notes for Version 2, Revision 0 			April 7, 1987

Added the following from the wish list:

-A slider gadget is implemented to adjust the 3-dimensional

distance between the planes of the windows.

-A slider gadget is implemented to adjust shadow darkness.  If

desired, full independent control of the shadow colors is

possible.

I had some problems with the latter, so I now increase the

task priority while the darkness slider is selected.  It

has not been tested thoroughly.

-----	-----------------------------------------   -----

Notes for Version 1, Revision 1				March 12,1987

When Windows are moved, created, sized, deleted, or depth-

arranged, Intuition calls layers.library.  This program

intercepts those calls to determine that the shadow needs

to be recalculated.  Before this version, this recalculating

and redrawing was done before Intuition got a chance to

redraw Window borders and Gadgets.

This version of DropShadow performs the work on its schedule,

and is only signaled by the code which intercepts the

layers calls.  This makes for a smoother and less dangerous

implementation.

-----	-----------------------------------------   -----

Notes for Version 1, Revision 0				March 1, 1987

The program on this disk creates and maintains multi-level

transparent drop shadows of windows on the Workbench screen.

The program creates its own window, and is terminated by closing

that window.  The program, named DropShadow, may be run from

the CLI, or if associated with an icon, from the Workbench.

The shadows are created by attaching an additional bitplane to

the Workbench's display bit map, but doing so in a way which

is transparent to the Workbench or its application programs.

Unfortunately, this third bitplane brings the Workbench screen

to a point which causes processor cycles to be stolen by the

display generation hardware.  Thus, this program is probably

best used only for demos and having fun, at least for users

without "Fast" RAM.

Enhancements which are planned include:

-The program detects that the shadow image needs to be

recalculated by monitoring the layers library.  That section

of code should check that the layers activity is in fact in

the Workbench screen.

-A way to detect color changes in preferences would allow the

shadow colors to track user changes to the Workbench colors.

In the meantime, the program must be restarted to initialize the

shadow colors.  Commodities Exchange can be used for this.

-A technique of unknown type is planned for user control of

the light source which casts shadows.  Shadows cast according

to the position of the sun (time of day and season) are possible

but unlikely.

-This program is not yet a safe Library vector stealer.  It should

refuse to exit if some other program has displaced its vectors,

lest that other program call DropShadow after it has exited.

Jim Mackraz

7101 Rainbow Drive #8

San Jose, CA  95129