//			Troubleshooting

//

//   In the intervening time between releases, I have learned a

//   lot about how the glove behaves in different situations.

//   If you are experiencing problems such as jumpiness or

//   dimensions not responding, read the following very carefully.

//   Most problems with glove accuracy can be attributed to

//   one or more of the following conditions (in order of

//   commonness):

//

//	1) Sensors on or near hard, smooth surfaces.

//   This problem can be difficult to correct.  The ideal

//   place to use the glove is in a 20' x 20' "room" with

//   ALL WALLS (and ceiling) CARPETED.  You want to get as close

//   to this ideal as you can.  Try taping a thick blanket up

//   against a wall and attaching the sensors directly to the

//   blanket.  If its still not good enough, start putting blankets

//   up on other walls.  You will notice dramatically better

//   response the "softer" your make your environment.  The problem

//   results because the ultrasonic waves emitted by the glove

//   (and measured by the sensors) tend to bounce off smooth

//   hard surfaces.  The circuitry interprets the glove

//   position based on the assumption that the waves are coming

//   DIRECTLY FROM THE GLOVE.  Waves that have been reflected off

//   ANY object in the room WILL be interpreted incorrectly, and

//   WILL cause the glove to seem "glitchy" or prevent the

//   glove cursor from sensing certain positions correctly.

//   "Soft" surfaces made of thick fabric dampen the waves

//   enough to keep the sensors from getting confused.

//   I know it is disappointing to learn that the sensors do

//   NOT belong on a computer monitor, or for that matter a TV,

//   but I'm telling you the truth.

//

//      2) Objects between glove and sensors.

//   It should be obvious that the ultrasonic waves cannot travel

//   through objects on their path to the sensors.  Less obviously,

//   objects that are simply in the vicinity of the path between

//   the glove and the sensors will reflect ultrasonic waves and

//   cause problems as described in #1.

//

//	3) User too near or too far from sensors.

//   You should keep the glove between five and ten feet away from

//   the sensors.  If the glove gets too close, it will become

//   unable to sense certain positions, and perhaps even fail

//   to detect changes on an entire axis.  If the glove gets too

//   far away, it will "move" even when you hand is still.  Also,

//   the position reported will be "grainier" that is, it will

//   lose resolution.

//

//	4) Connector is wired incorrectly.

//   If you have verified all of the above and the glove still

//   fails to work as expected, you should check the wiring.

//   Another easily overlooked factor is the voltage.  Though I

//   have not experimented with this, I've read messages which

//   state that the voltage MUST be between +4.5V and +5.5V.

//   If you are using the 5V wire from your computer, there's

//   nothing to worry about as long as your computer behaves

//   normally.  The keyboard connector and the joystick connector

//   also have reliable 5V wires.

//

//   The only other possible problem could be that your computer is

//   too slow for the software (very unlikely) or the software is

//   incompatible with your computer, which is nearly impossible.

//

//   GLITCHY RESPONSE IS NOT THE FAULT OF THIS CODE!  There is VERY

//   little I could do with software to eliminate the problems

//   with shadowing (#1 and #2 above).  But you need NOT get used

//   to it!  My glove is responding near the accuracy of a mouse!!

//   Yours can too!

//

//   Thanks to Chris Babcock for suggesting the use of "soft"

//   surfaces in the glove's environment.

//

//	Copyright 1992   Mark Thomas Pflaging