Most problems with glove accuracy can be attributed to one or

more of the following conditions (in order of commonness):

1) Sensors on or near hard, smooth surfaces.  This problem can

be difficult to correct.  The ideal place to use the glove is in

a 20' x 20' "room" with ALL WALLS (and ceiling) CARPETED.  You

want to get as close to this ideal as you can.  Try taping a

thick blanket up against a wall and attaching the sensors

directly to the blanket.  If its still not good enough, start

putting blankets up on other walls.  You will notice

dramatically better response the "softer" your make your

environment.  The problem results because the ultrasonic waves

emitted by the glove (and measured by the sensors) tend to

bounce off smooth hard surfaces.  The circuitry interprets the

glove position based on the assumption that the waves are coming

DIRECTLY FROM THE GLOVE.  Waves that have been reflected off ANY

object in the room WILL be interpreted incorrectly, and WILL

cause the glove to seem "glitchy" or prevent the glove cursor

from sensing certain positions correctly.  "Soft" surfaces made

of thick fabric or foam dampen the waves enough to keep the

sensors from getting confused.  It can be disappointing to learn

that the sensors may not belong on a computer monitor, or for

that matter a TV, but this problem is function of the design of

the Power Glove.  I have not tried anechoic foam yet, but I have

heard that putting some behind the sensors will stop almost all

the glitches.

2) Objects between glove and sensors.  It should be obvious that

the ultrasonic waves cannot travel through objects on their path

to the sensors.  Less obviously, objects that are simply in the

vicinity of the path between the glove and the sensors will

reflect ultrasonic waves and cause problems as described in #1.

3) Glove too near or too far from sensors.  You should keep the

glove between five and ten feet away from the sensors.  If the

glove gets too close, it will become unable to sense certain

positions, and perhaps even fail to detect changes on an entire

axis.  If the glove gets too far away, it will "move" even when

you hand is still.  Also, the position reported will be

"grainier" that is, it will lose resolution.

4) Connector is wired incorrectly.

This problem can only occur if have built the connector

yourself!  If you have verified all of the above and the glove

still fails to work as expected, you should check the wiring.

Another easily overlooked factor is the voltage.  Though I have

not experimented with this, I've read messages which state that

the voltage MUST be between +4.5V and +5.5V.    If you are using

the 5V wire from your computer, there's nothing to worry about

as long as your computer behaves normally.  The keyboard

connector and the joystick connector also have reliable 5V

wires.  I've also read messages stating that the connector

should be RFI shielded.  Personally, I think that if you solder

the connector and keep the wires outside of your computer case,

you should be OK.

5) Glove is defective - RARE!!!    Toys R' Us and other places

are selling "used" (perhaps reconditioned?) gloves that can be

identified by a very beat-up looking box.  Be suspicious of any

glove that came in such a box, or any glove that fails to

respond AT ALL after repeated verification of the connector.

6) Weird hardware - RARE!!!    A few kinds of parallel ports are

not "bidirectional".  There is no sure way to test this except

to read the documentation that came with the computer (or

parallel port add-on).  The only other possible problem could be

that your computer is too slow for the software (very unlikely)

or the software is incompatible with your computer, which is

nearly impossible. (Rend386 only runs on '386s or better, Object

Glove demos only on '286s or better.)

Copyright 1992   Mark Thomas Pflaging