PowerGlove Interface for Parallel Port on IBM

This file contains instructions for constructing a

connector to go from a Nintendo Power Glove to a

parallel port, presumably on a PC.  It also contains

instructions for creating a "two glove" connector,

which connects two separate Power Gloves to one

parallel port.

BIG NOTE:

Assemble at YOUR OWN RISK.  I have assembled this

cable successfully from these directions, BUT any

damage resulting from construction or use of this

cable is YOUR problem, not mine.

(just in case... <g>)

The Connector:

* 1

7 * * 2

6 * * 3     Looking AT the connector on glove cable

5 * * 4

Pinout:

1: Ground

2: Clock

3: Latch

4: Data

5: N/C (lightgun)

6: N/C (lightgun)

7: +5V

Connecting it to Parallel Port:

Glove      Parallel Port

-----      -------------

1           18  GND

2            2  D0

3            3  D1

4           13  SELECT (input)

Power:

Connect pin 7 on glove to any location with +5V from

the computer.  Pin 1 on any gameport is a good place.

Or, use the keyboard connector.  The +5V on keyboard

should be pin 5 (may be wrong here, I'm using an XT

pinout) as follows:

Looking at back of computer:

3   1

5   4

2

The Cable:

I highly recommend buying one of the extender cables (the

Curtis Super Extendo cable is around $7 for 2 cables) to

chop up for the connector.  This makes it easy to also use

other Nintendo devices and allows TESTING the glove and

still being able to exchange it if defective.  (Plus you get

a long cable to the control box).

Chris Babcock

CompuServe 72657,2126

10/18/91

Two Gloves:

Connecting them to Parallel Port:

1st Glove      Parallel Port

---------      -------------

1           18  GND

2            2  D0

3            3  D1

4           13  SELECT (input)

2nd Glove      Parallel Port

---------      -------------

1           18  GND  (connect to same pin as 1st glove)

2            4  D2

3            5  D3

4           12  PAPER OUT (input)

You should note that the "1st Glove" is exactly the same as the

single glove hookup given by Chris B.  So if you already have a

single glove connector, you don't need to disconnect any of the

wires that are already there.

Alternate Connection:

It should be possible to simply wire an identical "1st Glove"

connector and connect it to a spare parallel port.  The software

should detect and use the 2nd glove as if it were connected to

the same parallel port.  I HAVE NOT TESTED THIS since I only have

one parallel port, but it really should work.  Let me know if

you can't get it to work.

Power:

Connect pin 7 on glove 2 to any location with +5V.  Obviously,

you should be able to use the same location as the first glove's.

Twice as much current will be drawn.  (If you're using a battery,

it'll wear out twice as fast.)  If you're using the keyboard or

gameport power as Chris suggests, there should be no problem.

The Cable:

See the above section.  You should get a second Super Extendo

Cable, but maybe you already have one since they come in packs

of two!

Although no left handed Power Gloves were ever made, it is

possible to build one from the parts of a Power Glove - I've

built two "better" gloves this way.  I also changed the code

from a right-right systen (where two different people are

using the gloves), to a left-right system where one

person has both hands in gloves.  For that purpose, I may

refer to the "1st Glove" as the "Right Glove" and the "2nd

Glove" as the "Left Glove".  If you have a hood for your

parallel port connector, you may wish to label the two

Nintendo Cables as "left" and "right" before putting the hood

back on.

The Sensors:

Where do you position the second set of sensors?  (In case you

don't know, the sensors are housed in the L-shaped bar that was

designed to go on top of a TV.  It's the piece that has the little

red lights.)  It is easiest to simply piggyback

the sensors, and hold them together with some duct tape or similar

adhesive.  (Now that I think about it, those little Velcro pads

might work OK.)  Basically, you want the sensors in close

proximity, but make sure they do not block each other!  Here's

a diagram of how mine are set up:

/----\	            /-----\

|    |=================|       \

\----/	            \--+ o  |

/----\	               /-----\ |o o |

|    |================|       \| o  |

\----/	               \--+ o  |o o |

|o o |----/

| o  | ||

|o o | ||

\----/ ||

||   ||

||   ||

||   ||

||   ||

||   ||

|| /----\

|| |    |

|| \----/

/----\

|    |

\----/

I had to use 5 pieces of tape to get them to stay together

well, but I'm sure there's other ways.

Software:

I have modified REND386 and the DOS demo and made a new Windows

app that uses the "two glove" setup.  They all access the

same driver which "seeks out" gloves on three different

parallel port addresses, then uses the "alternate polling

technique" for avoiding conflict between the two gloves.

One glove is polled, then the other glove is polled, then

the first glove, etc, etc.  So, the ultrasonic pulses from

one glove are sensed before the pulses from the second glove

are sent out.