TABLE OF CONTENTS

parallel.device/command/Clear

parallel.device/command/Flush

parallel.device/command/Query

parallel.device/command/Read

parallel.device/command/Reset

parallel.device/command/SetParams

parallel.device/command/Start

parallel.device/command/Stop

parallel.device/command/Write

parallel.device/function/OpenDevice

parallel.device/command/Clear                 parallel.device/command/Clear

NAME

Clear -- clear the parallel port buffer

FUNCTION

This command just RTS's (no buffer to clear)

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_CLEAR (05)

parallel.device/command/Flush                 parallel.device/command/Flush

NAME

Flush -- clear all queued I/O requests for the parallel port

FUNCTION

This command purges the read and write request queues for the

parallel device.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_FLUSH (08)

parallel.device/command/Query                 parallel.device/command/Query

NAME

Query -- query parallel port/line status

FUNCTION

This command return the status of the parallel port lines and

registers.

IO REQUEST

io_Message      must have mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      PDCMD_QUERY (09)

RESULTS

io_Status        BIT  ACTIVE  FUNCTION

0     high   printer busy toggle (offline)

1     high   paper out

2     high   printer selected on the A1000

printer selected & serial "Ring

Indicator" on the A500/A2000

Use care when making cables.

3      -     read=0,write=1

4-7            reserved

BUGS

In a earlier version of this AutoDoc, BUSY and PSEL were reversed.

The function has always been correct.

parallel.device/command/Read                   parallel.device/command/Read

NAME

Read -- read input from parallel port

FUNCTION

This command causes a stream of characters to be read from the

parallel I/O register. The number of characters is specified in

io_Length.

The parallel.device has no internal buffer; if no read request has

been made, pending input (i.e. handshake request) is not

acknowledged.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_READ (02)

io_Flags        If IOF_QUICK is set, driver will attempt Quick IO

io_Length       number of characters to receive.

io_Data         pointer where to put the data.

RESULTS

io_Error -- if the Read succeded, then io_Error will be null.

If the Read failed, then io_Error will contain an error code.

SEE ALSO

parallel.device/command/Read

parallel.device/command/SetParams

parallel.device/command/Reset                 parallel.device/command/Reset

NAME

Reset -- reinitializes the parallel device

FUNCTION

This command resets the parallel device to its freshly initialized

condition. It aborts all I/O requests both queued and current and

sets the devices's flags and parameters to their boot-up time

default values.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_RESET (01)

RESULTS

Error -- if the Reset succeded, then io_Error will be null.

If the Reset failed, then the io_Error will be non-zero.

parallel.device/command/SetParams         parallel.device/command/SetParams

NAME

SetParams -- change parameters for the parallel device

FUNCTION

This command allows the caller to change parameters for the

parallel port device. It will disallow changes if any reads or

writes are active or queued.  The PARB_EOFMODE bit of io_ParFlags

controlls whether the io_PTermArray is to be used as an additional

termination criteria for reads and writes.  It may be set directly

without a call to SetParams, setting it here performs the

additional service of copying the PTermArray into the device

default array which is used as the initial array for subsequent

device opens. The Shared bit can be changed here, and overrides the

current device access mode set at OpenDevice time.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       preset by OpenDevice

io_Unit         preset by OpenDevice

io_Command      PDCMD_SETPARAMS (0A)

NOTE that the following fields of your IORequest

are filled by Open to reflect the parallel device's

current configuration.

io_PExtFlags    must be set to zero, unless used

io_ParFlags     see definition in parallel.i or parallel.h

NOTE that x00 yields exclusive access, termarray

inactive.

io_PTermArray   ASCII descending-ordered 8-byte array of

termination characters. If less than 8 chars

used, fill out array w/lowest valid value.

Terminators are used only if EOFMODE bit of

io_Parflags is set. (e.g. x512F040303030303 )

This field is filled on OpenDevice only if the

EOFMODE bit is set.

RESULTS

io_Error -- if the SetParams succeded, then io_Error will be null.

If the SetParams failed, then io_Error will be non-zero.

parallel.device/command/Start                 parallel.device/command/Start

NAME

Start -- restart paused I/O over the parallel port

FUNCTION

This command restarts the current I/O activity on the parallel

port by reactivating the handshaking sequence.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_START (07)

SEE ALSO

parallel.device/command/Stop

parallel.device/command/Stop                   parallel.device/command/Stop

NAME

Stop -- pause current activity on the parallel device

FUNCTION

This command halts the current I/O activity on the parallel

device by discontinuing the handshaking sequence.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_STOP (06)

SEE ALSO

parallel.device/command/Start

BUGS

Using any other parallel.device command will restart IO.

parallel.device/command/Write                 parallel.device/command/Write

NAME

Write -- send output to parallel port

FUNCTION

This command causes a stream of characters to be written to the

parallel output register. The number of characters is specified in

io_Length, unless -1 is used, in which case output is sent until

a zero byte in the data: note that this is independent of setting

EOFMODE in io_ParFlags and using the PTermArray to terminate the

write.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_WRITE (03)

io_Flags        If IOF_QUICK is set, driver will attempt Quick IO

io_Length       number of characters to transmit, or if set

to -1 send until zero byte encountered

io_Data         pointer to block of data to transmit

RESULTS

io_Error -- If the Write succeded, then io_Error will be null.

If the Write failed, then io_Error will contain an error code.

SEE ALSO

parallel.device/command/Read

parallel.device/command/SetParams

parallel.device/function/OpenDevice

NAME

Open -- a request to open the parallel port

SYNOPSIS

error = OpenDevice("parallel.device", unit, ioExtPar, flags)

D0                  A0                D0    A1        D1

FUNCTION

This is an exec call that starts up the parallel.device.

This function allows the requestor software access to the parallel

device.  Unless the shared-access bit (bit 5 of io_ParFlags) is

set, exclusive use is granted and no other access is allowed

until the owner closes the device.  The PTermArray of the ioExtPar

is initialized only if the EOFMODE bit is set in io_ParFlags.

INPUTS

"parallel.device" - a pointer to literal string "parallel.device"

unit - Must be zero for future compatibility

ioExtPar - pointer to an IO Request block of structure IOExtPar

to be initialized by the Open routine.  (see devices/parallel.h for

definition)

The io_ParFlags field must be set as desired (see shared-access

description, above).  Note that this is not a standard IO Request

structure.

flags - Must be zero for future compatibility

RESULTS

d0 -- same as io_Error

io_Error -- if the Open succeded, then io_Error will be null.

If the Open failed, then io_Error will be non-zero.

SEE ALSO

exec/function/CloseDevice