- Sampler.snd  (demo sample)

AMIGA STEREO SAMPLER

--------------------

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

!!                                                                     !!

!!  IMPORTANT: Building and using this device is at your own risc !!   !!

!!             I can't and will not take any responsebility on any     !!

!!             damage caused by this device !! (usual message :) )     !!

!!                                                                     !!

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

HISTORY

-------

I wanted to build me a cheap sampler for the Amiga and tried

serveral samplers. In these experimental days I received a small

circuit design for a stereo-sampler. After building this sampler,

I was stunned, what a great quality for only $14 !!

Since the other (PD) samplers didn't match my wishes, I decided

to draw the circuit and give it to you, the other Amiga Freaks

in the world,...

FILES INCLUDED

--------------

You'll find the following files:

* Sampler.IFF        ; picture of the device

* Sampler.DOC        ; this text-file

PARTS

-----

You'll need the following parts:

1x ZN449E (cheap AD-Converter)

1x HCF4066BE (makes the stereo possible)

4x 10uF El. Capacitor

1x 33pF Capacitor

1x 1n4148 Diode

2x (red-red-red) 2K2 Resistor

1x (brown-black-red) 1K Resistor

1x (brown-green-orange) 15K Resistor

1x DB-25 Male (for your printer port, so watch it A1000 users!!)

2x RCA-in (Audio in)

HOW IT WORKS

------------

First of all,... I made a PCB for my sampler, but it will work just as

fine if you do it with wiring. The layout of my PCB matches the

IFF-picture (left: COMPONENT SIDE!, right: SOLDERSIDE!). I'm considering

to add Mike Kopachs little amplifier to make incoming sound adjustable.

Even a little LED-display (VU) would be nice,... but that I leave for the

future,... Well that's all,.. eh... I think this is easy to build,.. please

report suggestions and problems to:

Hans Luyten

't Hofflandt 36

4851TC Ulvenhout

The Netherlands

or to:   LUYTEN@KUB.NL

Have fun,...