Welcome to:

\                |              /

-   The ultimate PC-Mouse package   -

/                |              \

A lot of people don't like the standard Amiga mouse (and I don't blame 'm).

And if they do like him (I like to 'personalize' my mouse) he (she?) dies

within a short period of time. The market for specific Amiga mice is very

small. Most people like the PC mice and trackballs. I 'm using a PC mouse

of Q-tec, after my Amiga mouse died some years ago. To make it a bit easier

for people to use PC mice and trackballs on the Amiga, I decided to write

this packadge for the use of PC-mice on the Amiga. It contains the

following things (in this doc or the archive):

* A serial mouse driver for both Microsoft mice and Mouse System mice.

* A description for converting 9-pin Sub-D serial to 25-pin serial.

* A description for converting 9-pin Sub-D BUS-mouse to 9-pin Amiga

mouse.

* A description for converting 9-pin Mini-Din BUS-mouse to 9-pin Amiga

mouse.

The SerialMouse-driver

----------------------

The SerialMouse-driver is called "SerMouse" and should be included in this

package. It can only be used with Kickstart version 2.0 or above. You can

start it from the command-line by typing "SerMouse" or from the WorkBench

by double-clicking on it's icon. To deactivate it, just restart the

program. If you want to activate it every time your computer is switched

on, add the command in your s:user-startup or drag the icon to the

WBStartup drawer (do not remove the DONOTWAIT tooltype!).

You can specify the following things either as command-line argument or as

icon tooltype:

3BYTES or 5BYTES

This indicates the mouse-type you're using. 5BYTES is the default and most

used type. If you've got a 3-button mouse, it's deffenetely a 5BYTES type.

If the mouse doesn't work, just try the other type... I named the types

by the number of bytes they sent for one event.

DEVICE

The device to be used for reading serial data. Default is the

serial.device. SerMouse has been tested with the VECTOR Connection multi

I/O card and it worked without any problems.

UNIT

The device-unit to be used for reading serial data. Default is unit 0.

Also tested with the VECTOR board.

PRIORITY

This is the task-priority of the mouse-driver. The default is 20. This

should be OK. It's the same priority as the input.device eg. the standard

mouse.

The program SerMouse is (c) by me (K.P. van Beem) and is distributed as

FreeWare. This means, you may freely use and distribute it, as long as the

programm and this documentation is left unchanged. The source of the

programm is available as GiftWare. Contact me if you're interested in it.

It's partly C and partly assembler.

Converting 9-pin Sub-D serial to 25-pin serial

----------------------------------------------

Most serial mice do have a 9-pin Sub-D serial connector (that's the two-row

connector, NOT a din!). The Amiga has a 25-pin serial connector. In most

stores, you can by adapters from 9 to 25 pins. For those who want to build

them in stead of by them, or just want to connect a 25-pin connector

directly to the wire, this is how you should connect it (the pin-numbers

are normally written on the connectors):

9-pin           25-pin

D-Sub           D-Sub

1       <==>     8          Carrier Detect

2       <==>     3          Recieve

3       <==>     2          Transmit

4       <==>    20          Data Terminal Ready

5       <==>     7          Signal Ground

6       <==>     6          Data Set Ready

7       <==>     4          Request To Send

8       <==>     5          Clear To Send

9       <==>    22          Ring indicator

And don't forget the shield-ground.

Converting 9-pin Sub-D BUS-mouse to 9-pin Amiga mouse

-----------------------------------------------------

Besides serial mice, there's an other type of mouse that is used for the

PC. Although not often these days. It's called the BUS-mouse. It's basicly

the same mouse as the Amiga (and the Atari) is using, but differently

wired. You can easily rewire the mouse (or make an adapter) and connect the

mouse on the mouse-port of the Amiga, just like you do with your standard

Amiga mouse. This way, you can use this type of mouse without a separate

driver. This is the way I use my PC mouse on the Amiga. You should rewire

the mouse as described below:

Amiga side            Mouse side

D-Sub                  D-Sub

5 4 3 2 1             5 4 3 2 1

9 8 7 6               9 8 7 6

1         <==>          4       Vertical pulse

2         <==>          1       Horizontal pulse

3         <==>          3       Vertical pulse Qualifier

4         <==>          2       Horizontal pulse Qualifier

5         <==>          5       Midle button

6         <==>          6       Left button

7         <==>          7       +5V

8         <==>          8       Ground

9         <==>          9       Right button

Converting 9-pin Mini-Din BUS-mouse to 9-pin Amiga mouse

--------------------------------------------------------

For a description of the BUS mouse, see above. Besides the Sub-D plug, most

modern computers do have a mini-Din to connect the bus-mouse to the

computer. Of cause, these can also be rewired. Belose is described how. I

haven't tested this, but I took it from a German magazin.

Mouse          Amiga

Mini-DIN        D-SUB

1     <==>     7      +5V

2     <==>     2      Horizontal pulse

3     <==>     4      Horizontal pulse Qualifier

4     <==>     1      Vertical pulse

5     <==>     3      Vertical pulse Qualifier

6     <==>     6      Left button

7     <==>     5      Middle button

8     <==>     9      Right button

9     <==>     8      GND

I'm not sure the pin-nubers are written on the Mini-Din, besides, if you

don't make an adapter but just rewire it, you won't have the bus. So I'l

give the numbers here. You are looking at the soldier side of the bus, so

at the pin-side of the plug!

|__|

7   8  9

3  4  5  6

__ 1   2  __

|      |

Credits

-------

I want to thank the following people (in alphabetical order) who helped me

with this mice stuff:

Marcel Duyn, who lent me a serial mouse for testing the driver.

Stefan Johannesdal, who tested the 5BYTES part of the driver.

Christian Stieber, who provided me with some technical info about the

5BYTES serial mouse protocol.

For questions, suggestions or bug-reports, you can always reach me at the

adresses below.

Have fun! (Easy, 'cause you own an Amiga!)

_            _              _

/_)          /_)          __// Hobby programmer...  Fido: 2:280/464.2

/ atrick van /_)eem        \X/  e-mail: patrick.van.beem@aobh.xs4all.nl

Postal address: Parlevinker 5

2152 LC  Nieuw-Vennep

The Netherlands