AMIGA STEREO SAMPLER

--------------------

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

!!                                                                     !!

!!  IMPORTANT: Building and using this device is at your own risc !!   !!

!!             I can't and will not take any responsebility on any     !!

!!             damage caused by this device !! (usual message :) )     !!

!!                                                                     !!

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

HISTORY

-------

I wanted to build me a cheap sampler for the Amiga and tried

serveral samplers. In these experimental days I received a small

circuit design for a stereo-sampler. After building this sampler,

I was stunned, what a great quality for only $18 !!

It can sample at all rates possible using Audiomaster IV.

Since the other samplers didn't match my wishes, I decided

to draw the circuit and give it to you, the other Amiga Freaks

in the world,...

This Sampler is an update to my previous sampler, I added

2 amps for volume control on the input (Thanks to: Mike Kopack,

he used this amp in his sampler aswell).

FILES INCLUDED

--------------

You'll find the following files:

* Sampler_COMPONENTSIDE.IFF   ; a black&white IFF of the component-

side of the device pcb

* Sampler_SOLDERSIDE.IFF      ; a black&white IFF of the solderside

* Sampler.DOC                 ; you're reading it

* SamplerDemo.IFF             ; a sample sampled with this sampler

(+- 20Khz,... Audiomaster IV)

* NEWIO-Files: - Bibliothek-dir

- Platine-dir

- patched 'newio.vorein' for Harddisk usage

(read the note !)

PARTS

-----

You'll need the following parts:

1x ZN449E (cheap AD-Converter)

1x HCF4066BE (makes the stereo possible)

2x LM386N (amps)

6x 10uF El. Capacitor

1x 33pF Capacitor

2x 0.1uF El. Capacitor

1x 1n4148 Diode

2x (red-red-red) 2K2 Resistor

1x (brown-black-red) 1K Resistor

1x (brown-green-orange) 15K Resistor

2x Var.resistor (Pot.meter) 100K

1x DB-25 Male (for your printer port, so watch it A1000 users!!)

2x RCA-in (Audio in)

HOW IT WORKS

------------

First of all,... I made a PCB for my sampler, but it will work just as

fine if you do it with wiring. The layout of my PCB matches the

IFF-pictures (note the difference between componentside en solderside!).

I added Mike Kopachs little amplifier to make incoming sound adjustable.

For best results: - use a METALcase grounded to audio GND

- use 2 seperate variable resistors

- keep all audio cables as short as possible

I also included the data-files for NEWIO 3.0 which I used to draw the

PCB,.. (don't forget the libraries)

A1200 users; Newio 3.0 can be started on your Amiga using the original

chipset (select at boot).

For harddisk useage patch the 'Newio.Vorein'.

You'll have to change every appearance of 'DF0:' in 'Newio.Vorein'

to 'PCB:'.

After that you assign PCB: to you work directory,

example:

assign PCB: DH1:CAD/Newio

Newio now works just fine on your harddisk.

Plans: a little LED-display (VU) would be nice,... (Suggestions ??)

Well that's all,.. eh... I think this is easy to build,.. please

report suggestions and bugs to:

Hans Luyten

't Hofflandt 36

4851TC Ulvenhout

The Netherlands

or to:   LUYTEN@KUB.NL

Have fun,...