6    TRACKDISK DEVICE

6.1  INTRODUCTION

All Amiga models are delivered with at least one internal

disk drive, but you may connect up to three extra disk drives.

The disk drivers are using normal double sided double density

(2DD) 3 1/2" disks. 5 1/4" disks may also be used, but this is

not supported by the trackdisk device.

The trackdisk device consists of a set of low level routines

which are used by other higher level processes like AmigaDOS.

When you want to read and write data you should normally only

use AmigaDOS or the special file commands in C. The trackdisk

device should only be used when you want to write disk

viewers/editors, or disk-copy programs.

Although the disk routines are normally considered to be very

slow on the Amiga, it should be noted that it is actually not

the disk drivers which are slow. AmigaDOS is using a very

flexible disk operating system and because of this the disk

drivers appears to be very slow. The low level routines are

on the other hand very quick and are turbocharged with the

"Blitter" (special coprocessor on the Amiga) which is

specialized in moving large quantities of data extremely fast.

To read and write so called "raw data" with the lowlevel

routines is much faster that to use the higher and more

sophisticated AmigaDOS routines. Many games use these low

level routines to quickly load graphics and sound effects.

Normal programs should, as said before, not use the lowlevel

routines described here. However, it can still be interesting

to read about it anyway, since it is always good to know what

is actually happening inside the Amiga.

6.2  AMIGA DISK DRIVERS

The trackdisk device was designed to handle normal double sided

double density (2DD) 3 1/2" disks (also called "1 megabyte

disks"). The disk can be logically (fysically too, but that is

not recommended) be divided into several small data areas where

the actual data is stored.

Each data area, normally called "Sector", can store 512 bytes

of data and 16 bytes of so called "label" data. The label area

is used to identify the sector and what is stored here, and can

therefore not be used to store raw data.

A group of eleven sectors is called "Cylinder" or "Track", and

there exist 80 cylinders on each side of the disk. In total you

can store 512 (bytes/sector) * 11 (sectors/cylinder) * 80

(cylinders/side) * 2 (double sided disks) = 901120 bytes = 880KB

(901120 / 1024).

The trackdisk device will only handle whole cylinders. Even if

you only want to read some sectors of a cylinder the complete

cylinder will be loaded into the trackdisk's own memory buffer.

This technique of reading and writing complete cylinders

greatly improves the storage capacity and speed of the drives.

The actual reading and writing process is very quick compared

to the time it takes for the drive to move the head to the right

position and start rotating the disk.

Your program will not notice that the trackdisk device is only

using complete cylinders. If you want to read some sectors, the

trackdisk device will read a complete cylinder, but gives you

only those sectors you wanted. If you later want to read some

sectors more it may happen that the device immediately can give

you the data you wanted without having to access the disk.

6.3  TRACKDISK DEVICE

The trackdisk device is controlled in the same manner as all

other devices. You send your commands to it with help of a

request block, and the device will send messages back to a

specific reply port. So, to use the trackdisk device you have

to: (Same as with most other devices.)

1. Create a message port with which the device can send

messages back to us. See chapter 1 "Devices" for more

information.

2. Allocate and preinitialize a request block.

3. Open the trackdisk device.

6.3.1  REQUESTBLOCK

A normal IOStdReq structure ("standard request block") is in

most cases possible to use, but if you want to use some special

features described further down you have to use the extended

request block "IOExtTD". This IOExtTD structure is declared in

header file "devices/trackdisk.h" as:

struct IOExtTD

{

struct IOStdReq iotd_Req;

ULONG iotd_Count;

ULONG iotd_SecLabel;

};

iotd_Req:      The top part of the request block consists of

a standard request block (IOStdReq structure).

iotd_Count:    Each disk drive you are working with is assigned

a counter value which is increased each time a

disk is removed. If you are using the "extended"

commands (described below) the trackdisk device

will check the driver's count value with this

value. If the counter is greater than this value

the request will be aborted.

If you are going to working with a disk it is

best to first get the driver's current count

value and store it here. You will then be sure

that the user will not change disks without you

knowing about it.

iotd_SecLabel: As described above, each sector has a 16 bytes

long "label" are. This label area is not used by

the track disk device. However, if you want to

read and/or write to this label area you should

use the "extended" read and write commands

described below, and this field should contain

a pointer to some memory where the label data

can be stored/read from.

NOTE! If you intend to read some sectors' label

areas you must make sure you have allocated a

buffer that is large enough. 16 bytes is needed

for every sector you intend to read. If you want

to write new sector labels the memory must of

course also be large enough, but also

initialized with the values you want to write.

Since this is an extended request block you have to use the

CreateExtIO() function rather than CreateStdIO().

Synopsis: ext_req = CreateExtIO( msg_port, size );

ext_req:  (struct IORequest *) Pointer to the new extended

request block, or NULL if the request block could

not be created.

msg_port: (struct MsgPort *) Pointer to the message port

the device should use to communicate with you.

size:     (long) The number of bytes that should be allocated

for the extended request block. Use the function

sizeof() to find the exact number of bytes needed.

If you are not going to use the "extended" commands which are

described below, but only the normal commands, you do not need

to use this extended request block. A normal standard request

block will be enough. However, if you later would change your

program so it starts to use the extended commands you have to

remember to also change the type of request block. Usually it

is best to always use an extended request block, although it

sometimes it not strictly necessary.

6.3.2  OPEN THE TRACKDISK DEVICE

Once you have opened a (reply) message port and created the

request block you can open the trackdisk device. This is done

with help of the now for us famous OpenDevice() function.

Synopsis: error = OpenDevice( name, unit, req, flags );

error:    (long) If OpenDevice() managed to open the device

it returns 0, else an error number is returned.

name:     (char *) Name of the device you want to open.

The name of the trackdisk device is (surprise)

"trackdisk.device". This name has been defined

is the header file as "TD_NAME".

When you are opening devices you should always try

to use the defined names rather than the actual

names. If Commodore would some day change the name

of the device (very unlikely though) you only have

to recompile your program with the new header

files, and your program will work again.

unit:     (long) Which disk drive you want to use. All Amigas

are sold with at leas one internal drive (df0:),

but many have bought at lest one more drive. In

total there may exist up to four disk drivers.

Drive df0: has been given the unit number 0, df1:

the number 1, and so on...

If you are trying to access a drive which does

not exist, OpenDevice() will fail. See Example 2.

req:      (struct IORequest *) Pointer to the request block

you have created and initialized.

flags:    (long) This field is ignored by the trackdisk

device.

6.3.3  CLEAN UP

As always, do NOT forget to clean up after yourself! All opened

message ports, request blocks, devices etc. must be closed and

removed before your program may terminate. Here is what you

have to do:

1. Make sure that all your requests have been completed or

aborted.

2. Close the trackdisk device with help of the CloseDevice()

function.

Synopsis: CloseDevice( req );

reg:      (struct IORequest *) Pointer to the device's

request block.

3. Delete all request blocks you have created. Note that if

you have allocated extended request blocks you must use

the DeleteExtIO() function, but if you have allocated

standard request blocks you can use the DeleteStdIO()

function. As recommended above you should only use the

extended request blocks with the trackdisk device.

Synopsis: DeleteExtIO( std_req, size );

std_req:  (struct IOStdReq *) Pointer to the extended

request block you want to delete.

size:     (long) The size of the request block, in bytes.

4. Close all message ports you have opened. Do this by

calling the DeletePort() function. Note that all messages

still left at the port must be removed before you may

close the message port.

Synopsis: DeletePort( msg_port );

msg_port: (struct MsgPort *) Pointer to the MsgPort

structure that should be deallocated.

6.4  COMMANDS

When the trackdisk device has successfully been opened you may

start to send requests to it. The trackdisk device can do a lot

of useful stuff, so there exist several commands that could be

used. These commands can be divided into two groups. The first

group consists of the "normal" commands. These commands will

not check if the disk has been removed or changed. Note also

that these commands can not be used to read or write the label

areas of the sectors.

The second group consists of the "extended" commands - those

commands that needs the extended IOExtTD structure. These

commands will check the unit counter too see if the disk has

been changed, and if so, the commands will automatically be

aborted. These commands can be used if you want to access

the sectors' label areas.

When you want to read and write data you have to give the

"io_Data" field of the request block a pointer to some memory

where all data you read will be placed, or if you are writing,

where the data that should be written is stored. This buffer

should be "word aligned" which means that it must start on

an even byte address. The memory must also be of the type

"Chip Memory" since the blitter will be used to speed up the

operations. To fulfill both demands you should allocate the

memory with help of the AllocMem function. See examples further

down for more information.

The "io_Offset" value contains the offset value from which

the trackdisk device will start to read/write from. The

correct offset value is calculated with this formula:

TD_SECTOR*( NUMSECS * 2 * cylinder + NUMSECS * head + sector )

The words in capital letters are defined in the header file

"devices/trackdisk.h" as:

TD_SECTOR   512 (512 bytes / sector)

NUMSECS      11 (11 sectors / cylinder)

For example, if you want to start to read at side 0, cylinder 5,

and sector 3, you should set the offset value to:

TD_SECTOR * ( NUMSECS * 2 * 5 + NUMSECS * 0 + 3 );

You tell the trackdisk device to read/write a specific number

of bytes by setting the "io_Length" field to the number of

bytes that should be transferred. Note that you must read

complete sectors, and the length should therefore be set to:

TD_SECTOR * "nr of desired sectors". (The trackdisk device

will only read complete cylinders, but has nothing to do with

this restriction of only reading complete sectors.)

The "io_Actual" filed will contain the number of actual bytes

written/read. If this value is not the same as the "io_Length"

something has happened. Check the "io_Error" field to find

any error values. See below for more inforamtion about error

messages.

6.4.1  READ

Probably one of the most commonly used commands is undoubtedly

the read command. If you simply want to read some data, and

you do not care if the user has changed disks, you should use

the "CMD_READ" command. However, most times it is best to check

if the disk has been changed, before you start to read, and

then you should use the extended "ETD_READ" command.

Here is an example: ("exreq" is a pointer to the request block,

"buffer" is a pointer to some memory which is word aligned and

is of the type chip memory. The buffer must at least be 2 * 512

bytes.)

/* We want to read: */

exreq->iotd_Req.io_Command = ETD_READ;

/* Pointer to our buffer: */

exreq->iotd_Req.io_Data = (APTR) buffer;

/* Read two sectors: */

exreq->iotd_Req.io_Length = TD_SECTORS * 2;

/* Start to read at side 0, cylinder 5, and sector 3: */

exreq->iotd_Req.io_Offset = (LONG)

TD_SECTOR*( NUMSECS * 2 * 5 + NUMSECS * 0 + 3 );

/* Do our request: */

DoIO( exreq );

6.4.2  WRITE

Same as with reading, there exist two commands to write data.

The shorter "CMD_WRITE" will simply write to the disk which is

currently in the drive. The extended "ETD_WRITE" will check

that the disk has not been changed.

Here is a similar example as the one above. This time we will

try to write two sectors of data:

/* We want to write: */

exreq->iotd_Req.io_Command = ETD_WRITE;

/* Pointer to our buffer: */

exreq->iotd_Req.io_Data = (APTR) buffer;

/* Write two sectors: */

exreq->iotd_Req.io_Length = TD_SECTORS * 2;

/* Start to write at side 0, cylinder 5, and sector 3: */

exreq->iotd_Req.io_Offset = (LONG)

TD_SECTOR*( NUMSECS * 2 * 5 + NUMSECS * 0 + 3 );

/* Do our request: */

DoIO( exreq );

6.4.3  MOTOR ON/OFF

You can tell the trackdisk device to start or stop the motor of

the disk drive. Although you do not need to start the motor

(this will be done automatically if you start to read or write),

you must make sure to stop the drive after you have used it.

You can either use the command "TD_MOTOR" or "ETD_MOTOR". The

later will check that the disk has not been changed.

If the "io_Length" field is set to 0 the drive will be turned

off. If the field is set to 1 the drive will be turned on. Here

is an example ("exreq" is a pointer to the request block):

/* Turn motor on/off: */

exreq->iotd_Req.io_Command = ETD_MOTOR;

/* Turn the motor on: (0 = off, 1 = on) */

exreq->iotd_Req.io_Length = 1;

/* Do our request: */

DoIO( exreq );

Remember to always turn off the motor after you have used the

drive! (If the motor already was on before you started to use

it you do not need to turn it off. This means that some other

device is also using the drive.)

6.4.4  UPDATE THE DISK

When you write data to the disk it will not immediately be

stored. The trackdisk device will as explained above only read

and write complete cylinders, and if you do not write exactly

an even number of cylinders, some data will be temporarily

stored in the trackdisk device's internal memory buffer. When

you later change cylinder the buffer will be copied out before

the head is moved.

You can tell the trackdisk device to immediately copy the

data in the buffer to the disk by sending a "CMD_UPDATE" or

"ETD_UPDATE" command. The later command will as usual check if

the disk has been removed or changed before the buffer is

copied onto the disk.

After you have written something to the disk and before you

turn the motor off you should use this command. You will then

be sure that all data you have written will actually be on the

disk.

Here is an example:

/* Update the disk (move any data still left in the buffer */

/* to the disk:                                            */

exreq->iotd_Req.io_Command = ETD_UPDATE;

/* Do our request: */

DoIO( exreq );

6.4.5  CLEAR BUFFER

You can tell the trackdisk device to clear it's internal

buffer by sending a "CMD_CLEAR" or "ETD_CLEAR" command. Be

sure that you do not clear anything that still has not been

moved out onto the disk.

This clear command is recommended to use after you have noticed

that the disks have been changed. This will ensure that old

data from another disk will not be written to the new disk.

Here is an example:

/* Clear the temporary buffer: */

exreq->iotd_Req.io_Command = ETD_CLEAR;

/* Do our request: */

DoIO( exreq );

6.4.6  POSITION THE HEAD

You can move the head to a specific position by sending a

"TD_SEEK" or "ETD_SEEK" command. The head will simply be moved

to the specified position in the "io_Offset" field, but no

data will be read or written. This command is usually not

needed since the head will always be placed on the correct

position when you read or write data.

Here is an example:

/* Position the head: */

exreq->iotd_Req.io_Command = ETD_SEEK;

/* Move to position: side 1, cylinder 2, and sector 4: */

exreq->iotd_Req.io_Offset = (LONG)

TD_SECTOR*( NUMSECS * 2 * 2 + NUMSECS * 1 + 4 );

/* Do our request: */

DoIO( exreq );

6.4.7  FORMAT

With help of the "TD_FORMAT" or "ETD_FORMAT" command you can

format a cylinder. You give the "io_Data" field a pointer to

some data which will be written onto the new cylinder, and

set the "io_Length" to NUMSECS * TD_SECTOR (one cylinder). The

"io_Offset" field should be set to an offset value to the

cylinder you want to format.

If you write data to a cylinder and you receive a hard write

error (the cylinder is not formatted), you can try to reformat

that cylinder. Note that it is usually best to tell the user

to use some other disk rather than trying to correct this one.

The user can then try to use "Diskdoctor" or similar programs

to solve the disk problem.

Here is an example:

/* We want to format one cylinder: */

exreq->iotd_Req.io_Command = ETD_FORMAT;

/* Pointer to our buffer: (Must be at least one  */

/* cylinder of bytes large, NUMSECS * TD_SECTOR. */

exreq->iotd_Req.io_Data = (APTR) buffer;

/* Format one cylinder: */

exreq->iotd_Req.io_Length = NUMSECS * TD_SECTOR;

/* Format cylinder 5 on side 0: */

exreq->iotd_Req.io_Offset = (LONG)

TD_SECTOR*( NUMSECS * 2 * 5 + NUMSECS * 0 );

/* Do our request: */

DoIO( exreq );

6.4.8  REMOVE

You can send a "TD_REMOVE" command to the trackdisk device,

and it will then increase the driver's count value. It will

look like that the user has changed disks.

Here is an example:

/* Change disks: */

exreq->iotd_Req.io_Command = TD_REMOVE;

/* Do our request: */

DoIO( exreq );

6.4.9  GET THE DISK'S CURRENT COUNT NUMBER

The "extended" commands check if the disk has changed or not

before they are executed. If the disk has changed the command

is aborted. The commands compare the "iotd_Count" value of the

request block with the disk's current count value. If the

disk's count value is larger than the "iotd_Count" the command

is aborted and the error flag "TDERR_DiskChanged" is set.

Before you use these extended commands you should therefore

get the disk's current count value and store it in the request

block. To get the disk's current count value you send an

extended request block with the "TD_CHANGENUM" command set to

the trackdisk device. The device will as soon as possible

return your request with a copy of the current count value in

the "io_Actual" field of the request block.

Here is an example:

/* Store the current count value here: */

ULONG count_value;

/* Get the disk's change count value: */

extreq->iotd_Req.io_Command = TD_CHANGENUM;

/* Do our request, and return when completed: */

DoIO( extreq );

/* Save the count value in the variable:  */

count_value = extreq->iotd_Req.io_Actual;

After you have got the current count value you should give it

the the request blocks which should later be used. The request

block can then use the extended commands. Example:

/* Give the request block the current count value: */

extreg->iotd_Count = count_value;

After your program has received a "TDERR_DiskChanged" you should

try to get the new count value before you attempt to access the

new disk.

6.4.10  CHECK IF THERE IS A DISK IN THE DRIVE OR NOT

With help of the trackdisk device can you check if there is a

disk in a drive or not. To do this you use the "TD_CHANGESTATE"

command. If there is a disk in the drive will the "io_Actual"

field of the request will contain zero. On the other hand, if

there is not a disk in the drive the field will contain a non

zero value.

/* Check if there is a disk in the drive or not: */

extreq->iotd_Req.io_Command = TD_CHANGESTATE;

/* Do our request, and return when completed: */

DoIO( extreq );

/* Is there a disk in the drive? */

if( extreq->iotd_Req.io_Actual )

printf( "No disk in the drive!\n" );

else

printf( "There is a disk in the drive!\n" );

6.4.11  CHECK IF THE DISK IS WRITE PROTECTED OR NOT

It can sometimes be very useful to know if the disk is write

protected or not. With help of the "TD_PROTSTATUS" command we

can easily check this. If the disk is write protected the

"io_Actual" field will contain a non zero value. On the other

hand, if the disk is not write protected the field will contain

zero.

/* Check if the disk is write protected or not: */

extreq->iotd_Req.io_Command = TD_PROTSTATUS;

/* Do our request, and return when completed: */

DoIO( extreq );

/* Is the disk write protected or not? */

if( extreq->iotd_Req.io_Actual )

printf( "The disk is write protected!\n" );

else

printf( "The disk is not write protected!\n" );

6.4.12  GET DRIVE TYPE

The trackdisk device can also be used to tell you what type of

disk drives are connected. If you send a request block with the

"TD_GETDRIVETYPE" command set, the "io_Actual" field of the

request block will either contain the flag "DRIVE3_5" or

"DRIVE5_25" when it is returned. The "DRIVE3_5" means that it

is a normal 3 1/2" disk drive, and the "DRIVE5_25" means that

it is a 5 1/4 (IBM) disk drive.

/* Check drive type: */

extreq->iotd_Req.io_Command = TD_GETDRIVETYPE;

/* Do our request, and return when completed: */

DoIO( extreq );

/* What type is it? */

if( extreq->iotd_Req.io_Actual == DISK3_5 )

printf( "It is a normal 3 1/2\" disk drive.\n" );

else

if( extreq->iotd_Req.io_Actual == DISK5_25 )

printf( "It is a 5 1/4\" disk drive.\n" );

else

printf( "It is a very strange disk drive!\n" );

6.4.13  GET THE NUMBER OF TRACKS

The "TD_GETNUMTRACKS" command is used to check how many tracks

are used by the drive. When the request is returned you can

check the "io_Actual" field of the request to get the number of

tracks. A normal 3 1/2 disk drive uses 160 tracks (80 tracks /

side).

/* Get the number of tracks this drive is using: */

extreq->iotd_Req.io_Command = TD_GETNUMTRACKS;

/* Do our request, and return when completed: */

DoIO( extreq );

/* How many tracks? */

printf( "No tracks: %d\n", extreq->iotd_Req.io_Actual );

6.5  ERRORS

When you are using the trackdisk device you will most definitely

receive many errors. The "io_Error" field of the requestblock

will contain 0 if the request was successfully executed, but if

something failed it will contain one of the following error

numbers: (Defined in the header file "devices/trackdisk.h".)

Error code           Description

-------------------------------------------------------------

TDERR_NotSpecified   A strange error has occurred.

TDERR_NoSecHdr       Could not find a sector.

TDERR_BadSecPreamble Strange sector.

TDERR_BadSecID       Also a strange sector.

TDERR_BadHdrSum      Strange header.

TDERR_BadSecSum      Strange data areas.

TDERR_TooFewSecs     Missing some sectors.

TDERR_BadSecHdr      Yet another strange sector.

TDERR_WriteProt      The disk is write protected.

TDERR_DiskChanged    The disk has been removed/changed.

TDERR_SeekError      Could not reach track zero.

TDERR_NoMem          Not enough memory.

TDERR_BadUnitNum     Wrong unit number.

TDERR_BadDriveType   Strange drive type.

TDERR_DriveInUse     Someone is already using the drive.

TDERR_PostReset      Tea time, the system is resetted.

There exist also four general device errors which all devices

may use: (Defined in header file "exec/errors.h".)

IOERR_OPENFAIL       Could not open the device.

IOERR_ABORTED        Request aborted.

IOERR_NOCMD          Not a valid command.

IOERR_BADLENGTH      Bad length or value.

-------------------------------------------------------------

6.6  EXAMPLES

Example 1

This program will use the Trackdisk Device to turn on and off

the internal disk drive's motor.

Example 2

This program demonstrates how you can check what went wrong

while you were using the Trackdisk Device. This example will

try to use drive DF3:, which most of us does not have, and

thus we will receive an error message. (Well if you have four

disk drives connected to your Amiga there will not be any

error message.)

Example 3

This example demonstrates how you can read data with help of

the Trackdisk Device. You give this program four arguments

(drive, head, cylinder and sector), and it will print out all

data in that sector. You only have to expand this program a

little and you will end up with a nice disk viewer.

Example3 drive (0-3) head (0-1) cylinder (0-79) sector (0-10)

Example 4

This example contains a lot of small and useful functions

that does almost everything you ever would like to do with

the trackdisk device. The example has been written so you can

easily use the functions in your own programs.