CPUClear V3.100

COPYRIGHT

This program is based on the original  CPUClear-version by Oliver

Wagner. The few changes I wanted to make have grown into a nearly

completely rewritten program.

My version of CPUClear is Freeware, so anybody may use it without

any charge. You are welcome to give this program to your friends,

as long as you do not charge any money for  it.  All  rights  are

reserved to MAXON Computer GmbH, Germany.

WHAT IS IT ?

CPUClear is a small hack, inspired by  CPUBlit, that replaces the

BltClear()-Routine  of  the  "graphics.library"  with  a   highly

optimized 68020 (or higher) version.  This  gains  (in  a  simple

test) approx. 60% speed and should be even faster  on  an  A3000,

because the CPU works more efficiently with 32-Bit ChipRAM.

NEW FEATURES

A complete list of  changes  since  V1.0  can  be  found  in  the

source-code (CPUClear.asm). But the main change is, that CPUClear

got faster and faster! Especially  this  V3.1-version  should  be

approx. 30% faster than V2.0. Unfortenatly, this isn't  reflected

in the easy SpeedTest-programm, included in the archive.

CPUClear V3.1 is under bad  circumstances  (not  longword-aligned

address and blocksize) much faster than all previous releases.

INSTALLATION

All you have to do to install the patch is to  call  CPUClear  in

your User-Startup! (No  RUN  is  needed.)  Executing  CPUClear  a

second time will remove the patch again. (If you  have  installed

another BltClear-Patch while  mine  is  running,  CPUClear  can't

idenitify the routine and will install itself again...)

SYSTEM REQUIREMENTS

CPUClear requires at least Kickstart V37.xxx and a 68020 CPU.  If

these requirements aren't fullfilled, CPUClear will quietly fail.

THE UNSECURE VERSION

Everybody who prefers this, may  use  the  "unsecure"-version  of

CPUClear, which lacks most security checks plus  the  68020-test.

Additionally,  I  reduced  the  screen  output,  which  made  the

executable some bytes shorter.

THE '40 VERSION

On machines, equipped with a 68040-CPU, the clear-process is done

in the copyback-cache and not in the "real" RAM. This  will  make

the DMA (screendisplay) getting wrong values!

This  can  be  avoided,  using  the  MMU  to  mark   ChipRAM   as

non-cachable. Amigas, running such a patch can use  the  ordinary

CPUClear-Version.

But if you do  not  run  this  patch,  you'll  need  the  special

68040-versions  included  in  this  archive,  which   flush   the

copyback-cache after clearing the memory!  Of  course  this  will

reduce the performance...

THE AUTHOR

If you have any comments for me or just want to tell me that  you

hate my program, don't hesitate to connect me under  one  of  the

following addresses:

Snail-Mail: Peter Simons      E-Mail: simons@peti.GUN.de (Usenet)

Europaring 20             2:242/7.25       (FIDO-Net)

D-5300 Bonn 1

Germany            Voice: Tel. 0228 / 746061