Personal Communications Library

For Borland/Turbo Pascal

(PCL4P)

USERS MANUAL

Version  5.1

June 5, 1996

This software is provided as-is.

There are no warranties, expressed or implied.

Copyright (C) 1996

All rights reserved

MarshallSoft Computing, Inc.

Post Office Box 4543

Huntsville AL 35815

Voice : 205-881-4630

FAX : 205|880|0925

BBS : 205-880-9748

email : help@marshallsoft.com

Anon FTP : ftp.marshallsoft.com

web : www.marshallsoft.com

_______

____|__     |                (R)

--+       |    +-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

--+--+   |   +---------------------

|___|___|    MEMBER

PCL4P Users Manual                                                Page 1

C O N T E N T S

Chapter                                                   Page

1.0 Introduction................................................3

1.1 User Support............................................4

1.2 ASP Ombudsman...........................................4

1.3 A Typical Application...................................5

1.4 Installation............................................6

2.0 Library Organization........................................7

2.1 Configuration...........................................7

2.2 Initialization & Termination............................7

2.3 Modem Control & Status..................................8

2.4 Serial I/O..............................................8

2.5 Error Detection.........................................9

2.6 General Support.........................................9

3.0 Library Overview...........................................10

3.1 Using the Library......................................10

3.2 Protected Mode.........................................10

3.3 Application Notes .....................................11

3.3.1 Terminal Programs................................11

3.3.1 Door Programs....................................11

3.3.2 BBS Programs.....................................11

3.4 Compiling & Linking....................................12

3.5 Turbo Vision...........................................12

4.0 Talking to Your Modem......................................13

4.1 Modem Standards........................................13

4.2 Flow Control...........................................14

4.3 MODEM_IO functions.....................................14

4.4 Modem Initialization...................................15

5.0 Problems...................................................16

6.0 Serial Communications......................................17

6.1 Communications Basics..................................17

6.2 Standard Port Addresses................................18

6.3 Running 3 or 4 Ports Concurrently......................19

6.4 Using Multiport Cards..................................20

6.4.1 The DigiBoard....................................20

6.4.2 The BOCA Board...................................20

6.5 Transmitter Interrupts.................................21

6.6 RS232 Signals..........................................22

6.7 National INS8250, INS16450, and INS16550 UARTs.........23

6.8 Register Summary.......................................24

7.0 Example Programs...........................................26

7.1 MINIMAL................................................26

7.2 SIMPLE.................................................26

7.3 LOGIN..................................................26

7.4 DOOR...................................................26

7.5 SELFTEST...............................................26

8.0 Legal Issues...............................................27

8.1 Registration...........................................27

8.2 License................................................27

8.3 Warranty...............................................28

9.0 Summary....................................................29

9.1 Revision History.......................................29

9.2 Function Summary.......................................31

9.3 Further Reading........................................31

10.0 Other MarshallSoft Computing products for Pascal...........32

10.1 The Personal Protocol Library for Pascal..............32

10.2 The Personal Communication Library for DELPHI.........32

11.0 Other MarshallSoft Computing shareware products............32

PCL4P Users Manual                                                Page 2

1.0 Introduction

The  Personal  Communications Library for Borland/Turbo Pascal (PCL4P) is an

asynchronous  communications  library  designed  for  experienced   software

developers programming in Turbo or Borland Pascal. The PCL features:

o 36 communications and support functions.

o Supports 16-bit protected mode [requires Borland Pascal].

o Support for the high performance 16550 UART.

o Supports hardware (RTS/CTS) flow control.

o Interrupt driven receiver and transmitter.

o Supports 300 baud to 115,200 baud.

o Supports the DigiBoard PC/4 and PC/8.

o Supports the BOCA BB1004, BB1008, and BB2016 boards.

o Supports COM1 through COM8 (through COM20 with multiport board)

o Adjustable receive queues from 8 bytes to 32 KB.

o Control-BREAK error exit.

o Use IRQ2 through IRQ15 with any UART address.

o 18 communications error conditions trapped.

o Allows 4 ports to run concurrently (20 with multiport board).

o Complete modem control & status.

o Written in assembly language for small size & high speed.

o Terminal program featuring ASCII (with XON/XOFF), XMODEM, YMODEM,

and YMODEM-G. [Source in protocol library PPL4P].

Why should you buy PCL4P ?  Several good reasons are:

COMPLETE - PCL4P is complete since it provides absolute control

of the serial ports (including the high  performance

INS16550).

COMPACT - PCL4P  is  very compact  at  less  than  8  KB. Your

application doesn't carry a lot of excess code.

FAST - PCL4P is fast since it will  run  at  38400  baud  on

even  slow  8088 PCs (4.77 MHZ) and at 115200 baud on

most everything else.

SUPPORT - If you get stuck,  you talk  to  the programmer  that

wrote the code, not a  person  hired  to  answer  the

phone.

BBS - A BBS is available (2400 to 14400 baud, N81) in order

to provide immediate support as necessary. You can also

email us at help@marshallsoft.com.

NEWSLETTER - MSC  newsletter discusses communications problems and

solutions (published quarterly).

PRICE - You get PCL4P for a very reasonable price ($75) !

UPGRADES - Once you buy PCL4P, you can always update to  the  most

recent version very inexpensively ($25 plus shipping).

PCL4P Users Manual                                                Page 3

1.1 User Support

We want you to be successful in developing your applications using PCL4P! We

depend  on  our  customers to let us know what they need in a communications

library.  This means we are committed to providing the  best  communications

library  that we can. If you have any suggestions or comments, please let us

know!

We provide customer support for registered users by voice, FAX, BBS,  email,

and  mail.   We  provide  limited  support  for unregistered users by voice,

email, and BBS only.

If  you  are  having  a problem using PCL4P, call us at 205-881-4630 between

1:30 PM and 9:30 PM (CST) Monday through Friday. You can also call at  other

times and leave a message, and call back later for a reply. Registered users

(ONLY) can also FAX us at 205-880-0925 at any time (24 hours). You can  also

email us at help@marshallsoft.com.

However,  we  can  only  answer  questions  with  respect to using the PCL4P

library.  We cannot help you program your application, but we'll be glad  to

discuss it with you.

You  may  also  call  our User Support BBS (2400 to 14400 baud, no parity, 8

data bits, 1 stop bit) at 205-880-9748 and leave a message  (address  it  to

the SYSOP).  We will usually have a reply ready for you within 24 hours.

The BBS is available 24 hours per day. All files are in standard ZIP format.

The  BBS  will  contain  the  latest  shareware  version of all MarshallSoft

Computing products as well as related files such as:

BUGS.ZIP     -  Bug report.

NEWS.ZIP     |  Latest news regarding our products.

PRODUCTS.ZIP -  List of all shareware products.

The MarshallSoft  Computing,  Inc.   newsletter  "Comm  Talk"  is  published

quarterly.  It discusses various communications problems and solutions using

PCL4P  as  well  as related information. "Comm Talk" is published on our Web

site www.marshallsoft.com and is available in text form on our BBS  and  FTP

site (ftp.marshallsoft.com/marshallsoft).

1.2 ASP Ombudsman

MarshallSoft Computing, Inc.  is a member of the  Association  of  Shareware

Professionals  (ASP).   ASP  wants to make sure that the shareware principle

works for you.  If you are unable to  resolve  a  shareware-related  problem

with  an  ASP  member  by contacting the member directly, ASP may be able to

help. The ASP Ombudsman can help you resolve a dispute or  problem  with  an

ASP  member,  but  does not provide technical support for members' products.

Please write to the ASP Ombudsman at  545  Grover  Road,  Muskegon,  MI  USA

49442-9427,  Fax  616-788-2765,  or send a CompuServe message via CompuServe

Mail to ASP Ombudsman 70007,3536.

PCL4P Users Manual                                                Page 4

1.3 A Typical Application

In general, there are two classes of applications that use a  communications

library  like PCL4P - those that use a modem to connect to the outside world

and those that connect directly to a peripheral device. In  either  case,  a

typical  application  program using PCL4P might look like the following code

outline:

+----------------------------------------------------------------+

|                                                                |

| program YourProgram                                            |

|                                                                |

| uses PCL4P;                                                    |

| ...                                                            |

| var BufPtr : Pointer;                                          |

|     BufSeg : Integer;                                          |

|                                                                |

| ...                                                            |

| begin (* YourProgram *)                                        |

|    (* allocate RX buffer memory *)                             |

|    GetMem(BufPtr,1024+16);                                     |

|    BufSeg := (Seg(BufPtr^)+1) + (Ofs(BufPtr^) SHR 4);          |

|    RetCode := SioRxBuf(Port,BufSeg,Size1024);                  |

|    (* allocate TX buffer memory *)                             |

|    GetMem(BufPtr,128+16);                                      |

|    BufSeg := (Seg(BufPtr^)+1) + (Ofs(BufPtr^) SHR 4);          |

|    RetCode := SioTxBuf(Port,BufSeg,Size128);                   |

|    (* initialize port *)                                       |

|    RetCode := SioParms(Port,NoParity,OneStopBit,WordLength8);  |

|    RetCode := SioReset(Port,Baud2400);                         |

|    ...                                                         |

|    ... ( application code )                                    |

|    ...                                                         |

|    RetCode := SioDone(Port);                                   |

| end. (* YourProgram *)                                         |

+----------------------------------------------------------------+

In  the  above example, SioRxBuf is called to set up the a 1024 byte receive

buffer. SioTxBuf is called to  set  up  the  128  byte  transmitter  buffer.

SioParms  is  called  to set up the parity, stop bit count, and word length.

SioReset is called to  set  the  baud  rate  to  2400  and  reset  the  UART

(Universal Asynchronous Receiver / Transmitter).

Before leaving your application, SioDone is  called  to  restore  the  prior

state of the serial communications system.

If  you  are using a modem, you also need to be concerned about initializing

your  modem  correctly  and  handling  any required flow control.  Refer  to

Chapter 4.0, "Talking to Your Modem" for detailed information.

PCL4P Users Manual                                                Page 5

1.3 Installation

(1) Before installation of PCL4P, your compiler should already be  installed

on  your system and tested. If you are not familiar with makefiles, refer to

your compiler manual.  Examine the file "FILES.LST" for a list  of  all  the

distribution files.

(2)  Make  a  backup  copy  of  your  distribution  disk.  Put your original

distribution disk in a safe place.

(3) Create a work directory on your work disk (normally your harddisk).  For

example,  to create a work directory named PCL4P, we first log onto the work

disk and then type:

MKDIR PCL4P

(4) Copy all the files from your backup copy of  the  distribution  disk  to

your  work  directory.   For example, to copy from the A: drive to your work

directory, we type:

CD PCL4P

COPY A:*.*

(5) The library unit must be created by running:

TP_REAL.BAT : Creates PCL4P.TPU [real mode] using Turbo Pascal.

BP_REAL.BAT : Creates PCL4P.TPU [real mode] using Borland Pascal.

BP_PROT.BAT : Creates PCL4P.TPP [prot mode] using Borland Pascal.

(6) Compile SIMPLE.PAS:

TPC SIMPLE  - if using Turno Pascal.

BPC SIMPLE  - if using Borland Pascal.

SIMPLE.PAS should compile without any problems.

(7)  The  recommended  way  to  test  SIMPLE  is  to run it on two computers

connected by a null modem cable.  Whatever is typed on one  computer  should

be displayed on the other. SIMPLE can also be tested by connecting your port

to a modem.

(8)  Compile  and  run the remaining example programs. In partcular, compile

the protected mode program SIMPLE16.PAS if using Borland Pascal.

BPC /CP SIMPLE16.PAS

PCL4P Users Manual                                                Page 6

2.0 Library Organization

The  PCL4P  library  is organized into six categories of functions. Refer to

the PCL Reference Manual (PCL4P.REF) for details on individual functions.

2.1 Configuration

There are three functions in the configuration category.  SioPorts  is  used

to  set  the number of PC ports, specify the first DigiBoard (or BOCA board)

port, and set the DigiBoard (or BOCA board) status register address. SioUART

is used to change the UART base address  for  a  communications  port  to  a

non-standard  address, while SioIRQ is used to assign a nonstandard IRQ line

to a port.  (See Chapter 6.0, Serial  Communications  for  more  details  on

standard UART addresses and IRQ lines).

The  configuration  functions  SioPorts,  SioUART  and SioIRQ must be called

before calling any other library functions.  Be very careful in using  these

functions.  Remember that your serial hardware must support the UART and IRQ

that you specify.

SioPorts - Set # PC ports, 1st DigiBoard (or BOCA board) port.

SioUART  | Sets the UART base address.

SioIRQ   - Assigns an IRQ line to a port.

THE IRQ GOLDEN  RULE:  You  may  open (via SioReset) only one port per IRQ

(except for the DigiBoard and BOCA board).

2.2 Initialization & Termination

There are eight functions in the initialization  and  termination  category.

Together,  SioParms,  SioFIFO,  SioRxBuf,  SioTxBuf, and SioReset initialize

your serial communications system.  Your application must call SioParms  and

SioRxBuf  before  calling  SioReset,  and SioReset must be called before any

serial I/O processing can be done.

After initialization, SioParms and SioBaud can be called again to change the

communications parameters without resetting the serial port. SioFlow can  be

called to enable hardware flow control.

Before  exiting  from  your  application, SioDone must be called. Failure to

call SioDone can crash your system later.

SioRxBuf   - Sets up receive buffer.

SioTxBuf   | Sets up transmitter buffer.

SioFIFO    | Sets the interrupt level for the INS16550.

SioParms   | Sets parity, stop bits, and word length.

SioReset   | Initialize a serial port for processing.

SioDone    | Terminates further serial processing.

SioBaud    | Sets the baud rate of the selected port.

SioFlow    - Enables / disables flow control.

PCL4P Users Manual                                                Page 7

2.3 Modem Control & Status

There are nine functions in the modem  control  and  status  category  which

provide  your  application with complete control over the status and control

bits of your modem.

There are two modem control bits, "Data Terminal Ready" (DTR)  and  "Request

To  Send"  (RTS).  These  bits  can  be  read, set, or cleared by SioDTR and

SioRTS.

There are four modem status bits, "Data Set Ready" (DSR),  "Clear  To  Send"

(CTS),  "Ring Indicator" (RI), and "Data Carrier Detect" (DCD). SioModem can

read any of the modem status bits.  SioDSR, SioCTS, SioRI,  and  SioDCD  can

only  read  their respective modem status bit. SioGetDiv reads the baud rate

divisor register so that the baud rate can be determined. SioRead reads  any

of the 7 UART registers.

Refer  to  the  chapter entitled "RS232 Signals" for a discussion of each of

the control and status bits.

SioDTR    - Set, clear, or read the Data Terminal Ready (DTR) bit.

SioRTS    | Sets, clears, or reads the Request to Send (RTS) line.

SioModem  | Reads the modem status register.

SioDSR    | Reads the Data Set Ready (DSR) modem status bit.

SioCTS    | Reads the Clear to Send (CTS) modem status bit

SioDCD    | Reads the Data Carrier Detect (DCD) modem status bit.

SioRI     | Reads the Ring Indicator (RI) modem status bit.

SioRead   | Reads the contents of the 7 UART registers.

SioGetDiv - Reads the baud rate divisor registers.

2.4 Serial I/O

There  are  nine  library  functions  in the serial I/O category.  Together,

these functions give the programmer complete control over serial I/O. Higher

level functions such as protocols and  smart  modem  communications  can  be

completely  implemented  in  terms of these functions.  Refer to the example

code.

SioGetc  and  SioPutc perform all the actual serial I/O.  SioUnGetc "ungets"

the last serial byte  read.   SioRxClear  clears  the  receive  queue  while

SioTxClear  clears  the  transmit queue.  SioTxFlush forces all bytes in the

transmit queue to be transmitted. SioRxQue returns the number  of  bytes  in

the receive queue while SioTxQue returns the number of bytes in the transmit

queue. SioLine can be used to test for UART errors.

SioGetc    - Reads the next character from the serial line.

SioPutc    | Transmit a character over a serial line.

SioUnGetc  | "Un-gets" (puts back) a specified character.

SioRxClear | Clears the receive buffer.

SioRxQue   | Returns the number of characters in the receive queue.

SioTxClear | Clears the transmit buffer.

SioTxFlush | Flushes the transmit buffer.

SioTxQue   | Returns the number of characters in the transmit queue.

SioLine    - Reads the line status register.

PCL4P Users Manual                                                Page 8

2.5 Error Detection

There are four functions in the error detection category. They are concerned

with  detecting  or reporting communications errors.  Use of these functions

can make your application significantly more robust.

SioBrkKey can  be  used  as  an  "emergency"  exit  from  your  application.

SioBrkSig  can  read  or  modify  the  UART  break  bit.  This is useful for

signalling  the  remote  system  that  a  fatal  condition   has   occurred.

SioLoopBack  can  be  used  to  test  the  integrity of your UART.  SioError

displays a error message corresponding to an  error  code  returned  from  a

PCL4P function.

SioBrkKey - Returns non-zero if the Control-BREAK key was pressed

SioBrkSig | Asserts, cancels, or detects  BREAK  signal.

SioError  - Displays  error  in  text.

SioLoopBack - Performs a UART loopback test.

2.6 General Support

There are three functions in the general support category.  Strictly speaking,

they  are not communications functions, but they take up a very small amount

of additional memory.  Registered users can remove these functions from  the

library if needed.

SioInfo     - Returns the library version, RX & TX interrupts, etc.

SioTimer    | Returns the number of system clock tics.

SioDelay    - Delays one or more timer tics (18.2 tics per second).

PCL4P Users Manual                                                Page 9

3.0 Library Overview

3.1 Using the Library

The  PCL4P  library  has  been  tested on a Tandy 1000 (4.77 MHZ 8088 IBM PC

clone), a Tandy 3000 (80286 IBM AT clone), a Tandy 1400LT (IBM XT clone),  a

Gateway  2000 386(25 MHZ 80386-DX), and a Gateway 2000 486(66 MHX 80486-DX).

PCL4P has been tested under MSDOS 2.11, 3.2, 3.3, 4.01, 5.0, 6.0,  and  6.2.

The  PCL4P  library has also been compiled and tested using Turbo Pascal 6.0

and 7.0 as well as Borland Pascal 7.0.

Please examine the PCL4P.PAS file. Note that COM1 is defined as  port  zero,

not  port  one.   The  user  must  assume  the responsibilty for passing the

correct information when calling PCL4P functions.

3.2 Protected Mode

Protected mode programming takes advantage of the segmented architecture  of

286  and  up  Intel processors.  In contrast to normal real mode programming

which limits programs to an address space of 1 MB (1024 KB), protected  mode

allows  up  to  16 MB (in 16-bit protected mode). Protected mode also offers

some measure of protection of one program from another.

A program that can excecute in  protected  mode  under  DOS  requires  three

modules:

(1) A DPMI (DOS Protected Mode Interface) driver loaded in memory.

(2) A protected mode runtime library linked to your program.

(3) A runtime module which provides protected mode equivalents of:

a) A subset of DOS interrupt 21H calls.

b) A subset of BIOS calls.

c) Additional functions such as protected mode memory management.

The  Personal  Communications Library (PCL4P16.TPP) supports protected mode.

Be sure to create PCL4P16.TPP and USE_DPMI.TPP by  running  the  batch  file

BP_PROT.BAT.   An  application program can then be compiled with the Borland

Pascal compiler, which supports 16-bit protected mode.

Examine the example program SIMPLE16.PAS, which is a protected mode  version

of  the  normal  DOS  based  SIMPLE.PAS. Compare it to SIMPLE.PAS to see the

differences necessary to run in protected mode.

In particular, note the manner in which memory is  allocated  for  use.  The

transmit  and  receive  buffers must be allocated in conventional DOS memory

(the first 1MB) since serial interrupts can occur during both protected mode

and real mode.

Compile the protected mode program SIMPLE16.PAS using

BPC /CP SIMPLE16.PAS

PCL4P Users Manual                                                Page 10

3.3 Applications Notes

3.3.1 Terminal Programs

The  "terminal  program" is the most common class of communications program.

It is used to call up a BBS or on-line service such as  CompuServe,  America

On-Line,  etc.   Refer  to  the  programs  SIMPLE and LOGIN in section 7 for

examples of simple terminal programs.

A  more  sophisticated terminal program featuring ASCII, XMODEM, YMODEM, and

ZMODEM protocol file transfers can be found in our  sister  product  --  The

Personal  Protocol Library (PPL4P). Source code is included in the shareware

distribution for everything except ZMODEM and the script interpreter.

3.3.2 Door Programs

In order to write a door program which "takes over" a  serial  port  without

resetting  the  port or changing the baud rate, call SioReset() with NORESET

as the second argument rather than the baud rate. Call  SioGetDiv()  to  get

the  baud  rate divisor if the baud rate must be determined. Be sure to call

SioDone() before returning to the invoking program.  Refer to  the  DOOR.PAS

example program.

3.3.3 BBS Programs

If you are designing a BBS program (also known as HOST  programs),  consider

using  16550  UARTS.   You  should  also choose a multiport card such as the

DigiBoard or BOCA board if you wish to run more than 4 ports simultaneously.

If  you  are using an error correcting modem, then you should be sure to set

flow control and fix your baud rate at the highest possible transfer  rates.

For  14,400  modems, this means 19200 or 38400. For 28,800 modem, this means

38400 or 57600. Consult your modem manual for the correct  baud  rate.   You

may also need a 16550 UART in order to run at the higher speed.

If you are using an older multi-speed modem (say  1200,  2400,  4800,  9600)

that doesn't use flow control, you should change your baud rate to match the

CONNECT message baud rate.

PCL4P Users Manual                                                Page 11

3.4 Compiling and Linking

The  Personal  Communications  Library for Pascal supports both Turbo Pascal

and Borland Pascal. Substitute BCP for  TPC  in  the  following  if  Borland

Pascal is being used.

Registered  users  may  wish  to assemble PCL4P.ASM. To create the real mode

library object with transmitter interrupts enabled:

TASM PCL4P.ASM,PCL4P.OBJ /DPASCAL_MODEL;

TPC PCL4P.PAS

To create the real mode library object with transmitter interrupts disabled:

TASM PCL4P.ASM,PCL4P.OBJ /DPASCAL_MODEL /DNO_TBE;

TPC PCL4P.PAS

For protected mode [Borland Pascal only]:

TASM USE_DPMI.ASM /MX;

TASM PCL4P.ASM,PCL4P16.OBJ /DPASCAL_MODEL /DDPMI /MX;

To create the real mode library PCL4P.TPU

TPC PCL4P.PAS

To create the protected mode [Borland Pascal only] library PCL4P16.TPP

BPC /CP USE_DPMI.PAS

BPC /CP PCL4P16.PAS

To compile the sample programs:

TPC SIMPLE.PAS

To compile the protected mode [Borland Pascal only] program SIMPLE16.PAS

BPC /CP SIMPLE16.PAS

3.5 Turbo Vision

The PCL4P library can also be used in Turbo  Vision  programs.  Examine  the

TVTERM.PAS for a simple example program analogous to the SIMPLE.PAS program.

TVTERM  copies any keyboard input to the serial port and copies any incoming

serial port data to the screen. Type Alternate-X to exit.

PCL4P Users Manual                                                Page 12

4.0 Talking to Your Modem

A modem is used to extend the  distance  over  which  you  may  communicate.

Without  a  modem, your RS232 cable is limited to a maximum of approximately

50 feet.  But with a modem, you can communicate literally around the world.

Also refer to the Section 4.3, "MODEM_IO  functions"  for  details  on  MODEM_IO

functions. These functions facilitate communications with modems.

4.1 Modem Standards

Two modems can communicate over a telephone line only if they are both using

the same signaling frequencies and modulation, which are determined  by  the

the  modem  standards used.  Modem standards can be divided into three sets:

(1) speed, (2) data compression used, and (3) error control.

The Bell standards  (103  &  212A)  are  those  of  AT&T.   The  CCITT  (The

International  Consultative Committee for Telephone and Telegraph) standards

are designated as "V. ".

Speed

Bell 103  -   300 baud

Bell 212A |  1200 baud

V.21      |   300 baud

V.22bis   |  1200 & 2400 baud

V.32      |  4800 & 9600 baud

V.32bis   |  4800, 7200, 9600, 12000, and 14400 baud

V.34      -  through 28800 baud

Data Compression

MNP 5     -  Microcom Networking Protocol (proprietary).

V.42bis   -  International data compression standard.

Error Control

MNP 2,3,4 -  Three level error correction (public domain).

V.42      -  International error correction standard.

Most  of  the  newer  high  speed modems use several of the above standards.

However, not all combinations of modem makes communicate  easily  with  each

other, especially at high speed (9600 and up).

PCL4P Users Manual                                                Page 13

4.2 Flow Control

With modems using data compression, the modem to modem connection  will  run

at  various  speeds  depending  on the quality of the line.  The computer to

modem connection will be at a fixed baud rate. Therefore, a  protocol  (flow

control)  is  necessary to synchronize the data flow between a modem and the

computer to  which  it  is  connected.   Refer  to  your  modem  manual  for

information on flow control protocols supported.

Two  flow  control  protocols  are  used  by  most modems which require flow

control. Software flow control is called  "XON/XOFF"  (other  software  flow

control  character  pairs  are defined but operate the same as XON/XOFF) and

hardware flow control is called "RTS/CTS".  Most modems which  require  flow

control enable hardware flow control by default.

In XON/XOFF (software) flow control, the computer suspends transmitting data

if  it  receives  a  XOFF  character  (13 hex) from the modem, and continues

transmitting when it receives a XON character  (11  hex).   Similiarly,  the

computer  can  signal  the modem not to send any more data by transmitting a

XOFF to it, and can tell the modem to continue  transmission  be  sending  a

XON.

In  RTS/CTS (hardware) flow control, the RTS line is used by the computer to

signal the modem , while the CTS line is used by the  modem  to  signal  the

computer.   The  RTS  line  is  set OFF by the computer to tell the modem to

suspend  transmission,  and  set  to  ON  to  tell  the  modem  to  continue

transmission.   The CTS line is set to OFF by the modem to tell the computer

to stop transmitting, and set  to  ON  to  tell  the  computer  to  continue

transmitting.

Given  the  choice,  always  choose hardware flow control over software flow

control so that all data transmission  is  transparent.   If  hardware  flow

control  is  not  the default (which it almost always is), you should modify

your modem initialization string to turn hardware flow control on.

4.3 MODEM_IO Functions

The  file  MODEM_IO.PAS  contains  several functions that ease communicating

with your modem.  Look in the LOGIN.PAS code for examples of their use.

ModemSendTo   : Sends string (including control chars) to the modem.

ModemWaitFor  : Waits for a particular string from the modem, passing

all else through.

ModemQuiet    : Waits for continuous quiet of specified duration.

ModemHangup   : Hangs up the modem.

ModemCmdState : Goes into the modem's command state.

ModemEcho     : Echos all serial incoming bytes to the display.

PCL4P Users Manual                                                Page 14

4.4 Modem Initialization

If your application uses a modem (as opposed to using a null  modem  cable),

then  you should always send an initialization string to your modem if it is

a programmable modem such as those made by  Hayes.   Communication  programs

such as PROCOMM and TELIX always send such a string automatically as soon as

they start up.

The particular initialization string depends on the make of your modem.  For

Hayes  and  Hayes  AT  command  set  compatible modems, the following string

(followed by a carriage return) should work:

AT E1 S7=60 S11=60 V1 X1 Q0 S0=0

Refer to your Modem User's Guide for a full discussion of these commands.  A

brief description is as follows:

AT     Modem attention command.

E1     Modem will echo what you send to it.

S7=60  Wait 60 seconds for carrier and/or dial tone.

S11=60 Use 60 milliseconds for tone dialing duration & spacing.

V1     Display result code as words (not numbers).

X1     Use the extended result message (CONNECT XXXX) set.

Q0     Modem returns result codes.

S0=0   Do not answer RING.

If  your application will answer incoming calls, then set the S0 register to

the ring on which to automatically answer.

If you send the above codes by using SioPutc (as opposed to typing them from

the keyboard), then follow these guidelines:

(1) Send an initial carriage return before the initialization string.

(2) Pause at least two tics (18 tics to the  second)  after  each  character

sent  as  your  modem needs the time to perform its own internal processing.

Pause a little longer if your modem is  not  accepting  your  initialization

string.

(3)  Pause  one  and a half seconds after sending any initialization command

such as ATZ or AT&F since your modem must do quite a bit of processing.

If you experience any problems in initializing your Hayes modem, you  should

first reset it to factory settings by sending:

AT&F

Refer  to  the  TERM  program  (functions  SendTo  and  WaitFor  in the file

MODEM_IO.PAS) for an example of sending an initialization string to a  Hayes

compatible modem.

PCL4P Users Manual                                                Page 15

5.0 Problems

If  you  cannot  get your application to run properly, first compile and run

the terminal emulator program TERM provided on your distribution  disk.   If

you are using a null modem cable or a non-programmable modem, be sure not to

define  AT_COMMAND_SET  in DEFINES.PAS.  If you are using a Hayes compatible

modem, then do define AT_COMMAND_SET {$DEFINE AT_COMMAND_SET}.  If  you  are

using  a  programmable  modem  which  is not Hayes compatible, then you must

modify the initialization string for your particular modem.

If your application does not run but TERM runs correctly, then you have most

likely  made  a  programming  mistake  in  your  application.   MarshallSoft

Computing  cannot  debug  your  application,  especially over the telephone!

However, consider each of the following when searching for an error in  your

application.

1.  Did you include the "uses PCL4P" statement ?

2.  Is your receive buffer large enough ? If you are using 1K data blocks in

YMODEM,  then  your  receive buffer should be at least 1K ( 2K if baud rates

above 38400 are to be used ).

4.  Have you selected too high a baud rate ( if you are using a slow PC )  ?

If  only  one COM port is being run, you should be able to run at 38400 baud

on 8088 machines and 115200 on most 286 and all 386 and 486 machines.

5.  Are you attempting to run another application in the background  ?   Try

running  without  any  other programs running in the background ( unload all

TSR programs ).

6.  If you are running two COM ports simultaneously, are you using  separate

receive buffers ? ( you should ).

7.   Did SioReset return a zero value ?  If not, then you must call SioReset

again. See TERM.PAS for an example.

8.  Did you send the proper initialization string to your modem  ?  Did  you

set DTR and RTS ? ( you should ).

9.   Do  you  have more than one COM1 port, etc.  For example, if you have a

COM1 port on your motherboard, you cannot add another  COM1  port  or  modem

board that uses COM1 without first disabling the COM1 on the motherboard.

10. Are you passing the proper segment of the receive (or transmit) buffer?.

See SIMPLE.PAS or TERM.PAS for an example.

Registered  users  can  call  (205)  881  - 4630 from 1:30 PM to 9:30 PM CST

Monday through Friday for help.

PCL4P Users Manual                                                Page 16

6.0 Serial Communications

6.1 Communications Basics

The heart of serial  communications  is  the  UART  (Universal  Asynchronous

Receiver  Transmitter).   The  IBM PC/XT/AT and compatibles use the INS8250,

INS16450, or the INS16550 UART.  The purpose of the UART is:

(1) To convert bytes from the CPU (Central Processing Unit), into  a  serial

format  by  adding  the  necessary start, stop, and parity bits to each byte

before transmission, and to then transmit each bit at the correct baud rate.

(2) To convert the incoming stream (at a specified baud rate) of serial bits

into bytes by removing the start, stop, and parity bits  before  being  made

available to the CPU.

The  UART  is part of the serial interface circuitry which allows the CPU to

send and receive signals over the RS232 lines. This  can  be  diagrammed  as

follows:

Serial Interface

+-------------------+

|                   |

+-----+  Data Bus  |     +------+      |    RS232 Signals

| CPU +------------+     | UART |      +----------------*

+-----+            |     +------+      |

|                   |

+-------------------+

The  INS8250/16450/16550  UART  is capable of operating in one of two modes,

"polled" and "interrupt driven".  The serial communications functions in the

BIOS uses the polled method.  In this approach, the CPU is  typically  in  a

loop  asking  the  UART  over  and over again if it has a byte ready. If its

does, the polling code returns the byte.  But, if the  next  byte  comes  in

before the polling code is executing again, then that byte is lost.

In  the  interrupt driven approach (used by PCL4P for incoming data), when a

byte is received by the  UART,  an  "Interrupt  Service  Routine"  (ISR)  is

executed immediately, suspending temporarily whatever else is executing. The

ISR  then  moves  the  byte to a buffer so that your application program can

later read it.  Refer to Sections 6.6 and 6.7 entitled "RS232  Signals"  and

"National  INS8250,  INS16450  and INS16550 UARTs", respectively for further

information on these topics.

PCL4P Users Manual                                                Page 17

6.2 Standard Port Addresses

There  are  a  few  things to know about how serial communications ports are

used by IBM PC/XT/AT and compatible computers.  The  standard  IBM  PC/XT/AT

configuration values are as follows:

Port     Reg Base   IRQ Line   Vector

COM1        3F8H        4        12

COM2        2F8H        3        11

COM3        3E8H        4        12

COM4        2E8H        3        11

(Refer  to your DigiBoard manual for DigiBoard addresses, or your BOCA board

manual for BOCA port addresses).

PCL4P assumes the above values.  If necessary, the UART base address can  be

changed by SioUART, and IRQ lines can be re-assigned by SioIRQ. Refer to the

PCL4P Reference Manual for specific details.

When  installing  new  communications  cards,  the  following guidelines are

recommended:

(1) Be sure to read the documentation for the hardware you  are  installing.

Pay special attention to UART base addresses and IRQ lines.

(2)  If  you  have  a  choice in base addresses and IRQ lines, always choose

standard values as defined above.

(3) The first port should be COM1, the second COM2, etc.  Do NOT  skip  over

any port. This problem has caught several users.

(4)  Use SioUART to zero all unused ports (for example, call SioUART(COM4,0)

if there is no COM4 port installed).

(5) Be carefull not to configure two ports for the  same  address.  This  is

easier to do than you may believe.

(6)  Choose  an  external  modem over an internal one.  It is much easier to

debug problems with an external modem than an internal one.

(7) Select hardware flow control (RTS/CTS) if flow control is  required  and

hardware flow control is not the default.

(8) Always test your port as soon as it is installed.

PCL4P Users Manual                                                Page 18

6.3 Running 3 or 4 Ports Concurrently

PCL4P supports up to 4 serial ports running concurrently (more if you have a

DigiBoard or BOCA board).  One free interrupt for each port is required. The

following assumes that you don't have a DigiBoard.

Interrupts  IRQ4  and  IRQ3  are  dedicated to the communications ports in a

standard IBM PC/XT/AT configuration.  IRQ4 is shared between COM1  and  COM3

while IRQ3 is shared between COM2 and COM4.  This means that you can run two

ports simultaneously provided that they don't share an interrupt.

Suppose that you wish to run 3 ports simultaneously. To begin, you must have

3  serial  UARTs  installed  on your computer.  Assume, for purposes of this

discussion, that COM1 is installed on your motherboard, and  that  you  have

purchased a new 2 port serial communications board.

You  should  be able to configure the first serial board port as COM2, which

uses IRQ3.  Refer to the manual that came with your serial board.

In order to run the third serial port concurrently with  the  first  two,  a

unused  interrupt  must be found.  If your serial card can only use IRQ3 and

IRQ4, then there is no way to run a third line since IRQ4 and IRQ3 are  used

for COM1 and COM2.

However,  many serial cards can use other IRQs, typically IRQ2 through IRQ5.

Since IRQ5 is normally used  for  a  second  printer  port,  it  is  a  good

candidate  for  COM3.  To use IRQ5 for the third serial port, first set your

serial card to use IRQ5 for COM3 (refer to your serial card manual) and then

add the following line to your applications code before calling SioReset:

RetCode := SioIRQ(COM3,IRQ5);

Don't forget to disable any device that might use IRQ5,  such  as  a  second

printer  port  or  a  music  card.   Unfortunately,  there is no easy way to

determine that you have no conflicts until you actually attempt to  use  the

IRQ.  If  there  are  conflicts, your system will probably hang and you will

have to reboot.

To run a fourth serial port, another  free  IRQ  must  be  found.   On  some

systems, IRQ7 can be used. To use IRQ7 for the fourth serial port, first set

your serial card to use IRQ7 for COM4 and then add:

RetCode := SioIRQ(COM4,IRQ7);

To  summarize,  your  serial  card must be able to generate the correct IRQ,

which is not already being used. Refer to the entry for the SioIRQ  function

in the PCL4P Reference Manual.

PCL4P Users Manual                                                Page 19

6.4 Using the DigiBoard

PCL4P  supports the the dumb DigiBoard (PC/4 & PC/8) and the dumb BOCA board

(BB1004, BB1008, and BB2016).

6.4.1 The DigiBoard

Your PCs ports must be partitioned into "standard" PC  ports  and  DigiBoard

ports.  Remember that standard PC ports cannot share IRQs like the DigiBoard

can.  If  you  are  using IRQ4 and IRQ3 for standard PC ports COM1 and COM2,

then you cannot use either for DigiBoard ports (try IRQ5 or IRQ7).

Suppose that COM1 through COM2 are standard PC ports (using IRQ4  and  IRQ3)

and  you  have  installed  a  PC/8  DigiBoard  that you wish to use for COM3

through COM10 using interrupt line IRQ5.  You choose to use the  recommended

DigiBoard UART addresses starting at $100:

Code := SioPorts(10,COM3,$140,DIGIBOARD);

Address := $100;                (* 1st DigiBoard UART address *)

for Port:=COM3 to COM10 do      (* look at each port *)

begin

Code := SioUART(Port,Address);     (* set the UART address *)

Address := Address + 8;            (* next DigiBoard UART *)

Code := SioIRQ(Port,IRQ5);         (* set the IRQ & ISR *)

end

The  DigiBoard uses $140 for the status address for odd interrupts and 0x141

for even interrupts.

DigiBoard may be contacted at 6400 Flying  Cloud  Drive,  Eden  Prairie,  MN

55344.  Telephone 612-943-9020. FAX 612-943-5398.

6.4.2 The BOCA Board

PCL4P  supports  the  dumb  BOCA  board.  As  with  the  DigiBoard, you must

configure PCL4P before using the BOCA board.

For example, to configure the BOCA BB2016 to use COM1  to  COM16  with  base

addresses starting at $100 and using IRQ15:

Code := SioPorts(16,COM1,$107,BOCABOARD);

Address := $100;                (* 1st DigiBoard UART address *)

for Port:=COM1 to COM16 do      (* look at each port *)

begin

Code := SioUART(Port,Address);     (* set the UART address *)

Address := Address + 8;            (* next DigiBoard UART *)

Code := SioIRQ(Port,IRQ15);        (* set the IRQ & ISR *)

end

BOCA  may  be  contacted at BOCA Research, Inc., 6413 Congress Avenue, Suite

130, Boca Raton, FL 33487. Phone 407-241-8088, FAX 407-997-0918.

PCL4P Users Manual                                                Page 20

6.5 Transmitter Interrupts

Beginning  in  version 4.0 of PCL4P, transmitter interrupts are supported by

the  library.   Separate  libraries  are  provided,  one  with   transmitter

interrupts  enabled  (the  default  beginning  with  version  5.1)  and  one

without.  When transmitter interrupts are NOT enabled, the  following  logic

occurs everytime you call SioPutc:

1.  Wait  for  transmit buffer to become empty. The transmit buffer may not

be empty if the previous transmit is not completed (the UART breaks down the

byte & sends 1 bit at a time). 2. When the transmit  buffer  is  empty,  the

byte from the SioPutc call is loaded into the transmit buffer and control is

returned to the caller.

Note that you can not write to the UART any faster the the UART baud rate.

When transmitter interrupts are enabled, the byte from SioPutc is put into a

previously  prepared  (by SioTxQue) transmitter queue. The interrupt service

routine fetches bytes from this queue as soon as the previous byte has  been

sent.

While  you  can  now call SioPutc faster than the baud rate, bytes are still

transmitted at the given baud rate.

The   above   sounds   like  transmitter  interrupts  are  the  way  to  go.

Unfortunately, this is often NOT the case.  Most applications  will  perform

better if transmitter interrupts are NOT enabled.

The  reason  is that transmitter interrupts double the amount of code in the

time critical interrupt service routines.  While the library is processing a

transmitter interrupt (which can take a while), incoming bytes  can  not  be

processed.  What this means is that a given machine can run at a higher baud

rate without transmitter interrupts. This problem is compounded when running

multiple ports simultaniously.

However, there are some application areas where transmitter  interrupts  are

preferable.   If  your  application  will  be transmitting blocks of data at

fairly slow baud rates you might profit from enabling transmitter interrupts

provided that there is something else for the processor to do (which is  NOT

the case in most protocols under DOS).

The  default  library is with transmitter interrupts enabled.  To create the

PCL4P  library  without  TX  interrupts  enabled,  copy  "PCL4PN.OBJ"   over

"PCL4P.OBJ"  in  TP_REAL.BAT  (or  BP_REAL.BAT) and then execute TP_REAL (or

BP_REAL).

PCL4P Users Manual                                                Page 21

6.6 RS-232 Signals

RS-232 is the name of the serial data interface  standard  used  to  connect

computers  to modems.  Most IBM compatible computers are built with at least

one serial port and use either DB9 (9 pin) or DB25 (25 pin) connectors.

A summary of these pins and  their  function  follows.   For  more  detailed

information, refer to one of the many books dealing with RS-232 interfacing.

Signal Ground Pin 7 (DB25), Pin 5 (DB9)

The  SG  line  is  used  as  the  common  signal  ground, and must always be

connected.

Transmit Data Pin 2 (DB25), Pin 3 (DB9)

The TX line is used to carry data from the computer to the modem.

Receive Data Pin 3 (DB25), Pin 2 (DB9)

The RX line is used to carry data from the modem to the computer.

Data Terminal Ready Pin 20 (DB25), Pin 4 (DB9)

The DTR line is used by the computer to signal the modem that it  is  ready.

DTR should be set high when talking to a modem.

Data Set Ready Pin 6 (DB25), Pin 6 (DB9)

The DSR line is used by the modem to signal the computer that it is ready.

Request to Send Pin 4 (DB25), Pin 7 (DB9)

The  RTS  line  is used to "turn the line around" in half duplex modems, and

for hardware flow control in most modems that require flow control.

Clear to Send Pin 5 (DB25), Pin 8 (DB9)

The CTS line is used to "turn the line around" in half  duplex  modems,  and

for hardware flow control in most modems that require flow control.

Data Carrier Detect Pin 8 (DB25), Pin 1 (DB9)

The DCD line is used by the modem to signal the computer that a data carrier

signal is present.

Ring Indicator Pin 22 (DB25), Pin 9 (DB9)

The RI line is asserted when a 'ring' occurs.

PCL4P Users Manual                                                Page 22

6.7 National INS8250, INS16450, and INS16550 UARTs

The  Personal  Communications  Library  is  based  on  the standard National

INS8250, INS16450, and INS16550 UARTs. The 8250 was the original  UART  used

in  the IBM PC, whereas the 16450 is a faster version found on most 286 & up

machines. The 16550 contains a 16 byte FIFO to further reduce communications

overhead. These UARTs consists of 8 register ports as follows:

Offset    R/W   Register

0       R/W   Receiver (read) / Transmitter (write)

1       R/W   Interrupt Enable (read)

2       R     Interrupt Identification

2       W     FIFO control (INS16550 only)

3       R/W   Data Format (Line Control)

4       R/W   RS-232 (Modem) Control

5       R/W   Line Status

6       R/W   RS-232 (Modem) Status

7       R/W   Not used.

The UART registers are based at 3F8 (COM1), 2F8 (COM2), 3E8 (COM3), and  2E8

(COM4).  COM1  and COM3 share interrupt vector 12 and interrupt request line

IRQ4 while COM2 and COM4 share interrupt vector  11  and  interrupt  request

line  IRQ3.   This  means that COM1 and COM3 cannot both be used at the same

time. Similarly, COM2 and COM4 cannot both be used at the same time.

Port     Reg Base   IRQ Line   Vector

COM1        3F8H        4        12

COM2        2F8H        3        11

COM3        3E8H        4        12

COM4        2E8H        3        11

Four sources of interrupts  are  possible  with  the  8250  and  16550:  (1)

receiver error or BREAK, (2) receiver data ready, (3) ready to transmit, and

(4)  RS232  input.   These  four  sources  of  interrupts  are summarized as

follows:

Source of Interrupt        Action Required to Clear

Receiver error or BREAK.   Read Line Status register.

Receiver data.             Read data from data register.

Transmitter Buffer Empty.  Write to data register or read IID reg.

RS232 input.               Read Modem Status register.

However, PCL4P only enables the receiving data interrupt.  This  means  that

interrupts can only be caused by incoming data.

If you are not familiar with the INS8250, several good books are  available.

Refer  to  Section  9.3,  Further  Reading  for recommendations.  Although a

knowledge of the 8250 is not necessary to use PCL4P, a general knowledge  of

the theory of asynchronous serial communications is recommended.

PCL4P Users Manual                                                Page 23

6.8 Register Summary

REG 0 : Data Register

Reading from the data register fetches the  next  input  byte,  once  it  is

ready.   Writing  to the data register transmits the byte written to it over

the serial line.

REG 1 : Interrupt Enable

The Interrupt Enable register enables each of four types of interrupts  when

the appropriate bit is set to a one.

bit 3 : Enable  interrupt  on  RS232  input.

bit 2 : Enable interrupt on receiver error or break.

bit 1 : Enable  interrupt  on  transmitter  buffer empty(TBE).

bit 0 : Enable interrupt on received data (RxRDY).

REG 2 : Interrupt Identification (IID)

Reading  the Interrupt Identification (read only) register once an interrupt

has occurred identifies the interrupt as follows:

Bit 2  Bit 1  Bit 0  Priority    Interrupt

0      0      1      none      none

1      1      0      0 (high)  Serialization or break.

1      0      0      1         Received data.

0      1      0      2         Transmitter Buffer Empty.

0      0      0      3 (low)   RS232 Input.

In the INS16650, REG 2 (write only)  is  also  the  FIFO  control  register.

Writing bits 6 & 7 will set the FIFO trigger level (number of bytes received

before an interrupt is generated).

Bit 7  Bit 6   Trigger             Bit 7  Bit 6   Trigger

0      0      1 byte               1      0      8 bytes

0      1      4 bytes              1      1      14 bytes

REG 3 : Line Control

RS232 line parameters are selected by writing to this register.

bit 7 : DLAB = 0

bit 6 : BREAK on(1), off(0).

bits 5-3: Parity None(000),ODD(001),EVEN(011),MARK(101),SPACE(111)

bit 2 : One stop bit(0), two stop bits(1).

bits 1-0: Data bits = 5 (00), 6(01), 7(10), 8(11).

When  the Divisor Latch Access Bit (DLAB) is 1, registers 0 and 1 become the

LS and MS bytes of the Baud Rate Divisor registers.

Baud   Divisor      Baud  Divisor      Baud  Divisor

300    0180        4800   0018       38400   0003

1200    0060        9600   000C       57600   0002

2400    0030       19200   0006      115200   0001

PCL4P Users Manual                                                Page 24

REG 4 : Modem Control

RTS, DTR, loopback testing, and  General  Purpose  Outputs  #1  and  #2  are

controlled by the Modem Control register as follows:

bit 4 : Enable local loopback.

bit 3 : Enable GP02. Necessary for 8250 interrupts.

bit 2 : Enable GP01.

bit 1 : Set / clear RTS.

bit 0 : Set / clear DTR.

REG 5 : Line Status

Reading  the  Line Status register provides status information as follows (1

for TRUE, 0 for FALSE) :

bit 6 : Transmitter Empty.

bit 5 : Transmitter Buffer Empty (TBE).

bit 4 : BREAK detect.

bit 3 : Framing error.

bit 2 : Parity error.

bit 1 : Overrun error.

bit 0 : Data Ready.

REG 6 : Modem Status

Reading the Modem Status register provides the following status  information

(1 for TRUE, 0 for FALSE) :

bit 7 : DCD status.

bit 6 : RI status.

bit 5 : DSR status.

bit 4 : CTS status.

bit 3 : Delta DCD status.

bit 2 : Delta RI status.

bit 1 : Delta DSR status.

bit 0 : Delta CTS status.

The  delta  bits  (bits 0 through 3) are set whenever one of the status bits

(bits 4 through 7) changes (from 0 to 1 or from 1 to 0) since the last  time

that  the  Modem Status register was read. Reading the Modem Status register

clear the delta bits.

REG 7 : Scratch Register

There is no function associated with register 7.  It does not exist in early

versions of the 8250.

PCL4P Users Manual                                                Page 25

7.0 Example Programs

Five example programs are include with PCL4P.   In  addition,  the  Personal

Protocol Library for Pascal (PPL4P) includes the terminal program TERM which

features  ASCII,  XMODEM,  YMODEM,  and ZMODEM protocol transfers.  Complete

source code is included in the shareware product for all  of  the  protocols

above  except  ZMODEM.   In  order  to  get  ZMODEM  source,  PPL4P  must be

registered.

7.1 MINIMAL

MINIMAL is the simpliest possible communications program. It reads from  the

serial port & displays on the screen and reads from the keyboard & sends out

over the serial line. COM1 and 9600 baud are hard coded for simplicity.

7.2 SIMPLE

SIMPLE  is  a simple terminal program. It operates like MINIMAL, except that

you specify both a port and a baud rate. For example.

SIMPLE 1 9600

7.3 LOGIN

LOGIN is programmed to dial our support BBS (205-880-9748)  and  log  on  as

GUEST.  Start LOGIN like SIMPLE by providing a COM port and a baud rate. For

example,

LOGIN 1 38400

7.4 DOOR

The  DOOR program is a simple communications program which can "take over" a

serial port.  For example, say you  are  running  PROCOMM,  PCPLUS,  or  any

communications  program  with  a  DOS  gateway.  Select the DOS gateway from

PROCOMM (or whatever) and then type

DOOR 1

at the DOS prompt to take over port COM1.

7.5 SELFTEST

The SELFTEST program is designed to test your serial ports provided that you

have two ports which can be connected together  with  a  null  modem  cable.

SELFTEST can also be used to test your multiport board.

For example, to test PC port COM1 against COM2, type:

SELFTEST PC 1 2

SELFTEST  may  need  to  be  configured  for  non-standard  PC ports or your

multiport board. Refer to the SELFTEST source code for more information.

PCL4P Users Manual                                                Page 26

8.0 Legal Issues

8.1 Registration

If you wish to register the PCL4P library, please send $75 plus $5 S&H  ($10

outside of North America) to:

MarshallSoft Computing, Inc.

Post Office Box  4543

Huntsville AL 35815

Multiple copies are available: $55 for 3 to 9, $45 for 10 to 19, and $35 for

20  or more.  A site license is also available for $675 (includes 20 sets of

printed documentation). We pay shipping.

We  accept  VISA,  MasterCard,  and  American   Express   (account   number,

expiration  date, exact name on your card, and complete card billing address

required), checks in US dollars drawn on a US bank,  purchase  orders  (POs)

from recognized US schools and companies listed in Dun & Bradstreet, and COD

(street  address and phone number required) within the USA (plus a $4.50 COD

charge).

You can also order PCL4P from The Public Software Library  (PSL)  with  your

MC,  Visa,  AmEx,  or  Discover card by calling 800-242-4PSL (from overseas:

713-524-6394) or by FAX at 713-524-6398 or  by  CompuServe  at  [71355,470].

THESE  NUMBERS  ARE  FOR  ORDERING  ONLY.  PSL's product number for PCL4P is

10909.  Please have your shipping address and credit  card  billing  address

ready.

If you wish to update from an older version of PCL4P, send $25 plus  $5  S&H

($10  outside  of  North  America).   Updates  must be ordered directly from

MarshallSoft Computing.

The registered package includes:

o  Assembler source code for the library.

o  Printed Users Manual.

o  Printed Reference Manual.

o  Technical support for one year.

Print the file INVOICE.DOC if an invoice is needed. The registered user will

receive the latest version of PCL4P shipped by two day priority mail (packet

airmail overseas).  A 3.5" diskette is provided unless a 5.25"  diskette  is

requested.

PCL4P Users Manual                                                Page 27

8.3 License

MarshallSoft Computing, Inc. grants the registered user of PCL4P  the  right

to  use  one copy of the PCL4P library (in object form) on a single computer

in the development of any software product (other  than  libraries  such  as

PCL4P).  The  user  may not use the library on more than one computer at the

same time.  The source code for the library (PCL4P.ASM)  is  copyrighted  by

MarshallSoft and may not be released in whole or in part.

Products developed using PCL4P can include the object form  of  the  library

and may be distributed without any royalty.

8.4 Warranty

MARSHALLSOFT COMPUTING, INC.  DISCLAIMS  ALL  WARRANTIES  RELATING  TO  THIS

SOFTWARE,  WHETHER  EXPRESSED  OR  IMPLIED, INCLUDING BUT NOT LIMITED TO ANY

IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR  PURPOSE,

AND  ALL  SUCH WARRANTIES ARE EXPRESSLY AND SPECIFICALLY DISCLAIMED. NEITHER

MARSHALLSOFT COMPUTING, INC. NOR ANYONE ELSE WHO HAS BEEN  INVOLVED  IN  THE

CREATION,  PRODUCTION,  OR DELIVERY OF THIS SOFTWARE SHALL BE LIABLE FOR ANY

INDIRECT, CONSEQUENTIAL, OR INCIDENTAL DAMAGES ARISING OUT  OF  THE  USE  OR

INABILITY  TO  USE  SUCH  SOFTWARE EVEN IF MARSHALLSOFT COMPUTING, INC.  HAS

BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES OR CLAIMS. IN NO EVENT SHALL

MARSHALLSOFT COMPUTING, INC.'S LIABILITY FOR ANY SUCH  DAMAGES  EVER  EXCEED

THE  PRICE  PAID FOR THE LICENSE TO USE THE SOFTWARE, REGARDLESS OF THE FORM

OF THE CLAIM. THE PERSON USING THE SOFTWARE BEARS ALL RISK AS TO THE QUALITY

AND PERFORMANCE OF THE SOFTWARE.

Some states do not allow  the  exclusion  of  the  limit  of  liability  for

consequential  or  incidental damages, so the above limitation may not apply

to you.

This agreement shall be governed by the laws of the  State  of  Alabama  and

shall  inure  to  the  benefit  of  Marshallsoft  Computing,  Inc.   and any

successors, administrators, heirs and  assigns.  Any  action  or  proceeding

brought  by either party against the other arising out of or related to this

agreement shall be brought only in a STATE or  FEDERAL  COURT  of  competent

jurisdiction  located in Madison County, Alabama. The parties hereby consent

to in personam jurisdiction of said courts.

PCL4P Users Manual                                                Page 28

9.0 Summary

9.1 Revision History

Version 1.0 - 14 January 1991 - original release.

Version 1.1 - 11 March 1991

o  Added SioUnGetc() function to library.

Version 1.2 - 1 June 1991

o  Name changed to Personal Communications Library.

o  Minor bug fixes.

Version 1.3 - 1 July 1991

o  Added NORESET option to SioReset.

o  Added SioDSR, SioCTS, SioDCD, and SioRI.

o  Added SioLoopBack function to library.

o  Added LOOPBACK.PAS example program.

Version 2.0 - 1 Nov 1991

o  All example code released in shareware package.

o  Fixed bug due to Microsoft Assembler (MASM 5.0,5.1) error.

Version 3.0 - 15 Jan 1991

o  Added XMODEM & YMODEM to example code.

o  Added SioUART function.

o  Added "UART undefined" error code.

o  Added "Bad or missing UART" error code.

o  Added "Port already enabled" error code.

o  Added "Cannot enable both COM1 & COM3 ..." error code.

o  Fixed several minor bugs ( using new automated testing ).

Version 3.1 - 1 March 1992

o  Added SioFIFO ( INS16550 only ).

o  Added SioIRQ function.

o  Increased maximum receive buffer size to 32K bytes.

Version 3.2 - 1 May 1992

o  Modified SioReset so that it no longer clears DTR & RTS.

o  Modified SioModel & renamed to SioInfo.

o  Fixed bug in SioDone when using 2 ports simultaneously.

o  Added SioFlow to library.

o  Added YMODEM-G protocol to TERM program.

PCL4P Users Manual                                                Page 29

9.1 Revision History (continued)

Version 3.3 - 3 August 1992

o  Fixed bug in SioUnGet when using 2 ports simultaneously.

o  Add SioRead function.

Version 3.4 - 4 Jan 1993

o  Library modified to use up to four ports simultaneously.

o  SioIRQ was modified to include  third argument.

o  EXAMPORT utility distributed to registered users.

Version 3.5 - 15 May 1993

o  Supports DigiBoard PC/4 and PC/8.

o  Two new error traps added ("No such IRQ" & "No such ISR").

o  ASCII file transfer protocol added to TERM (with XON/XOFF).

Version 4.0 - 18 Oct 1993

o  The library supports transmitter interrupts.

o  Corrects bug in Ver 3.5 requiring calling SioIRQ for COM3/4.

o  The SioIRQ function has been simplified.

Version 4.1 - 1 May 1994

o  Transmitter FIFO enabled.

o  Minor internal modifications.

o  Supports dumb BOCA boards (BB1004, BB1008, & BB2016).

o  Port definition extended to COM16.

Version 4.2 - 1 Sept 1994

o  Flow contro bug fixed.

o  SioGetDiv function added.

o  SioRxBuf and SioTxBuf function modified.

Version 4.3 - 2 April 1995

o  BREAK detection bug fixed.

o  Port definitions extended to COM20.

o  Support for IRQ8 through IRQ15.

o  Line status bits preserved.

Version 5.1 - 5 June 1996

o  Supports 16-bit protected mode.

o  SioRxFlush renamed to SioRxClear.

o  SioTxFlush renamed to SioTxClear.

o  Added (new) SioTxFlush [forces TX].

o  Added more choices to SioInfo.

o  Modified ISR logic for improved reliability.

PCL4P Users Manual                                                Page 30

9.2 Function Summary

Refer to the PCL4P Reference Manual (PCL4P.REF) for detailed information  on

the  communications  and  support  functions.  A  one  line  summary of each

function follows:

SioBaud     Sets the baud rate of the selected port.

SioBrkKey   Returns non-zero if the Control-BREAK key was pressed.

SioBrkSig   Asserts, cancels, or detects BREAK signal.

SioCTS      Reads the Clear to Send (CTS) modem status bit.

SioDCD      Reads the Data Carrier Detect (DCD) modem status bit.

SioDelay    Delays one or more timer tics (18.2 tics per second).

SioDone     Terminates further serial processing.

SioDSR      Reads the Data Set Ready (DSR) modem status bit.

SioDTR      Set, clear, or read the Data Terminal Ready (DTR) bit.

SioError    Displays error in text.

SioFIFO     Sets the interrupt level for the INS16550.

SioFlow     Enables / disables hardware flow control.

SioGetc     Reads the next character from the serial line.

SioGetDiv   Reads the baud rate divisor.

SioInfo     Returns library version number.

SioIRQ      Assigns an IRQ line and interrupt service to a port.

SioLine     Reads the line status register.

SioLoopBack Performs a UART loopback test.

SioModem    Reads the modem status register.

SioParms    Sets parity, stop bits, and word length.

SioPorts    Sets # ports, 1st DigiBoard / BOCA port & status reg.

SioPutc     Transmit a character over a serial line.

SioReset    Initialize a serial port for processing.

SioRI       Reads the Ring Indicator (RI) modem status bit.

SioRTS      Sets, clears, or reads the Request to Send (RTS) line.

SioRead     Read any of the 7 UART registers.

SioRxBuf    Sets up receive buffer.

SioRxClear  Clears the receive buffer.

SioRxQue    Returns the number of characters in the receive queue.

SioTimer    Returns the number of system clock tics.

SioTxBuf    Sets up transmit buffer.

SioTxClear  Clears the transmit buffer.

SioTxFlush  Flushes the transmit buffer.

SioTxQue    Returns the number of characters in the transmit queue.

SioUART     Sets the UART base address.

SioUnGetc   "Un-gets" (puts back) a specified character.

9.3 Further Reading

The best way to learn about serial communications is to read a good book  on

the  subject.  Several good texts are available.  Two that I like are (sorry

but most communications books use C for their examples) :

(1) C Programmers's Guide to Serial Communications by Joe Campbell (SAMS)

(2) Mastering Serial Communications by Peter Gofton (SYBEX).

If you are interested in Protected Mode:

(3) Borland Pascal 7.0 Language Manual

(4) Extending DOS - A Programmer's Guild To Protected Mode DOS by Ray Duncan

(Addison Wesley)

PCL4P Users Manual                                                Page 31

10.0 Other MarshallSoft Computing Products for Pascal

10.1 The Personal Protocol Library for Pascal

The Personal Protocol Library  for  Pascal  (PPL4P)  consists  of  a  Pascal

language  library  which  implements  ASCII,  XMODEM, XMODEM-CRC, XMODEM-1K,

XMODEM-G, YMODEM, YMODEM-G, and ZMODEM file transfer  protocols.   A  script

compiler  and interpreter is also included which is capable of such tasks as

automatically logging onto a BBS and downloading a file or retrieving  mail.

Three example script programs are included.

The protocol library (PPL4P) requires the  Personal  Communications  Library

for Pascal (PCL4P).

The  Personal  Protocol  Library for Pascal is available for $40 plus $5 S&H

($10 S&H overseas).

10.2 The Personal Communications Library for DELPHI

Coming this summer (1996) !

11.0 Other Shareware Products

We have DOS based communications and protocol libraries for  C/C++,  Pascal,

Power Basic, and Visual Basic.

We  have  Windows  based  products  for  C/C++ and Visual Basic, with DELPHI

coming soon.

All  of  our shareware products and example code is available on our support

BBS  (205-880-9748,  14.4KB),   our   Internet   anonymous   FTP   site   at

(ftp.marshallsoft.com/marshallsoft),  as  well  as many information services

such as CompuServe, America Online, ExecPc, etc.

PCL4P Users Manual                                                Page 32