Personal Communications Library

For Visual BASIC / Windows

(PCLVBW)

REFERENCE MANUAL

Version  1.1

July 1, 1996

This software is provided as-is.

There are no warranties, expressed or implied.

Copyright (C) 1996

All rights reserved

MarshallSoft Computing, Inc.

Post Office Box 4543

Huntsville AL 35815

Voice : 205-881-4630

FAX : 205|880|0925

BBS : 205-880-9748

email : info@marshallsoft.com

web : www.marshallsoft.com

_______

____|__     |                (R)

--+       |    +-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

--+--+   |   +---------------------

|___|___|    MEMBER

PCL4VB   Reference   Manual                                       Page 1

C O N T E N T S

Chapter                                     Page

Table of Contents.............................2

Introduction..................................3

SioBaud....................................4

SioBrkSig..................................4

SioCTS.....................................5

SioDCD.....................................5

SioDone....................................6

SioDSR.....................................6

SioDTR.....................................7

SioFIFO....................................7

SioFlow....................................8

SioGetc....................................8

SioGetDiv..................................9

SioInfo....................................9

SioIRQ....................................10

SioLine...................................10

SioLoopBack...............................11

SioModem..................................11

SioParms..................................12

SioPorts..................................12

SioPutc...................................13

SioRead...................................13

SioReset..................................14

SioRI.....................................14

SioRTS....................................15

SioRxBuf..................................15

SioRxClear................................16

SioRxQue..................................16

SioTxBuf..................................17

SioTxClear................................17

SioTxFlush................................18

SioTxQue..................................18

SioUART...................................19

SioUnGetc.................................19

Function Summary.............................20

Error Code Summary...........................21

Code Examples................................21

PCL4VB   Reference   Manual                                       Page 2

Introduction

This  manual  lists all of the PCLVBW functions in alphabetical order. Every

library function returns a value as follows:

1.  Negative values for error conditions. The last page of this manual lists

all error values and their meanings.

2.  Non-negative values when returning data (eg: SioLine).

3.  Zero otherwise. Note that a zero or positive return value  is  never  an

error.

When  debugging an application, all return values should be tested. Refer to

page 23 "Error Code Summary" for a textual description of the error codes.

There are two versions of the  library.  One  version  is  with  transmitter

interrupts  disabled  and  one  with them enabled (the default). The default

library is normally chosen.

Refer to the User's Manual (PCLVBW.USR) for additional information.

It  is  also  recommended  that the two example programs SIMPLE and LOGIN be

studied as examples of calling the library functions.  Refer  to  the  Users

Manual for a description of the example programs.

All data and function types are integer unless otherwise specified.

( DEFINT A-Z ).

IMPORTANT

Be  sure  to  read  the  README.DOC file. It contains last release notes and

other last minutes changes and/or additions.

PCL4VB   Reference   Manual                                       Page 3

+-------------+------------------------------------------------------------+

|   SioBaud   |  Sets the baud rate of the selected port.                  |

+-------------+------------------------------------------------------------+

Syntax    FUNCTION SioBaud(ByVal Port,ByVal BaudCode)

REM Port:      Port selected (COM1 - COM20)

REM BaudCode:  Baud code [not baud rate]

Remarks    The  SioBaud  function sets the baud rate without resetting  the

port. It is used to change the baud rate after calling SioReset.

Code   Rate    Name             Code   Rate   Name

0     300    Baud300            5    9600   Baud9600

1     600    Baud600            6   19200   Baud19200

2    1200    Baud1200           7   38400   Baud38400

3    2400    Baud2400           8   57600   Baud57600

4    4800    Baud4800           9  115200   Baud115200

Returns     -4 : No such port. Expect 0 to MaxPort.

-11 : Bad baud rate code. See above code values.

See Also    SioReset

+-------------+------------------------------------------------------------+

|  SioBrkSig  |  Asserts, cancels, or detects BREAK signal.                |

+-------------+------------------------------------------------------------+

Syntax    FUNCTION SioBrkSig(ByVal Port,ByVal Cmd)

REM Port:      Port selected (COM1 thru COM20)

REM char Cmd:  ASSERT, CANCEL, or DETECT

Remarks    The SioBrkSig function controls the BREAK bit in the line status

register. The legal commands are:

ASSERT_BREAK ('A') to assert BREAK

CANCEL_BREAK ('C') to cancel BREAK

DETECT_BREAK ('D') to detect BREAK

ASSERT_BREAK, CANCEL_BREAK,  and  DETECT_BREAK  are  defined  in

PCLVBW.BAS. See TERM.BAS for an example of the use of SioBrkSig.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

-6 : Illegal command. Expected 'A', 'C', or 'D'.

>0 : BREAK signal detected (DETECT command only)

See Also    SioBrkKey

PCL4VB   Reference   Manual                                       Page 4

+-------------+------------------------------------------------------------+

|   SioCTS    |  Reads the Clear to Send (CTS) modem status bit.           |

+-------------+------------------------------------------------------------+

Syntax    FUNCTION SioCTS(ByVal Port)

REM Port: Port selected (COM1 thru COM20)

Remarks    The SioCTS function is used to read  the  Clear  to  Send  (CTS)

modem status bit.

The  CTS  line  is  used  by  some  error  correcting  modems to

implement hardware flow control.  CTS is dropped by the modem to

signal the computer not to send data  and is  raised  to  signal

the computer to continue.

Refer to the User's Manual for a discussion of flow control.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port.  Expect 0 to MaxPort.

0 : CTS is clear.

>0 : CTS is set.

See Also    SioFlow, SioDSR, SioRI, SioDCD, and SioModem.

+-------------+------------------------------------------------------------+

|   SioDCD    |  Reads the Data Carrier Detect (DCD) modem staus bit.      |

+-------------+------------------------------------------------------------+

Syntax    FUNCTION SioDCD(ByVal Port)

REM Port: Port selected (COM1 thru COM20)

Remarks    The  SioDCD  function is  used to read the Data Carrier Detect

(DCD) modem status bit. Also see SioModem.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port.  Expect 0 to MaxPort.

0 : DCD is clear.

>0 : DCD is set.

See Also    SioDSR, SioCTS, SioRI, and SioModem.

PCL4VB   Reference   Manual                                       Page 5

+-------------+------------------------------------------------------------+

|   SioDone   |  Terminates further serial processing.                     |

+-------------+------------------------------------------------------------+

Syntax    FUNCTION SioDone(ByVal Port)

REM Port: Port selected (COM1 thru COM20)

Remarks    The  SioDone  function  terminates  further  serial  processing.

SioDone  MUST  be called before exiting your application so that

interrupts can be restored to their original state.  Failure  to

do  this  can crash the operating system.  If you forget to call

SioDone before exiting, be sure to re-boot your  computer.   You

can  call SioDone even if SioReset has not been called, so it is

good  practice  to  always  call  SioDone  before  exiting  your

application.

Also  note  that  SioDone  dereferences the transmit and receive

buffers set up by SioRxQue and SioTxQue.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

See Also    SioReset.

+-------------+------------------------------------------------------------+

|    SioDSR   |  Reads the Data Set Ready (DSR) modem status bit.          |

+-------------+------------------------------------------------------------+

Syntax    FUNCTION SioDSR(ByVal Port)

REM Port: Port selected (COM1 thru COM20)

Remarks    The SioDSR function is used to read the  Data  Set  Ready  (DSR)

modem status bit.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port.  Expect 0 to MaxPort.

0 : DSR is clear.

>0 : DSR is set

See Also    SioCTS, SioRI, SioDCD, and SioModem

PCL4VB   Reference   Manual                                       Page 6

+-------------+------------------------------------------------------------+

|    SioDTR   |  Set, clear, or read Data Terminal Ready (DTR) status bit. |

+-------------+------------------------------------------------------------+

Syntax    FUNCTION SioDTR(ByVal Port,ByVal Cmd)

REM Port: Port selected (COM1 thru COM20)

REM Cmd:  DTR command (SET, CLEAR, or READ)

Remarks    The SioDTR function controls the Data Terminal Ready (DTR) bit

in the modem control register.  DTR should always  be  set  when

communicating with a modem.  Commands  (defined  in  PCLVBW.BAS)

are:

SET_LINE ('S')  to set DTR (ON)

CLEAR_LINE ('C')  to clear DTR (OFF)

READ_LINE ('R')  to read DTR

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

-5 : Not one of 'S', 'C', or 'R'.

0 : DTR is OFF (READ_LINE Command).

>0 : DTR is ON (READ_LINE Command).

See Also    SioRTS.

+-------------+------------------------------------------------------------+

|   SioFIFO   |  Sets the FIFO trigger level (16550 UART only).            |

+-------------+------------------------------------------------------------+

Syntax    FUNCTION SioFIFO(ByVal Port,ByVal LevelCode)

REM Port:      Port selected (COM1 thru COM20)

REM LevelCode: FIFO level code (see below)

Remarks    The SioFIFO function  is  used to enable both transmit & receive

FIFOs for 16550 UARTS, and to set the  trigger  level  at  which

receive interrupts are generated.

For  example, if the FIFO level is set to 8, then the 16550 UART

will not generate an interrupt until 8 bytes have been received.

This reduces the  number  of  interrupts  generated  and  allows

faster   processing   with   slower  machines.

In order to test if your port is a 16550 UART, call SioFIFO with

a  LevelCode  of  other than FIFO_OFF. SioFIFO can be called for

the 8250 and 16450 UART without ill effect.

Code  PCLVBW.BAS Name Trigger Level

-1      FIFO_OFF     Disable FIFO

0      LEVEL_1      1 byte

1      LEVEL_4      4 bytes

2      LEVEL_8      8 bytes   [Recommended]

3      LEVEL_14     14 bytes

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port.  Expect 0 to MaxPort.

>0 : FIFO level set.

0 : FIFO level not set (not #16550).

PCL4VB   Reference   Manual                                       Page 7

+------------+-------------------------------------------------------------+

|   SioFlow  |  Sets hardware (RTS/CTS) flow control.                      |

+------------+-------------------------------------------------------------+

Syntax    FUNCTION SioFlow(ByVal Port,ByVal Tics)

REM Port: Port selected (COM1 thru COM20)

REM Tics: # tics before TX timeout [due to CTS=0]

Remarks    The SioFlow function is used to enable or disable hardware  flow

control.  Hardware flow control uses RTS and CTS to control data

flow  between  the  modem  and the computer.  To enable hardware

flow control, call SioFlow with Tics > 0. "Tics" is  the  number

of  timer  tics  (18.2 per second) before a call to SioPutc will

time out because CTS was not set.

In order for hardware flow control to work correctly, your modem

must also be configured to work with hardware flow control,  and

your  computer  to  modem  cable  must  have  RTS  and CTS wired

straight through. If you enable hardware flow  control,  do  not

modify  the  RTS  line  (by  calling  SioRTS).

Flow control is disabled after resetting a port.  To  explicitly

disable hardware flow control, call SioFlow with Tics=-1.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port.  Expect 0 to MaxPort.

-1 : Flow control disabled.

>0 : Flow control enabled.

See Also    SioPutc

+------------+-------------------------------------------------------------+

|   SioGetc  |  Reads the next character from the serial line.             |

+------------+-------------------------------------------------------------+

Syntax    FUNCTION SioGetc(ByVal Port,ByVal Tics)

REM Port: Port selected (COM1 thru COM20)

Remarks    The SioGetc function reads a byte from the selected serial port.

The function will wait for the number of system  tics  given  by

the  'Tics'  argument  before  returning 'timed out'.  There are

18.2  tics  to  the  second.  Be careful not to pass too large a

value for Tics.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

-1 : If timed out.

>=0 : Character read.

See Also    SioUnGetc and SioPutc.

PCL4VB   Reference   Manual                                       Page 8

+---------------+----------------------------------------------------------+

|   SioGetDiv   |  Reads the baud rate divisor.                            |

+---------------+----------------------------------------------------------+

Syntax    FUNCTION SioGetDiv(ByVal Port)

REM Port: Port selected (COM1 thru COM20)

Remarks     The  SioGetDiv  function reads the baud rate divisor. The baud

rate can then be determined by the divisor:

Baud   Divisor      Baud  Divisor      Baud  Divisor

300    0180        4800   0018       38400   0003

1200    0060        9600   000C       57600   0002

2400    0030       19200   0006      115200   0001

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

>0 : Baud rate divisor.

See Also    SioParms.

+-------------+------------------------------------------------------------+

|   SioInfo   |  Returns PCLVBW library information.                       |

+-------------+------------------------------------------------------------+

Syntax    FUNCTION SioInfo(ByVal Cmd)

REM Cmd: Command (See below)

Remarks    The  SioInfo  function returns an integer code  corresponding to

the Cmd as follows

Command   SioInfo returns

-1       Cannot recognize command.

ASC("V")  Library version number.

ASC("I")  TRUE (not 0) if transmitter interrupts are enabled.

ASC("T")  The accumulative total number of transmitter interrupts.

ASC("R")  The accumulative total number of receiver interrupts.

Returns

See remarks above.

PCL4VB   Reference   Manual                                       Page 9

+-------------+------------------------------------------------------------+

|    SioIRQ   |  Assigns an IRQ line to a port.                            |

+-------------+------------------------------------------------------------+

Syntax    FUNCTION SioIRQ(ByVal Port,ByVal IRQcode)

REM Port:    Port selected (COM1 thru COM20)

REM IRQcode: IRQ number [IRQ2..IRQ15]

Remarks    The SioIRQ function assigns an IRQ line to  a  port.   That  is,

SioIRQ  maps  an  IRQ  to a port.  SioIRQ (like SioUART) must be

called before calling SioReset. Unless you have  a  non-standard

(COM1  &  COM3  use IRQ4 while COM2 & COM4 use IRQ3) serial port

configuration  (or  don't  want  to  run  more  than   2   ports

concurrently),  you  will not need to call SioIRQ.  Be EXTREMELY

careful with SioIRQ as it can lock  your  machine  up  if  given

incorrect information.

In particular, remember that your port  hardware  must  generate

the interrupt that you have specified.  You should refer to your

serial   board  hardware  manual  for  specfic  instructions  in

configuring your hardware.  Be sure to call  SioPorts  first  to

setup  DigiBoard  ports  if  you have them.  Refer to the PCLVBW

Users Manual for additional information.

Returns    -4 : No such port. Expect 0 to MaxPort.

-15 : Port already enabled.  SioReset has already been called.

|17 : No such IRQ.

-18 : ISR limit (maximum of 4 PC IRQs) exceeded.

0 : Otherwise

See Also    SioUART and SioPorts.

+-------------+------------------------------------------------------------+

|   SioLine   |  Reads the line status register.                           |

+-------------+------------------------------------------------------------+

Syntax    FUNCTION SioLine(ByVal Port)

REM Port: Port selected (COM1 thru COM20)

Remarks    The  SioLine  function  reads  the  line  status  register.  The

individual bit masks are as follows:

&H40  = Transmitter empty.

&H20  = Transmitter buffer empty.

&H10  = Break detected.

&H08  = Framming error.

&H04  = Parity error.

&H02  = Overrun error.

&H01  = Data ready.

The above are documented in the file PCLVBW.BAS.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

>0 : Line status (rightmost byte of word).

See Also    SioModem.

PCL4VB   Reference   Manual                                       Page 10

+-------------+------------------------------------------------------------+

| SioLoopBack |  Does a UART loopback test.                                |

+-------------+------------------------------------------------------------+

Syntax    FUNCTION SioLoopBack(ByVal Port)

REM Port: Port selected (COM1 thru COM20)

Remarks    SioLoopBack makes use of the built in loopback  test  capability

of  the  INS8250  family  UART.  Normally SioLoopBack will never

need to be called unless you suspect that your UART is bad. Many

UARTs must be reset after running a loopback test.

Returns      0 : Loopback test is successfull.

-2 : Port not enabled. Call SioReset first.

-4 : No such port.  Expect 0 to MaxPort.

-12 : Loopback test fails.

+-------------+------------------------------------------------------------+

|   SioModem  |  Reads the modem status register.                          |

+-------------+------------------------------------------------------------+

Syntax    FUNCTION SioModem(ByVal Port, ByVal Mask)

REM Port: Port selected (COM1 thru COM20)

REM Mask: Modem function mask

Remarks    The  SioModem  function  reads  the  modem  register.    The bit

definitions for the function mask are as follows:

Bit Name  Function               Bit  Name      Function

7  DCD   Data Carrier Detect     3   DeltaDCD  DCD has changed

6  RI    Ring Indicator          2   DeltaRI   RI has changed

5  DSR   Data Set Ready          1   DeltaDSR  DSR has changed

4  CTS   Clear To Send           0   DeltaCTS  CTS has changed

Bits  4  through  7  represent  the  absolute  state  of   their

respective  RS-232  inputs.  Bits 0 through 3 represent a change

in the state of their respective RS-232 inputs since last  read.

Once UART register 3 is read, the Delta bits are cleared.  Thus,

it is best not to depend on the Delta bits.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

>0 : Modem status (rightmost byte of word).

See Also   SioCTS, SioDCD, SioDSR and SioRI.

PCL4VB   Reference   Manual                                       Page 11

+-------------+------------------------------------------------------------+

|  SioParms   |  Sets parity, stop bits, and word length.                  |

+-------------+------------------------------------------------------------+

Syntax     FUNCTION SioParms(ByVal Port,ByVal Parity,

ByVal StopBits,ByVal DataBits)

REM Port:     Port selected (COM1 - COM20)

REM Parity:   Parity code [0,1,2]

REM StopBits: Stop bits code [0,1]

REM DataBits: Word length code [0,1,2,3]

Remarks    The SioParms function sets  the  parity,  stop  bits,  and  word

length.   If  the  default parity (none), stop bits (1), or word

length (8) is not  acceptable,  then  they  can  be  changed  by

calling SioParms.  SioParms can be called either before or after

calling SioReset. See file PCLVBW.BAS. (8 = default)

Value   Description    PCLVBW.BAS Name

ParityCode:    *0      no parity      NoParity

1      odd parity     OddParity

3      even parity    EvenParity

StopBitsCode:  *0      1 stop bit     OneStopBit

1      2 stop bits    TwoStopBits

WordLengthCode: 0      5 data bits    WordLength5

1      6 data bits    WordLength6

2      7 data bits    WordLength7

*3      8 data bits    WordLength8

Returns     -4 : No such port. Expect 0 to MaxPort.

-7 : Bad parity code selected. Expecting  0 to 2.

|8 : Bad stop bits code. Expecting  0 or 1.

-9 : Bad word length code. Expecting  0 to 3.

See Also    SioReset.

+-------------+------------------------------------------------------------+

|  SioPorts   |  Sets number of PC & Digiboard / BOCA Board ports.         |

+-------------+------------------------------------------------------------+

Syntax    FUNCTION SioPorts(ByVal NbrPorts,ByVal FirstPort,

ByVal StatusReg,ByVal Code)

REM NbrPorts:  Total number of ports

REM FirstPort: First DigiBoard port

REM StatusReg: DigiBoard Status Register

REM Code:      PC_PORTS, DIGIBOARD, or BOCABOARD

Remarks    The SioPorts function must be called  before  ANY  other  serial

functions.   The  purpose of the SioPorts function is to set the

total number of ports, the first DigiBoard (or BOCA board)  port

and the DigiBoard (or BOCA board) status register address.

Once SioPorts is called, all COM ports starting with "FirstPort"

will be treated as DigiBoard (or BOCA board) ports.  The default

setup is 4 standard PC ports and no DigiBoard or  BOCA  ports  [

SioPorts(4,4,0,PC_PORTS) ].

Refer  to  the  PCLVBW users maniual for more information on the

DigiBoard and BOCA board.

Returns    -4 : No such port. Expect 0 to 9.

0 : No error (sets MaxPort to NumberPorts-1).

See Also    SioUART, SioIRQ, and Code Examples.

PCL4VB   Reference   Manual                                       Page 12

+-------------+------------------------------------------------------------+

|   SioPutc   |  Transmit a character over a serial line.                  |

+-------------+------------------------------------------------------------+

Syntax    FUNCTION SioPutc(ByVal Port,ByVal Ch)

REM Port: Port selected (COM1 thru COM20)

REM Ch:   Character to send [ASC(char)]

Remarks    The SioPutc function transmits one character  over the  selected

serial line.

If  flow  control has been enabled, then SioPutc may return a -1

(time out) if the  number  of  tics  specified  in  the  SioFlow

function was exceeded waiting for the modem to raise CTS.

Refer to the User's Manual for a discussion of flow control.

If  transmitter  interrupts  are  enabled  (there  are  separate

versions  of  the  library  for transmitter interrupts enabled),

then the  byte  is  placed  in  the  transmit  buffer,  awaiting

transmission by the PCLVBW interrupt service routine.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

-1 : Timed out waiting for CTS (flow control enabled)

See Also    SioGetc and SioFlow.

+-------------+------------------------------------------------------------+

|   SioRead   |  Reads any UART register.                                  |

+-------------+------------------------------------------------------------+

Syntax    FUNCTION SioRead(ByVal Port,ByVal Reg)

REM Port: Port selected (COM1 thru COM20)

REM Reg:  UART register (0 to 7)

Remarks    The SioReset function directly reads the contents of any of  the

7  UART  registers.   This  function  is  useful  when debugging

application  programs  and  as  a  method  for  verifying   UART

contents.  Refer to the PCLVBW Users Manual for a discussion  of

the 7 UART registers.

The  line  status  register  (register  5) can also be read with

SioLine while the modem status register (register 6) can also be

read with SioModem.  Refer to the PCLVBW  User's  Manual  for  a

discussion of the UART registers.

Returns     -3 : No buffer available. Call SioRxBuf and SioTxBuf first.

-4 : No such port. Expect 0 to MaxPort.

See Also    SioLine and SioModem.

PCL4VB   Reference   Manual                                       Page 13

+-------------+------------------------------------------------------------+

|  SioReset   |  Initialize a serial port for processing.                  |

+-------------+------------------------------------------------------------+

Syntax    FUNCTION SioReset(ByVal Port,ByVal BaudCode)

REM Port:     Port selected (COM1 thru COM20)

REM BaudCode: Baud code or -1

Remarks    The SioReset function  initializes  the  selected  serial  port.

SioReset should be called after calling SioParm and SioRxBuf but

before  making  any  other  calls  to PCLVBW.  SioReset uses the

parity, stop bits, and  word  length  value  previously  set  if

SioParm  was  called, otherwise the default values (see SioParm)

are used.

Recall that COM1 and COM3 share the same  interrupt  vector  and

therefore  cannot  operate  simultaneously. Similiarly, COM2 and

COM4 cannot operate simultaneously. Any other combination of two

ports can be used.

By specifing NORESET (-1) for the baud rate code, the port  will

NOT  be  reset.   This is used to "take over" a port from a host

communications program that allows a  "DOS  gateway".   External

protocols  can  be implemented this way.  See SioBaud for a list

of the baud rate codes, or see "PCLVBW.BAS".

Returns     -3 : No buffer available. Call SioRxBuf & SioTxBuf first.

-4 : No such port. Expect 0 to MaxPort.

-11 : Bad baud rate code selected. Expecting  0 to 9.

|13 : UART undefined.  SioUART(Port,0) was called previously.

|14 : Bad or missing UART.  You may not have hardware present.

|15 : Port already enabled.  SioReset has already been called.

-16 : Interrupt already in use.

See Also    SioBaud, SioParms, SioRxBuf, SioDone, and SioUART.

+-------------+------------------------------------------------------------+

|    SioRI    |  Reads the Ring Indicator (RI) modem status bit.           |

+-------------+------------------------------------------------------------+

Syntax    FUNCTION SioRI(ByVal Port)

REM Port: Port selected (COM1 thru COM20)

Remarks    The SioRI function is used to read the Ring  Indicator

(RI) modem status bit. Also see SioModem.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port.  Expect 0 to MaxPort.

0 : RI is clear.

>0 : RI is set (RING has occurred).

See Also    SioDSR, SioCTS, SioDCD, and SioModem.

PCL4VB   Reference   Manual                                       Page 14

+-------------+------------------------------------------------------------+

|    SioRTS   |  Sets, clears, or reads the Request to Send (RTS) line.    |

+-------------+------------------------------------------------------------+

Syntax    FUNCTION SioRTS(ByVal Port, ByVal Cmd)

REM Port: Port selected (COM1 thru COM20)

REM Cmd:  RTS command (SET, CLEAR, or READ)

Remarks    The SioRTS function controls the Request to Send (RTS bit in the

modem control register.

The  RTS  line  is  used  by  some  error  correcting  modems to

implement hardware flow control.  RTS is dropped by the computer

to signal the modem not to send data, and is  raised  to  signal

the modem to continue. RTS should be set when communicating with

a modem unless Flow Control is being used.

Refer  to  the  User's  Manual for a discussion of flow control.

Commands (defined in PCLVBW.BAS) are:

SET_LINE     ASC("S")   - set RTS (ON)

CLEAR_LINE   ASC("C")   | clear RTS (OFF)

READ_LINE    ASC("R")   - read RTS

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

-5 : Command is not one of 'S', 'C', or 'R'.

0 : RTS is OFF (READ_LINE Command).

>0 : RTS is ON  (READ_LINE Command).

See Also    SioFlow and SioDTR.

+------------+-------------------------------------------------------------+

|  SioRxBuf  |  Sets up receive buffers.                                   |

+------------+-------------------------------------------------------------+

Syntax    FUNCTION SioRxBuf(ByVal Port,ByVal Selector,ByVal SizeCode)

REM Port:      Port selected (COM1 thru COM20)

REM Selector:  Receive buffer selector

REM SizeCode:  Buffer size code

Remarks     The  SioRxBuf  function passes the address selector and size of

the receive buffer to PCLVBW.  Recall  that  PCLVBW  requires  a

receive buffer for each port in simultaneous operation since the

receive  function is interrupt driven.  It must be called before

any incoming characters can be received. SioRxBuf must be called

before SioReset.  Buffer size codes are listed in "PCLVBW.BAS".

Size Code       Buffer Size    PCLVBW.BAS Name

4            128 bytes        Size128

5            256 bytes        Size256

6            512 bytes        Size512

7           1024 bytes        Size1024 or Size1K

8           2048 bytes        Size2048 or Size2K

9           4096 bytes        Size4096 or Size4K

10           8192 bytes        Size8192 or Size8K

11          16384 bytes        Size16384 or Size16K

12          32768 bytes        Size32768 or Size32K

Returns    -4 : No such port. Expect 0 to MaxPort.

-10 : Bad buffer size code. Expecting 0 to 11.

See Also    SioReset and Code Examples.

PCL4VB   Reference   Manual                                       Page 15

+------------+-------------------------------------------------------------+

| SioRxClear |  Clears the receive buffer.                                 |

+------------+-------------------------------------------------------------+

Syntax    FUNCTION SioRxClear(ByVal Port)

REM Port: Port selected (COM1 thru COM20)

Remarks    The SioRxClear  function  will  delete  any  characters  in  the

receive  buffer  for  the  specified port.  After execution, the

receive buffer will be empty.  Call SioRxClear after resetting a

port in order to delete any spurious characters.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

See Also    SioRxQue.

+-------------+------------------------------------------------------------+

|  SioRxQue   |  Returns the number of bytes in the receive queue.         |

+-------------+------------------------------------------------------------+

Syntax    FUNCTION SioRxQue(ByVal Port)

REM Port: Port selected (COM1 thru COM20)

Remarks    The  SioRxQue  function  will return the number of characters in

the receive queue at the time of the call. It  can  be  used  to

implement XON/XOFF flow control.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

See Also     SioRxFlush.

PCL4VB   Reference   Manual                                       Page 16

+------------+-------------------------------------------------------------+

|  SioTxBuf  |  Sets up transmitter buffer.                                |

+------------+-------------------------------------------------------------+

Syntax    FUNCTION SioTxBuf(ByVal Port,ByVal Selector,ByVal SizeCode)

REM Port:     Port selected (COM1 thru COM20)

REM Selector: Receive buffer selector

REM SizeCode: Buffer size code

Remarks    The  SioTxBuf  function passes the address selector and  size  of

the transmit buffer to PCLVBW, provided that you will link  with

a  PCLVBW  library  with transmitter interrupts enabled.  PCLVBW

requires  a  transmit  buffer  for  each  port  in  simultaneous

operation  if  you  are  using  the  version of the library with

transmitter interrupts enabled (PCLVBW2.LIB).   SioTxBuf()  must

be called before SioReset.

Size Code       Buffer Size    PCLVBW.BAS Name

4            128 bytes        Size128

5            256 bytes        Size256

6            512 bytes        Size512

7           1024 bytes        Size1024 or Size1K

8           2048 bytes        Size2048 or Size2K

9           4096 bytes        Size4096 or Size4K

10           8192 bytes        Size8192 or Size8K

11          16384 bytes        Size16384 or Size16K

12          32768 bytes        Size32768 or Size32K

This  function is not used unless the transmitter interrupts are

enabled (the  default  library).   Refer  to  the  PCLVBW  Users

Manual for information on transmitter (TX) interrupts.

Returns     -4 : No such port. Expect 0 to MaxPort.

-10 : Bad buffer size code. Expecting 0 to 11.

See Also    SioRxBuf, SioReset and Code Examples.

+------------+-------------------------------------------------------------+

| SioTxClear |  Clears the transmitter buffer.                             |

+------------+-------------------------------------------------------------+

Syntax    FUNCTION SioTxClear(ByVal Port)

REM Port: Port selected (COM1 thru COM20)

Remarks    The SioTxClear  function  will  delete  any  characters  in  the

transmit  buffer  for the specified port, provided that you will

link with a PCLVBW library with transmitter interrupts  enabled.

After execution, the transmit buffer will be empty.

Once  this  function  is  called,  any character in the transmit

buffer (put there by SioPutc) will be  lost  and  therefore  not

transmitted.   This  function is not used unless the transmitter

interrupts are enabled Refer to  the  PCLVBW  Users  Manual  for

information on transmitter (TX) interrupts.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

See Also    SioTxQue & SioTxFlush.

PCL4VB   Reference   Manual                                       Page 17

+------------+-------------------------------------------------------------+

| SioTxFlush |  Flushes the transmitter buffer characters.                 |

+------------+-------------------------------------------------------------+

Syntax    FUNCTION SioTxFlush(ByVal Port)

REM Port: Port selected (COM1 thru COM20)

Remarks    The SioTxFlush  function  will  transmit any  characters  in  the

transmit  buffer before returning. In order to clear the buffer,

call SioTxClear instead.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

See Also    SioTxQue & SioTxClear.

+------------+-------------------------------------------------------------+

|  SioTxQue  |  Returns the number of bytes in the transmit queue.         |

+------------+-------------------------------------------------------------+

Syntax    FUNCTION SioTxQue(ByVal Port)

REM Port: Port selected (COM1 thru COM20)

Remarks    The SioTxQue function will return the number  of  characters  in

the  transmit  queue  at the time of the call, provided that you

will link with a  PCLVBW  library  with  transmitter  interrupts

enabled.

This  function is not used unless the transmitter interrupts are

enabled.  Refer to the PCLVBW Users Manual  for  information  on

transmitter (TX) interrupts.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

See Also     SioTxFlush.

PCL4VB   Reference   Manual                                       Page 18

+------------+-------------------------------------------------------------+

|  SioUART   |  Sets the UART base address.                                |

+------------+-------------------------------------------------------------+

Syntax    FUNCTION SioUART(ByVal Port,ByVal Address)

REM Port:    Port selected (COM1 thru COM20)

REM Address: UART address

Remarks    The  SioUART  function  sets  the  UART  base  address  for  the

specified port.  SioUART must be called before SioReset in order

to  have  effect.   Be extremely sure that you know what you are

doing!   Note  that  PCLVBW  uses  the  standard  PC/XT/AT  port

addresses,   interrupt  request  lines,  and  interrupt  service

vectors.   Therefore,  this  function   is   only   needed   for

non-standard ports.

Returns    >0 : The previous base address for this port.

-4 : No such port.  Expect 0 to MaxPort.

-15 : Port already enabled.  SioReset has already been called.

See Also    SioPorts, SioIRQ, and SioReset.

+------------+-----+-------------------------------------------------------+

| SioUnGetc  |  "Un|gets" the last character read with SioGetc().          |

+------------+-----+-------------------------------------------------------+

Syntax    FUNCTION SioUnGetc(ByVal Port,ByVal Ch)

REM Port: Port selected (COM1 thru COM20)

REM Ch:   Character to unget

Remarks    The SioUnGetc function returns  ("pushes")  the  character  back

into  the serial input buffer.  The character pushed will be the

next character returned by SioGetc.  Only one character  can  be

pushed back. This function works just like the "ungetc" function

in the C language.

Returns    -2 : Port not enabled. Call SioReset first.

-4 : No such port. Expect 0 to MaxPort.

See Also    SioReset.

PCL4VB   Reference   Manual                                       Page 19

Function Sumary

+-------------+----------+----------+----------+----------+

| Function    |   Arg1   |   Arg2   |   Arg3   |   Arg4   |

+-------------+----------+----------+----------+----------+

| SioBaud     | Port     | BaudCode |          |          |

| SioBrkSig   | Port     | Cmd      |          |          |

| SioCTS      | Port     |          |          |          |

| SioDCD      | Port     |          |          |          |

| SioDone     | Port     |          |          |          |

| SioDSR      | Port     |          |          |          |

| SioDTR      | Port     | Cmd      |          |          |

| SioFIFO     | Port     | LevelCode|          |          |

| SioFlow     | Port     |          |          |          |

| SioGetc     | Port     | Tics     |          |          |

| SioGetDiv   | Port     |          |          |          |

| SioInfo     | Cmd      |          |          |          |

| SioIRQ      | Port     | IRQcode  |          |          |

| SioLine     | Port     |          |          |          |

| SioLoopBack | Port     |          |          |          |

| SioModem    | Port     | Mask     |          |          |

| SioParms    | Port     | Parity   | StopBits | DataBits |

| SioPorts    | NbrPorts | FirstDBP |StatusReg | Code     |

| SioPutc     | Port     | Ch       |          |          |

| SioRead     | Port     | Register |          |          |

| SioReset    | Port     | BaudCode |          |          |

| SioRI       | Port     |          |          |          |

| SioRTS      | Port     | Cmd      |          |          |

| SioRxBuf    | Port     | BufSeg   | SizeCode |          |

| SioRxClear  | Port     |          |          |          |

| SioRxQue    | Port     |          |          |          |

| SioTxBuf    | Port     | BufSeg   | SizeCode |          |

| SioTxClear  | Port     |          |          |          |

| SioTxFlush  | Port     |          |          |          |

| SioTxQue    | Port     |          |          |          |

| SioUART     | Port     | Address  |          |          |

| SioUnGetc   | Port     | Ch       |          |          |

+-------------+----------+----------+----------+----------+

PCL4VB   Reference   Manual                                       Page 20

Error Code Summary

+------+--------------------------------------------------------+

| Code |  Description                                           |

+------+--------------------------------------------------------+

|   0  |  No error.                                             |

|  -1  |  Timeout waiting for I/O.                              |

|  |2  |  Port not enabled. Call SioReset first.                |

|  |3  |  No buffer available. Call SioRxBuf and SioTxBuf first |

|  |4  |  No such port. Expect COM1 to COM20.                   |

|  |5  |  Expected 'S', 'C', or 'R' as 2nd argument.            |

|  |6  |  Expected 'A', 'C', or 'D' as 2nd argument.            |

|  |7  |  Bad parity code specified. Expected 0 to 7.           |

|  |8  |  Bad stop bits code specified. Expected 0 or 1.        |

|  -9  |  Bad wordlength code specified. Expected 0 to 3.       |

| -10  |  Bad buffer size code specified. Expected 0 to 11.     |

| |11  |  Bad baud rate code. Expected 0 to 9.                  |

| |12  |  Loopback test fails.                                  |

| |13  |  UART undefined.                                       |

| |14  |  Missing or bad UART. (no UART hardware ?)             |

| |15  |  Port already enabled.                                 |

| |16  |  ISR (interrupt) already in use.                       |

| |17  |  No such IRQ. (Should be 2 to 15)                      |

| |18  |  ISR limit (maximum of 4 PC IRQs) exceeded.            |

+-+----+--------------------------------------------------------+

Code Examples

DigiBoard Example

'status reg uses &H140 for odd IRQs & &H141 for even IRQs

RC = SioPorts(8,COM1,&H140,DIGIBOARD)

FOR I = COM1 TO COM20

'set DigiBoard UART addresses

RC = SioUART(i,&H100+8*i)

'set DigiBoard for IRQ5

RC = SioIRQ(I,IRQ5)

NEXT I

BOCA Board Example

'status reg uses base port + 7

RC = SioPorts(16,COM1,&H100+7,BOCABOARD)

FOR I = COM1 TO COM16

'set BOCA UART addresses

RC = SioUART(i,&H100+8*i)

'set BOCA for IRQ5

RC = SioIRQ(I,IRQ5)

NEXT I

PCL4VB   Reference   Manual                                       Page 21