SETDMA.EXE FREEDMA.EXE

Source: setdma.c 	source to setdma.exe

freedma.c 	source to freedma.exe

doenv.c		source for affecting environment

sfdma.m		make/nmake build file

sfdma.doc	this document

Build:	cl -c setdma.c

cl -c freedma.c

cl -c doenv.c

link setdma.obj doenv.obj;

link freedma.obj doenv.obj;

These two complementary programs are used together to obtain and return

memory from the system.

They do not use any standard means of accomplishing this!

There are three areas that are especially non-standard:

1. They use the (officially) undocumented function

to retrieve the address of the DOS "List of Lists"

in order to obtain the address of the first MCB in

the chain of memory control blocks.

This is actually the standard means of accomplishing

such, although it uses an "undocumented" int 21h call.

2. It updates the Master Environment block.

There is no official, undocumented or in any way

"bulletproof" means of locating this area of memory.

The means used here is merely one of of several which

can fail under different system configurations.

3. It directly affects the MCB chain; it does not use

DOS to allocate or release memory.

Of course, this sort of activity must be "frowned upon".

Without writing a TSR, allocating memory without having

it freed upon exit is otherwise impossible.  But, the

definition of a memory control block is straightforward,

so this works.

The whole point of this exercise is to allocate an area of memory

to be used for DMA transfers with the Pro Audio Spectrum series of

sound boards from Media Vision.

Because the DMA buffer must not cross a 64K segment, it is not possible

to simply allocate a block of memory using DOS and use it because it may

cross such a boundary.  If you were to try to do this "early" in the

boot sequence, you may have success;  after loading a number of TSR's,

though, who knows?

SETDMA will search all the way to the top of memory, back up 16K, and

take that block from the system (if it determines that it is okay to

do so).  DOS does not provide the ability to allocate from the top of

memory reliably (the last-fit strategy has been reported to me as not

always implemented).

It takes the memory by simply adjusting the size of the last MCB and

creating a new MCB and marking it as "owned".  That's it.

Since you have the source code, you can modify the program to take

as or as little memory as possible.

Oh, and you will notice that after the program traces the MCB chain

for conventional memory, it follows on up through the Upper Memory

(memory above A000).  You might want to modify the program to allocate

a block in this area, thereby freeing up that 16K in conventional

(actually, that is what I am going to do)!

FREEDMA will release the memory allocated by setdma.  You either

specify the address of memory (not the MCB;  the MCB + 1 paragraph)

on the command line, or let the program find the environment variable

"DMABUF".

The program "TPCM.exe" (which uses "PCM.com", the TSR for PCM services)

can be told to use a specific area of memory for its DMA buffer, which

is the whole point between these two programs.  It will use the environment

variable "DMABUF" (if so directed), and when SETDMA is modified to use

UMB's, PCM output will no longer take 64K+ of buffer space!

WARNING!  Use these programs at your own risk!

Distributed for Informational Purposes Only.

Copyright (c) 1992.  Media Vision, Inc.  All Rights Reserved.

Programmer: Bart Crane