TPCM - Test PCM

Syntax

tpcm [-T] [-dqmL] [-iname] [-r#l#s] [-v#p#c#] [-B#D#] [-R#] file ...

Options

-T	output on Thunderboard (requires "set BLASTER="

-d	debug mode, output status

-q	quiet mode (no sound)

-m	toggle use Monochrome Screen Buffer for DMA

-L	Loop sound continuously

-iname	name= name of input PCM file (use -r#, -l# and -s)

-r#	#= default samplerate (decimal) default: 11025

-l#	#= size of sample in bits (decimal) default: 8

-s	treat as stereo (default: mono)

-w#	#= delay in clock ticks before timing-out;  default: 4

-R[X]	record mode, X= compression method;  default: none

-t#	#= number of clock ticks to specify recording-time

-v[#]	#= video display image # (1-3, see ref) default: 1

-p#	#= palette # (1-8) default: 2 (6 if -v2 specified)

-c#	#= palette cycle delay (default: 1)

-B#	#= DMA Buffer size (KB), default: 8K if video, else 16K

-D#	#= number of DMA Divisions, default: 4 if video, else 8

wavfiles ...	filenames to play (overrides -R and -s)

Note: -iname and "wavfiles ..." are mutually exclusive

-b###	###= base port address for PAS (for multiple boards)

Examples

Playback

C:> tpcm sound.wav

This simply plays the file "sound.wav".

C:> tpcm -T sound.wav

This plays the file "sound.wav" on the ThunderBoard

(SoundBlaster) DAC.

The "pcm.com" driver looks for "BLASTER=" in the

environment to configure for the SB.

C:> tpcm -v sound.wav

This displays and plays the file "sound.wav".

C:> tpcm -v *.wav

This displays and plays all .wav files.

C:> tpcm -vp7c600 *.wav

This displays and plays all .wav files using palette 7

with a delay of "600" (computer speed dependent).

C:> tpcm -isound.snd -R22050 -s

This plays the PCM file "sound.snd" at 22050Hz in stereo.

C:> tpcm -vm *.wav

This displays and plays all sound files, but uses

the monochrome screen memory for the DMA buffer.

Recording

C:> tpcm -R x.wav

This records into the file "x.wav" at 11025Hz, 8-bit mono.

C:> tpcm -v -R -r44100 -s -l16 cdqualty.wav

This records into the file "cdqualty.wav" at 44100Hz,

16-bit stereo, while displaying the waveform.

C:> tpcm -RA -r44100sl16 cdqualty.cmp

This records into the file "cdqualty.cmp" at 44100Hz,

16-bit stereo, but compresses using the ALAW method.

Playback Control

During playback of the waveform, several keys are processed:

w	wait for keypress, loops on DMA buffer

Not particularly glamorous, but can be irritating to those

around you.

<>	slow/speed playback rate

Use this key to slowdown/speedup the playback/record rate.

m	display "mark", ticks/bytes/samples

Use this key to display the current offset into the playback/

record buffer.

l	turn on/off "looping" at end of file

Display Control

During display of the waveform, several keys are processed:

P	palette select - the program waits for either:

0= no palette, 1 through 8= palettes 1 through 8

c,C	auto cycle delay (+1/10), increasing delay as you hold key

r	reverse cycle, c,C now decreases delay as you hold key

SPACE	stop cycling

f,F	manually cycle one way (256/16 colors)

b,B	manually cycle the other (256/16 colors)

u,d	change RGB up/down 1

U,D	change RGB up/down 10

CR	restore palette

ESC	exit

Operation

This program no longer requires the TSR pcm.com to be loaded in memory.

Instead, the program will load "pcm.com" it if it is not already so found.

This requires only that "pcm.com" be found in the "PATH" (tpcm.exe and

pcm.com should be in the same directory: "\bin" or "\util").

Use this program to play .wav files, and display them in real time.

The program will accept compressed 16-bit .wav files (see "cmpwav.exe").

The program will also record using the compression methods

in "cmpwav.exe" format, although not all are real-time.

Use this program to manually specify many parameters, such as the

location and size of the DMA buffer, samplerate and datasize, IRQ

activity, and others during the playback or record.

Options

As you can see, there are many options available to affect the startup

conditions of the program.

-T

Use the -T option to output the sound on the SB part of the PAS, or

on a ThunderBoard, or on a Sound Blaster.

You must set the variable "BLASTER" in the environment before pcm.com

loads and before running tpcm.exe.  The following defines the meaning

of this variable:

C:> set BLASTER=A220 D1 I5 T3

|   	| |  ||	|| ||

|    | |  ||	|| `--- Product Number "3"

|   	| |  ||	`------ IRQ Number "5"

|   	| |  `-w-------- DMA Channel "1"

|   	`-------------- Port Address "220h"

`------------------- Variable Name "BLASTER"

-d

Use the -d option to observe the queueing of data and the waiting for

DMA to finish.  When redirecting this output to a file on a ramdrive,

you can see how much time there is available for additional processing.

Use this option when attempting to locate valid DMA/IRQ combinations.

Display and Sound Output from the program will help you determine valid

combinations.  This is useful with the "proas.exe" program, which will

reprogram the card to any dma/irq combination.

-q

Use the -q option to suppress the output of sound and have the program

simply run through the file.  Note that there will be no display (if the

-v option was specified) as there is no DMA activity and so no User

Function calls.  Use the -d option to see the progress of loading the

file and the associated simulated DMA transfers.

-m

Use the -m option to cause the program to use monochrome screen memory

for the DMA buffer.  Note: this memory may be used as DOS UMB (loadhigh),

and overwriting this area with sound data is not suggested!  However, if

you do have a monochrome monitor, then you can watch the data slam by.

Why use the monochrome screen buffer for DMA?  Well, nowadays,

with VGA and multi-megabyte-memory machines, the B segment is

a nice 32K block of memory commonly unused, and perfect for

scratch memory (like sound, where overwrites are merely noisy,

not fatal).

Unfortunately, this memory is too slow for use with 44,100 Hz

sounds, so this is not a perfect solution.  Also, some machines

generate a high-frequency pitch when accessing this memory

during DMA transfers.

-L

Use the -L option to loop on the sound file continuously.  Press ESCAPE

to exit the loop and proceed to the next file (if any).

Use this option with the 'm' key to display the current timeticks/bytes/

samples of playback or record.  This gives you the ability to jot notes

on paper of where interesting clips occur in a sound file.

While looping, you can press the "<>" keys (or ",.") to

slowdown or speedup the samplerate, and so achieve a

higher degree of resolution when specifying "marks".

-i

Use the -i option to specify a filename (no DOS wildcards allowed).

Use the -s, -l and -r options to specify stereo, the sample size in bits,

and the sample rate.

-r

Use the -r option to specify the samplerate for .wav files specified

with the -i option, and for non .wav files, and for recording.  This

value can range from 4000 to 88200 for mono files, and 8000 to 44100

for stereo files.

-l

Use the -l option to specify the bit-size of the ram data samples.

Use this option when processing sound files or by using the "-i" option.

-s

Use the -s option to cause the program to treat the -i specified file or

raw PCM files as stereo.  This option has no effect on files whose

formats are known, unless the file is specified using the -i option.

-v

Use the -v option to select a picture to display:

1	tpcm.scr	256-color gradient

2	tpcm2.scr	me

3	tpcm3.scr	lame-looking "deck" with useless knobs

These must be simple "memory dump" files, 320x200x8bit.

-B

Use the -B option to specify the size of the DMA buffer in KB.  This

number is expressed in decimal, and should be either 4, 8, 16, 32 or 64.

This number is forced to at least 4, unless the -v option is given, in

which case it is made to be 4 for stereo output, and 2 for mono.

This number usually specifies the DMA buffer size in KB, but if the value

given is greater than 64, then it specifies the actual buffer size.

To get a 1-second DMA buffer with a 22050Hz, 16-bit mono

sound, use "-B44100".

-D

Use the -D option to specify the number of parititions for the DMA buffer.

This number is expressed in decimal, and should be a power of 2 greater

than 1.  If the -V option is given, then it is made to be 2 in order to

ensure synchronous sound and display output.

The DMA buffer routines in pcm.com accept buffer sizes that

are not multiples of 1024, and recognize treat any passed

size that is greater than 0xFF to be the size in bytes

instead of Kbytes).  Thus -B15750 is valid.

Also, pcm.com handles number-of-divisions of the DMA buffer

that are not powers of 2.  Thus, -D5 is valid.

Note that the size of the DMA buffer should be a multiple

of the number of divisions to prevent buffer overruns.

Mono/Stereo display is selected automatically according to the channels

for recognized .wav files.

You can specify multiple files to process, if the -i option is not also

used, by naming them at the end of the command line.  You can use DOS

wildcards: "tpcm *.wav".  These files are examined to determine their

type, and .wav files have their format information used to specify the

number of channels and samplerate, but the command line parameters or

default values are used for non-.wav files.

Record Mode

Use the -R to put the program into "record" mode.

Most options are valid in this mode, with exception of -l (looping) and

-iname ("input file name"), and that only one filename is processed.

The program will attempt to compress in real-time using the format

specified with the -R option:

-RA	- use ALAW

-RM	- use MULAW

-RI	- use IMA

-RW	- use Microsoft ADPCM (not real-time - but implemented).

-RP	- no compression

This output format is compatible with the input, so you can always use

tpcm to play/record these files, and "cmpwav.exe" (from MediaVision) to

control their format.

Return Value

Playback

errorlevel 0 if the file played with no error.

errorlevel 1 if the file played but not properly (IRQ mismatch)

errorlevel 255 if it failed to find or load the pcm.com TSR.

other nonzero errorlevel for syntax error, bad memory, etc.

Record

errorlevel 0 if the file recorded with no error.

other nonzero errorlevel for syntax error, bad memory, etc.

Requirements

MediaVision Pro Audio Spectrum (any model).

This program requires a graphics monitor with the ability to display

320x200x256 colors in order to display the waveform.

Secrets

DMA Buffer Location

It is possible to specify the location of the DMA buffer manually using

two options not documented above, however, this can be very dangerous.

Use the -S# and -O# options to specify the segment and offset (in hex)

of the DMA buffer (use the -B# and -D# option to specify the size and

divisions).  You can then have the program play or record and direct

the DMA buffer to A000:0, thus seeing the PCM data appear on the

screen in graphics mode.  Interesting?  Maybe, but since we can, why not?

If you have the "setdma.exe" and "freedma.exe" programs,

you can use them to allocate and deallocate a 16K block of

memory from the very top (thereby eliminating any waste).

The "setdma.exe" program displays the segment to specify

with the tpcm -S option (and to "freedma.exe").

The -S and -O options ask you to confirm that you are manually

specifying these values to prevent accidental confusion with

the -s and -o options, unless you insert a "!" between the "S"

or "O", as in "-S!####" or "-O!####".

Using monochrome screen memory as the DMA buffer can be "noisy".

Multiple PAS Boards

Also, you can specify a "Board Address" to gain access to multiple

PAS boards by using "-b###".  Each board requires an instance of

"pcm.com" or "pcmfun.com" loaded with the same board address.

"tpcm.exe" will use the first available pcm.com (pcmfun.com) if no

particular board address is specified.  If an address is given, then

the first pcm.com (pcmfun.com) that controls that board will be used.

If no suitable handler is found, then the program will install "pcm.com"

for that particular board address.  The default address is 388h, but

the TSR will locate at the first found address.

The following sequence will demonstrate the logic implemented:

C:> proas -b388 d:5 q:15 s:1,220,1,5 v:70

C:> proas -b384 d:6 q:11 s:1,230,3,7 v:70

These two lines initialize two PAS16 boards,

one at 384h, one at 388h

C:> tpcm sound.wav		# no handler, load pcm.com at 388h

With no handler installed, the program loads pcm.com,

and with no board address specified, the default is 388h.

C:> tpcm -b384h	sound.wav	# no handler, load pcm.com at 384h

With no handler installed for the particular address,

pcm.com is loaded for that particular board.

C:> pcm -b384 -i		# install handler for board 384h

When a handler is installed, that board is the default

board when no address is specified.

C:> tpcm sound.wav		# is handler, plays on board 384h

With a handler installed, and no board address specified,

the sound is played on that handler's board.

C:> tpcm -b388 sound.wav	# no handler, load pcm.com at 388h

A handler is installed, but not for the specified board,

so pcm.com is loaded and the sound plays on that board.

C:> pcm -b384 -u		# uninstall handler for board 384h

C:> pcm -b388 -i		# install handler for board 388h

The default board to play on when no address is specified

was 384h, and is now 388h.

C:> tpcm sound.wav		# is handler, plays on board 388h

No address specified, plays on first handler's board.

C:> tpcm -b384 sound.wav	# no handler, load pcm.com at 384h

No handler for that board installed, so pcm.com is

loaded and the sound is output on that board.

C:> pcm -b384 -i		# install handler for board 384h

Now there are two handlers installed, one for each board

C:> tpcm sound.wav		# play on first found (384h PAS)

The first handler found (is the last one loaded) is at

at 384h, so since no address is specified, output comes

out of the board at 384h.

C:> tpcm -b384 -T sblast.wav	# play sound on board 384h TB

Address specified, and handler exists.

C:> proas -b380 d:7 q:12 s:1,220,0,3

C:> pcm -b380 -i

Uh, oh, another board is initialized, at 380h, and

a handler is installed for it.

Since this is the last handler, it is the first

found, and so is the default board when no address

is specified.

C:> tpcm sound.wav

No address specified, output on 380h.

With "srs.exe", you can use the 'm' Mark valuse to cut sounds clips

into other files, and then add them together to create a new sound

track, all from the command line.

C:> tpcm -L sound.wav

... during playback, slow samplerate using "<" ...

... during loopback, press the 'm' key to display

... ticks/bytes/samples offset at desired points ...

C:> srs save from sound.wav at 44376 upto 83000 into sound.0

... 44376 is the byte offset to start cutting ...

... 83000 is the byte offset of the end of the cut ...

C:> tpcm -L sound.0

... loop on the cut clip ...

C:> srs add sound.0 into soundtrk.wav at 10 secs save

... add the sound clip into the sound track being edited ...

New Stuff

-T, the Thunder Board (Sound Blaster) output.

"<>" during playback.

pcmfun.com interface.

multiple boards.

Programmer Output

dasj;l;kare-0423m saldf, 3129;as,.4pdfp

I did that!

Whew!  If you are reading this, then you are my kind of user!

I feel like I can tell you things... things I haven't told anyone else!

Except for everyone who will listen, that is!

Seriously (why do I keep backspacing and undoing, instead of just

getting on with it!).  (Okay, so I just finished the "copyright" stuff

below... talk about conjuctive phrasing!)

Okay, so what I really want to say is, if you have gotten this far,

don't stop now!

First, a little about the programmer of this stuff.  Me.  SRS.EXE,

PCM.COM and PCMFUN.COM, TPCM.EXE, proas.exe, WAVCMP.LIB... all I can

say is: "there you go".

I use'm all the time.

My only regret is that I only use them to build them!  I wish, instead,

I was in your place (maybe), and had cause to use them!  I can imagine

being a recording engineer, or database programmer, and having a real

need for this stuff.  Or, as an independent developer writing a game or

other application.

Tell me about some other program that will record at 88,200 Hz, please!

Or will stat all .wav or .voc files, please!  Or (un)compress!

Now that I can play the SB and record it and all other inputs on the

PAS, in the background(!), well, my head is five feet wide!

Here's a little joke I played for awhile: I recorded random FM at

88,200, then played it back at 12,000-4,000 (nearly so-to-truly subsonic)

whenever people came into my cube regarding bugs in "-------.exe", in the

background, so no one knew!  (Maybe I make people uncomfortable, anyway.)

The following text is merely for informational purposes, and really only

for those that use "tail" on documents to see what is at the end, and I

don't want those of you who read all of the above to be cheated by those

who skip to the last screen!

This software is copyrighted, and unauthorized use or application may or

may not warrant or require remedy or remuneration if unlawful or illegal

copies or replications are made or distributed, subject to civil or

criminal penalties.

If you would like to become a registered user of this and its associated

softwar, please post some mail that includes your first name and your

age to

Media Vision

3185 Laurelview Court

Fremont, CA  94538

Attn: Software Engineering, DOS

or fax the same to

(510) 226-2582

----------------------------------------------------------------------------

Copyright (c) 1992-1993.  Media Vision, Inc.  All Rights Reserved.

Programmer: Bart Crane