WINDOWS DRIVER UPDATE

Aug 17, 1992

Version 1.4 BETA

This is the latest release of improved Windows drivers for the Pro

AudioSpectrum family of audio cards from Media Vision.  Media Vision

will continue to do testing, bug-fixing, and enhancements.

We recommend that you make a backup of your existing drivers before

using the ones provided with this release.  If you discover any

further bugs, please notify us via CompuServe ID 75300,2772.

WARNING:  The powerful nature of Media Vision's Windows drivers exceed

the ability of Windows' Control Panel Driver Installation

applet.  We have provided SETUP.EXE to automate the

installation process.  We strongly recommend that you

use SETUP to install the drivers.

If you are comfortable with editing system.ini yourself,

you may wish to install the drivers manually.

If you insist on using the control panel to install, use

the following procedure:

TO INSTALL USING THE CONTROL PANEL APPLET:

1) From Windows, run the Control Panel's drivers applet.

2) Click on the Add button.

2 1/2) Now would be a good time to add the [MCI] CD Audio driver if

you have a CD

3) Select 'Unlisted or Updated Driver'

4) Select browse and find the directory where the new drivers are

5) Select the appropriate WAVE/MIDI/AUX driver for your hardware.

(This will cause all other drivers for your hardware to load as

well)  Allow the system to overwrite per-existing drivers.

6) Restart Windows

TO INSTALL USING THE MEDIA VISION SETUP UTILITY:

1) Insert the Windows Drivers Disk.

2) CD to the drive that contains that disk

3) Type WIN SETUP

TO INSTALL WINDOWS DRIVERS MANUALLY:

1) Copy *.drv, *.dll, *.cfg and *.386 from the Windows drivers disk

to the \WINDOWS\SYSTEM directory.

2) Copy *.exe from the Windows drivers disk to the \WINDOWS directory.

3) Make the following changes to your system.ini file:

[386Enh]

device=*vdmad            <-- make sure this line is first (Win 3.1 only)

device=vpasd.386         <--- add this line for PAS PLUS, 16 or CDPC

dmabuffersize=64         <--- add this line

device=vadlibd.386       <-- remove this entry if you have it

device=vsbd.386          <-- remove this entry if you have it

device=vtbd.386          <--  add this line for ThunderBoard

[mci]

WaveAudio=mciwave.drv 9  <--- should already be there

Sequencer=mciseq.drv     <--- should already be there

CDAudio=mcicda.drv       <--- should already be there

mixer=mcimixer.drv       <--------- add this line

[drivers]

timer=timer.drv

midimapper=midimap.drv

midi=opl3.drv            <--- for PAS Plus, PAS 16 and CDPC only

midi=mvfm.drv            <--- add this if you have the original PAS

midi=adlib.drv           <--- add this for ThunderBoard or Thunder

& Lightning

midi1=mvproaud.drv       <--- add this line

Wave=mvproaud.drv        <--- add this line for PAS Plus, 16 or CDPC

Wave=tbwave.drv          <--- add this line for ThunderBoard only

Wave=tlwave.drv          <--- add this line for Thunder & Ligtning only

Mixer=mvmixer.drv        <--- add this line

aux=mvproaud.drv         <--- add this line

Wave1=sbwave.drv         <--- for PAS 16 only (adds 2nd wave driver)

Also you must add the following line to system.ini's

[mvproaud.drv] section:

buffer=32

4) Add the following executeables to your Windows desktop:

PMIX.EXE, PREC.EXE, PROMIX.EXE, MIX.EXE

GENERAL NOTES

1) Central Point's BACKUP and PCTOOLS 7.1 are reported to be

installing their own VDMAD.386 over the Windows 3.1.

Central Point's VDMAD.386 has the wrong version number and will

prevent mvproaud.drv from loading.

2) After changing one or more MIDI drivers, check the control

panel's MIDImapper applet.  Channels 1 thru 10 should be patched

to the MIDI interface and channels 13 thru 16 should be patched

to the on-board synthesizer (MVFM.DRV or OPL3.DRV

When playing 'Old Style' MIDI files (not General MIDI) use

the Vintage MIDI file patch map.

3) Pro Mixer has two extra sliders on the Pro-16 that are not

documented in the manual.  The BLSTR channel is the volume

selector for the Snd Blaster compatible DAC output.  The REC IN

slider controls the recording level of the sum of the signals

mixed for recording.

4) Pocket Recorder has greyed-out compression controls.  They will

be enabled when compression becomes part of the Multimedia

Windows architecture.

5) TrakBlaster Pro does not support the higher IRQ's or DMA's.

6) We have provided a small program called DMAFIX.EXE that restores

the DMA channels to a safe state when run after running

TrakBlaster Pro.  We suggest you make a file called TBPRO.BAT

that has 3 lines in it:

@ECHO OFF

FILE_REQ

DMAFIX

Make sure you copy DMAFIX.EXE to your TrakBlaster Pro

subdirectory.  TrakBlaster Pro leaves the DMA channel in a state

that causes Windows to slow to a crawl.  This program is a

temporary fix.  We will contact Volker Zinke, the author of

TrakBlaster Pro, to make the correct fix.

**********************************************************************

CONTACTING MEDIA VISION

**********************************************************************

Media Vision, Inc        Main:         (800) 348-7116 or (510) 770-8600

3815 LaurelView Ct.      Sales:        (800) 845-5870

Fremont, CA 94539        Tech Support: (800) 638-2807 or (510) 770-9905

FAX:          (510) 770-8648 or (510) 770-9592

Developer Assistance Program:  (800) 472-6147

BBS: (510) 770-0968 2400 (8 bits, no parity, 1 stop bit)

(510) 770-1661 2400 (8 bits, no parity, 1 stop bit)

(510) 770-0527 9600 (8 bits, no parity, 1 stop bit)

CIS: GO MEDIAVISION or send CIS mail to 75300,2772

**********************************************************************

INSTALLATION FILES        WINDOWS DRIVERS        WINDOWS APPLICATIONS

------------------       -----------------       --------------------

file1.lis                 mcimixer.drv              pmix.exe

file2.lis                 mvmixer.drv               prec.exe

file3.lis                 mvproaud.drv              promix.exe

file4.lis                 vadmad.386                mix.exe

file5.lis                 vpasd.386

install.ins               vtbd.386

setup.exe                 opl3.drv

oemsetup.inf              mmmixer.dll

tbwave.drv

tlwave.drv

mvfm.drv

sbwave.drv

DOCUMENTATION             MISCELANEOUS

--------------           ----------------

mixapi.wri                midimap.cfg

prec.wri                  mmmixer.h

readme.doc                musicbox.ini

mmmixer.lib

WINDOWS DRIVERS

---------------

MCIMIXER.DRV    This driver provides a set of extended MCI commands

for accessing mixer capabiities.

See MCIMIXER.WRI for details of the command set.

MVMIXER.DRV     This is Media Vision's Mixer driver.

The mixer driver adds mixer state information to

your WIN.INI file:

[mvmixer.drv]

LineIn1=SYNTH     MUS]  PLAY     L=100% R=100%  @=0000

LineIn2=MIXER     MIX]  PLAY     L= 72% R= 72%  @=0000

LineIn3=AUX       AUX|  PLAY     L=100% R=100%  @=0000

LineIn4=CD        CDA|  PLAY     L=  0% R=  0%  @=0000

LineIn5=MIC       MIC|  PLAY     L=  0% R=  0%  @=0000

LineIn6=DIGITAL   WAV]  RECORD   L=  0% R=  0%  @=0000

LineIn7=SPKR      SPK]  PLAY     L=100% R=100%  @=0000

LineIn7=BLSTR     WAV]  PLAY     L=100% R=100%  @=0000

LineOut1=MASTER   AMP|  PLAY     L= 79% R= 79%  @=0000

LineOut2=RECORD   WAV|  PLAY     L=100% R=200%  @=0000

StereoEnhance=ON

Loudness=ON

Bass=50

Treble=78

SaveSettingsOnWindowsExit=YES

LINE FIELD:

There are 7 input Lines and 1 output line.

Note: the Pro AudioSpectrum 16 has a second output

line.

PATCH NAME:

The field immediately after the '=' is the patch name.

These names (ie. SYNTH) appear in promix.exe at the

bottom of each slider pair.

DEVICE TYPE

The next field is the device type field.  The device

type determines the type of icon displayed in MIX.EXE

Each device type has a standard 3-letter mnemonic name

that can be obtained by calling mixGetDeviceName.  The

current list of types is:

N/C   No Connection             SPK        PC speaker

CDA   Compact Disk Player       AAT        Analog Audio Tape

AUX   Auxillary                 DAT        Digital Audio Tape

MIC   Microphone                TEL        Telephone

VTR   Video Tape Recorder       MIX        Mixer

VDP   Video Disc Player         EQU        Equalizer

MUS   Musical Instrument        AMP        Amplifier

WAV   Digital Waveform Player   LPA        Phonograph Audio

RAD   Radio                     TVA        Television Audio

UNK   Unknown Device Type

The fourth character indicates whether the user is

responsible for making the physical device connection.

Think of the character as the mounting bracket of an audio

card.  A hole, the '|' character, represents a mini

phone jack.  If the fourth character is ']', the connection

is made by the driver.  The '|' character indicates a jack

or connector must be hooked up before audio can be

heard.  User-friendly applications may use this information

to remind the user to connect a particular device.

OUTPUT CONNECTION

The next field corresponds to the internal mixer

connection info.  The word PLAY indicates that input

is routed to the PLAY output (LineOut1).  RECORD

means the input is routed to the ADC circuitry.

VOLUMES

If it isn't obvious, the L= and R= fields specify the

volumes of the left and right channels at start-up.

ASSOCIATION

This field is used to save information that specifies

a relationship between a mixer input and some other

device in the system.  For example, if there are two

CD's in the system and one is connected to LineIn4

and the other is connected to LineIn3, the

dwAssociation can be used to determine which MCI CD

device is on which input.

MISC

The other lines are self explanatory.  In the future

we will let users change the settings through control

panel's driver setup.

CHANGING WIN.INI

As an example, if you connect your Walkman to LineIn3

(AUX), you can replace the word AUX with RADIO.

Then, change the device type field to RAD|

/////////////////////////////////////////////////////

MVPROAUD.DRV    This driver supports WAVE (digital sound) audio, MIDI I/O

and auxillary volume control.

This driver keeps configuration information in your

system.ini file under the header [mvproaud.drv]

If it fails to find that entry, it will

check to see if MVSOUND.SYS is loaded and use it's

settings.  Failing that, it will look for the

outmoded [mvwave.drv] and [mvmidi.drv] entries.

As a last resort, the driver will default to IRQ=7

and DMA=1.

When our aux driver is the only one referred to

in the drivers section, a call to auxGetNumDevs

returns a value indicating 4 aux devices are in the system:

Device 0 is internal CD-ROM volume

Device 1 is auxillary (line in) volume

Device 2 is microphone volume

Device 3 is master volume

While we provide a Multimedia Mixer library, we also

support the existing volume control calls, auxGetVolume

and auxSetVolume, when this driver is installed.

VADMAD.386        system patch file required for digital audio under

Windows 3.0

VPASD.386       This driver provides extra buffer space needed to handle

16-bit digital audio.  If you experience problems

with using audio during a Windows DOS session, remove

this driver from system.ini.

VTBD.386        This driver provides device contention services for

the ThunderBoard.

OPL3.DRV        OPL-3 is the name of the Yamaha FM-synthesizer chip

on the PAS Plus, PAS 16 and CDPC.  This driver treats

the OPL-3 as a stereo MIDI device.

MMMIXER.DLL     MMMIXER.DLL is like MMSYSTEM.DLL in a couple of ways.

First, it provides a driver-independent layer for all

mixer API's.  Second, it loads any mixer device drivers

in the system.

This DLL was developed to provide MultiMedia developers

better control over audio mixing functionality than

is afforded by the current volume API's.  We have

offered this DLL to Microsoft and the MPC council as a

standard for Multimedia Windows volume control.  There

See the MIXSPEC directory for detailed information on the

Multimedia Mixer API.

OEMSETUP.INF    This file allows you to add drivers to Windows using

the drivers applet in the control panel.

TBWAVE.DRV      This is the .WAV driver for the ThunderBoard.

TLWAVE.DRV      This is the .WAV driver for the Thunder & Lightning.

MVFM.DRV        This driver is for the original Pro AudioSpectrum.

It supports the two OPL-2 chips on that card as a

single stereo MIDI device.

SBWAVE.DRV      This is a Snd Blaster compatible WAVE driver for the

Sound Blaster compatible section of the Pro

AudioSpectrum 16. It can be used in addition to the

mvproaud.drv wave driver giving Windows the

capability to play two digital audio sounds at once.

WINDOWS APPLICATIONS

---------------------

MIX.EXE         This program allows you to modify those mixing controls

that you'll want to change most often.

Its main difference from promix is that it doesn't

cover nearly as much screen area.

PROMIX.EXE      ProMix can be launced from the WIN.INI 'load' line.

If you have Windows 3.1, use the Program Manager's

File/Preferences menu item to select the open

minimized option.

Here's a nifty feature:  When running Windows, you can

use the keyboard to affect the volume and the

equalization

CTRL-ALT-U                raises master volume

CTRL-ALT-D                lowers master volume

CTRL-ALT-M                 Mutes the amplifier

CTRL-ALT-S                Toggles Stereo Enhance

CTRL-ALT-L                Toggles Loudness

CTRL-ALT-B                Sets BASS mode

CTRL-ALT-T                Sets TREBLE mode

When in Treble or Bass mode, you can affect the level:

CTRL-ALT +                Increases BASS or TREBLE level

CTRL-ALT -                Reduces   BASS or TREBLE level

ProMix now features a real-time VU meter that will

display the current recording level.  The VU meter

will be active all the time on the PAS Plus

and PAS 16 but on the CDPC and origintal PAS, the VU

meter will only be active during recording and

playback of digital audio.

PMIX.EXE         Pocket Mixer - another mixer application

PREC.EXE        Pocket Recorder - a Windows WAV file recorder.

See PREC.WRI for more info

MISCELLANEOUS

----------------

MMMIXER.H        Header file for access to the Multimedia Mixer

API documented in MIXAPI.DOC

MMMIXER.LIB        Library file for access to the Multimedia Mixer

API documented in MIXAPI.DOC

MIDIMAP.CFG        Configuration file for the Control panel's MIDI

Mapper applet

MUSICBOX.INI        CD definition file for Microsoft's MUSICBOX.EXE

application (only distributed with Win 3.0 MME's)

DOCUMENTATION

----------------

PREC.WRI        This file contains information on Pocket Recorder

MCIMIXER.WRI    This file describes the MCI Mixer command set.

Examples command strings are included.

MIXAPI.WRI      This file describes the Multimedia Mixer API.

_____________________________________________________________________

CHANGES THIS RELEASE:

1) Using MCI player to play short .WAV files no longer skips

2) MediaPlayer 'thumb' not correct on playback of some WAV formats

3) "Audio dropouts when running Microsoft Works, Kings Quest V,

Grandma and Me and BattleChess" have been fixed.

4) Pocket Recorder's level indicators now work.

5) Pocket Recorder now supports CUT, COPY and PASTE

---------------------------------------------------------------------

--------------- FREQUENTLY ASKED QUESTIONS --------------------------

---------------------------------------------------------------------

Q:  What is the format of .WAV file?

A:  Here's a sample header from a common WAV file.

00 : 52 49 46 46 94 6C 00 00-57 41 56 45 66 6D 74 20   RIFF.l..WAVEfmt

10 : 10 00 00 00 01 00 01 00-22 56 00 00 22 56 00 00   ........"V.."V..

20 : 01 00 08 00 64 61 74 61-70 6C 00 00 80 80 80 80   ....datapl......

30 : 80 80 80 80 80 80 80 80-80 80 80 80 80 80 80 80   ................

OFFSET   SIZE        NAME                  DESCRIPTION

----------------------------------------------------------------------

00     DWORD        dwRIFF         Must be ascii values for 'RIFF'

04     DWORD        dwRIFFSize     size in bytes of all data following this

dword

08     DWORD        dwRIFFType     Must be ascii values for 'WAVE'

<< start of first chunk which is 'fmt ', the wave format chunk >>

0C     DWORD        dwChunkType    Must be ascii values for 'fmt '

10     DWORD        dwChunkSize    size of format chunk (currently 10h)

14     FORMATCHUNK                 see format chunk broken down below

<< start of second chunk which is 'data ', the wave format chunk >>

24     DWORD        dwChunkType    Must be ascii values for 'data '

(may be different for other format types)

28     DWORD        dwChunkSize    size of data chunk

2C     DATACHUNK                   actual sound data.  80h is centerline

value for 8-bit. 0000h is centerline

for 16-bit.

======================================================================

This describes the format chunk of a standard RIFF .WAV file:

OFFSET  SIZE     NAME                    DESCRIPTION

----------------------------------------------------------------------

0      WORD     wFormatTag        Defines format of data.  Currently

only one type is defined: PCM (a value of 1)

2      WORD     nChannels         1 for mono, 2 for stereo

4      DWORD    nSamplesPerSec    usually 11025, 22050 or 44100 but can

be any value.

8      DWORD    nAvgBytesPerSec   this value is calculated by

(samplerate * nChannels) * (wBitsPerSample/8)

12     WORD     nBlockAlign       this value is calculated by

(nChannels) * (wBitsPerSample/8)

14     WORD     wBitsPerSample    common values are 8 and 16.  You code

should handle the 12 case.

All numbers above are in base 10 (decimal).

----------------------------------------------------------------------

Be aware that all of the above information is based on standard PCM

uncompressed wave files.  Evolving digital audio file formats will

invalidate some of the information described above.

**********************WAV FILE FORMAT***********************

4 bytes         'RIFF'

4 bytes         <length>

4 bytes         'WAVE'

4 bytes         'fmt '

4 bytes         <length>        ; 10h - length of 'data' block

2 bytes         01              ; format tag

2 bytes         01              ; channels (1=mono, 2=stereo)

4 bytes         xxxx            ; samples per second

4 bytes         xxxx            ; average samples per second

2 bytes         01              ; block alignment

2 bytes         08              ; bits per sample

4 bytes         'data'

4 bytes         <length>

bytes         <sample data>

----------------------------------------------------------------------

----------------------------------------------------------------------

Q:  What CD-ROM drives are supported?

A:  *****************SUPPORTED SCSI CD-ROM DRIVES***************

INTERNAL DRIVES:

Chinon CDS-431.  Contact Chinon at 310-533-0274 to get the

40-pin  to 50-pin converter.

Hitachi CDR-3650

NEC CDR-80,  CDR-82, CDR-83, CDR-83J, CDR-83M, CDR-84

Sony    CDU-541

Texel DM-3020, DM-3021

Toshiba XM-3201B

EXTERNAL DRIVES:

Chinon CD-SC(Sony CDU-8001), CD-SC Plus (Sony CDU-8002)

CD Technologies Porta-Drive

Chinon CDX-431

Denon DRD-253

Hitachi CDR-1553S, CDR-1650, CDR-1750S

IBM 3510

Laser Magnetics CM212,  CM214,  CM231

NEC CDR-35, CDR-36, CDR-36M, CDR-72, CDR-73, CDR-73M, CDR-

74, CDR-77

Panasonic CR-501-S

Pioneer DRM-600 6-disc changer

Sony CDU-6211, CDU-7211

Texel DM3010, DM-3120, DM-5020, DM-5021, DM-5120, DM-7120A,

DM-7120B

Toshiba XM-3201A, XM-3301, XM-5100A

If your CD-ROM drive is not on this list, it will NOT work

with the PAS16 (the SONY CDU-535 and the Laser Library are

not SCSI).  If it is on this list and you would like to use

the PAS16's SCSI interface to control the CD-ROM drive,

please contact Media Vision Sales at 1-800-845-5870 to order

an Internal SCSI Cable/Driver Kit for $39.00 or an External

SCSI Cable/Driver Kit for $79.00.

**************WINDOWS DRIVERS AND POCKET TOOLS**************

The Windows Drivers disk uses InstallSHIELD v1.3 by The

Stirling Group.

1.      Q:      What do I need to do before I install the PAS16 Windows

Drivers disk?

A:      If you have the PAS8 Windows drivers installed or have

an older version of PAS16 drivers,  go into the Control

Panel, Drivers, and remove every driver which

references Pro AudioSpectrum or Media Vision.  Restart

WIN31 and then install the new drivers.

Or, if you have Norton Desktop For Windows which will

interfere with our Windows Driver's INSTALL program.

To work around the problem, go into SYSTEM.INI, [boot]

section.  Replace "SHELL=NDW.EXE" with

"SHELL=PROGMAN.EXE".  Install the Windows drivers,

restart WIN31, and make sure that all sounds work.

Replace the "PROGMAN" by "NDW" in SYSTEM.INI.  Restart

WIN31.  Go into Program Manager, File, Run to execute

"SETUP /P" so that Norton will register the Multimedia

Tools Group which has just been created by the INSTALL

program.

Or, if you are using DR DOS 6.0's SHARE which causes

our INSTALL program to crash the system.  To work

around the problem, remove SHARE.  Re-boot the

computer.  Then install our Windows drivers.

2.      Q:      While installing the PAS16 WIN31 drivers, I get the

error "You must first load Multimedia Extensions."

A:      First, go into the Control Panel, Drivers applet to

check the installed drivers list.  There must be MIDI

Mapper, Timer, [MCI] Sound, [MCI] MIDI Sequencer.

Second, check the SYSTEM.INI file.  Under the [boot]

section, there should be "DRIVERS=MMSYSTEM.DLL".  If

any of these items are missing, WIN31 has not been

properly installed.  Installing WIN31 over an older

version of windows may have caused this problem.  The

only solution is to install WIN31 from scratch and then

install the PAS16 WIN31 drivers again.

3.      Q:      After the PAS16 WIN31 drivers installation, runing

WIN31 causes the error "Can't find ver. 2.00 of

VDMAD.386."

A:      Some Windows applications replace this driver by their

own DMA driver, such as Norton tape backup software

which puts under the [386Enh] section of the

SYSTEM.INI, the statement "DEVICE=VDMADX.386".  First,

put a semi-colon ";" infront of  the whole statement.

Second, add the statement "DEVICE=*VDMAD" to the

beginning of the [386Enh] section.  Restart WIN31 and

try again.

4.      Q:      After the PAS16 WIN31 drivers installation, running

WIN31 causes the computer to lock up.

A:      This is caused by a DMA and/or IRQ conflict.  Check for

all possible conflicts; a bus mouse is usally set for

IRQ 5 which will conflict with the SB IRQ 5 default

setting.   Use a text editor to go into SYSTEM.INI.  Go

the the end of the file to locate [MVPROAUD.DRV] and

change the DMA and IRQ settings.  Bring up WIN31 and

try again.

5.      Q:      Do I need MVSOUND.SYS for Windows?

A:      Currently, yes.  In the future, the PAS16's Windows

drivers will not have to depend on MVSOUND.SYS.

6.      Q:      Can I use different IRQ and DMA settings under Windows,

than under DOS?

A:      The DMA set in SYSTEM.INI for Windows can be different

from the DMA set in CONFIG.SYS for DOS as long as WIN31

is started in 386-Enhanced mode.  However , if you

start WIN31 is started in standard mode, you make make

the settings the same.

7.      Q:      What is the advantage of using a high-order DMA and

IRQ?

A:      High-order DMA allows 16-bit DMA transfer which enables

the MVPROAUD.DRV sound driver to handle digital audio

more efficiently and also to provide more available DMA

selections.  High-order IRQ only provides more

available IRQ selections.

8.      Q:      I have just received the latest version of the PAS16

Windows Drivers Disk.  Do I have to remove the older

version first?

A:      Yes.  After removing the old drivers, restart Windows and

then install the new drivers.

9.      Q:      Sound plays normally under DOS using the selected DMA

and IRQ setttings, but loops under Windows using the

same DMA and IRQ settings.

A:      This is caused by an IRQ conflict.  Edit the SYSTEM.INI

file to change the IRQ setting under [mvproaud.drv]

section.

10.     Q:      Pro Mixer's wave meter points at the beginning of the

red area instead of at zero.

A:      Our future release of WIN31 drivers v1.4 will correct

this problem.  Please check with Customer Service after

9/1.

11.     Q:      What do I have to do before I can record using Pocket

Recorder or Sound Recorder?

A:      Go into Pro Mixer or Pocket Mixer to set the audio

input source to recording mode.

12.     Q:      When I record at 16-bit mode, there is very bad

clipping or it does not work at all.  8-bit recording

works fine.

A:      If you are using an low-order DMA then change to a 16-

bit DMA under the [mvproaud.drv] section of SYSTEM.INI

and try again.

13.     Q:      Recording at 8-bit is very scratchy.  16-bit recording

does not have this problem.

A:      The PAS16 has a 16-bit PCM DAC.  Recording at 8-bit

using a 16-bit DAC is definitely more noisy than

recording at 8-bit on the PAS8 which only has an 8-bit

DAC.  In the next release, Pocket recorder will have a

feature to convert a 16-bit recorded file to 8-bit to

resolve the problem.  Please check with Customer

Service after 9/1 to find out if WIN31 Drivers and

Pocket Tools v1.4 is available which will have this

feature.

14.     Q:      Does the Pocket Recorder support the Cut, Copy and

Paste features?

A:      These features will be supported in the next release.

Please check with Customer Service after 9/1 to find

out if WIN31 Drivers and Pocket Tools v1.4 is

available.

15.     Q:      I get the errors "Can't find input device with pid."

and "Can't find output device with pid." when trying to

run Pocket Recorder.

A:      Make sure the MVSOUND.SYS statement appears in

CONFIG.SYS and that the driver loads properly when the

computer boots up.

16.     Q:      I get the error "PREC has caused a General Protection

Fault error in USER.EXE..."

A:      This error occurs most frequently in 286 computers and

also in faster computers which have less than 2MB of

virtual memory under Windows.  To help us troubleshoot

this problem, first, please download PRECGPF.ZIP

(PREC.EXE with GPF error checking) from the BBS.

Second, run DRWATSON.EXE (a WIN31 utility).  Third, run

the new Pocket Recorder.  When WIN31 issues the GPE,

all the necessary information are stored in

DRWATSON.LOG.  Fourth, print out this log and fax it to

Technical Support.  Upon receiving the information, we

will forward to Engineering and await their response.

17.     Q:      If Pocket Recorder is used to record a 16-bit wav file,

and is saved then played from Media Player, the file

will play properly to the end but the thumb button on

the position slider will not have eached the end.  The

thumb button usually stops around the halfway mark.  If

the position button is dragged to the right of this

mark, the message "The specified parameter is out of

range for the specified command" will pop up.  Dragging

past the halfway mark and playing the file sometimes

results in white noise.

A:      This problem has been reported to Engineering and will

be fixed in the future release of WIN31 Drivers v1.4.

Please contact Customer Service after 9/1 to find out

if this is available.

18.      Q:     While recording under Pocket Recorder, if I

open/close/use other applications, there are

licking/popping sounds that also get recorded.

A:      This problem has been reported to Engineering and is

currently being researched.

19.     Q:      Pocket Recorder's OLE (Object Linking & Embedding) does

not work properly.

A:      The OLE feature is not fully implemented.  Please

document the problems in detail and fax or mail to

Technical Support to be forwarded to Engineering.

20.     Q:      Pocket Recorder issues "Cannot write WAV files".

A:      This problem is caused by slow system throughput.

First, make sure VPASD.386 is installed in SYSTEM.INI.

This driver allocates a 32KB buffer for faster DMA

transfer.  Second, take out SMARTDRV and STACKER, if

installed.  Third, increase the CPU speed, if possible.

21.     Q:      A file recorded using RECFILE.EXE (PAS16's DOS utility)

plays back fine under Pocket Recorder but this same

file will not play back under Media Player.  And under

Sound Recorder, loading this file will cause the

"General Protection Fault Error In Sound Recorder".

A:      A new version of RECFILE.EXE, dated 6-22-92 or later,

does not have this problem.  On BBS, download

P16REC.ZIP; on Compuserve, download TOOLS.ZIP.

22.     Q:      How do I play CD-Audio under WIN31?

A:      First, put a CD-Audio disk into the CD-ROM drive.

Failure to do this will cause the [MCI] CD-Audio driver

to not install properly  Second, go into the Control

Panel, Drivers.  Click on the Add button.  Select

"[MCI] CD-Audio" driver.  Click on the OK button.

WIN31 will prompt you for the Microsoft Windows Disk 4.

Third, after this driver has been properly installed,

go into Media Player.  Select Device.  CD-Audio should

be one of the choices for you to select from.  If  not,

edit MPLAYER.INI to change "CDAUDIO=0," to "CDAUDIO=17,

CDAUDIO".

23.     Q:      I do not get any FM sound when I run DOS games under

the DOS shell of Windows.

A:      The OPL3 FM synthesizer chip on the PAS16 can operate

in either OPL3 (4-operator) mode or 3812 (2-operator)

mode.  Under Windows, Voyetra's OPL3.DRV driver puts

this FM chip into the OPL3 mode to produce rich FM

sounds.  However, existing games only support the 3812

mode.  This driver will put the OPL3 chip back in 3812

mode upon exiting WIN31 to go back to DOS.  However,

wihle still under Windows, if the user attempts to run

a DOS game or application which supports AdLib, FM

sound will not play.  To work around this problem, go

into SYSTEM.INI, [drivers] section, change

MIDI=OPL3.DRV to become MIDI=MVFM.DRV.  Then restart

Windows.  Go into the Control Panel, MIDI Mapper.

Click on New.  Give it a name like "3812 mode".  Under

the Port Name column, select "Media Vision FM Synth"

for channels 1-10 and 13-16 and then save the setup.

Now DOS games and applications can produce FM sounds.

24.     Q:      Under Media Player, CANYON.MID plays properly but when

I play other MIDI files, I don't hear any music.

A:      WIN31 only supports General MIDI.  The MIDI files that

you have may be Non-General MIDI.  Try the following.

Go into Control Panel, MIDI Mapper.  Select the Pro

AudioSpectrum setup.  Click on Edit.  Under the Port

Name column, select "Media Vision FM Synth" for

channels 1-10 and 13-16 and then save the setup.  Now

all MIDI songs should play; however, they may not sound

correct.

25.     Q:      Do you have PAS16 drivers for Windows 3.0 with

Multimedia Extentions?

A:      Download from the BBS a file called MVPROAUD.ZIP.  This

file contains a special version of MVPROAUD.DRV for

Windows 3.0 with MME.  Replacing the MVPROAUD.DRV that

came with the PAS16 WIN31 Drivers disk with this

version is all you need to do.

*************************PAS.EXE****************************

1.      Q:      How do I adjust the volume for the SB section of the

sound card.

A:      Run "PAS SET SB TO XX" where xx is 0-100.

2.      Q:      DOS software produces no volume or very low volume,

especially after exiting WIN31.

A:      Run "PAS RESET" and try again.

3.      Q:      When I run "PAS *", how do I enable the recording mode

for an input source?

A:      Use the up and down arrow keys to move to the input

source desired.  Then press the Enter key to toggle

between PLAY and PLAY & RECORD.

*********************RECFILE, PLAYFILE**********************

NOTE:  Before running RECFILE.EXE, make sure that you have

run PAS * to enable the recording mode for the input

source(s).  See the PAS.EXE section above.

1.      Q:      How do I use the command parameters Vxxx, V+xxx and Fx

for RECFILE.EXE?

A:      Vxxx, where xxx is a number from 000-127 for voice-

activated level.  Only sounds at and above this level

get recorded.

V+xxx, where xxx is 000-127.  Record all when voice is

activated at threshold.

Fx, where x is between 0-6 and selected using the

following guidelines:

Filter Value    Limiting Frequency   Sampling Rate

Frequency Range       18 KHz              44 KHz     near CD

5              16 KHz              32 KHz     cassette

4              12 KHz              24 KHz     FM radio

3               9 KHz              18 KHz     AM radio

2               6 KHz              12 KHz     telephone

1               3 KHz               6 KHz     male voice

0               0 KHz                         mute

2.      Q:      When PLAYFILE.EXE starts and stops, it produces a

"popping" sound.

A:      Get a newer version of  PLAYFILE.EXE.  Please download

P16REC.ZIP from the BBS.

3.      Q:      I am having trouble recording 16-bit wave files using

the V+xxx parameter.

A:      Get a newer version of RECFILE.EXE.  Please download

P16REC.ZIP from the BBS.

4.      Q:      I do not hear any sound when I play back under WIN31's

Media Player a file which has been recorded using

RECFILE.EXE.

A:      Get a newer version of RECFILE.EXE.  Please download

P16REC.ZIP from the BBS.

5.      Q:      I get the error "File is full or blocked" when I run

RECFILE.EXE.

A:      This problem is caused by slow system throughput.

First, try running RECFILE with the /M parameter to

allocate as much memory to RECFILE as possible.

Second, reduce to a generic CONFIG.SYS and

AUTOEXEC.BAT.  Third, make sure the hard disk has been

optimized (unfragmented).  Fourth, try at a lower

sampling rate or try recording to a ramdrive (use DOS's

RAMDRIVE.SYS).

*********************CD-ROM APPLICATIONS********************

1.      Q:      After running Compton's Multimedia Encyclopedia, I only

hear sounds out of one channel.

A:      When Compton's plays digital audio (represented by .WAV

file extension), it puts the CD-ROM drive into mono

mode and also shuts down one channel.  For a temporary

solution, you need to do a cold boot to reset the CD-

ROM drive.  To resolve this problem, download

COMPTONS.ZIP from the BBS or contact Customer Service

to have this file sent via mail.

2.      Q:      While installing MS Bookshelf, at 99% completion,

installation aborts with an error message in SETUP.INF

file.

A:      First, back up SETUP.INF.  Second, go into SETUP.INF

and remove all lines that start with a semi-colon ";"

and then save it and re-install Bookshelf

3.      Q:      National Geographic Society's Presidents Starting With

George runs fine but does not produce audio.

A:      This application uses IBM Linkway and supports SB.

During installation, the driver statement

"DEVICE=SNDBK12.SYS 1" gets added to CONFIG.SYS.  This

driver defaults to IRQ 7 and I/O port 220h and DMA 1.

The SB section of the sound card defaults to IRQ 5 and

I/O port 220h.  Edit the CONFIG.SYS to show

"DEVICE=SNDBK12.SYS 1 5 220", where 5 is for IRQ 5 and

220 is for I/O port 220h, or which ever settings you

have at J10 and J12 on the sound card.

********************BUS MASTERING DEVICE********************

1.      Q:      I hear periodic clicking sound or static noise whenever

the hard disk's light goes on (while drive is being

accessed).

A:      Lower the bus on-time and increase the bus off-time by

the same amount so that the on-time is smaller than the

off-time, or experiment to find the best ratio.  We

discovered this from using the Adaptec 1542B SCSI Hard

Disk Controller.  The install program for this card

adds the following statements to CONFIG.SYS:

DEVICE=C:\ASPI4DOS.SYS /N11 /F5

DEVICE=C:\ASPIDISK.SYS

Where N11 is 11/16ms and F5 is 5/16ms (default

settings).  The problem was resolved when we changed to

N5 and F11.

********************OS/2 DRIVER SUPPORT*********************

1.      Q:      Are there any plans to support OS/2 v2.0?

A:      Yes, we are currently working on it.  Please contact

Customer Service after 10-1-92.

********************WINDOWS NT SUPPORT**********************

1.      Q:      Are there any plans to support Windows NT?

A:      Please check with Customer Service after 10-1-92.