TABLE OF CONTENTS

ioblix.resource/--background--

ioblix.resource/AddIRQHook

ioblix.resource/AllocChipList

ioblix.resource/FindChip

ioblix.resource/FreeChipList

ioblix.resource/ObtainChip

ioblix.resource/ObtainChipShared

ioblix.resource/ReleaseChip

ioblix.resource/ReleaseChipShared

ioblix.resource/RemIRQHook

ioblix.resource/--background--                   ioblix.resource/--background--

PURPOSE

The ioblix.resource is created by SetupIOBlix during startup and provides

several functions to gain exclusive as well as shared access to any chip

the IOBlix multi I/O boards provide. If you ever want to use one of the

IOBlix chips you should ALWAYS use either ObtainChip() or

ObtainChipShared() to prevent other ioblix.resource-conform programs from

interferring with your program. All device drivers being delivered with

your IOBlix board follow this rule. And hopefully all third-party drivers

will also do.

To access these functions just call OpenResource(IOBLIXRESNAME) and use

the returned pointer in the same manner as library bases are used.

ioblix.resource/AddIRQHook                           ioblix.resource/AddIRQHook

NAME

AddIRQHook  - add a private interrupt function to the IOBlix interrupt

chain

SYNOPSIS

void = AddIRQHook( node )

A0

void AddIRQHook( struct IRQHookNode * );

FUNCTION

Adds the given node to the IOBlix interrupt chain. This is necessary

because normal interrupt handling by AmigaOS will loose some interrupts on

heavy system load.

Just imagine two applications using the IOBlix board (eg one serial and

one parallel port). When adding your own interrupt function with Exec's

AddIntServer() the system will just recognize the first of two

simultaneously happening interrupts, because the first function will return

a successfull interrupt handling and Exec will cease any further handling

within its chain. But at this time the second interrupt has still not been

processed. IOBlix' own interrupt chain avoids this problem by checking all

active chips before returning control to Exec.

The function given in ihn_HookFunc will be passed the data given in

ihn_HookUserData on the stack. Just as Exec's interrupt handling functions

a return value of 0 indicates that no interrupt has happened. Any other

return value indicates a successfull interrupt handling.

INPUTS

node     -- a correctly initialized IRQHookNode

EXAMPLE

/* add a private interrupt function */

ULONG __saveds myIRQFunc( APTR userData);

APTR myUserData;

struct IRQHookNode *node;

if (IOBlixBase = OpenResource(IOBLIXRESNAME)) {

node->ihn_Node.ln_Name = "TestHook";

node->ihn_Node.ln_Pri = 5; /* priority within the chain */

node->ihn_HookFunc = myIRQFunc;

node->ihn_HookUserData = myUserData;

AddIRQHook(node);

/* do whatever you want */

RemIRQHook(node);

}

ULONG myIRQFunc( APTR userData )

{

/* check wether an interrupt has happened or not                   */

/* if yes, then do all necessary things and return a value of != 0 */

/* else just return 0                                              */

}

SEE ALSO

RemIRQHook

ioblix.resource/AllocChipList                     ioblix.resource/AllocChipList

NAME

AllocChipList -- get a list of all available chips

SYNOPSIS

list = AllocChipList( void )

D0

struct List *AllocChipList( void );

FUNCTION

This function returns a copy of the internal list of all available chips.

You are allowed to iterate through the list, but you must not change

anything. All nodes in the list are of type (struct IOBlixChipNode *). No

chip in the list is obtained in a ObtainChip()-like manner.

RESULTS

chipNode -- a list of all available chips, or NULL if the call failed

EXAMPLE

/* print information about all IOBlix chips in the system */

struct IOBlixResource *IOBlixBase;

struct List *list;

struct IOBlixChipNode *node;

if (IOBlixBase = OpenResource(IOBLIXRESNAME)) {

if (list = AllocChipList()) {

node = (struct IOBlixChipNode *)chipList->lh_Head;

while (node->icn_Node.ln_Succ) {

switch (node->icn_Type) {

case ICT_Z2_SERIAL_CHIP:

printf("IOBlix Z2 UART, %s, %ld bytes FIFO\n",

node->icn_Description, node->icns_FIFOSize);

break;

case ICT_Z2_PARALLEL_CHIP:

printf("IOBlix Z2 parallel port, %s, %ld bytes FIFO\n",

node->icn_Description, node->icnp_FIFOSize);

break;

default:

break;

printf("chip is in use by %s\n",

(node->icn_Owner) ? node->icn_Owner : "nobody");

}

node = (struct IOBlixChipNode *)node->icn_Node.ln_Succ);

}

FreeChipList(list);

} else {

printf("failed to allocate chip list\n");

}

}

SEE ALSO

FreeChipList

ioblix.resource/FindChip                               ioblix.resource/FindChip

NAME

FindChip -- find a chip in the resource's database

SYNOPSIS

chipNode = FindChip(chipType, chipNum)

D0                  D0        D1

struct IOBlixChipNode *FindChip( ULONG, ULONG );

FUNCTION

This function searches the internal list for the chip matching the given

values for chip type and number. If the search is successfull the chip's

information block is returned, but the chip is NOT obtained!

You may use this function just to check if a certian chip is available

and to get some information about it. All fields in IOBlixChipNode are

considered READ-ONLY, you must not change anything.

INPUTS

chipType -- the type of chip you want to find

chipNum  -- internal number of the chip you want to find

valid range is 0..IOBLIX_Z2_NUM_SERUNITS for UARTs

0..IOBLIX_Z2_NUM_PARUNITS for parallel ports

0..IOBLIX_Z2_NUM_FIFOUNITS for external FIFOs

to obtain a chip on multiple IOBlix boards use

boardNum*10+chipNum as value, ie 13 is the third UART on the

second board

RESULTS

chipNode -- a pointer to the chip's information block, or NULL if the

chip is not available.

EXAMPLE

/* try to find serial unit 0 on first IOBlix board */

struct IOBlixResource *IOBlixBase;

struct IOBlixChipNode *chipInfo;

if (IOBlixBase = OpenResource(IOBLIXRESNAME)) {

if (chipInfo = FindChip(ICT_Z2_SERIAL_CHIP, 0)) {

/* chip is available, let's see what it is able to do */

printf("UART is a %s with %ld bytes FIFO"\n,

chipInfo->icn_Description, chipInfo->icns_FIFOSize);

printf("UART is in use by %s\n",

(chipInfo->icn_Owner) ? chipInfo->icn_Owner : "nobody");

} else {

printf("serial port #0 is not available\n");

}

}

SEE ALSO

ReleaseChip, FindChip

ioblix.resource/FreeChipList                       ioblix.resource/FreeChipList

NAME

FreeChipList -- free a previously allocated chip list

SYNOPSIS

void = FreeChipList( list )

A0

void FreeChipList( struct List * );

FUNCTION

This function frees the given list again.

INPUTS

list     -- a list previously allocated by AllocChipList(). Passing NULL

is a no-op.

SEE ALSO

AllocChipList

ioblix.resource/ObtainChip                           ioblix.resource/ObtainChip

NAME

ObtainChip -- obtain a chip for exclusive use

SYNOPSIS

chipNode = ObtainChip(chipType, chipNum, newOwner, oldOwner)

D0                    D0        D1          A0        A1

struct IOBlixChipNode *ObtainChip( ULONG, ULONG, UBYTE *, UBYTE ** );

FUNCTION

This function attempts to obtain the specified chip for exclusive use.

The name newOwner is used as an identifier to tell other programs who

currently has obtained the chip.

If the chip is not in use by anybody then the IOBlixChipNode of this chip

is returned to show a successfull allocation. If the chip is already in

use then the return value will be NULL, and the identifier of the using

program is retured in oldOwner, if a valid pointer is given. If also

oldOwner is still NULL then the chip is not available.

INPUTS

chipType -- the type of chip you want to obtain

chipNum  -- internal number of the chip you want to obtain

valid range is 0..IOBLIX_Z2_NUM_SERUNITS for UARTs

0..IOBLIX_Z2_NUM_PARUNITS for parallel ports

0..IOBLIX_Z2_NUM_FIFOUNITS for external FIFOs

to obtain a chip on multiple IOBlix boards use

boardNum*10+chipNum as value, ie 13 is the third UART on the

second board

newOwner -- a string to identify you as the current owner of the chip

oldOwner -- pointer to a string where the identfier of a previous owner

will be placed. If NULL then it will not be used.

RESULTS

chipNode -- If the chip is available and not in use then chipNode will be

a pointer to IOBlixChipNode, which contains various

information about the chip. If the chip is either not

available or already obtained by someone else, this function

will return NULL. Use oldOwner to check if the chip is

already in use.

oldOwner -- If given a valid pointer and the chip is obtained by someone

else, the previous allocators identifier will be placed here.

EXAMPLE

/* try to obtain serial unit 0 on first IOBlix board */

struct IOBlixResource *IOBlixBase;

struct IOBlixChipNode *chipInfo;

UBYTE *oldOwner = NULL;

if (IOBlixBase = OpenResource(IOBLIXRESNAME)) {

if (chipInfo = ObtainChip(ICT_Z2_SERIAL_CHIP, 0, "Test", &oldOwner)) {

/* chip is obtained now, do whatever you want */

ReleaseChip(chipInfo);

} else {

if (oldOwner) {

printf("serial port #0 is in use by %s\n", oldOwner);

} else {

printf("serial port #0 is not available\n");

}

}

}

SEE ALSO

ObtainChipShared, ReleaseChip, ReleaseChipShared, FindChip

ioblix.resource/ObtainChipShared               ioblix.resource/ObtainChipShared

NAME

ObtainChipShared -- obtain a chip for shared use

SYNOPSIS

chipNode = ObtainChipShared(chipType, chipNum, newOwner, oldOwner)

D0                    D0        D1          A0        A1

struct IOBlixChipNode *ObtainChipShared( ULONG, ULONG,

UBYTE *, UBYTE ** );

FUNCTION

This function attempts to obtain the specified chip for shared use.

The name newOwner is used as an identifier to tell other programs who

currently has obtained the chip.

If the chip is not in use by anybody or already obtained in shared mode

then the IOBlixChipNode of this chip is returned to show a successfull

allocation. If the chip is already obtained in exclusive mode the return

value will be NULL, and the identifier of the using program is retured in

oldOwner, if a valid pointer is given. If also oldOwner is still NULL then

the chip is not available.

When using a chip in shared mode you MUST use the semaphore contained in

the returned IOBlixChipNode while accessing the chip with Exec's semaphore

functions to gain exclusive access for a short time. After that you MUST

give access back to the system again with exec.library/ReleaseSemaphore.

When you got access to the chip you should save the important registers of

the chip (ie the control register of a parallel port), then put the values

you need there. Before calling ReleaseSemaphore() on the IOBlixChipNode's

semaphore you should put the previously saved values back to the important

registers to leave the chip in a specified state.

When this call succeeds the ICFF_SHARED flag will be set in the chip

node's icn_Flags field. It will remain set unless the last user has

released the chip with ReleaseChipShared(). The list

icn_SharedAccessorList contains all users, while icn_Owner just contains

the string "shared use".

INPUTS

chipType -- the type of chip you want to obtain

chipNum  -- internal number of the chip you want to obtain

valid range is 0..IOBLIX_Z2_NUM_SERUNITS for UARTs

0..IOBLIX_Z2_NUM_PARUNITS for parallel ports

0..IOBLIX_Z2_NUM_FIFOUNITS for external FIFOs

to obtain a chip on multiple IOBlix boards use

boardNum*10+chipNum as value, ie 13 is the third UART on the

second board

newOwner -- a string to identify you as the current owner of the chip

oldOwner -- pointer to a string where the identfier of a previous owner

will be placed. If NULL then it will not be used.

RESULTS

chipNode -- If the chip is available and not in use then chipNode will be

a pointer to IOBlixChipNode, which contains various

information about the chip. If the chip is either not

available or already obtained by someone else, this function

will return NULL. Use oldOwner to check if the chip is

already in use.

oldOwner -- If given a valid pointer and the chip is obtained by someone

else, the previous allocators identifier will be placed here.

EXAMPLE

/* try to obtain serial unit 0 on first IOBlix board */

struct IOBlixResource *IOBlixBase;

struct IOBlixChipNode *chipInfo;

struct UARTRegisters *uart;

UBYTE *oldOwner = NULL;

UBYTE scratch

if (IOBlixBase = OpenResource(IOBLIXRESNAME)) {

if (chipInfo = ObtainChipShared(ICT_Z2_SERIAL_CHIP, 0,

"Test", &oldOwner)) {

/* chip is obtained in shared mode now */

uart = (struct UARTRegisters *)chipInfo->icn_ChipRegisters;

/* now access the chip exclusively for a short time */

ObtainSemaphore(&chipInfo->icn_SharedAccessSema);

/* first save some importan registers */

scratch = *uart->ur_scr;

/* now do whatever you want */

*uart->ur_scr = 0x42;

/* put back the saved registers */

*uart->ur_scr = scratch;

/* release the chip from exclusive access */

ReleaseSemaphore(&chipInfo->icn_SharedAccessSema);

/* completely release the chip */

ReleaseChipShared(chipInfo, "Test");

} else {

if (oldOwner) {

printf("serial port #0 is in use by %s\n", oldOwner);

} else {

printf("serial port #0 is not available\n");

}

}

}

SEE ALSO

ObtainChip, ReleaseChip, ReleaseChipShared, FindChip

ioblix.resource/ReleaseChip                         ioblix.resource/ReleaseChip

NAME

ReleaseChip -- release a previously obtained chip from exclusive use

SYNOPSIS

ReleaseChip(chipNode)

A0

void ReleaseChip( struct IOBlixChipNode * );

FUNCTION

ReleaseChip() is the inverse of ObtainChip(). After calling ReleaseChip()

the given chip is available again for others.

Each ObtainChip() call must be balanced by exactly one ReleaseChip() call.

Needless to say, havoc breaks out if the task releases a chip more times

than it has obtained it.

INPUTS

chipNode -- the node previously returned by ObtainChip(). Passing NULL is

a no-op.

NOTES

Never release a shared obtained chip with ReleaseChip()!

SEE ALSO

ObtainChip, ObtainChipShared, ReleaseChipShared

ioblix.resource/ReleaseChipShared             ioblix.resource/ReleaseChipShared

NAME

ReleaseChipShared -- release a previously obtained chip from shared use

SYNOPSIS

ReleaseChipShared(chipNode, owner)

A0        A1

void ReleaseChipShared( struct IOBlixChipNode *, UBYTE * );

FUNCTION

ReleaseChipShared () is the inverse of ObtainChipShared().

Each ObtainChipShared() call must be balanced by exactly one

ReleaseChipShared() call. Needless to say, havoc breaks out if the task

releases a chip more times than it has obtained it. The chip remains in

shared mode unless all obtainers have released it.

You should call ReleaseChipShared() with the same owner name as

ObtainChipShared(), else the chip will remain obtained.

When the last obtainer calls ReleaseChipShared() the ICFF_SHARED flag will

be reset.

INPUTS

chipNode -- the node previously returned by ObtainChipShared(). Passing

NULL is a no-op.

owner    -- the same owner as used for ObtainChipShared()

NOTES

Never release an exclusively obtained chip with ReleaseChipShared()!

SEE ALSO

ObtainChip, ObtainChipShared, ReleaseChip

ioblix.resource/RemIRQHook                           ioblix.resource/RemIRQHook

NAME

RemIRQHook  - remove a previously installed interrupt function

SYNOPSIS

void = RemIRQHook( node )

A0

void RemIRQHook( struct IRQHookNode * );

FUNCTION

Removes the given node from IOBlix' own interrupt chain.

INPUTS

node     -- a correctly initialized IRQHookNode

SEE ALSO

AddIRQHook