TABLE OF CONTENTS

ioblixpar.device/AllocECPInfo

ioblixpar.device/CMD_CLEAR

ioblixpar.device/CMD_FLUSH

ioblixpar.device/CMD_READ

ioblixpar.device/CMD_RESET

ioblixpar.device/CMD_START

ioblixpar.device/CMD_STOP

ioblixpar.device/CMD_WRITE

ioblixpar.device/FreeECPInfo

ioblixpar.device/GetChipInfo

ioblixpar.device/OpenDevice

ioblixpar.device/PDCMD_QUERY

ioblixpar.device/PDCMD_SETPARAMS

ioblixpar.device/AllocECPInfo                     ioblixpar.device/AllocECPInfo

NAME

AllocECPInfo -- get the IEEE1284 probing information

SYNOPSIS

epi = AllocECPInfo(ioRequest)

D0                     A0

struct ECPProbeInformation *AllocECPInfo( struct IORequest * );

FUNCTION

Modern parallel port devices are able to send some information about

themself back to the controlling machine. These information include

manufacturer, model, command sets, current state, etc.

The information block contains all these data in parsed form as well as

the string that was originally read from the device.

INPUTS

ioRequest        - pointer to a valid ioRequest after a successfull

OpenDevice() call.

RESULTS

epi  - pointer to struct ECPProbeInformation, or NULL if the call

failed. If connected device is IEEE1284 compatible then

epi->epi_IsIEEE1284Compatible will be TRUE and the other fields

contain all the data that where read from the device.

EXAMPLE

/* try to get ECP probe information */

struct IORequest *ioreq;

struct ECPProbeInformation *epi;

if (!OpenDevice(IOBLIXPAR_NAME, 0, ioreq, 0) {

if (epi = AllocECPInfo(ioreq)) {

if (epi->epi_IsIEEE1284Compatible) {

printf("device %s was manufactured by %s\n",

epi->epi_Description, epi->epi_Manufacturer);

} else {

printf("no IEEE1284 compatible device found\n");

}

FreeECPInfo(epi);

} else {

printf("AllocECPInfo() failed\n");

}

CloseDevice(ioreq);

}

SEE ALSO

ioblixpar.device/CMD_READ, FreeECPInfo

ioblixpar.device/CMD_CLEAR                           ioblixpar.device/CMD_CLEAR

NAME

Clear -- clear the parallel port buffer

FUNCTION

This command just RTS's (no buffer to clear)

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_CLEAR (05)

ioblixpar.device/CMD_FLUSH                           ioblixpar.device/CMD_FLUSH

NAME

Flush -- clear all queued I/O requests for the parallel port

FUNCTION

This command purges the read and write request queues for the

parallel device. The currently active request is not purged.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_FLUSH (08)

ioblixpar.device/CMD_READ                             ioblixpar.device/CMD_READ

NAME

Read -- read input from parallel port

FUNCTION

This command causes a stream of characters to be read from the

parallel I/O register. The number of characters is specified in

io_Length. The EOF and EOL modes are supported, but be warned that

using these modes can result in a buffer overflow if the proper

EOL or EOF character is not received in time. These modes should

be used only when the sender and receiver have been designed to

cooperate. A safety guard can be implemented to EOF by setting

io_Length to a maximum allowed value. That cannot be done with EOL

since the EOL mode is identified by io_Length=-1.

The ioblixpar.device has no internal buffer; if no read request has

been made, pending input (i.e. handshake request) is not

acknowledged.

ioblixpar.device does IEEE1284 reads. This means if the connected

device is not IEEE1284 compatible io_Error will be set to

ParErr_NotIEEE1284. On success the unparsed data will be returned in

io_Data with io_Length set to the amount of data read.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_READ (02)

io_Flags        If IOF_QUICK is set, driver will attempt Quick IO

io_Length       number of characters to receive.

io_Data         pointer where to put the data.

RESULTS

io_Error -- if the Read succeded, then io_Error will be null.

If the Read failed, then io_Error will contain an error code.

SEE ALSO

ioblixpar.device/PDCMD_SETPARAMS, ioblixpar.device/AllocECPInfo

ioblixpar.device/CMD_RESET                           ioblixpar.device/CMD_RESET

NAME

Reset -- reinitializes the parallel device

FUNCTION

This command resets the parallel device to its freshly initialized

condition. It aborts all I/O requests both queued and current and

sets the devices's flags and parameters to their boot-up time

default values. At boot-up time the PTermArray is random, and it

will be so also here.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_RESET (01)

RESULTS

Error -- if the Reset succeded, then io_Error will be null.

if the Reset failed, then the io_Error will be non-zero.

ioblixpar.device/CMD_START                           ioblixpar.device/CMD_START

NAME

Start -- restart paused I/O over the parallel port

FUNCTION

This command restarts the current I/O activity on the parallel

port by reactivating the handshaking sequence.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_START (07)

SEE ALSO

ioblixpar.device/CMD_STOP

ioblixpar.device/CMD_STOP                             ioblixpar.device/CMD_STOP

NAME

Stop -- pause current activity on the parallel device

FUNCTION

This command halts the current I/O activity on the parallel

device by discontinuing the handshaking sequence. The stop and

start commands may not be nested.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_STOP (06)

SEE ALSO

ioblixpar.device/CMD_START

ioblixpar.device/CMD_WRITE                           ioblixpar.device/CMD_WRITE

NAME

Write -- send output to parallel port

FUNCTION

This command causes a stream of characters to be written to the

parallel output register. The number of characters is specified in

io_Length, unless -1 is used, in which case output is sent until

a zero byte occurs in the data. This is independent of, and may be

used simultaneously with setting the EOFMODE in io_ParFlags and using

the PTermArray to terminate the read or write.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_WRITE (03)

io_Flags        If IOF_QUICK is set, driver will attempt Quick IO

io_Length       number of characters to transmit, or if set

to -1 send until zero byte encountered

io_Data         pointer to block of data to transmit

RESULTS

io_Error -- If the Write succeded, then io_Error will be null.

If the Write failed, then io_Error will contain an error code.

SEE ALSO

ioblixpar.device/PDCMD_SETPARAMS

ioblixpar.device/FreeECPInfo                       ioblixpar.device/FreeECPInfo

NAME

FreeECPInfo -- free the IEEE1284 probing information

SYNOPSIS

FreeECPInfo(epi)

A0

void FreeECPInfo( struct ECPProbeInformation * );

FUNCTION

Free the gathered ECP probing information again. Each AllocECPInfo()

call should be balanced by exactly one FreeECPInfo() call to avoid

memory leaks.

INPUTS

epi - pointer to a previously allocated ECP information block. Passing

NULL is a no-op.

SEE ALSO

AllocECPInfo

ioblixpar.device/GetChipInfo                       ioblixpar.device/GetChipInfo

NAME

GetChipInfo -- get the chip's information block

SYNOPSIS

chipInfo = GetChipInfo(ioRequest)

D0                     A0

struct IOBlixChipNode *GetChipInfo(struct IORequest *);

FUNCTION

Get the chip's information block, as it is returned by

ioblix.resource/ObtainChip() and ioblix.resource/FindChip(). All fields

are considered READ-ONLY, you must not change anything.

INPUTS

ioRequest        - pointer to a valid ioRequest after a successfull

OpenDevice() call.

RESULTS

chipInfo  - the chip's information block

SEE ALSO

ioblix.resource/ObtainChip, ioblix.resource/FindChip

ioblixpar.device/OpenDevice                         ioblixpar.device/OpenDevice

NAME

Open -- a request to open the parallel port

SYNOPSIS

error = OpenDevice("ioblixpar.device", unit, ioExtPar, flags)

D0                  A0                 D0    A1        D1

FUNCTION

This function allows the requestor software access to the parallel

device.  Unless the shared-access bit (bit 5 of io_ParFlags) is

set, exclusive use is granted and no other access is allowed

until the owner closes the device.

The PTermArray of the ioExtPar is initialized only if the EOFMODE

bit (bit 1 of io_ParFlags) is set. The PTermArray can be further

modified using the PDCMD_SETPARAMS command.

INPUTS

"ioblixpar.device" - a pointer to literal string "ioblixpar.device"

unit - Must be zero for future compatibility

ioExtPar - pointer to an IO Request block of structure IOExtPar to

be initialized by the Open() function. The io_ParFlags

field must be set as desired.

flags - Must be zero for future compatibility

RESULTS

d0 -- same as io_Error

io_Error -- if the Open succeded, then io_Error will be null.

If the Open failed, then io_Error will be non-zero.

SEE ALSO

exec/CloseDevice

ioblixpar.device/PDCMD_QUERY                       ioblixpar.device/PDCMD_QUERY

NAME

Query -- query parallel port/line status

FUNCTION

This command return the status of the parallel port lines and

registers.

IO REQUEST

io_Message      must have mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      PDCMD_QUERY (09)

RESULTS

io_Status        BIT  ACTIVE  FUNCTION

0     high   printer busy toggle (offline)

1     high   paper out

2     high   printer selected

3      -     read=0,write=1

4-7            reserved

ioblixpar.device/PDCMD_SETPARAMS               ioblixpar.device/PDCMD_SETPARAMS

NAME

SetParams -- change parameters for the parallel device

FUNCTION

This command allows the caller to change the EOFMODE parameter for

the parallel port device. It will disallow changes if any reads or

writes are active or queued.

The PARB_EOFMODE bit of io_ParFlags controlls whether the

io_PTermArray is to be used as an additional termination criteria

for reads and writes.  It may be set directly without a call to

SetParams, setting it here performs the additional service of

copying the PTermArray into the device default array which is used

as the initial array for subsequent device opens. The Shared bit

can be changed here, and overrides the current device access mode

set at OpenDevice time.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       preset by OpenDevice

io_Unit         preset by OpenDevice

io_Command      PDCMD_SETPARAMS (0A)

NOTE that the following fields of your IORequest

are filled by Open to reflect the parallel device's

current configuration.

io_PExtFlags    must be set to zero, unless used

io_ParFlags     see definition in parallel.i or parallel.h

NOTE that 0x00 yields exclusive access, PTermArray

inactive.

io_PTermArray   ASCII descending-ordered 8-byte array of

termination characters. If less than 8 chars

used, fill out array w/lowest valid value.

Terminators are used only if EOFMODE bit of

io_Parflags is set. (e.g. x512F040303030303 )

This field is filled on OpenDevice only if the

EOFMODE bit is set.

RESULTS

io_Error -- if the SetParams succeded, then io_Error will be null.

if the SetParams failed, then io_Error will be non-zero.