!!!!! *YOU* *MUST* read the last paragraph about problems !!!!!

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

!!!!! Don`t wonder about the new size of the Updates and that !!!!!

!!!!! Updater doesn`t print all module versions anymore.      !!!!!

!!!!! This is NO bug. The modules are still there.            !!!!!

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

New Upgrade features:

o WideTermination check

A lot people don`t seem to have read the manual correctly

and use Wide/Narrow bridges without upper Widebus termination

which leads to not working reselection.

(Syndrom: 44.2 works perfectly with your devices but only slowly)

If the check detects such broken SCSI configuration the device

completely disables reselection for all units.

(CDROM is reselection by default)

ATTENTION:

Correct Termination checks are NOT 100%.

o New Unit ID standard so it works with WIDE HDs.

Board * 10*1000*1000

Lun   * 10*1000

Unit  * 10

+ 8 (8 in the last digit switches the format)

so if you wanna access Unit 13 Lun 5 it would look like

500138

or Unit 8 Lun 0

88

or Unit 1 Lun 0

18 (NewStyle)

or

1 (OldStyle)

The old Format was

Board * 100

Lun * 10

Unit * 1

so everybody sees that you could only use 9 Units and Wide

provides 15. I know the the use of Unit=8 as a flag isn`t

elegant but it`s the only compatible system i found.

o Key "2" disables the processor on an 1200 ONLY.

o Key "s" pressed during booting disables the SCSI

also disables the bootmenu, bootmenu settings,

31khz monitor and so on.

o Key "m" pressed during booting disables the memory

o Key "a" pressed during booting disables all

o BootMenu which can be invoked by holding

the Key <ESC> key during the system boot.

(Only works on systems with SCSI)

The Bootmenu now ALWAYS pops up in 31khz mode, so

folks with an A2024 parallel to their normal monitor

will have bad luck. But I assume that a generell 31Khz

Boot monitor is more useful than this problem for users

with that setup.

Then you see a menu with the 3 entries

o SCSI

o List with 8 or 16(WideSCSI) Units.

The entries show the current settings for the UnitID.

The Controller UnitID entry is marked differently.

If you click on the UnitID you get into a simular menu

like Unitcontrol`s Option menu.

If you click on the controller UnitID entry you get into

a simular menu like Unitcontrol`s Prefs menu.

Unit prefs:

o NO LUN

1 LUN

2 LUNs

3 LUNs

4 LUNs

5 LUNs

6 LUNs

7 LUNs

8 LUNs

-> This Entry can be used to disable the scan for the whole

unit for more Luns which can speed up booting a lot.

o Offset

Here you specify the synchron handshake.

o Period

Here you specify the synchron clock.

o ASynchron/Synchron

ASynchron

Synchron

Here you enable the synchron mode, so you

can change Offset and Period now.

The ASynchron/Synchron mode means that the

synchron mode is activated if specified by

the RDB for example.(The AUTO mode)

o Auto Buswidth

8-Bit

16-Bit

Here you specify the buswidth for a unit.

Well..anything more than Auto Mode isn`t really

needed.

o Auto Reselection

No Reselection

Reselection

Disabling reselection for units with problems is

possible here. Auto Mode means that reselection

is activated if the RDB flag is set or if it has

no RDB but is a normal unit like a cdrom.

Scanners and other esotheric units don`t use

reselection in AUTO mode.

o CDBoot Pri

define the cdrom boot priority. Check the System Bootmenu for

the boot priority of other devices.

o CDRom Device Name

define the device name with which the cdrom should be mounted.

o No CDRom Bount

CDRom Mount

CDRom Boot

The buildin CDRom filesystem is the old Phase5 CDRive filesystem.

Its feature set may not be enough for your needs.

Controller Prefs:

o BusID

Controller BusID..normally 7.

The BusID of the controller. This doesn`t mean you

can use SCSI Nets as these don`t work in general.

o SCSIResetDelay

The seconds the device waits after the first

SCSI Reset/Init.

o ErrorDelay

The seconds the device waits after an Inquiry Error

o RemovableDelay

The second intervall for a removable check

o ErrorDelayRetry

The retry count for an Inquiry after an Error.

o ErrorRetry

The retry count for an SCSI Operation after an SCSI Error.

o TimeOut

(not yet used)

o CDFS

o No CDFS HD RDB Scan

CDFS HD RDB Scan

Update the filesystem.resource with CDFS filesystems when these

are placed in the RDB of HDs **BEFORE** the CD Drive at the same

controller. This way you can theoretically use other cdfilesystems.

*Theoretically* means that i haven`t tested if other cd filesystem

work with such startup as they may need to find their prefs.

The CDFS dostype is

#define ID_CDFS_DISK	(0x43444653L) /* 'CDFS' */

The new SCSIConfig has an automatic filesystem entry for this.

o Cache Sets

How many Cache Sets are allocated.

CacheSets*CacheSetBlocks*2048 is the memory needed.

Only useful for CDFS

o Cache Set Blocks

The number of of each cache set.

A cache set is the line of block at Sector & ~(CacheSetBlocks-1) which

is fetched into the cache.

Only useful for CDFS

o RAM

o MemorySpeed 70ns

-> standard waitstates

MemorySpeed 60ns

-> for 50Mhz 060 / 66Mhz PowerPC cards this means

o 68k No Read Waitstate

o 68k No Write Waitstate

o PPC No Read Waitstate

o PPC No Write Waitstate

Free Config

o 68k Read Waitstate

68k No Read Waitstate

-> possible with at least 70ns rams

o 68k Write Waitstate

68k No Write Waitstate

-> possible with at least 70ns rams

o 68k PreCharge

68k No PreCharge

possible with at least 60ns rams  but not guranteed

to work with every ram.

(Enabled only for boards supporting this)

o PPC Read Waitstate

PPC No Read Waitstate

possible with at least 60ns rams

o PPC Write Waitstate

PPC No Write Waitstate

possible with at least 60ns rams

o PPC PreCharge

PPC No PreCharge

possible with at least 60ns rams  but not guranteed

to work with every ram.

(Enabled only for boards supporting this)

o SYSTEM

o No MapRom

MapRom

-> *current* kickstart ROM is mapped

o No MacPatch

MacPatch

-> the mapped kickstart`s chipram starts at 0x4000

to avoid shapeshifter/fusion hack tools.

Only tested with 40.68

o No SCSIPatch

SCSIPatch

-> the mapped kickstart`s scsi.device is disabled.

This means for the A4000 that IDE is off

and for A3000 that the on board SCSI is off.

o No NCR SCSIPatch

NCR SCSIPatch

-> the mapped kickstart`s NCR scsi.device is disabled.

This only works on the A4000T and means that

the on board SCSI is off.

o No 31Khz Screen

31Khz Default Screen

->Uses the 31khz 640*400 BootMonitor now also for the

AmigaOS BootMenu and for the initial shell.

When IPrefs is started the 31Khz Bootmonitor patch

get inactive.

*ATTENTION*

I set the hardware into best mode, which means

that it boots into AGA mode, if a Boot31Khz Screen

is opened. Normally BestMode is set with setpatch,

so setting it before may have consequences for old

games.

o SuperScalar No

SuperScalar Yes

->Enables superscalar mode to speed up booting.

This mode is activated with the 68060.library later

anyway.

o BranchCache No

BranchCache Yes

->Enables the branchcache to speed up booting.

This mode is activated with the 68060.library later

anyway.

o WriteBuffer No

WriteBuffer Yes

->Enables the writebuffer to speed up booting.

This mode is activated with the 68060.library later

anyway.

o SAVE | USE | CANCEL

- SAVE saves the settings into the flashrom which

means that these settings are used for every coldboot

or warmboot if not changed or *NEW FLASH UPDATE* is

done. With the help of the 2nd joystickport`s button

you can completely disable scsi device and settings

if you have for some reason a problem to get into the

new bootmenu after playing around with special settings.

So you SHOULD HAVE the PPCUpdate/MK3Update on a disk so

you can quickly reinstall it without a hd which is

connected to the CyberStorm.

- USE uses the current changed settings for the current boot.

The settings aren`t saved and are lost in the next reset.

- CANCEL doesn`t use the settings.

How to solve Problems ?

o If you have weird crashes you probably have a power problem.

Even if you have a bigger power supply the problem can be

occur because the small power connection to the A1200 board

by certain towers can`t transport the needed power so the

ppc board gets unstable. To work around this problem

you should create a 2nd connection from the A1200 to the

power supply. Several people used the floppy connectors.

o If you have problems with 60ns mode your simms need significant

more power in 60ns mode which is a problem.

Read the first section how to fix about power problems.

Besides this...60ns printed on a Simm don`t necessary mean

that it`s a real 60ns Simm.

o If you`re using at least 1 Ultra unit on the bus be *very*

careful with the length of the cable.

o If you use a 68/50 adapter and use more than one unit on

this new chain or let the wide chain end here with one unit

this adapter MUST BE active terminated on the wide lines.

o At least one unit must provide TERM power.

o The controller is NOT terminated

o If you have problems and can`t boot with a certain setup

try to disable units in the Bootmenu until you`re able to

boot. Then use Unitcontrol Rescan and you may get device

error requesters.

If you get parity errors for example you have a serious

cable/term problem.

The device has absolutely nothing to do with that.

o IBM and (Seagate ?..not mine) drives have problems with

the device when the SDTR Jumpers are set.

I can`t see what is exactly happening then because i

don`t have such drive.

**************************************************************************

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

--------------------------------------------------------------------------

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

**************************************************************************

o You use AmiCDFS or other CDFilesystems and the new BlizzardPPC or

CyberstormPPC FlashUpdate is installed.

It does NOT matter if you have a SCSI or not...it also effects IDE.

Change the 0x43444653 (CDFS) dostype in the (AmiCDFS) mountlist.

Add a 1 at the last number for example.

OR

add the line FORCELOAD = 1

otherwise it`s using the Phase5 CDFilesystem which may

not do what you want it to do.

The CDRive filesystem is always added to the filesystem.resource

with the 0x43444653 ID

**************************************************************************

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

--------------------------------------------------------------------------

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

**************************************************************************

o SYSIHack Problem...SYSIHack says that iprefs was already

started which it wasn`t. (Caused by opening gadtools.library

in the rom)

Enter the BootMenu GUI, then leave it...SysIHack won`t

start because it checks if the gadtools.library is already

used. The gadtools.library can`t be expunged so there`s

no cure for this problem.

o Only use the real Setpatch 40.16. The last CBM had released.

o Don`t use skick as people reported problems with it.

Use cybermap instead.