Chapter 2

# MIB Definition

RFC1213-MIB { iso org(3) dod(6) internet(1) mgmt(2) 1 }

DEFINITIONS ::= BEGIN

IMPORTS

mgmt, NetworkAddress, IpAddress, Counter, Gauge,TimeTicks

FROM RFC1155-SMI

OBJECT-TYPE

FROM RFC-1212;

This MIB module uses the extended OBJECT-TYPE macro as defined in [14];

MIB-II (same prefix as MIB-I)

mib-2			OBJECT IDENTIFIER ::=	{ mgmt 1 }

textual conventions

DisplayString ::=

OCTET STRING

This data type is used to model textual information taken from the NVT ASCII character  set.  By convention, objects with this syntax are declared as having

SIZE (0..255)

PhysAddress ::=

OCTET STRING

This data type is used to model media addresses.  For many types of media, this will be  in a binary representation. For example, an ethernet address would be represented as a  string of 6 octets.

groups in MIB-II

system		OBJECT IDENTIFIER ::=	{ mib-2 1 }

interfaces	OBJECT IDENTIFIER ::=	{ mib-2 2 }

at				OBJECT IDENTIFIER ::=	{ mib-2 3 }

ip				OBJECT IDENTIFIER ::=	{ mib-2 4 }

icmp			OBJECT IDENTIFIER ::=	{ mib-2 5 }

tcp			OBJECT IDENTIFIER ::=	{ mib-2 6 }

udp			OBJECT IDENTIFIER ::=	{ mib-2 7 }

egp			OBJECT IDENTIFIER ::=	{ mib-2 8 }

—historical (some say hysterical)

—cmot		OBJECT IDENTIFIER ::=	{ mib-2 9 }

transmission	OBJECT IDENTIFIER ::=	{ mib-2 10 }

snmp			OBJECT IDENTIFIER ::=	{ mib-2 11 }

private		OBJECT IDENTIFIER ::=	{ internet 4 }

enterprises	OBJECT IDENTIFIER ::=	{ private 1 }

system		OBJECT IDENTIFIER ::=	{ mib 1 }

interfaces	OBJECT IDENTIFIER ::=	{ mib 2 }

at				OBJECT IDENTIFIER ::=	{ mib 3 }

ip				OBJECT IDENTIFIER ::=	{ mib 4 }

icmp			OBJECT IDENTIFIER ::=	{ mib 5 }

tcp			OBJECT IDENTIFIER ::=	{ mib 6 }

udp			OBJECT IDENTIFIER ::=	{ mib 7 }

egp			OBJECT IDENTIFIER ::=	{ mib 8 }

synoptics		OBJECT IDENTIFIER ::=	{ enterprises 45 }

products		OBJECT IDENTIFIER ::=	{synoptics 1 }

registration	OBJECT IDENTIFIER ::=	{synoptics 3 }

temporary	OBJECT IDENTIFIER ::=	{synoptics 2 }

series1000	OBJECT IDENTIFIER ::=	{products 1}

s3snmpAgent	OBJECT IDENTIFIER ::=	{products 2}

series3000	OBJECT IDENTIFIER ::=	{products 3}

ieee8023		OBJECT IDENTIFIER ::=	{temporary 1}

s3000Chassis	OBJECT IDENTIFIER ::=	{series3000 1}

s3000Ethernet	OBJECT IDENTIFIER ::=	{series3000 2}

s3000TokenRing	OBJECT IDENTIFIER ::=	{series3000 3}

s3000Bridge	OBJECT IDENTIFIER ::=	{series3000 5}

s3agentSW	OBJECT IDENTIFIER ::=	{s3snmpAgent 1}

s3agentNetProtocol	OBJECT IDENTIFIER ::=	{s3snmpAgent 2}

trapRcvrTable 	OBJECT IDENTIFIER ::=	{s3snmpAgent 3}

s3agentHW	OBJECT IDENTIFIER ::=	{s3snmpAgent 4}

s3agentSpecific	OBJECT IDENTIFIER ::=	{s3snmpAgent 5}

s3agentIpProtocol	OBJECT IDENTIFIER ::=	{s3agentNetProtocol 1}

s3recother	OBJECT IDENTIFIER ::=	{registration 1}

s3rec3000	OBJECT IDENTIFIER ::=	{registration 2}

s3rec3030	OBJECT IDENTIFIER ::=	{registration 3}

s3rec3138	OBJECT IDENTIFIER ::=	{registration 4}

s3rec332X	OBJECT IDENTIFIER ::=	{registration 5}

s3rec332XS	OBJECT IDENTIFIER ::=	{registration 6}

s3rec3356	OBJECT IDENTIFIER ::=	{registration 7}

—Common Bridge Objects

brdgcommon	OBJECT IDENTIFIER ::=	{s3000Bridge 1 }

—Remote Bridge Objects

oemid		OBJECT IDENTIFIER ::=	{s3000Bridge 3 }

retix			OBJECT IDENTIFIER ::=	{enterprises 72 }

product		OBJECT IDENTIFIER ::=	{retix 8 }

prembridge	OBJECT IDENTIFIER ::=	{product 99 }

station		OBJECT IDENTIFIER ::=	{ oemid 1 } — Station Group

lapb			OBJECT IDENTIFIER ::=	{ oemid 2 } — LAPB Group

ieee8023		OBJECT IDENTIFIER ::=	{ oemid 3 } — IEEE 802.3 Group

phySerIf		OBJECT IDENTIFIER ::=	{ oemid 4 } — Physical Serial Group

mlink			OBJECT IDENTIFIER ::=	{ oemid 5 } — Multilink Group

lan			OBJECT IDENTIFIER ::=	{ oemid 6 } — LAN Group

bridge		OBJECT IDENTIFIER ::=	{ oemid 7 } — Bridge Group

product		OBJECT IDENTIFIER ::=	{ oemid 8 } — Product Group

br12			OBJECT IDENTIFIER ::=	{ oemid 9 } — Br12 Bridge Group

router		OBJECT IDENTIFIER ::=	{ oemid 10 } — Router Group

boot			OBJECT IDENTIFIER ::=	{ oemid 11 } — Bootp Group

boothelper	OBJECT IDENTIFIER ::=	{ oemid 12 } — Boothelper Group

remote		OBJECT IDENTIFIER ::=	{ oemid 13 } — Remote Group

ipx			OBJECT IDENTIFIER ::=	{ remote 1 } — IPX group

decnet		OBJECT IDENTIFIER ::=	{ remote 2 } — Decnet group

rmtLapb		OBJECT IDENTIFIER ::=	{ remote 3 } — Remote Lapb group

x25			OBJECT IDENTIFIER ::=	{ remote 4 } — x25 group

— object types

— the Product group

p4942	OBJECT IDENTIFIER ::=	{ product 1 }

p2244m2	OBJECT IDENTIFIER ::=	{ product 2 }

p2265	OBJECT IDENTIFIER ::=	{ product 3 }

p4660	OBJECT IDENTIFIER ::=	{ product 4 }

p4820	OBJECT IDENTIFIER ::=	{ product 5 }

p4880	OBJECT IDENTIFIER ::=	{ product 6 }

p4850	OBJECT IDENTIFIER ::=	{ product 7 }

p4760	OBJECT IDENTIFIER ::=	{ product 8 }

p4941	OBJECT IDENTIFIER ::=	{ product 9 }

## System grouptc "System group"§

Implementation of the System group is mandatory for all systems.  If an agent is not configured to have a value for any of these variables, a string of length 0 is returned.

sysDescr OBJECT-TYPE

SYNTAX	DisplayString (SIZE (0..255))

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“A textual description of the entity.  This value should include the full name and version identification of the system’s hardware type, software operating-system, and networking software.  It is mandatory that this only contain printable ASCII characters.”

::= { system 1 }

sysObjectID OBJECT-TYPE

SYNTAX	OBJECT IDENTIFIER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The vendor’s authoritative identification of the network management subsystem contained in the entity.  This value is allocated within the SMI enterprises subtree (1.3.6.1.4.1) and provides an easy and unambiguous means for determining ‘what kind of box’ is being managed.  For example, if vendor ‘Flintstones, Inc.’ was assigned the subtree 1.3.6.1.4.1.4242, it could assign the identifier 1.3.6.1.4.1.4242.1.1 to its ‘Fred Router’.”

::= { system 2 }

sysUpTime OBJECT-TYPE

SYNTAX	TimeTicks

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The time (in hundredths of a second) since the network management portion of the system was last re-initialized.”

::= { system 3 }

sysContact OBJECT-TYPE

SYNTAX	DisplayString (SIZE (0..255))

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“The textual identification of the contact person for this managed node, together with information on how to contact this person.”

::= { system 4 }

sysName OBJECT-TYPE

SYNTAX	DisplayString (SIZE (0..255))

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“An administratively-assigned name for this managed node.  By convention, this is the node’s fully-qualified domain name.”

::= { system 5 }

sysLocation OBJECT-TYPE

SYNTAX	DisplayString (SIZE (0..255))

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“The physical location of this node (e.g., `telephone closet, 3rd floor’).”

::= { system 6 }

sysServices OBJECT-TYPE

SYNTAX	INTEGER (0..127)

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“A value which indicates the set of services that this entity primarily offers. The value is a sum. This sum initially takes the value zero, Then, for each layer, L, in the range 1 through 7, that this node performs transactions for, 2 raised to (L - 1) is added to the sum. For example, a node which performs primarily routing functions would have a value of 4 (2^(3-1)). In contrast, a node which is a host offering application services would have a value of 72 (2^(4-1) + 2^(7-1)). Note that in the context of the Internet suite of protocols, values should be calculated accordingly:

layer functionality

1	physical (e.g., repeaters)

2	datalink/subnetwork (e.g., bridges)

3	internet (e.g., IP gateways)

4	end-to-end (e.g., IP hosts)

7	applications (e.g., mail relays)

For systems including OSI protocols, layers 5 and 6 may also be counted.”

::= { system 7 }

## Interfaces grouptc "Interfaces group"§

Implementation of the Interfaces group is mandatory for all systems.

ifNumber OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of network interfaces (regardless of their current state) present on this system.”

::= { interfaces 1 }

### the Interfaces tabletc "the Interfaces table"§

The Interfaces table contains information on the entity’s interfaces. Each interface is thought of as being attached to a ‘subnetwork’. Note that this term should not be confused with ‘subnet’ which refers to an addressing partitioning scheme used in the Internet suite of protocols.

ifTable OBJECT-TYPE

SYNTAX	SEQUENCE OF IfEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
“A list of interface entries. The number of entries is given by the value of ifNumber.”

::= { interfaces 2 }

ifEntry OBJECT-TYPE

SYNTAX	IfEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
“An interface entry containing objects at the subnetwork layer and below for a particular interface.”

INDEX		{ ifIndex }

::= { ifTable 1 }

IfEntry ::=

SEQUENCE {

ifIndex

INTEGER,

ifDescr

DisplayString,

ifType

INTEGER,

ifMtu

INTEGER,

ifSpeed

Gauge,

ifPhysAddress

PhysAddress,

ifAdminStatus

INTEGER,

ifOperStatus

INTEGER,

ifLastChange

TimeTicks,

ifInOctets

Counter,

ifInUcastPkts

Counter,

ifInNUcastPkts

Counter,

ifInDiscards

Counter,

ifInErrors

Counter,

ifInUnknownProtos

Counter,

ifOutOctets

Counter,

ifOutUcastPkts

Counter,

ifOutNUcastPkts

Counter,

ifOutDiscards

Counter,

ifOutErrors

Counter,

ifOutQLen

Gauge,

ifSpecific

OBJECT IDENTIFIER

}

ifIndex OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“A unique value for each interface. Its value ranges between 1 and the value of ifNumber. The value for each interface must remain constant at least from one re-initialization of the entity’s network management system to the next re- initialization.”

::= { ifEntry 1 }

ifDescr OBJECT-TYPE

SYNTAX	DisplayString (SIZE (0..255))

ACCESS	read-only

STATUS	mandatory

DESCRIPTION 
“A textual string containing information about the interface.  This string should include the name of the manufacturer, the product name and the version of the hardware interface.”

::= { ifEntry 2 }

ifType OBJECT-TYPE

SYNTAX	INTEGER {

other(1),	none of the following

regular1822(2),

hdh1822(3),

ddn-x25(4),

rfc877-x25(5),

ethernet-csmacd(6),

iso88023-csmacd(7),

iso88024-tokenBus(8),

iso88025-tokenRing(9),

iso88026-man(10),

starLan(11),

proteon-10Mbit(12),

proteon-80Mbit(13),

hyperchannel(14),

fddi(15),

lapb(16),

sdlc(17),

ds1(18),	T-1

e1(19),	european equiv. of T-1

basicISDN(20),

primaryISDN(21),	proprietary serial

propPointToPointSerial(22),

ppp(23),

softwareLoopback(24),

eon(25),	CLNP over IP [11]

ethernet-3Mbit(26),

nsip(27),	XNS over IP

slip(28),	generic SLIP

ultra(29),	ULTRA technologies

ds3(30),	T-3

sip(31),	SMDS

frame-relay(32)

}

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The type of interface, distinguished according to the physical/link protocol(s) immediately `below’ the network layer in the protocol stack.”

::= { ifEntry 3 }

ifMtu OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The size of the largest datagram which can be sent/received on the interface, specified in octets.  For interfaces that are used for transmitting network datagrams, this is the size of the largest network datagram that can be sent on the interface.”

::= { ifEntry 4 }

ifSpeed OBJECT-TYPE

SYNTAX	Gauge

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“An estimate of the interface’s current bandwidth in bits per second.  For interfaces which do not vary in bandwidth or for those where no accurate estimation can be made, this object should contain the nominal bandwidth.”

::= { ifEntry 5 }

ifPhysAddress OBJECT-TYPE

SYNTAX	PhysAddress

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The interface’s address at the protocol layer immediately `below’ the network layer in the protocol stack.  For interfaces which do not have such an address (e.g., a serial line), this object should contain an octet string of zero length.”

::= { ifEntry 6 }

ifAdminStatus OBJECT-TYPE

SYNTAX	INTEGER {

up(1),	ready to pass packets

down(2),

testing(3)	in some test mode

}

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The desired state of the interface.  The testing(3) state indicates that no operational packets can be passed.”

::= { ifEntry 7 }

ifOperStatus OBJECT-TYPE

SYNTAX	INTEGER {

up(1),	ready to pass packets

down(2),

testing(3)	in some test mode

}

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The current operational state of the interface. The testing(3) state indicates that no operational packets can be passed.”

::= { ifEntry 8 }

ifLastChange OBJECT-TYPE

SYNTAX	TimeTicks

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The value of sysUpTime at the time the interface entered its current operational state.  If the current state was entered prior to the last re- initialization of the local network management subsystem, then this object contains a zero value.”

::= { ifEntry 9 }

ifInOctets OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of octets received on the interface, including framing characters.”

::= { ifEntry 10 }

ifInUcastPkts OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of subnetwork-unicast packets delivered to a higher-layer protocol.”

::= { ifEntry 11 }

ifInNUcastPkts OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of non-unicast (i.e., subnetwork- broadcast or subnetwork-multicast) packets delivered to a higher-layer protocol.”

::= { ifEntry 12 }

ifInDiscards OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of inbound packets which were chosen to be discarded even though no errors had been detected to prevent their being deliverable to a higher-layer protocol.  One possible reason for discarding such a packet could be to free up buffer space.”

::= { ifEntry 13 }

ifInErrors OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of inbound packets that contained errors preventing them from being deliverable to a higher-layer protocol.”

::= { ifEntry 14 }

ifInUnknownProtos OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of packets received via the interface which were discarded because of an unknown or unsupported protocol.”

::= { ifEntry 15 }

ifOutOctets OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of octets transmitted out of the interface, including framing characters.”

::= { ifEntry 16 }

ifOutUcastPkts OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of packets that higher-level protocols requested be transmitted to a subnetwork-unicast address, including those that were discarded or not sent.”

::= { ifEntry 17 }

ifOutNUcastPkts OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of packets that higher-level protocols requested be transmitted to a non- unicast (i.e., a subnetwork-broadcast or subnetwork-multicast) address, including those that were discarded or not sent.”

::= { ifEntry 18 }

ifOutDiscards OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of outbound packets which were chosen to be discarded even though no errors had been detected to prevent their being transmitted.  One possible reason for discarding such a packet could be to free up buffer space.”

::= { ifEntry 19 }

ifOutErrors OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of outbound packets that could not be transmitted because of errors.”

::= { ifEntry 20 }

ifOutQLen OBJECT-TYPE

SYNTAX	Gauge

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The length of the output packet queue (in packets).”

::= { ifEntry 21 }

ifSpecific OBJECT-TYPE

SYNTAX	OBJECT IDENTIFIER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“A reference to MIB definitions specific to the particular media being used to realize the interface.  For example, if the interface is realized by an ethernet, then the value of this object refers to a document defining objects specific to ethernet.  If this information is not present, its value should be set to the OBJECT IDENTIFIER { 0 0 }, which is a syntatically valid object identifier, and any conformant implementation of ASN.1 and BER must be able to generate and recognize this value.”

::= { ifEntry 22 }

## Address Translation grouptc "Address Translation group"§

Implementation of the Address Translation group is mandatory for all systems.  Note however that this group is deprecated by MIB-II. That is, it is being included solely for compatibility with MIB-I nodes, and will most likely be excluded from MIB-III nodes.  From MIB-II and onwards, each network protocol group contains its own address translation tables.

The Address Translation group contains one table which is the union across all interfaces of the translation tables for converting a NetworkAddress (e.g., an IP address) into a subnetwork-specific address.  For lack of a better term, this document refers to such a subnetwork-specific address as a ‘physical’ address.

Examples of such translation tables are: for broadcast media where ARP is in use, the translation table is equivalent to the ARP cache; or, on an X.25 network where non-algorithmic translation to X.121 addresses is required, the translation table contains the NetworkAddress to X.121 address equivalences.

atTable OBJECT-TYPE

SYNTAX	SEQUENCE OF AtEntry

ACCESS	not-accessible

STATUS	deprecated

DESCRIPTION
“The Address Translation tables contain the NetworkAddress to `physical’ address equivalences. Some interfaces do not use translation tables for determining address equivalences (e.g., DDN-X.25 has an algorithmic method); if all interfaces are of this type, then the Address Translation table is empty, i.e., has zero entries.”

::= { at 1 }

atEntry OBJECT-TYPE

SYNTAX	AtEntry

ACCESS	not-accessible

STATUS	deprecated

DESCRIPTION
“Each entry contains one NetworkAddress to `physical’ address equivalence.”

INDEX		{ atIfIndex,

::= { atTable 1 }

AtEntry ::=

SEQUENCE {

atIfIndex

INTEGER,

atPhysAddress

PhysAddress,

atNetAddress

NetworkAddress

}

atIfIndex OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	deprecated

DESCRIPTION
“The interface on which this entry’s equivalence is effective.  The interface identified by a particular value of this index is the same interface as identified by the same value of ifIndex.”

::= { atEntry 1 }

atPhysAddress OBJECT-TYPE

SYNTAX	PhysAddress

ACCESS	read-write

STATUS	deprecated

DESCRIPTION
“The media-dependent `physical’ address. Setting this object to a null string (one of zero length) has the effect of invaliding the corresponding entry in the atTable object.  That is, it effectively dissasociates the interface identified with said entry from the mapping identified with said entry.  It is an implementation-specific matter as to whether the agent removes an invalidated entry from the table. Accordingly, management stations must be prepared to receive tabular information from agents that corresponds to entries not currently in use. Proper interpretation of such entries requires examination of the relevant atPhysAddress object.”

::= { atEntry 2 }

atNetAddress OBJECT-TYPE

SYNTAX	NetworkAddress

ACCESS	read-write

STATUS	deprecated

DESCRIPTION
“The NetworkAddress (e.g., the IP address) corresponding to the media-dependent `physical’ address.”

::= { atEntry 3 }

## IP grouptc "

## IP group"§

Implementation of the IP group is mandatory for all systems.

ipForwarding OBJECT-TYPE

SYNTAX	INTEGER {

forwarding(1),	acting as a gateway

not-forwarding(2)	NOT acting as a gateway }

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“The indication of whether this entity is acting as an IP gateway in respect to the forwarding of datagrams received by, but not addressed to, this entity.  IP gateways forward datagrams.  IP hosts do not (except those source-routed via the host). Note that for some managed nodes, this object may take on only a subset of the values possible. Accordingly, it is appropriate for an agent to return a `badValue’ response if a management station attempts to change this object to an inappropriate value.”

::= { ip 1 }

ipDefaultTTL OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“The default value inserted into the Time-To-Live field of the IP header of datagrams originated at this entity, whenever a TTL value is not supplied by the transport layer protocol.”

::= { ip 2 }

ipInReceives OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of input datagrams received from interfaces, including those received in error.”

::= { ip 3 }

ipInHdrErrors OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of input datagrams discarded due to errors in their IP headers, including bad checksums, version number mismatch, other format errors, time-to-live exceeded, errors discovered in processing their IP options, etc.”

::= { ip 4 }

ipInAddrErrors OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of input datagrams discarded because the IP address in their IP header’s destination field was not a valid address to be received at this entity.  This count includes invalid addresses (e.g., 0.0.0.0) and addresses of unsupported Classes (e.g., Class E).  For entities which are not IP Gateways and therefore do not forward datagrams, this counter includes datagrams discarded because the destination address was not a local address.”

::= { ip 5 }

ipForwDatagrams OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of input datagrams for which this entity was not their final IP destination, as a result of which an attempt was made to find a route to forward them to that final destination. In entities which do not act as IP Gateways, this counter will include only those packets which were Source-Routed via this entity, and the Source- Route option processing was successful.”

::= { ip 6 }

ipInUnknownProtos OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of locally-addressed datagrams received successfully but discarded because of an unknown or unsupported protocol.”

::= { ip 7 }

ipInDiscards OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of input IP datagrams for which no problems were encountered to prevent their continued processing, but which were discarded (e.g., for lack of buffer space).  Note that this counter does not include any datagrams discarded while awaiting re-assembly.”

::= { ip 8 }

ipInDelivers OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of input datagrams successfully delivered to IP user-protocols (including ICMP).”

::= { ip 9 }

ipOutRequests OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of IP datagrams which local IP user-protocols (including ICMP) supplied to IP in requests for transmission.  Note that this counter does not include any datagrams counted in ipForwDatagrams.”

::= { ip 10 }

ipOutDiscards OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of output IP datagrams for which no problem was encountered to prevent their transmission to their destination, but which were discarded (e.g., for lack of buffer space).  Note that this counter would include datagrams counted in ipForwDatagrams if any such packets met this (discretionary) discard criterion.”

::= { ip 11 }

ipOutNoRoutes OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of IP datagrams discarded because no route could be found to transmit them to their destination.  Note that this counter includes any packets counted in ipForwDatagrams which meet this `no-route’ criterion.  Note that this includes any datagarms which a host cannot route because all of its default gateways are down.”

::= { ip 12 }

ipReasmTimeout OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The maximum number of seconds which received fragments are held while they are awaiting reassembly at this entity.”

::= { ip 13 }

ipReasmReqds OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of IP fragments received which needed to be reassembled at this entity.”

::= { ip 14 }

ipReasmOKs OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of IP datagrams successfully re- assembled.”

::= { ip 15 }

ipReasmFails OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of failures detected by the IP re- assembly algorithm (for whatever reason: timed out, errors, etc).  Note that this is not necessarily a count of discarded IP fragments since some algorithms (notably the algorithm in RFC 815) can lose track of the number of fragments by combining them as they are received.”

::= { ip 16 }

ipFragOKs OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of IP datagrams that have been successfully fragmented at this entity.”

::= { ip 17 }

ipFragFails OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of IP datagrams that have been discarded because they needed to be fragmented at this entity but could not be, e.g., because their Don’t Fragment flag was set.”

::= { ip 18 }

ipFragCreates OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of IP datagram fragments that have been generated as a result of fragmentation at this entity.”

::= { ip 19 }

### the IP address tabletc "

### the IP address table"§

The IP address table contains this entity’s IP addressing information.

ipAddrTable OBJECT-TYPE

SYNTAX	SEQUENCE OF IpAddrEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
“The table of addressing information relevant to this entity’s IP addresses.”

::= { ip 20 }

ipAddrEntry OBJECT-TYPE

SYNTAX	IpAddrEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
“The addressing information for one of this entity’s IP addresses.”

INDEX		{ ipAdEntAddr }

::= { ipAddrTable 1 }

IpAddrEntry ::=

SEQUENCE {

ipAdEntAddr

IpAddress,

ipAdEntIfIndex

INTEGER,

ipAdEntNetMask

IpAddress,

ipAdEntBcastAddr

INTEGER,

ipAdEntReasmMaxSize

INTEGER (0..65535)

}

ipAdEntAddr OBJECT-TYPE

SYNTAX	IpAddress

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The IP address to which this entry’s addressing information pertains.”

::= { ipAddrEntry 1 }

ipAdEntIfIndex OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The index value which uniquely identifies the interface to which this entry is applicable.  The interface identified by a particular value of this index is the same interface as identified by the same value of ifIndex.”

::= { ipAddrEntry 2 }

ipAdEntNetMask OBJECT-TYPE

SYNTAX	IpAddress

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The subnet mask associated with the IP address of this entry.  The value of the mask is an IP address with all the network bits set to 1 and all the hosts bits set to 0.”

::= { ipAddrEntry 3 }

ipAdEntBcastAddr OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The value of the least-significant bit in the IP broadcast address used for sending datagrams on the (logical) interface associated with the IP address of this entry.  For example, when the Internet standard all-ones broadcast address is used, the value will be 1.  This value applies to both the subnet and network broadcasts addresses used by the entity on this (logical) interface.”

::= { ipAddrEntry 4 }

ipAdEntReasmMaxSize OBJECT-TYPE

SYNTAX	INTEGER (0..65535)

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The size of the largest IP datagram which this entity can re-assemble from incoming IP fragmented datagrams received on this interface.”

::= { ipAddrEntry 5 }

### c.the IP routing table

The IP routing table contains an entry for each route presently known to this entity.

ipRouteTable OBJECT-TYPE

SYNTAX	SEQUENCE OF IpRouteEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
“This entity’s IP Routing table.”

::= { ip 21 }

ipRouteEntry OBJECT-TYPE

SYNTAX	IpRouteEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
“A route to a particular destination.”

INDEX		{ ipRouteDest }

::= { ipRouteTable 1 }

IpRouteEntry ::=

SEQUENCE {

ipRouteDest

IpAddress,

ipRouteIfIndex

INTEGER,

ipRouteMetric1

INTEGER,

ipRouteMetric2

INTEGER,

ipRouteMetric3

INTEGER,

ipRouteMetric4

INTEGER,

ipRouteNextHop

IpAddress,

ipRouteType

INTEGER,

ipRouteProto

INTEGER,

ipRouteAge

INTEGER,

ipRouteMask

IpAddress,

ipRouteMetric5

INTEGER,

ipRouteInfo

OBJECT IDENTIFIER

}

ipRouteDest OBJECT-TYPE

SYNTAX	IpAddress

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“The destination IP address of this route.  An entry with a value of 0.0.0.0 is considered a default route.  Multiple routes to a single destination can appear in the table, but access to such multiple entries is dependent on the table- access mechanisms defined by the network management protocol in use.”

::= { ipRouteEntry 1 }

ipRouteIfIndex OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“The index value which uniquely identifies the local interface through which the next hop of this route should be reached.  The interface identified by a particular value of this index is the same interface as identified by the same value of ifIndex.”

::= { ipRouteEntry 2 }

ipRouteMetric1 OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“The primary routing metric for this route.  The semantics of this metric are determined by the routing-protocol specified in the route’s ipRouteProto value.  
If this metric is not used, its value should be set to -1.”

::= { ipRouteEntry 3 }

ipRouteMetric2 OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“An alternate routing metric for this route.  The semantics of this metric are determined by the routing-protocol specified in the route’s ipRouteProto value.  
If this metric is not used, its value should be set to -1.”

::= { ipRouteEntry 4 }

ipRouteMetric3 OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“An alternate routing metric for this route.  The semantics of this metric are determined by the routing-protocol specified in the route’s ipRouteProto value.  
If this metric is not used, its value should be set to -1.”

::= { ipRouteEntry 5 }

ipRouteMetric4 OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“An alternate routing metric for this route.  The semantics of this metric are determined by the routing-protocol specified in the route’s ipRouteProto value.  
If this metric is not used, its value should be set to -1.”

::= { ipRouteEntry 6 }

ipRouteNextHop OBJECT-TYPE

SYNTAX	IpAddress

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“The IP address of the next hop of this route. (In the case of a route bound to an interface which is realized via a broadcast media, the value of this field is the agent’s IP address on that interface.)”

::= { ipRouteEntry 7 }

ipRouteType OBJECT-TYPE

SYNTAX	INTEGER

{other(1),	none of the following

invalid(2),	an invalidated route

route to directly

direct(3),	connected (sub-)network

route to a non-local

indirect(4)	host/network/sub-network

}

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“The type of route.  Note that the values direct(3) and indirect(4) refer to the notion of direct and indirect routing in the IP architecture. Setting this object to the value invalid(2) has the effect of invalidating the corresponding entry in the ipRouteTable object.  That is, it effectively dissasociates the destination identified with said entry from the route identified with said entry.  It is an implementation-specific matter as to whether the agent removes an invalidated entry from the table. Accordingly, management stations must be prepared to receive tabular information from agents that corresponds to entries not currently in use. Proper interpretation of such entries requires examination of the relevant ipRouteType object.”

::= { ipRouteEntry 8 }

ipRouteProto OBJECT-TYPE

SYNTAX	INTEGER {

other(1),	none of the following non-protocol information, e.g., manually

local(2),	configured entries set via a network management protocol obtained

netmgmt(3),	via ICMP, e.g., Redirect the remaining values are all gateway routing

icmp(4),	protocols

egp(5),

ggp(6),

hello(7),

rip(8),

is-is(9),

es-is(10),

ciscoIgrp(11),

bbnSpfIgp(12),

ospf(13),

bgp(14)

}

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The routing mechanism via which this route was learned.  Inclusion of values for gateway routing protocols is not intended to imply that hosts should support those protocols.”

::= { ipRouteEntry 9 }

ipRouteAge OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“The number of seconds since this route was last updated or otherwise determined to be correct. Note that no semantics of `too old’ can be implied except through knowledge of the routing protocol by which the route was learned.”

::= { ipRouteEntry 10 }

ipRouteMask OBJECT-TYPE

SYNTAX	IpAddress

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“Indicate the mask to be logical-ANDed with the destination address before being compared to the value in the ipRouteDest field.  For those systems that do not support arbitrary subnet masks, an agent constructs the value of the ipRouteMask by determining whether the value of the correspondent ipRouteDest field belong to a class-A, B, or C network, and then using one of:

mask	network

255.0.0.0	class-A

255.255.0.0	class-B

255.255.255.0	class-C

If the value of the ipRouteDest is 0.0.0.0 (a default route), then the mask value is also 0.0.0.0.  It should be noted that all IP routing subsystems implicitly use this mechanism.”

::= { ipRouteEntry 11 }

ipRouteMetric5 OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“An alternate routing metric for this route.  The semantics of this metric are determined by the routing-protocol specified in the route’s ipRouteProto value.  If this metric is not used, its value should be set to -1.”

::= { ipRouteEntry 12 }

ipRouteInfo OBJECT-TYPE

SYNTAX	OBJECT IDENTIFIER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“A reference to MIB definitions specific to the particular routing protocol which is responsible for this route, as determined by the value specified in the route’s ipRouteProto value.  If this information is not present, its value should be set to the OBJECT IDENTIFIER { 0 0 }, which is a syntatically valid object identifier, and any conformant implementation of ASN.1 and BER must be able to generate and recognize this value.”

::= { ipRouteEntry 13 }

### the IP Address Translation tabletc "the IP Address Translation table"§

The IP address translation table contain the IpAddress to ‘physical’ address equivalences.  Some interfaces do not use translation tables for determining address equivalences (e.g., DDN-X.25 has an algorithmic method); if all interfaces are of this type, then the Address Translation table is empty, i.e., has zero entries.

ipNetToMediaTable OBJECT-TYPE

SYNTAX	SEQUENCE OF IpNetToMediaEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
“The IP Address Translation table used for mapping from IP addresses to physical addresses.”

::= { ip 22 }

ipNetToMediaEntry OBJECT-TYPE

SYNTAX	IpNetToMediaEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
“Each entry contains one IpAddress to `physical’ address equivalence.”

INDEX		{ ipNetToMediaIfIndex,ipNetToMediaNetAddress }

::= { ipNetToMediaTable 1 }

IpNetToMediaEntry ::=

SEQUENCE {

ipNetToMediaIfIndex

INTEGER,

ipNetToMediaPhysAddress

PhysAddress,

ipNetToMediaNetAddress

IpAddress,

ipNetToMediaType

INTEGER }

ipNetToMediaIfIndex OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“The interface on which this entry’s equivalence is effective.  The interface identified by a particular value of this index is the same interface as identified by the same value of ifIndex.”

::= { ipNetToMediaEntry 1 }

ipNetToMediaPhysAddress OBJECT-TYPE

SYNTAX	PhysAddress

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“The media-dependent `physical’ address.”

::= { ipNetToMediaEntry 2 }

ipNetToMediaNetAddress OBJECT-TYPE

SYNTAX	IpAddress

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“The IpAddress corresponding to the media- dependent `physical’ address.”

::= { ipNetToMediaEntry 3 }

ipNetToMediaType OBJECT-TYPE

SYNTAX	INTEGER {

other(1),	none of the following

invalid(2),	an invalidated mapping

dynamic(3),

static(4) }

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“The type of mapping. Setting this object to the value invalid(2) has the effect of invalidating the corresponding entry in the ipNetToMediaTable.  That is, it effectively dissasociates the interface identified with said entry from the mapping identified with said entry. It is an implementation-specific matter as to whether the agent removes an invalidated entry from the table.  Accordingly, management stations must be prepared to receive tabular information from agents that corresponds to entries not currently in use.  Proper interpretation of such entries requires examination of the relevant ipNetToMediaType object.”

::= { ipNetToMediaEntry 4 }

additional IP objects

ipRoutingDiscards OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of routing entries which were chosen to be discarded even though they are valid.  One possible reason for discarding such an entry could be to free-up buffer space for other routing entries.”

::= { ip 23 }

## ICMP grouptc "ICMP group"§

Implementation of the ICMP group is mandatory for all systems.

icmpInMsgs OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of ICMP messages which the entity received.  Note that this counter includes all those counted by icmpInErrors.”

::= { icmp 1 }

icmpInErrors OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP messages which the entity received but determined as having ICMP-specific errors (bad ICMP checksums, bad length, etc.).”

::= { icmp 2 }

icmpInDestUnreachs OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP Destination Unreachable messages received.”

::= { icmp 3 }

icmpInTimeExcds OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP Time Exceeded messages received.”

::= { icmp 4 }

icmpInParmProbs OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP Parameter Problem messages received.”

::= { icmp 5 }

icmpInSrcQuenchs OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP Source Quench messages received.”

::= { icmp 6 }

icmpInRedirects OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP Redirect messages received.”

::= { icmp 7 }

icmpInEchos OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP Echo (request) messages received.”

::= { icmp 8 }

icmpInEchoReps OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP Echo Reply messages received.”

::= { icmp 9 }

icmpInTimestamps OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP Timestamp (request) messages received.”

::= { icmp 10 }

icmpInTimestampReps OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP Timestamp Reply messages received.”

::= { icmp 11 }

icmpInAddrMasks OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP Address Mask Request messages received.”

::= { icmp 12 }

icmpInAddrMaskReps OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP Address Mask Reply messages received.”

::= { icmp 13 }

icmpOutMsgs OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of ICMP messages which this entity attempted to send.  Note that this counter includes all those counted by icmpOutErrors.”

::= { icmp 14 }

icmpOutErrors OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP messages which this entity did not send due to problems discovered within ICMP such as a lack of buffers.  This value should not include errors discovered outside the ICMP layer such as the inability of IP to route the resultant datagram.  In some implementations there may be no types of error which contribute to this counter’s value.”

::= { icmp 15 }

icmpOutDestUnreachs OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP Destination Unreachable messages sent.”

::= { icmp 16 }

icmpOutTimeExcds OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP Time Exceeded messages sent.”

::= { icmp 17 }

icmpOutParmProbs OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP Parameter Problem messages sent.”

::= { icmp 18 }

icmpOutSrcQuenchs OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP Source Quench messages sent.”

::= { icmp 19 }

icmpOutRedirects OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP Redirect messages sent.  For a host, this object will always be zero, since hosts do not send redirects.”

::= { icmp 20 }

icmpOutEchos OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP Echo (request) messages sent.”

::= { icmp 21 }

icmpOutEchoReps OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP Echo Reply messages sent.”

::= { icmp 22 }

icmpOutTimestamps OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP Timestamp (request) messages sent.”

::= { icmp 23 }

icmpOutTimestampReps OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP Timestamp Reply messages sent.”

::= { icmp 24 }

icmpOutAddrMasks OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP Address Mask Request messages sent.”

::= { icmp 25 }

icmpOutAddrMaskReps OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of ICMP Address Mask Reply messages sent.”

::= { icmp 26 }

## UDP grouptc "UDP group"§

Implementation of the UDP group is mandatory for all systems which implement the UDP.

udpInDatagrams OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of UDP datagrams delivered to UDP users.”

::= { udp 1 }

udpNoPorts OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of received UDP datagrams for which there was no application at the destination port.”

::= { udp 2 }

udpInErrors OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The number of received UDP datagrams that could not be delivered for reasons other than the lack of an application at the destination port.”

::= { udp 3 }

udpOutDatagrams OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of UDP datagrams sent from this entity.”

::= { udp 4 }

### the UDP Listener tabletc "the UDP Listener table"§

The UDP listener table contains information about this entity’s UDP end-points on which a local application is currently accepting datagrams.

udpTable OBJECT-TYPE

SYNTAX	SEQUENCE OF UdpEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
“A table containing UDP listener information.”

::= { udp 5 }

udpEntry OBJECT-TYPE

SYNTAX	UdpEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
“Information about a particular current UDP listener.”

INDEX		{ udpLocalAddress, udpLocalPort }

::= { udpTable 1 }

UdpEntry ::=

SEQUENCE {

udpLocalAddress

IpAddress,

udpLocalPort

INTEGER (0..65535)

}

udpLocalAddress OBJECT-TYPE

SYNTAX	IpAddress

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The local IP address for this UDP listener.  In the case of a UDP listener which is willing to accept datagrams for any IP interface associated with the node, the value 0.0.0.0 is used.”

::= { udpEntry 1 }

udpLocalPort OBJECT-TYPE

SYNTAX	INTEGER (0..65535)

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The local port number for this UDP listener.”

::= { udpEntry 2 }

## Transmission grouptc "

## Transmission group"§

Based on the transmission media underlying each interface on a system, the corresponding portion of the Transmission group is mandatory for that system. When Internet-standard definitions for managing transmission media are defined, the transmission group is used to provide a prefix for the names of those objects. Typically, such definitions reside in the experimental portion of the MIB until they are “proven”, then as a part of the Internet standardization process, the definitions are accordingly elevated and a new object identifier, under the transmission group is defined. By convention, the name assigned is:

type OBJECT IDENTIFIER	::= { transmission number }

where “type” is the symbolic value used for the media in the ifType column of the ifTable object, and “number” is the actual integer value corresponding to the symbol.

## SNMP grouptc "SNMP group"§

Implementation of the SNMP group is mandatory for all systems which support an SNMP protocol entity.  Some of the objects defined below will be zero-valued in those SNMP implementations that are optimized to support only those functions specific to either a management agent or a management station.  In particular, it should be observed that the objects below refer to an SNMP entity, and there may be several SNMP entities residing on a managed node (e.g., if the node is hosting acting as a management station).

snmpInPkts OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of Messages delivered to the SNMP entity from the transport service.”

::= { snmp 1 }

snmpOutPkts OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of SNMP Messages which were passed from the SNMP protocol entity to the transport service.”

::= { snmp 2 }

snmpInBadVersions OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of SNMP Messages which were delivered to the SNMP protocol entity and were for an unsupported SNMP version.”

::= { snmp 3 }

snmpInBadCommunityNames OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of SNMP Messages delivered to the SNMP protocol entity which used a SNMP community name not known to said entity.”

::= { snmp 4 }

snmpInBadCommunityUses OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of SNMP Messages delivered to the SNMP protocol entity which represented an SNMP operation which was not allowed by the SNMP community named in the Message.”

::= { snmp 5 }

snmpInASNParseErrs OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of ASN.1 or BER errors encountered by the SNMP protocol entity when decoding received SNMP Messages.”

::= { snmp 6 }

{ snmp 7 } is not used

snmpInTooBigs OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of SNMP PDUs which were delivered to the SNMP protocol entity and for which the value of the error-status field is `tooBig’.”

::= { snmp 8 }

snmpInNoSuchNames OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of SNMP PDUs which were delivered to the SNMP protocol entity and for which the value of the error-status field is `noSuchName’.”

::= { snmp 9 }

snmpInBadValues OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of SNMP PDUs which were delivered to the SNMP protocol entity and for which the value of the error-status field is `badValue’.”

::= { snmp 10 }

snmpInReadOnlys OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number valid SNMP PDUs which were delivered to the SNMP protocol entity and for which the value of the error-status field is `readOnly’.  It should be noted that it is a protocol error to generate an SNMP PDU which contains the value `readOnly’ in the error-status field, as such this object is provided as a means of detecting incorrect implementations of the SNMP.”

::= { snmp 11 }

snmpInGenErrs OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of SNMP PDUs which were delivered to the SNMP protocol entity and for which the value of the error-status field is `genErr’.”

::= { snmp 12 }

snmpInTotalReqVars OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of MIB objects which have been retrieved successfully by the SNMP protocol entity as the result of receiving valid SNMP Get-Request and Get-Next PDUs.”

::= { snmp 13 }

snmpInTotalSetVars OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of MIB objects which have been altered successfully by the SNMP protocol entity as the result of receiving valid SNMP Set-Request PDUs.”

::= { snmp 14 }

snmpInGetRequests OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of SNMP Get-Request PDUs which have been accepted and processed by the SNMP protocol entity.”

::= { snmp 15 }

snmpInGetNexts OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of SNMP Get-Next PDUs which have been accepted and processed by the SNMP protocol entity.”

::= { snmp 16 }

snmpInSetRequests OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of SNMP Set-Request PDUs which have been accepted and processed by the SNMP protocol entity.”

::= { snmp 17 }

snmpInGetResponses OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of SNMP Get-Response PDUs which have been accepted and processed by the SNMP protocol entity.”

::= { snmp 18 }

snmpInTraps OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of SNMP Trap PDUs which have been accepted and processed by the SNMP protocol entity.”

::= { snmp 19 }

snmpOutTooBigs OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of SNMP PDUs which were generated by the SNMP protocol entity and for which the value of the error-status field is `tooBig.’”

::= { snmp 20 }

snmpOutNoSuchNames OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of SNMP PDUs which were generated by the SNMP protocol entity and for which the value of the error-status is `noSuchName’.”

::= { snmp 21 }

snmpOutBadValues OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of SNMP PDUs which were generated by the SNMP protocol entity and for which the value of the error-status field is `badValue’.”

::= { snmp 22 }

{ snmp 23 } is not used

snmpOutGenErrs OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of SNMP PDUs which were generated by the SNMP protocol entity and for which the value of the error-status field is `genErr’.”

::= { snmp 24 }

snmpOutGetRequests OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of SNMP Get-Request PDUs which have been generated by the SNMP protocol entity.”

::= { snmp 25 }

snmpOutGetNexts OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of SNMP Get-Next PDUs which have been generated by the SNMP protocol entity.”

::= { snmp 26 }

snmpOutSetRequests OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of SNMP Set-Request PDUs which have been generated by the SNMP protocol entity.”

::= { snmp 27 }

snmpOutGetResponses OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of SNMP Get-Response PDUs which have been generated by the SNMP protocol entity.”

::= { snmp 28 }

snmpOutTraps OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The total number of SNMP Trap PDUs which have been generated by the SNMP protocol entity.”

::= { snmp 29 }

snmpEnableAuthenTraps OBJECT-TYPE

SYNTAX	INTEGER { enabled(1), disabled(2) }

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“Indicates whether the SNMP agent process is permitted to generate authentication-failure traps.  The value of this object overrides any configuration information; as such, it provides a means whereby all authentication-failure traps may be disabled. Note that it is strongly recommended that this object be stored in non-volatile memory so that it remains constant between re-initializations of the network management system.”

::= { snmp 30 }

## Station Grouptc "

## Station Group"§

stationTime OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(13))

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“The station time is the time as perceived by the station. The string returned is a subset of UTC Time as specified in the Draft Recommendation X.409 Message Handling Systems : Presentation Transfer Syntax and Notation (ASN 1).The formats acceptable when the time is being set by a manager are :

a. 11 octets

YYMMDDhhmmZ

b. 13 octets

YYMMDDhhmmssZ

where

YY are two digits representing the year,

MM are two digits representing the month

DD are two digits representing the day

hh are two digits representing the hour

mm are two digits representing the month

ss are two digits representing the seconds

Z is the ASCII character Z.

This time returned by the station in response to a Get or GetNext request will always be the 13 octet variant.”

::=  { station 1 }

stationResetCounter OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This parameter is a count of the number of times the station has been reset since the last station intialisation.”

::=  { station 2 }

freeBufferCount OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This parameter represents the number of free buffers remaining in this station.”

::=  { station 3 }

freeHeaderCount OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This parameter represents the number of free headers remaining in this station.”

::=  { station 4 }

physBlkSize OBJECT-TYPE

SYNTAX	INTEGER (300..1600)

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This parameter represents the current size in bytes of the physical block used by the station’s buffer manager.”

::=  { station 5 }

newPhysBlkSize OBJECT-TYPE

SYNTAX	INTEGER (300..1600)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This parameter represents the size in bytes of the physical block, that will be used by the station’s buffer manager, after the next station reset.”

::=  { station 6 }

resetStation OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object is used to reset the station. This action does not clear any configured data, so that after the reset, the box does not return to the factory defaults.Only two values are valid to use with this object. A value of 1 is always returned when a GET is performed upon the object. To force a reset,  a value of 2 must be used in the SET.  A SET with a value of 1 has no effect. “

::=  { station 7 }

initStation OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object is used to initialise the station. Any configured data is lost, and all data values are returned to factory default values.Only two values are valid to use with this object. A value of 1 is always returned when a GET is performed upon the object. To force the initialisation,  a value of 2 must be used in the SET.  A SET with a value of 1 has no effect.”

::=  { station 8 }

resetStats OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”The statistics counters are reset to zero by performing a set on this object. This reset occurs immediately. See the Statistics Objects section for a list of these counters.Only two values are valid to use with this object. A value of 1 is always returned when a GET is performed upon the object. To force a reset,  a value of 2 must be used in the SET.  A SET with a value of 1 has no effect.”

::=  { station 9 }

procLoadTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ProcLoadValue

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”List of processor load statistics.”

::=  { station 10 }

ProcLoadValue ::=

SEQUENCE {

procLoadTable

INTEGER

}

processorLoading OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”Each instance of this object gives the percentage loading on a particular processor. Currently the first instance gives the percentage loading on the station’s main processor and the second instance gives the loading on the station’s sub-processor.”

::=  { procLoadTable 1 }

trapDestinationTable OBJECT IDENTIFIER ::= { station 11 }

The trapDestinationTable object is used only as a place holder. It is replaced by the s3agentTrapRcvr stuff.

authenticationTrapStatus OBJECT-TYPE

SYNTAX	INTEGER (0..1)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object determines if the station will send a Trap upon detection of an authentication failure error.”

::=  { station 14 }

serialTxQueueSize OBJECT-TYPE

SYNTAX	INTEGER (1..127)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This parameter indicates the maximum number of LAPB frames that can be enqueued, waiting for transmission over the serial interface.”

::=  { station 15 }

internalQueueCurrentLength OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the length of the enqueued information specific to the internal operation of the bridge.”

::=  { station 16 }

queueUpperLimit OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This is the upper limit of the enqueued information, for all the queues, specific to the  operation of the bridge.”

::=  { station 17 }

lanQueueSize OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This variable reports the maximum number of frames that are enqueued waiting for transmission over the LAN interface.”

::=  { station 18 }

## 8023 group

ieee8023Table OBJECT-TYPE

SYNTAX	SEQUENCE OF Ieee8023Entry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”A list of IEEE 802.3 Interface entries.”

::= { ieee8023 1 }

ieee8023Entry OBJECT-TYPE

SYNTAXIeee8023Entry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION “”

INDEX		{ ieee8023Index }

::=  { ieee8023Table 1 }

Ieee8023Entry ::= SEQUENCE {

ieee8023Index

INTEGER,

ieee8023FramesTransmittedOks

Counter,

ieee8023SingleCollisionFrames

Counter,

ieee8023MultipleCollisionFrames

Counter,

ieee8023OctetsTransmittedOks

Counter,

ieee8023DeferredTransmissions

Counter,

ieee8023MulticastFramesTransmittedOks

Counter,

ieee8023BroadcastFramesTransmittedOks

Counter,

ieee8023Latecollisions

Counter,

ieee8023ExcessiveCollisionsColls

Counter,

ieee8023InternalMacTransmitErrors

Counter,

ieee8023CarrierSenseErrors

Counter,

ieee8023ExcessiveDeferals

Counter,

ieee8023FramesReceivedOks

Counter,

ieee8023OctetsReceivedOks

Counter,

ieee8023MulticastFramesReceivedOks

Counter,

ieee8023BroadcastFramesReceivedOks

Counter,

ieee8023FrameTooLongs

Counter,

ieee8023AlignmentErrors

Counter,

ieee8023FCSErrors

Counter,

ieee8023inRangeLengthErrors

Counter,

ieee8023outOfRangeLengthFields

Counter,

ieee8023InternalMACReceiveErrors

Counter,

ieee8023InitializeMAC

INTEGER,

ieee8023PromiscuousReceiveStatus

INTEGER,

ieee8023MACSubLayerStatus

INTEGER,

ieee8023TransmitStatus

INTEGER,

ieee8023MulticastReceiveStatus

INTEGER,

ieee8023MACAddress

OCTET STRING,

ieee8023SQETestErrors

Counter

}

ieee8023FramesTransmittedOks OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This contains a count of frames that are successfully transmitted. This counter is incremented when a frame is successfully transmitted, even if one of the collision Counters (following two objects) are also counted. In other words, this count includes transmissions that succeeded after some number of collisions. This count includes multicast and broadcast frames. “

::= { ieee8023Entry 2 }

ieee8023SingleCollisionFrames OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This contains a count of frames that are involved in a single collision, and are subsequently transmitted successfully. If this counter is incremented then the ieee8023FramesTransmittedOKs count will also be incremented AND ieee8023MultipleCollisionFrames will NOT be incremented.”

::= { ieee8023Entry 3 }

ieee8023MultipleCollisionFrames OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This contains a count of frames that are involved in more than one collision and are subsequently transmitted successfully. If this counter is incremented then the ieee8023FramesTransmittedOKs count will also be incremented AND ieee8023SingleCollisionFrames will NOT be incremented.”

::= { ieee8023Entry 4 }

ieee8023OctetsTransmittedOks OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This contains a count of data and padding octets of frames that are successfully transmitted. This counter includes octets in broadcast and multicast frames that are transmitted. This count does not include the address fields, length/type field and CRC field of the frame. In Ethernet-Speak, it is JUST the data field (IP header, etc).”

::= { ieee8023Entry 5 }

ieee8023DeferredTransmissions OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This contains a count of frames whose transmission was delayed on its first attempt because the medium was busy. Frames involved in collisions are NOT included in this count.”

::= { ieee8023Entry 6 }

ieee8023MulticastFramesTransmittedOks OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This contains a count of frames successfully transmitted to a group destination (multicast) address other than broadcast.”

::= { ieee8023Entry 7 }

ieee8023BroadcastFramesTransmittedOks OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This contains a count of frames successfully transmitted to a group destination (multicast) address other than broadcast.”

::= { ieee8023Entry 8 }

ieee8023LateCollisions OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This contains a count of frames successfully transmitted to the broadcast address. This does not include frames sent to a multicast address.”

::= { ieee8023Entry 9 }

ieee8023ExcessiveCollisions OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This contains a count of times that a collision has been detected later than 512 bit-times into the transmission of a packet. 512 bit-times is 51.2 micro-seconds. This type ofcollision is counted twice: both as a late collision and as a regular, run of the mill collision.”

::= { ieee8023Entry 10 }

ieee8023InternalMACTransmitErrors OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This contains a count of frames that due to excessive collisions are not transmitted successfully.”

::= { ieee8023Entry 11 }

ieee8023CarrierSenseErrors OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This contains a count of frames which would otherwise be transmitted by the station, but could not be sent due to an internal MAC sublayer transmit error. If this counter isincremented then none of the other Counters in this section are incremented. The exact meaning and mechanism for incrementing this counter is implementation dependent. In short, this is the count of errors for which there are no specific Counters.”

::= { ieee8023Entry 12 }

ieee8023ExcessiveDeferrals OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This contains a count of frames which deferred for an excessive period of time.”

::= { ieee8023Entry 13 }

ieee8023FramesReceivedOks OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This contains a count of frames that are successfully received. This does not include frames received with a frame-too-long, FCS, length, or alignment errors or frames lost due to internal MAC sublayer error. This counter includes broadcast and multicast frames that are received.”

::= { ieee8023Entry 14 }

ieee8023OctetsReceivedOks OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This contains a count of data and padding octets in frames that are successfully received. This does not include frames received with a frame-too-long, FCS, length, or alignment errors or frames lost due to internal MAC sublayer error. This counter includes octets in broadcast and multicast frames that are received. This counter does not include the octets in the address, length/type and CRC fields.”

::= { ieee8023Entry 15 }

ieee8023MulticastFramesReceivedOks OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This contains a count of frames that are successfully received and are directed to an active non-broadcast group address. This does  not include frames received with a frame-too-long, FCS, length, or alignment errors or frames lost due to internal MAC sublayer error. Broadcast packets are not included.”

::= { ieee8023Entry 16 }

ieee8023BroadcastFramesReceivedOks OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This contains a count of frames that are successfully received and are directed to the broadcast group address. This does not include frames received with a frame-too- long, FCS, length, or alignment errors or frames lost due to internal MAC sublayer error. Multicast packets are not included.”

::= { ieee8023Entry 17 }

ieee8023FrameTooLongs OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This contains a count of frames that are received and exceed the maximum permitted frame size.”

::= { ieee8023Entry 18 }

ieee8023AlignmentErrors OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This contains a count of frames that are not an integral number of octets in length and do not pass the FCS check.”

::= { ieee8023Entry 19 }

ieee8023FCSErrors OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This contains a count of frames that are an integral number of octets in length that do not pass the FCS check.”

::= { ieee8023Entry 20 }

ieee8023inRangeLengthErrors OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This contains a count of frames with a length field value between the minimum unpadded LLC data size and the maximum allowed LLC data size inclusive, and does not match the number of LLC data octets received. The counter also contains frames with a length field value less than the minimum unpadded LLC data size.”

::= { ieee8023Entry 21 }

ieee8023outOfRangeLengthFields OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This contains a count of frames with a length field value greater than the maximum allowed LLC data size.”

::= { ieee8023Entry 22 }

ieee8023InternalMACReceiveErrors OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This contains a count of frames which would otherwise be received by the station, but could not be accepted due to an internal MAC sublayer receive error. If this counter is incremented then none of the other Counters in this section are incremented. The exact meaning and mechanism for incrementing this counter is implementation dependent. In short, this is the count of errors for which there are no specific Counters.”

::= { ieee8023Entry 23 }

ieee8023InitializeMAC OBJECT-TYPE

SYNTAX	INTEGER {

initialize (1)

}

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”Resets the MAC layer counters. When set, only the value initialize may be used. When read, this variable always returns the most recent value to which ieee8023InitializeMAC was set to. At the moment, this function solely serves to zero the MAC counters.”

::= { ieee8023Entry 24 }

ieee8023PromiscuousReceiveStatus OBJECT-TYPE

SYNTAX	INTEGER {

enabled (1)

}

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This variable causes the MAC sublayer to enter promiscuous receive state.”

::= { ieee8023Entry 25 }

ieee8023MACSubLayerStatus OBJECT-TYPE

SYNTAX	INTEGER {

enabled (1)

}

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”Causes the MAC sublayer to enter the normal operational state at idle.”

::= { ieee8023Entry 26 }

ieee8023TransmitStatus OBJECT-TYPE

SYNTAX	INTEGER {

enabled (1),

disabled (2)

}

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This variable allows the MAC sublayer to be explicitly enabled/disabled for transmitting frames.”

::= { ieee8023Entry 27 }

ieee8023MulticastReceiveStatus OBJECT-TYPE

SYNTAX	INTEGER {

enabled (1)

}

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”Enables reception of multicast frames.”

::= { ieee8023Entry 28 }

ieee8023MACAddress OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE (6))

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This object contains the MAC station address (i.e. the Ethernet address).”

::= { ieee8023Entry 29 }

ieee8023SQETestErrors OBJECT-TYPE

SYNTAX	COUNTER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This contains a count of times that the SQE TEST ERROR status was received.”

::= { ieee8023Entry 30 }

## Physical Serial Grouptc "Physical Serial Group"§

phySerIfNumber OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The number of Physical Serial interfaces present on this system.”

::=  { phySerIf 1 }

phySerIfTable OBJECT-TYPE

SYNTAX	SEQUENCE OF PhySerIfEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”A list of interface entries. The number of entries is given by the value of phySerIfNumber.”

::=  { phySerIf 2 }

phySerIfEntry OBJECT-TYPE

SYNTAXPhySerIfEntry

ACCESS	not-accessible

STATUS	mandatory

INDEX		{ phySerIfIndex }

::=  { phySerIfTable 1 }

PhySerIfEntry ::= SEQUENCE {

phySerIfIndex

INTEGER,

phySerIfInterfaceType

INTEGER,

phySerIfMeasuredSpeed

INTEGER,

phySerIfIsSpeedsettable

INTEGER,

phySerIfPortSpeed

INTEGER,

phySerIfTransitDelay

INTEGER,

phySerIfT1clockSource

INTEGER,

phySerIfT1SlotLvalue

INTEGER,

phySerIfT1SlotHvalue

INTEGER,

phySerIfT1dRatePerChan

INTEGER,

phySerIfT1frameAndCode

INTEGER,

phySerIfMeasuredSpeed

INTEGER,

phySerIfPartnerAddress

OCTET STRING (SIZE (6))

}

phySerIfIndex OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The interface on which this entry’s equivalence is effective.”

::=  { phySerIfEntry 1 }

phySerIfInterfaceType OBJECT-TYPE

SYNTAX	INTEGER {

x21dte(1),

x21dce(2),

rs449(3),

g703(4),

v35(5),

v35btb(6),

rs232(7),

t1(8)

}

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”Each of the values above indicates a particular personality module and hence the physical interface present. It is only possible to modify certain values of this variable, as follows:

The value x21dte(1) may be modified to x21dce(2), and vice versa.

The value v35(5) may be modified to v35btb(6), and vice versa. The changes will occur after the next link reset.”

::=  { phySerIfEntry 2 }

phySerIfMeasuredSpeed OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This parameter is the value of the measured channel speed in bits per second.”

::=  { phySerIfEntry 3 }

phySerIfIsSpeedsettable OBJECT-TYPE

SYNTAX	INTEGER (1..2)

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This parameter indicates whether the speed is settable or not for the particular personality module. This parameter does not guarantee that this will be the value of the clock used over the serial link.”

::=  { phySerIfEntry 4 }

phySerIfPortSpeed OBJECT-TYPE

SYNTAX	INTEGER {

b1200(1200),

b2400(2400),

b4800(4800),

b9600(9600),

b19200(19200),

b24000(24000),

b32000(32000),

b48000(48000),

b64000(64000),

b256000(256000),

b512000(512000),

b1024000(1024000),

b2048000(2048000)

}

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”The port speed is the value of the personality module sourced clock. It may be accessed if the value of isSpeedSettable is 1. In many cases a special cable is needed for the parameter to affect station operation - see product specifications for details.For the RS232 personality module the port speed can be set to any of the values 2400 through 48000 listed above. For all the other personality modules for which port speed is a settable parameter, the values 19600 and 24000 are invalid.  These modules can assume any of the other speeds, dependent upon the product type - see product specifications for details.When a change has been made to this object, the new value will become effective after the next link reset.”

::=  { phySerIfEntry 5 }

phySerIfTransitDelay OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This parameter indicates the time taken for a frame to traverse the phySer link between the local and remote station. Its value is automatically determined by the station’s software.”

::=  { phySerIfEntry 6 }

phySerIfT1clockSource OBJECT-TYPE

SYNTAX	INTEGER(1..3)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This parameter indicates from where the clock is being derived. If its value is 1 then the clock signal comes from the data in the frame being received, if it is 2 then the clock is derived from an internal source and if its value is 3 the clock source is external. This parameter is only relevant for the T1 personality module. When a change has been made to this object, the new value will become effective after the next link reset.”

::=  { phySerIfEntry 7 }

phySerIfT1SlotLvalue OBJECT-TYPE

SYNTAX	INTEGER (1..24)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This is the  value of the lower time slot chosen within a frame of the 1.544MHz T1 service. This parameter is only relevant for the T1 personality module. When a change has been made to this object, the new value will become effective after the next link reset.”

::=  { phySerIfEntry 8 }

phySerIfT1SlotHvalue OBJECT-TYPE

SYNTAX	INTEGER (1..24)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This is the  value of the higher time slot chosen within a frame of the 1.544MHz T1 service. This parameter is only relevant for the T1 personality module. When a change has been made to this object the new value, will become effective after the next link reset.”

::=  { phySerIfEntry 9 }

phySerIfT1dRatePerChan OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object can have either of two values depending on the data rate required. If its value is 1 a data rate of 64k is provided, if its value is 5 then the data rate provided is 56k. This parameter is only relevant for the T1 personality module. When a change has been made to this object, the new value will become effective after the next link reset.”

::=  { phySerIfEntry 10 }

phySerIfT1frameAndCode OBJECT-TYPE

SYNTAX	INTEGER (1..4)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”There are four possible values of frame and code. These values represent the encoding on the 1.544MHz T1 service. The values and the codes they represent are as follows.

fcD4AndB8zs	1

fcD4Only	2

fcEsfAndB8zs	3

fcEsfOnly	4

This parameter is only relevant for the T1 personality module. When a change has been made to this object, the new value will become effective after the next link reset.”

::=  { phySerIfEntry 11 }

phySerIfpartnerAddress OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE (0..6))

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The mac address of the remote station to which this particular serial link is connected.”

::=  { phySerIfEntry 12 }

## Multilink Grouptc "Multilink Group"§

mlinkNumber OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The number of multilink interfaces, (regardless of their current state) present on this system.”

::=  { mlink 1 }

mlinkTable OBJECT-TYPE

SYNTAX	SEQUENCE OF MlinkEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”A list of multilink entries. The number of entries is given by the value of mlinkNumber.”

::=  { mlink 2 }

mlinkEntry OBJECT-TYPE

SYNTAX	MlinkEntry

ACCESS	not-accessible

STATUS	mandatory

INDEX		{ mlinkIndex }

::=  { mlinkTable 1 }

MlinkEntry ::= SEQUENCE {

mlinkIndex

INTEGER,

mlinkState

INTEGER,

mlinkSendSeq

INTEGER,

mlinkRcvSeq

INTEGER,

mlinkSendUpperEdge

INTEGER,

mlinkRcvUpperEdge

INTEGER,

mlinkFramesLost

Counter,

deletedMlinkFrames

Counter,

expressQueueCurrentLength

INTEGER,

expressQueueUpperLimit

INTEGER,

hiPriQueueCurrentLength

INTEGER,

hiPriQueueUpperLimit

INTEGER,

loPriQueueCurrentLength

INTEGER,

loPriQueueUpperLimit

INTEGER,

mlinkChanCompression

INTEGER,

mlinkChanCompressionRatio

OCTET STRING

}

mlinkIndex OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This value reports the particular instance of multi-link being referenced.”

::=  { mlinkEntry 1 }

mlinkState OBJECT-TYPE

SYNTAX	INTEGER (1..4)

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This parameter indicates the current state of the multi link connection. The states and corresponding codes are defined as follows.

negotiating	1

notInUse	2

inUseActive	3

inUseInactive	4"

::=  { mlinkEntry 2 }

mlinkSendSeq OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This parameter indicates the current state of the multi link connection. The states and corresponding codes are defined as follows.

negotiating	1

notInUse	2

inUseActive	3

inUseInactive	4"

::=  { mlinkEntry 3 }

mlinkRcvSeq OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This parameter represents the sequence number of the next-in-sequence multilink frame to be received.”

::=  { mlinkEntry 4 }

mlinkSendUpperEdge OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This parameter is the sum of the value of the last received acknowledgement and the value of the multilink window size.”

::=  { mlinkEntry 5 }

mlinkRcvUpperEdge OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This parameter is the sum of the value of the sequence number sent in the last acknowledgement and the receive window size.”

::=  { mlinkEntry 6 }

mlinkFramesLost OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This parameter represents the number of multilink packets lost because the out of sequence queue was full or the time spent waiting for an out of sequence frame (in order to re-establish the sequence) exceeded some implementation specific period. It is the count since the last statistics reset of station restart.”

::=  { mlinkEntry 7 }

deletedMlinkFrames OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the number of multilink frames which have been deleted, because they arrived out of sequence and were outside of the multi-link window.”

::=  { mlinkEntry 8 }

expressQueueCurrentLength OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the number of items enqueued in the multilink express queue.”

::=  { mlinkEntry 9 }

expressQueueUpperLimit OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the upper limit of the enqueued information specific to the multilink express queue.”

::=  { mlinkEntry 10 }

hiPriQueueCurrentLength OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the number of items enqueued in the multilink high priority queue.”

::=  { mlinkEntry 11 }

hiPriQueueUpperLimit OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the upper limit of the enqueued information specific to the multilink high priority queue.”

::=  { mlinkEntry 12 }

loPriQueueCurrentLength OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the number of items enqueued in the multilink low priority queue.”

::=  { mlinkEntry 13 }

loPriQueueUpperLimit OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the upper limit of the enqueued information specific to the multilink low priority queue.”

::=  { mlinkEntry 14 }

mlinkChanCompression OBJECT-TYPE

SYNTAX	INTEGER (1..2)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“Enable and disable serial line compression, can take the following values :

compression enabled 	1

compression disabled	2”

::= { mlinkEntry 16 }

mlinkChanCompressionRatio OBJECT-TYPE

SYNTAX	OCTET STRING

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“An octet string which, when compression is enabled returns the rateof compression acheived on this link.  An octet string is returned which indicates for each byte of compressed data transmitted how many uncompressed byteswhere processed.  The string will be in the form of a  decimal string with up to two decimal places. i.e. as follows :

‘1.23’ - indicates a compression ration of 1.23 uncompressed data bytes to each compressed byte.”

::= { mlinkEntry 17 }

mlinkWindow OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”The Multilink window size parameter defines the number of sequentially numbered multilink frames that the DCE or DTE may transfer to its SLP beyond the lowest numbered multilink frame which has not yet been acknowledged. This parameter is not instanced; it affects all instances of multilink.”

::=  { mlink 3 }

mlinkRxTimeout OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”The time, in 50ms units, after which the multilink resets due to a receive sequence error. This parameter is not instanced; it affects all instances of multilink.”

::=  { mlink 4 }

## Bridge Grouptc "Bridge Group"§

portNumber OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The number of ports supported by the bridging function in this station.”

::=  { bridge 1 }

bridgeStatsTable OBJECT-TYPE

SYNTAX	SEQUENCE OF BridgeStatsEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”A list of bridge statistics entries. The number of entries is given by the value of bridgeNumber. This table is only valid for the LAN ports.”

::=  { bridge 2 }

bridgeStatsEntry OBJECT-TYPE

SYNTAX	BridgeStatsEntry

ACCESS	not-accessible

STATUS	mandatory

INDEX		{ bridgeStatsIndex }

::=  { bridgeStatsTable 1 }

BridgeStatsEntry ::= SEQUENCE {

bridgeStatsIndex

INTEGER,

averageForwarded

Counter,

maxForwarded

Counter,

averageRejected

Counter,

maxRejected

Counter,

lanAccepts

Counter,

lanRejects

Counter,

deletedFrames

Counter

}

bridgeStatsIndex OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”A unique value for each bridge statistics instance.”

::=  { bridgeStatsEntry 1 }

averageForwarded OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This reports the average number of frames per second accepted from the LAN for forwarding to the serial ports over the sampling period.”

::=  { bridgeStatsEntry 2 }

maxForwarded OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the highest value of averageForwarded recorded since the previous station or statistics reset.”

::=  { bridgeStatsEntry 3 }

averageRejected OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This reports the average number of frames per second that have failed the filtering criterion for forwarding to the serial ports over the previous sampling period.”

::=  { bridgeStatsEntry 4 }

maxRejected OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This parameter is a count of the highest value of averageRejected recorded since the previous station or statistics reset.”

::=  { bridgeStatsEntry 5 }

lanAccepts OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This variable reports the total number of frames that have been accepted from the LAN for forwarding over the serial links since the last station or statistics reset.”

::=  { bridgeStatsEntry 6 }

lanRejects OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This reports the total number of frames that have failed the filtering criterion for forwarding to the serial ports since the last station or statistics reset.”

::=  { bridgeStatsEntry 7 }

deletedLanFrames OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The number of LAN frames which have been deleted, due to an internal lack of resource error,  since the last station restart or the last statistics reset.”

::=  { bridgeStatsEntry 8 }

stpTable OBJECT-TYPE

SYNTAX	SEQUENCE OF StpEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”A list of stp entries. The number of entries is given by the value of bridgeNumber.”

::=  { bridge 3 }

stpEntry OBJECT-TYPE

SYNTAX	StpEntry

ACCESS	not-accessible

STATUS	mandatory

INDEX		{ stpIndex }

::=  { stpTable 1 }

StpEntry ::= SEQUENCE {

stpIndex

INTEGER,

portSpatState

INTEGER,

pathCostMode

INTEGER,

pathCostAutoValue

INTEGER,

pathCostManValue

INTEGER,

portPriorityMode

INTEGER,

portPriorityAutoValue

INTEGER,

portPriorityManualValue

INTEGER

}

stpIndex OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The interface on which this this entry’s equivalence is effective.”

::=  { stpEntry 1 }

pathCostMode OBJECT-TYPE

SYNTAX	INTEGER(1..2)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This variable controls the mode of determining the path cost for the port. If set to 1, the path cost is determined automatically by the station, if set to 2, it is configured by the manager.”

::=  { stpEntry 2 }

pathCostAutoValue OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the value of path cost assigned to the port automatically by the station.”

::=  { stpEntry 3 }

pathCostManualValue OBJECT-TYPE

SYNTAX	INTEGER(0..65535)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This is the value of path cost assigned to the port by the manager.This parameter is also saved in non-volatile storage.”

::=  { stpEntry 4 }

portSpatState OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This parameter indicates the spanning tree state of a particular port. It may return one of the following five values.

Disabled	1

Blocking	2

Listening	3

Learning	4

Forwarding	5"

::=  { stpEntry 5 }

portPriorityMode OBJECT-TYPE

SYNTAX	INTEGER(1..2)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This variable controls the mode of determining the priority for the port. If set to 1, the priority is determined automatically by the station, if set to 2, it is configured by the manager.”

::=  { stpEntry 6 }

portPriorityAutoValue OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the value of priority assigned to the port automatically by the station.”

::=  { stpEntry 7 }

portPriorityManualValue OBJECT-TYPE

SYNTAX	INTEGER(1..255)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This is the value of priority assigned to the port by the manager.”

::=  { stpEntry 8 }

portDesignatedRoot OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(8))

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The bridge identifier of the of the root bridge recorded as the rootin hellog frames transmitted by the bridge on this port.”

::=  { stpEntry 9 }

portDesignatedCost OBJECT-TYPE

SYNTAX	INTEGER (0..65535)

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The path cost to the root from the designated port on the lan to which this port is attached.”

::=  { stpEntry 10 }

portDesignatedPort OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The port id of the bridge beleived to the designated port of the lan associated with the port.”

::=  { stpEntry 11 }

portDesignatedBridge OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(8))

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The bridge identifier of the of the designated bridge for the lan associatedwith the port.”

::=  { stpEntry 12 }

spanningTree OBJECT-TYPE

SYNTAX	INTEGER (1..2)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object describes whether or not the spanning tree protocol is enabled on this station. A value of 1 disables the spanning tree protocol, a value of 2 enables it.”

::=  { bridge 4 }

spatPriority OBJECT-TYPE

SYNTAX	INTEGER (1..65535)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”Each station has a priority, used by the spanning Tree protocol in determining the root bridge of a spanning tree network. The lower the value, which can range between the values 1 and 65535, the higher the priority.”

::=  { bridge 5 }

spatHelloTimer OBJECT-TYPE

SYNTAX	INTEGER (1..10)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This is the period of time, in seconds, between the transmission of subsequent configuration BPDUs, used by the spanning tree algorithm.”

::=  { bridge 6 }

spatResetTimer OBJECT-TYPE

SYNTAX	INTEGER (1..1800)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”The length of time, in seconds, after a serial link reset, for which the corresponding serial port’s path cost is forced to its maximum value.”

::=  { bridge 7 }

spatVersion OBJECT-TYPE

SYNTAX	INTEGER {

revisionC(3),

revision8(8)

}

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This variable determines the version of the spanning tree algorithm implemented in the station.”

::=  { bridge 8 }

spanningMcastAddr OBJECT-TYPE

SYNTAX	OCTET STRING ( SIZE (6))

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This is the multicast address of the frames used by the spanning tree protocol. The multicast address is in hexadecimal form, in network byte order.”

::=  { bridge 9 }

operatingMode OBJECT-TYPE

SYNTAX	INTEGER (1..2)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This determines whether the bridge will forward frames based only on the static addresses in the filtering database stored in its preconfigured table, or use its learning algorithm. It may take the following values:

learning mode	1

preconfigured mode	2"

::=  { bridge 10 }

preconfSourceFilter OBJECT-TYPE

SYNTAX	INTEGER (1..2)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This determines whether the bridge will forward frames based only on the static addresses in the filtering database stored in its preconfigured table, or use its learning algorithm. This parameter determines whether the bridge will filter on destination address alone, or destination and source addresses. A value of 1 disables filtering on the source address, whilst a value of 2 enables it.”

::=  { bridge 11 }

typeFilter OBJECT-TYPE

SYNTAX	INTEGER (1..2)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This parameter determines whether or not filtering is performed on frame type. A value of 1 disables this filtering, a value of 2 enables it.”

::=  { bridge 12 }

typePrioritisation OBJECT-TYPE

SYNTAX	INTEGER (1..2)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This parameter enables or disables the prioritisation of traffic destined for a remote LAN, dependent upon the type field of the frame. It can take on either of the following two values.

Disabled	1

Enabled	2"

::=  { bridge 13 }

dynamicLearningInLM OBJECT-TYPE

SYNTAX	INTEGER (1..2)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This parameter allows the enabling and disabling of learning when the station is configured to be in Learning mode. A value of 1 disables learning, whilst a value of 2 enables it.”

::=  { bridge 14 }

forgetAddressTimer OBJECT-TYPE

SYNTAX	INTEGER (24..1000)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This is the time, in 5 second units, after which, if an address has not been seen on the local LAN, it is forgotten from the learning address table.”

::=  { bridge 15 }

deleteAddressTimer OBJECT-TYPE

SYNTAX	INTEGER (1..20000)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This is the time, in 5 second units, after which, if an address has not been seen on the local LAN, it is deleted from the learning address table.”

::=  { bridge 16 }

multicastDisposition OBJECT-TYPE

SYNTAX	INTEGER (1..2)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This parameter determines the actions of the station upon receiving a multicast frame, when the station is set to preconfigured mode. If it is set to 2, multicasts are forwarded unconditionally to the serial link. If it is set to 1, they are handled like single cast addresses, being forwarded only if the multicast in question is present in the filtering database with the appropriate disposition.”

::=  { bridge 17 }

filterMatches OBJECT-TYPE

SYNTAX	INTEGER (1..2)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This parameter determines the action of the station when filtering on frame type. If it is set to 1, the frame is discarded if the type matches, and forwarded if it does not. If it is set to 2, the frame is forwarded if it matches, and discarded if it does not.”

::=  { bridge 18 }

ieeeFormatFilter OBJECT-TYPE

SYNTAX	INTEGER (1..2)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This variable determines whether IEEE802.3 frames will be forwarded or discarded, when type filtering is being performed and a match achieved, and may assume either of two values:

discard	1

forward	2"

::=  { bridge 19 }

priorityMatches OBJECT-TYPE

SYNTAX	INTEGER (1..2)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This variable defines the priority assigned to a frame if it is being forwarded after type matching, and can assume either of the two following values:

high	1

low	2"

::=  { bridge 20 }

ieeeFormatPriority OBJECT-TYPE

SYNTAX	INTEGER (1..2)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This variable defines the priority assigned to a IEEE 802.3 frame if it is being forwarded after type matching, and can assume either of the two following values:

high	1

low	2"

::=  { bridge 21 }

averagePeriod OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the averaging period over which the statistics in the BridgeStatsTable get updated.”

::=  { bridge 22 }

triangulation OBJECT-TYPE

SYNTAX	INTEGER(1..2)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This variable determines if the triangulation facility is enabled for this station, and can take either of the following values:

disabled	1

enabled	2"

::=  { bridge 23 }

adaptiveRouting OBJECT-TYPE

SYNTAX	INTEGER (1..2)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This variable determines whether the adaptive routing protocol is enabled for this station, and can assume either of the values:

disabled	1

enabled	2

It should be noted that in order to enable adaptive routing, the spanning tree algorithm  must  also be enabled.”

::=  { bridge 24 }

adaptiveMcastAddr OBJECT-TYPE

SYNTAX	OCTET STRING ( SIZE (6))

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This is the multicast address of the frames used by the adaptive routing protocol. The multicast address is in hexadecimal form, in network byte order.”

::=  { bridge 25 }

arAddressInfo OBJECT IDENTIFIER ::= { bridge 26 }

standbyRemote OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This object reports the total number of Adaptive Routing Destination Routes.”

::=  { arAddressInfo 1 }

standbyLocal OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This object reports the total number of Adaptive Routing Source Routes.”

::=  { arAddressInfo 2 }

activeRemote OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This object reports the total number of Adaptive Routing Remote Addresses.”

::=  { arAddressInfo 3 }

activeLocal OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This object reports the total number of Adaptive Routing Learned Addresses.”

::=  { arAddressInfo 4 }

maxSerialLoading OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object is used by the Adaptive Routing protocol. If the loading of a serial link exceeds this value, then any frames enqueued for this link by Adaptive Routing will be removed from the queue and enqueued on the other serial link. The value of this object is a measure of the percentage loading of the line.”

::=  { bridge 27 }

serialLoadPeriod OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This is the sampling period over which the serial links loading is measured, in seconds.”

::=  { bridge 28 }

serialLoading OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”Each instance of this object gives the percentage loading on a particular serial line. The instances are indexed by the value of if Index.”

::=  { bridge 29 }

SYNTAX SEQUENCE OF SerialLoadValue

ACCESS	not-accessible

STATUS	mandatory

INDEX		{ phySerIfEntry  }

DESCRIPTION
”List of serial load statistics.”

::=  { bridge 29 }

SerialLoadValue ::= SEQUENCE {

serialLoadTable INTEGER }

serialLoading OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”Each instance of this object gives the percentage loading on a particular serial line. The instances are indexed by the value of if Index.”

::=  { serialLoadTable 1 }

filteringDataBaseTable OBJECT IDENTIFIER ::= { bridge 30 }

filteringDbTable OBJECT-TYPE

SYNTAX	SEQUENCE OF FilteringDbEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”A list of filtering  data base entries.”

::=  { filteringDataBaseTable 1 }

filteringDbEntry OBJECT-TYPE

SYNTAX	SEQUENCE OF FilteringDbEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”This OCTET STRING contains one entry from the table. Each entry consists of a MAC address, filter disposition, filter status and type.”

INDEX		{ filteringDbMacAddress }

::=  { filteringDbTable 1 }

FilteringDbEntry ::= SEQUENCE {

filteringDbMacAddress

OCTET STRING,

filteringDbDisposition

INTEGER,

filteringDbStatus

INTEGER,

filteringDbType

INTEGER

}

filteringDbMacAddress OBJECT-TYPE

SYNTAX	OCTET STRING(6)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This OCTET STRING contains the MAC address in hexadecimal form, in network byte order.”

::=  { filteringDbEntry 1 }

filteringDbDisposition OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”The Filter Disposition byte has the following values :

0	discard

1	lan

2	channel1

4	channel2

7	flood”

::=  { filteringDbEntry 2 }

filteringDbStatus OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”The following values are available :

1	Learnt

2	Unlearnt

3	Static

4	Bridge Multicast address

5	Learnt - ar controlled

6	Unlearnt - ar controlled

7	Static - ar controlled

Only static entries may be entered in the table, remotely, via SNMP., i.e. a SET on this object may only have a value of 3.”

::=  { filteringDbEntry 3 }

filteringDbType OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”The type field can take either of the values shown below:

valid	1

invalid	2

For a Get or GetNext, this field is always returned as valid. For a Set, if the field is set to a value of valid, an entry is added to the table, whilst if it is set to invalid an entry is deleted.”

::=  { filteringDbEntry 4 }

filteringDbAction OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object is used to clear the Filtering Database Table of all static entries. Other entries are not affected.Only two values are valid to use with this object. A value of 1 is always returned when a GET is performed upon the object. To clear all static entries from the table, a value of 2 must be used in the SET.  A SET with a value of 1 has no effect.”

::=  {filteringDataBaseTable 2 }

filteringDbPage OBJECT-TYPE

SYNTAX	SEQUENCE OF FilteringDbPageValue

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”A list of paged entries from the filtering data base table. We shoulduse an object ‘filteringDbPageIndex’ as the index if one such exists, but since it doesnt we cannot.”

::=  { filteringDataBaseTable 3 }

FilteringDbPageValue ::= SEQUENCE {

filteringDbPage OCTET STRING ( SIZE (0..640))

}

filteringDbPageValue OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(0..640))

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This OCTET STRING contains up to 80 valid table entries. Each entry consists of a MAC address, filter disposition and filter status. Shown below is the first valid entry of a get on this object, as it is returned within the OCTET STRING :

Bytes 0 - 5	Byte 6	Byte 7

MAC Address	Filter Disposition	Filter Status

Each entry in the OCTET STRING is in the same order as that shown for the first entry above. The MAC address is in hexadecimal form, in network byte order.The Filter Disposition byte has the following values :

0	discard

1	lan

2	channel1

4	channel2

7	flood

The filter status byte can take the following values:

1	Learnt

2	Unlearnt

3	Static

4	Bridge Multicast address

5	Learnt - ar controlled

6	Unlearnt - ar controlled

7	Static - ar controlled”

::=  { filteringDbPage 1 }

priorityTable  OBJECT IDENTIFIER ::= { bridge 31 }

priorityTableEntry OBJECT-TYPE

SYNTAX	PriorityTableEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”This sequence defines one entry in the priority table.There is a maximum of 8 instances of this sequence.”

INDEX		{ priorityTableEntryValue }

::=  { prioritySubTable 1 }

PriorityTableEntry ::= SEQUENCE {

priorityTableEntryValue

INTEGER,

priorityTableEntryType

INTEGER

}

priorityTableEntryValue OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”The priority being added to, deleted or read from the table.”

::=  { priorityTableEntry 1 }

priorityTableEntryType OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object defines whether a priority is being added to or deleted from the priority table. It can assume the following values:

valid	1

invalid	2

A value of valid causes a priority to be added to the table, while a value of invalid causes it to be deleted. For a Get or Getnext, the value returned is always valid.”

::=  { priorityTableEntry 2 }

priorityTableAction OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object is used to clear the priority Table. Only two values are valid to use with this object. A value of 1 is always returned when a GET is performed upon the object. To clear the table,  a value of 2 must be used in the SET.  A SET with a value of 1 has no effect.”

::=  { priorityTable 2 }

filterTable OBJECT IDENTIFIER ::= { bridge 32 }

filterSubTable OBJECT-TYPE

SYNTAX	SEQUENCE OF FilterTableEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”This is a list of entries in the filter table.”

::=  { filterTable 1 }

filterTableEntry OBJECT-TYPE

SYNTAX	SEQUENCE OF FilterTableEntry

ACCESS	not-accessible

STATUS	mandatory

INDEX		{ filterTableEntryValue }

::=  { filterSubTable 1 }

FilterTableEntry ::= SEQUENCE {

filterTableEntryValue

INTEGER,

filterTableEntryType

INTEGER

}

filterTableEntryValue OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This sequence defines one entry in the filter table.There is a maximum of 8 instances of this object.”

::=  { filterTableEntry 1 }

filterTableEntryType OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This determines whether the value of a filter to be added to,  or deleted from. It can assume the following values:

valid	1

invalid	2

A value of valid causes the filter to be added. A value of invalid causes it to be deleted. For a Get, this value is always returned as valid.”

::=  { filterTableEntry 2 }

filterTableAction OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object is used to clear the filter Table of its static entries, all others are not affected.Only two values are valid to use with this object. A value of 1 is always returned when a GET is performed upon the object. To clear the table of static entries, a value of 2 must be used in the SET.  A SET with a value of 1 has no effect.”

::=  { filterTable 2 }

brdgBroadcastRX OBJECT-TYPE

SYNTAX	INTEGER (1..2)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”When set to 1 indicates that broadcast packets will be received onthe lan channel, when set to 2 they will be ignored.”

::=  { bridge 33 }

brdgLiveSpatHelloTimer OBJECT-TYPE

SYNTAX	INTEGER (1..10)

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The time, in seconds, between the transmission of configuration BPDU’sused by spanning tree.  If this bridge is the root bridge then all bridgeswill use this value.”

::=  { bridge 34 }

brdgHoldTime OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”After this time, in seconds, unacknowledged spanning tree bpdus willbe reset.”

::=  { bridge 35 }

brdgMaxAge OBJECT-TYPE

SYNTAX	INTEGER (6..40)

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The max time, in seconds, that a received STP protocol message can exist before it is discarded by the bridge.  If this bridge is theroot bridge, then this value will be used by all bridges in the network.”

::=  { bridge 36 }

brdgLiveMaxAge OBJECT-TYPE

SYNTAX	INTEGER (6..40)

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The max time, in seconds, that a received STP protocol message can exist before it is discarded by the bridge.  If this bridge is theroot bridge, then this value will be used by all bridges in the network. This is the current operation value and will be the sameas the initial Max Age of the root bridge of the network.”

::=  { bridge 37 }

brdgLiveBridgeForwardDelay OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The time spent in seconds in STP listening and learning states bythe bridge.  Thisis the current operation value and is assigned to thisbridge by the root bridge if this bridge is not the root.”

::=  { bridge 38 }

brdgTopologyChangeCount OBJECT-TYPE

SYNTAX	COUNTER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The count of the number of STP network topology changes since the lastbridge reset.”

::=  { bridge 39 }

brdgPriority OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”The spanning tree priority of the bridge.”

::=  { bridge 40 }

brdgPathCostScalingFactor OBJECT-TYPE

SYNTAX	INTEGER (1..10000)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”A scaling factor to be used when calculating path costs.  The IEEE specification (Draft 9) recommends a method for calculating the path cost for spanning tree ports.  This caculation is:

path cost = 1000 / data rateLan speed in Megabits/sec

It is the ‘1000’ of the above calculation which will be replaced by the configured value of brdgPathCostScalingFactorthe value of thisobject will be used for.”

::=  { bridge 40 }

## Boot group

bootpRetryCount  OBJECT-TYPE

SYNTAX	INTEGER ( 0..65535 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”The number of times a Bootp Request will be transmitted before the Bootp attempt is considered to have failed.  If this parameter is set to 0, Bootp is skipped and an immediate TFTP download attempted.”

::= { boot 1 }

downloadRetryCount  OBJECT-TYPE

SYNTAX	INTEGER ( 0..65535 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”The number of times the complete Bootp/TFTP download cycle is attempted before the download attempt is considered to have failed.”

::= { boot 2 }

downloadFilename  OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE( 0..128))

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This character string is inserted in the ‘boot file name’ field of the Bootp Request; its semantics is determined by the Boot Server.  If Bootp is skipped and an immediate TFTP download attempted, this parameter is used by TFTP as the name of the file to be downloaded.”

::= { boot 3 }

bootserverIpAddress  OBJECT-TYPE

SYNTAX	IpAddress

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”If this address is null (0.0.0.0), the Boot Request is sent to the IP broadcast address as determined by ipAdEntBcastAddr.  Otherwise it is sent to this address.  It is known that certain bootserver implementations do not respond to the standard limited broadcast, hence the need for this parameter.”

::= { boot 4 }

loadserverIpAddress  OBJECT-TYPE

SYNTAX	IpAddress

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”If this address is null (0.0.0.0), TFTP will request a download from the server address returned in the Bootp Response.  Otherwise it will use this address.”

::= { boot 5 }

uniqueBroadcastAddress  OBJECT-TYPE

SYNTAX	IpAddress

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”A unique broadcast address specified by the user which does not conform to the standard ip broadcast address format. If the user specifies a unique broadcast address, this will take precedence over the standard ip broadcast address.”

::= { boot 6 }

tftpRetryCount  OBJECT-TYPE

SYNTAX	INTEGER ( 0..65535)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”The number of times a PDU will be retransmitted during a TFTP transfer before declaring the transfer to have failed.”

::= { boot 7 }

tftpRetryPeriod  OBJECT-TYPE

SYNTAX	INTEGER (0..65535)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”The time  in units of 50ms that TFTP will wait for the next requested block before initiating error recovery.”

::= { boot 8 }

genericFilename  OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE( 0..128))

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”The bootfilename returned from the boot server following a bootprequest which will be used in a subsequent tftp download request.”

::= { boot 9 }

## RmtLapb grouptc "RmtLapb group"§

### Remote lapb configuration tabletc "Remote lapb configuration table"§

rmtLapbConfigTable OBJECT-TYPE

SYNTAX	SEQUENCE OF RmtLapbCTEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”A list of configurable LapbCT entries, one entry per interface.”

::= { rmtLapb 1 }

rmtLapbCTEntry OBJECT-TYPE

SYNTAX	RmtLapbCTEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”A list of configurable data, for each lapb interface.”

INDEX		{ rmtLapbCTIndex }

::= { rmtLapbConfigTable 1 }

RmtLapbCTEntry ::= SEQUENCE {

rmtLapbCTIndex

INTEGER,

rmtLapbCTLinkAddr

INTEGER,

rmtLapbCTExtSeqNumbering

INTEGER,

rmtLapbCTWindow

INTEGER( 1..127 ),

rmtLapbCTModeT1

INTEGER,

rmtLapbCTManualT1Value

INTEGER( 10..5000 ),

rmtLapbCTT3LinkIdleTimer

INTEGER( 1..65535 ),

rmtLapbCTN2RetryCount

INTEGER( 1..255 ),

rmtLapbCTLinkReset

INTEGER,

rmtLapbCTX25PortLineSpeed

INTEGER,

rmtLapbCTInitLinkConnect

INTEGER

}

rmtLapbCTIndex OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This defines the interface for which this entry is valid.”

::= { rmtLapbCTEntry 1 }

rmtLapbCTLinkAddr OBJECT-TYPE

SYNTAX	INTEGER ( 1..2 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object defines the link level logical address, and also the packet level logical address. It can take the following values :-

DTE	1

DCE	2"

::= { rmtLapbCTEntry 2 }

rmtLapbCTExtSeqNumbering OBJECT-TYPE

SYNTAX	INTEGER ( 1..2 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object enables/disables the use of extended frame sequence numbering at the link level. When enabled, modulo 128 numbering is used, as opposed to modulo 8 when disabled. It can take the following values :-

disabledenabled	1

enableddisabled	2"

::= { rmtLapbCTEntry 3 }

rmtLapbCTWindow OBJECT-TYPE

SYNTAX	INTEGER (1..127)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This parameter represents the maximum number of I-frames which can be transmitted before an acknowledgement is received.  It can have any value between 1 and 127, ( for this value to be greater than 7, extended sequence numbering must be invoked. This is the value of k as defined in the 1984 CCITT Recommendation X.25 VOLUME VIII - FASCILE VIII.3"

::= { rmtLapbCTEntry 4 }

rmtLapbCTModeT1 OBJECT-TYPE

SYNTAX	INTEGER (1..2)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”If the  value of modeT1 is 1 then T1 timer is determined automatically by the software. If the value of this object is 2 then the timer is settable by the manager. The value of T1 won’t change until the next reset of the station or the next link reset.”

::= { rmtLapbCTEntry 5 }

rmtLapbCTManualT1Value OBJECT-TYPE

SYNTAX	INTEGER (10..5000)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object allows the manager to set the value of the T1 timer as defined in the 1984 CCITT Recommendation X.25 VOLUME VIII -  FASCICLE VIII.3. The settable range is 10 - 5000 which represents the timer value in milliseconds. This object maybe set at any time, but it is only used if the value of modeT1 is 1.  This value will not become active until the next link or station reset.”

::= { rmtLapbCTEntry 6 }

rmtLapbCTT3LinkIdleTimer OBJECT-TYPE

SYNTAX	INTEGER (1..65535)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This is the value of the T3 timer, which defines the maximum time which a link can remain in the idle condition before being identified as non-operational.”

::= { rmtLapbCTEntry 7 }

rmtLapbCTN2RetryCount OBJECT-TYPE

SYNTAX	INTEGER (1..255)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This is the value of N2 as defined in the 1984 CCITT Recommendation X.25 VOLUME VIII -  FASCICLE VIII.3"

::= { rmtLapbCTEntry 8 }

rmtLapbCTLinkReset OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This parameter causes a link reset. The link reset occurs after a pre-determined period of time which is implementation specific.A value of 2 is used to SET this object and to cause the link reset. A value of 1 will be returned when a GET is performed on this object. If SET with a value of 1, no action is taken. No other values are valid.”

::= { rmtLapbCTEntry 9 }

rmtLapbCTX25PortLineSpeedOBJECT-TYPE

SYNTAX	INTEGER  {

b1200 (1200),

b2400 (2400),

b4800 (4800),

b9600 (9600),

b19200 (19200),

b32000 (32000),

b48000 (48000),

b64000 (64000)

}

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”The configured line speed.”

::= { rmtLapbCTEntry 10 }

rmtLapbCTInitLinkConnectOBJECT-TYPE

SYNTAX	INTEGER  {

enabled (1),

disabled (2)

}

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”Instructs the serial code to initiate a link connect, as opposed to waiting to be connected to. Effective only when x25 is enabled.”

::= { rmtLapbCTEntry 11 }

remote lapb statistics tabletc "remote lapb statistics table"§

rmtLapbStatsTable OBJECT-TYPE

SYNTAX	SEQUENCE OF RmtLapbSTEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”A list of LAPB interface entries.”

::= { rmtLapb 2 }

rmtLapbSTEntry OBJECT-TYPE

SYNTAX	RmtLapbSTEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”The LAPB table. An entry in this table is uniquely identified by the value of the rmtLapbSTIndex variable associated with the interface to which the statistics refer.”

INDEX		{ rmtLapbSTIndex }

::= { rmtLapbStatsTable 1 }

RmtLapbSTEntry ::= SEQUENCE {

rmtLapbSTIndex

INTEGER,

rmtLapbSTState

INTEGER,

rmtLapbSTAutoT1value

INTEGER,

rmtLapbSTLastResetTime

OCTET STRING (SIZE (13)),

rmtLapbSTLastResetReason

INTEGER,

rmtLapbSTResetCount

Counter,

rmtLapbSTSendFrameCount

Counter,

rmtLapbSTRcvFrameCount

Counter,

rmtLapbSTSendOctetCount

Counter,

rmtLapbSTRcvOctetCount

Counter,

rmtLapbSTAbortCount

Counter,

rmtLapbSTCrcErrorCount

Counter

}

rmtLapbSTIndex OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The interface on which this entry’s equivalence is effective. This object identifies the same physical interface as identified by the phySerIfIndex.”

::=  { rmtLapbSTEntry 1 }

rmtLapbSTState OBJECT-TYPE

SYNTAX	INTEGER (1..4)

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This parameter indicates the state of the LAPB link, it can take on one of the following four values.

inactive	1

active	2

unused	3

tooFast	4

Inactive indicates that LAPB initialisation was unsuccessful. Active indicates that LAPB initialisation on this link was successful. Unused indicates that, in the case where there are two parallel links between bridges one of which is slower, the slower link has been tagged ‘unused’. This is done to concentrate transmissions on the faster link and thus avoid a heavy re-ordering overhead due to frames arriving out of sequence. TooFast indicates that the clock on the received signal is too fast for the interface in question.”

::= { rmtLapbSTEntry 2 }

rmtLapbSTAutoT1value OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This parameter is the value of the T1 timer as defined in the 1984 CCITT Recommendation X.25 VOLUME VIII -  FASCICLE VIII.3 and is determined empirically by the software. It is the actual value of the timer in milliseconds.”

::= { rmtLapbSTEntry 3 }

rmtLapbSTLastResetTime OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE (13))

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”Time and date when the last link reset occurred. The format of this string is the same as that defined for stationTime.”

::= { rmtLapbSTEntry 4 }

rmtLapbSTLastResetReason OBJECT-TYPE

SYNTAX	INTEGER (1..9)

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The reason for the link reset. This number can have any one of nine values. The following is a list of the values and reset reasons they indicate :

noReason	1

xpcError	2

ctsLost	3

retryLimitExceeded	4

receivedIdle	5

personalityModule	6

softwareWatchdog	7

nmcInitiated	8

invalidConfiguration	9"

::= { rmtLapbSTEntry 5 }

rmtLapbSTResetCount OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The parameter represents the number of link resets performed since the last warm or cold restart of the station or the last statistics reset.”

::= { rmtLapbSTEntry 6 }

rmtLapbSTSendFrameCount OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The parameter represents the number of I-frames sent since the last station restart or the last statistics reset.”

::= { rmtLapbSTEntry 7 }

rmtLapbSTRcvFrameCount OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The parameter represents the number of I-frames sent since the last station restart or the last statistics reset.”

::= { rmtLapbSTEntry 8 }

rmtLapbSTSendOctetCount OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The parameter represents the number of bytes sent since the last station restart  or the last statistics reset.”

::= { rmtLapbSTEntry 9 }

rmtLapbSTRcvOctetCount OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The parameter represents the number of bytes received since the last station restart or the last statistics reset.”

::= { rmtLapbSTEntry 10 }

rmtLapbSTAbortCount OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This parameter represents the number of aborts received since last station restart or the last statistics reset.”

::= { rmtLapbSTEntry 11 }

rmtLapbSTCrcErrorCount OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The parameter represents the number of CRC errors received since the last station restart or the last statistics reset.”

::= { rmtLapbSTEntry 12 }

## X25 Grouptc "X25 Group"§

x25Operation OBJECT-TYPE

SYNTAX	INTEGER ( 1..2 )

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This object tells us whether X25 is enabled or disabled, at the packet level, for the device. The following values apply :-

enabled	1,

disabled	2

If X25PLP is turned on, this markedly affects the manner in which Lapb works on this interface. It also affects the relevance and effectiveness of some Lapb managed objects.”

::= { x25 1 }

x25OperNextReset OBJECT-TYPE

SYNTAX	INTEGER ( 1..2 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object tells us whether X25 is to be enabled or disabled, at the packet level,  for the device on the next Reset. The following values apply :-

enabled	1,

disabled	2

If X25PLP is turned on, this markedly affects the manner in which Lapb works on this interface. It also affects the relevance and effectiveness of some Lapb managed objects.”

::= { x25 2 }

x25 Connection set-up table

x25ConSetupTable OBJECT-TYPE

SYNTAX	SEQUENCE OF  X25CSTEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”This table holds configuration data used in setting up each X25 SVC.”

::= { x25 3 }

x25CSTEntry OBJECT-TYPE

SYNTAX	X25CSTEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”Each entry in the Table includes all the configurable objects for each interface.”

INDEX		{ x25CSTIndex }

::= { x25ConSetupTable 1 }

X25CSTEntry ::= SEQUENCE {

x25CSTIndex

INTEGER,

x25CST8084Switch

INTEGER,

x25CSTSrcDTEAddr

OCTET STRING ( SIZE (15) ),

x25CSTDestDTEAddr

OCTET STRING ( SIZE (15) ),

x25CST2WayLgclChanNum

INTEGER,

x25CSTPktSeqNumFlg

INTEGER,

x25CSTFlowCntrlNeg

INTEGER,

x25CSTDefaultWinSize

INTEGER,

x25CSTDefaultPktSize

INTEGER,

x25CSTNegWinSize

INTEGER,

x25CSTNegPktSize

INTEGER,

x25CSTCUGSub

INTEGER,

x25CSTLclCUGValue

INTEGER,

x25CSTRvrsChrgReq

INTEGER,

x25CSTRvrsChrgAcc

INTEGER

}

x25CSTIndex OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This index defines the interface for which this entry is valid. This index is  equivalent to the phySerIfNumber index representing the physical serial  interfaces on the device.”

::= { x25CSTEntry 1 }

x25CST8084Switch OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object is an operation switch, allowing the user to change the operating mode of x25 on this interface. A value of 1 sets the operating mode to 1980 operating mode, wheras a value of 2 sets it to 1984. “

::= { x25CSTEntry 2 }

x25CSTSrcDTEAddr OBJECT-TYPE

SYNTAX	OCTET STRING

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This is the address of the Source DTE associated with this interface. It can be any number, up to 15 ascii characters long, in the range 0-9.”

::= { x25CSTEntry 3 }

x25CSTDestDTEAddr OBJECT-TYPE

SYNTAX	OCTET STRING

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This is the address of the Destination DTE associated with this interface. It can  be any number, up to 15 ascii characters long, in the range 0-9.”

::= { x25CSTEntry 4 }

x25CST2WayLgclChanNum OBJECT-TYPE

SYNTAX	INTEGER ( 1.. 4095 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This number uniquely identifies the two-way logical channel number to be used in the DTE/DCE interface when establishing an SVC on this interface.”

::= { x25CSTEntry 5 }

x25CSTPktSeqNumFlg OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This flag allows the user to switch between Basic( modulo 8 ) and Extended ( modulo 128 ) Packet Sequence Numbering. A value of 1 indicates the use of Basic numbering, while a value of 4 specifies Extended numbering.”

::= { x25CSTEntry 6 }

x25CSTFlowCntrlNeg OBJECT-TYPE

SYNTAX	INTEGER ( 1..2 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object enables/disables Flow Control Negotiation on this interface. It can  have the following values:-

enabled	1

disabled	2

If disabled, the values of window size and packet size used on the connection are the specifed default values. If Negotiation is enabled, then this DTE offers its negotiation values and these may be negotiated downwards by the remote entity.”

::= { x25CSTEntry 7 }

x25CSTDefaultWinSize OBJECT-TYPE

SYNTAX	INTEGER ( 1..127 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This is the window size used if Negotiation is not enabled. If basic frame numbering is in operation, then it cannot have a value greater than 7. If extended frame numbering is enabled, then it can have a value up to  127.”

::= { x25CSTEntry 8 }

x25CSTDefaultPktSize OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This is the packet size used if Negotiation is not enabled. This must be less then or equal to the Physical Block size on the device. It can take the following values :

1 - 128 bytes.

2- 256 bytes.

3 - 512 byrtes.”

::= { x25CSTEntry 9 }

x25CSTNegWinSize OBJECT-TYPE

SYNTAX	INTEGER ( 1..127 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This is the window size requested when a connection is being initialised, if  Negotiation is enabled. If basic frame numbering is in operation, then it cannot have a value greater than  7. If extended frame numbering is enabled, then it can have a value up to 127.”

::= { x25CSTEntry 10 }

x25CSTNegPktSize OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This is the packet size requested at call setup time, if Negotiation is enabled. This should not be larger than the Physical Block size defined for this device. It can take the following values:

1 - 128 bytes.

2 - 256 bytes.

3 - 512 bytes.”

::= { x25CSTEntry 11 }

x25CSTCUGSub OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object defines whether we enable the use of the Closed User Group  selectrion facility for connections on this interface. It can have the following  values :-

disabled	1

CUGonly	4"

::= { x25CSTEntry 12 }

x25CSTLclCUGValue OBJECT-TYPE

SYNTAX	INTEGER ( 0..99 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This value is the number which uniquely defines the Closed User Group value to be used within the Closed User Group Selection facility of the call packet.”

::= { x25CSTEntry 13 }

x25CSTRvrsChrgReq OBJECT-TYPE

SYNTAX	INTEGER ( 1..2)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”When a connection is being set up, this device will request that Reverse  Charging of the call be accepted by the remote end, if this object is enabled.  It can have the following values:-

enabled	1

disabled	2"

::= { x25CSTEntry 14 }

x25CSTRvrsChrgAcc OBJECT-TYPE

SYNTAX	INTEGER ( 1..2)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”When a connection is being set up, this box will accept Reverse Charging of the  call, initiated by the remote end, if this object is enabled.  It can have the following values:-

enabled	1

disabled	2"

::= { x25CSTEntry 15 }

x25 Connection Control Table

x25ConControlTable OBJECT-TYPE

SYNTAX	SEQUENCE OF X25CCTEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”This table holds configurable data used in controlling each X25 SVC.”

::= { x25 4 }

x25CCTEntry OBJECT-TYPE

SYNTAX	X25CCTEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”Each entry contains configurable objects allowing the control of the status of the  SVC.”

INDEX		{ x25CCTIndex }

::= { x25ConControlTable 1 }

X25CCTEntry ::= SEQUENCE {

x25CCTIndex

INTEGER,

x25CCTManualConnect

INTEGER,

x25CCTManualDisconnect

INTEGER,

x25CCTCfgAutoConRetry

INTEGER,

x25CCTOperAutoConRetryFlg

INTEGER,

x25CCTAutoConRetryTimer

INTEGER,

x25CCTMaxAutoConRetries

INTEGER,

x25CCTCfgTODControl

INTEGER,

x25CCTAutoTODControlFlg

INTEGER,

x25CCTTODToConnect

OCTET STRING ( SIZE(4) ),

x25CCTTODToDisconnect

OCTET STRING ( SIZE(4) )

}

x25CCTIndex OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This index defines the interface for which this entry is valid. This index is  equivalent to the phySerIfNumber index representing the physical serial  interfaces on the device.”

::= { x25CCTEntry 1 }

x25CCTManualConnect OBJECT-TYPE

SYNTAX	INTEGER ( 1..2)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object, when set, causes a manual (re)connect of the X25 SVC .  The object  can have the following values:-

no action	1

connect	2

A Get on this object will return the value 1, i.e. no action, A Set with a value of 2,  will cause a (re)connect of the SVC. A Set with a value of 1 shall have no effect.”

::= { x25CCTEntry 2 }

x25CCTManualDisconnect OBJECT-TYPE

SYNTAX	INTEGER ( 1..2)

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object, when set, causes a manual disconnect of the X25 SVC .  The object  can have the following values:-

no action	1

connect	2

A Get on this object will return the value 1, i.e. no action, A Set with a value of 2,  will cause a disconnect of the SVC. A Set with a value of 1 shall have no effect.”

::= { x25CCTEntry 3 }

x25CCTCfgAutoConRetry OBJECT-TYPE

SYNTAX	INTEGER ( 1..2 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object will allow retries to take place when an SVC is being established. The object can have the following values:-

disabled	1,

enabled	2 “

::= { x25CCTEntry 4 }

x25CCTOperAutoConRetryFlgOBJECT-TYPE

SYNTAX	INTEGER ( 1..2 )

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”The current operation state of the x25 connection retry flag . The object can have the following values:-

disabled	1,

enabled	2 “

::= { x25CCTEntry 5 }

x25CCTAutoConRetryTimer OBJECT-TYPE

SYNTAX	INTEGER ( 1..3600 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This is the retry timer, which defines the delay, in seconds, between  consecutive attempts to establish the X25 SVC.”

::= { x25CCTEntry 6 }

x25CCTMaxAutoConRetries  OBJECT-TYPE

SYNTAX	INTEGER ( 0..255 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This defines the maximum no of attempts to set up a single X25 SVC allowed. A  value of 0 means there is no limit to the No. of retries allowed.”

::= { x25CCTEntry 7 }

x25CCTCfgTODControl OBJECT-TYPE

SYNTAX	INTEGER ( 1..2 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object enables/disables the Time-of-Day Control  mechanism, which can  control the time period for which an SVC may be active, between two devices. It  can have the following values:-

disabled	1

enabled	2"

::= { x25CCTEntry 8 }

x25CCTAutoTODControlFlg OBJECT-TYPE

SYNTAX	INTEGER ( 1..2 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”The current operation state of the time of day control flag. This is the current operational value. It  can have the following values:-

disabled	1

enabled	2"

::= { x25CCTEntry  9 }

x25CCTTODToConnect OBJECT-TYPE

SYNTAX	OCTET STRING ( 5 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This defines the time of day at which the Time-of-Day Control mechanism, when enabled, will attempt to establish an SVC to the remote end. If the SVC  exists at this time, then no action is taken. The format of this time should be as  follows :-

hh:mm

and should take values between 00:00 and 23:59 inclusive. These four octets  should be ascii characters within the ranges above.”

::= { x25CCTEntry 10 }

x25CCTTODToDisconnect  OBJECT-TYPE

SYNTAX	OCTET STRING ( 5 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This defines the time of day at which the Time-of-Day Control mechanism, when enabled, will attempt to disconnect an SVC to the remote end. If the SVC is  already disconnected at this time, then no action is taken. The format of this time  should be as follows :-

hh:mm

and should take values between 00:00 and 23:59 inclusive.These four octets  should be ascii characters within the ranges above.”

::= { x25CCTEntry 11 }

x25 Timer Table

x25TimerTable  OBJECT-TYPE

SYNTAX	SEQUENCE OF X25TTEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”This table holds the configurable timers used with each X25 connection  (SVC).”

::= { x25 5 }

x25TTEntry OBJECT-TYPE

SYNTAX	X25TTEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”Each entry holds the configurable values of timers and retry limits in operation  for each SVC.”

INDEX		{ x25TTIndex }

::= { x25TimerTable 1 }

X25TTEntry ::= SEQUENCE {

x25TTIndex

INTEGER,

x25TTT20Timer

INTEGER,

x25TTT21Timer

INTEGER,

x25TTT22Timer

INTEGER,

x25TTT23Timer

INTEGER,

x25TTR20Limit

INTEGER,

x25TTR22Limit

INTEGER,

x25TTR23Limit

INTEGER

}

x25TTIndex  OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION “”

::= { x25TTEntry 1 }

x25TTT20Timer  OBJECT-TYPE

SYNTAX	INTEGER ( 1..1024 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This defines the value of the restart retransmit timer, in seconds.”

::= { x25TTEntry 2 }

x25TTT21Timer  OBJECT-TYPE

SYNTAX	INTEGER ( 1..1024 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This defines the value of the call retransmit timer, in seconds.”

::= { x25TTEntry 3 }

x25TTT22Timer  OBJECT-TYPE

SYNTAX	INTEGER ( 1..1024 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This defines the value of the reset retransmit timer, in seconds.”

::= { x25TTEntry 4 }

x25TTT23Timer  OBJECT-TYPE

SYNTAX	INTEGER ( 1..1024 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This defines the value of the clear retransmit timer, in seconds.”

::= { x25TTEntry 5 }

x25TTR20Limit  OBJECT-TYPE

SYNTAX	INTEGER ( 1..255 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This defines the maximum value of the restart retransmit count, when operating as a DTE.”

::= { x25TTEntry 6 }

x25TTR22Limit  OBJECT-TYPE

SYNTAX	INTEGER ( 1..255 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This defines the maximum value of the reset retransmit count, when operating as a DTE.”

::= { x25TTEntry 7 }

x25TTR23Limit  OBJECT-TYPE

SYNTAX	INTEGER ( 1..255 )

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This defines the maximum value of the clear retransmit count, when operating as a DTE.”

::= { x25TTEntry 8 }

### x25 Status Table tc "x25 Status Table "§

x25StatusTable  OBJECT-TYPE

SYNTAX	SEQUENCE OF  X25StatusTableEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”This table holds status information for each X25 SVC  and for each interface.”

::= { x25 6 }

x25StatusTableEntry OBJECT-TYPE

SYNTAX	X25StatusTableEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”Each entry keeps status information on each Interface/SVC.”

INDEX		{ x25StatusIndex }

::= { x25StatusTable 1 }

X25StatusTableEntry ::= SEQUENCE {

x25StatusIndex

INTEGER,

x25StatusIfStatus

INTEGER,

x25StatusSVCStatus

INTEGER,

x25StatusWinSize

INTEGER,

x25StatusPktSize

INTEGER,

x25StatusCauseLastInClear

INTEGER,

x25StatusDiagLastInClear

INTEGER

}

x25StatusIndex  OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This index pertains to the interface/SVC for which this entry is valid.”

::= { x25StatusTableEntry 1 }

x25StatusIfStatus OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This object defines the current status of the interface. More specifically, this is  the state of the logical channel number 0, which governs the state of the entire 	 SVC.”

::= { x25StatusTableEntry 2 }

x25StatusSVCStatus OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This object defines the current status of the SVC, more specifically, the state of the active logical channel. This object has no meaning if the state of the Interface is not r1PktLevelReady.”

::= { x25StatusTableEntry 3 }

x25StatusWinSize  OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the window size in operation for the current SVC. This may be less than  the Configured Window sizes depending on whether Flow Control is enabled,  and the result of the negotiation.”

::= { x25StatusTableEntry 4 }

x25StatusPktSize  OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the Packet size in use for the current SVC. This may be less than the  Configured values, depending on whether Flow Control is enabled and if  negotiation has taken place.”

::= { x25StatusTableEntry 5 }

x25StatusCauseLastInClear  OBJECT-TYPE

SYNTAX	INTEGER( 0..255 )

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This object defines the Cause of the Last Incoming Clear Received on this  interface. It is in the form of a bit-map, the values and combinations of which are  too numerous to list  here. The values may be found by referring to the  appropriate section in the X25 PLP standard from CCITT.”

::= { x25StatusTableEntry 6 }

x25StatusDiagLastInClear  OBJECT-TYPE

SYNTAX	INTEGER( 0..255 )

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This object defines the Diagnostic of the Last Incoming Clear Received on this  interface. It is in the form of a bit-map, the values and combinations of which are  too numerous to list  here. The values may be found by referring to the  appropriate section in the X25 PLP standard from CCITT.”

::= { x25StatusTableEntry 7 }

### x25 Statistics Tabletc "x25 Statistics Table"§

x25StatsTable  OBJECT-TYPE

SYNTAX	SEQUENCE OF  X25StatsTableEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”This table holds all statistical data for each x25 SVC . There are two entries in  this table, for a 4942.”

::= { x25 7 }

x25StatsTableEntry OBJECT-TYPE

SYNTAX	X25StatsTableEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”Each entry in the Table includes all the statistical objects for each interface.”

INDEX		{ x25STSVCIndex }

::= { x25StatsTable 1 }

X25StatsTableEntry ::= SEQUENCE {

x25STSVCIndex

Counter,

x25STDataPktsTx

Counter,

x25STDataPktsRx

Counter,

x25STConnectReqPktsTx

Counter,

x25STIncomingCallPktsRx

Counter,

x25STClearReqPktsTx

Counter,

x25STClearIndPktsRx

Counter,

x25STResetReqPktsTx

Counter,

x25STResetIndPktsRx

Counter,

x25STRestartReqPktsTx

Counter,

x25STRestartIndPktsRx

Counter

}

x25STSVCIndex  OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This index pertains to the interface for which this entry is valid.”

::= { x25StatsTableEntry 1 }

x25STDataPktsTx  OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the no. of Data Pkts transmitted on this interface.”

::= { x25StatsTableEntry 2 }

x25STDataPktsRx  OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the number of Data Pkts received on this interface.”

::= { x25StatsTableEntry 3 }

x25STConnectReqPktsTx  OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the number of Connect Request Pkts transmitted on this interface.”

::= { x25StatsTableEntry 4 }

x25STIncomingCallPktsRx  OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the number of incoming Call request Pkts received  on this interface.”

::= { x25StatsTableEntry 5 }

x25STClearReqPktsTx  OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the number of Clear Request Pkts transmitted on this interface.”

::= { x25StatsTableEntry 6 }

x25STClearIndPktsRx  OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the number of Clear Indication Pkts received on this interface.”

::= { x25StatsTableEntry 7 }

x25STResetReqPktsTx  OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the number of Reset Request Pkts transmitted on this interface.”

::= { x25StatsTableEntry 8 }

x25STResetIndPktsRx  OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the number of Reset Indication Pkts received on this interface.”

::= { x25StatsTableEntry 9 }

x25STRestartReqPktsTx  OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the number of Restart Request Pkts transmitted on this interface.”

::= { x25StatsTableEntry 10 }

x25STRestartIndPktsRx  OBJECT-TYPE

SYNTAX	Counter

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
”This is the number of Restart Indication Pkts received on this interface.”

::= { x25StatsTableEntry 11 }

## Brdg common grouptc "Brdg common group"§

### ACCESSPolicy mib objectstc "ACCESSPolicy mib objects"§

bcomBpduTimer OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”The time in seconds between the generation of  bridge BPDU’s.  This is the current working value.”

::=  { brdgcommon 1 }

bcomDefaultBpduTimer OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object is non-functioning.”

::=  { brdgcommon 2 }

snmpAccessPolicyObject    OBJECT IDENTIFIER ::= {brdgcommon 3 }

snmpAccessPolicyTable OBJECT-TYPE

SYNTAX	SEQUENCE OF SnmpAccessPolicyEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”This table is a list of entries which are used to authenticate PDUs received by the station. This table contains a maximum of 8 entries.”

::=  { snmpAccessPolicyObject 1 }

snmpAccessPolicyEntry OBJECT-TYPE

SYNTAX	SnmpAccessPolicyEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
”This is an entry in the snmpAccessPolicyTable.”

INDEX		{ accessPolicyIndex }

::=  { snmpAccessPolicyTable 1 }

SnmpAccessPolicyEntry ::= SEQUENCE {

ACCESSPolicyIndex

INTEGER,

communityName

OCTET STRING (SIZE(0..22)),

ACCESSMode

INTEGER,

snmpAccessPolicyType

INTEGER

}

ACCESSPolicyIndex OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This index is used as the instance for a Get or Set operation on the table.”

::=  { snmpAccessPolicyEntry 1 }

communityName OBJECT-TYPE

SYNTAX	OCTET STRING ( SIZE (0..22))

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object is a string, against which the community field in a received PDU is compared. If there exists no community name in the table, with which the received community field forms a match, authentication of the PDU fails.”

::=  { snmpAccessPolicyEntry 2 }

accessMode OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This is a bit significant field, as follows:

bit 0	readAccess

bit 1	writeAccess

Setting the individual bit to 1 enables the type of access, setting it to 0 disables it.”

::=  { snmpAccessPolicyEntry 3 }

snmpAccessPolicyType OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
”This object determines what action is to be performed on a particular entry. A value of valid causes the entry to be added to the table, while a value of invalid causes the entry to be deleted.”

::=  { snmpAccessPolicyEntry 4 }

snmpAccessPolicyAction OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	write-only

STATUS	mandatory

DESCRIPTION
”This object is used to manipulate the table as a whole. It may take the following values:

clearTable	1"

::=  { snmpAccessPolicyObject 2 }

### S3snmpAgent grouptc "S3snmpAgent group"§

s3AgentType OBJECT-TYPE

SYNTAX	INTEGER {

other (1),	— none of the following

m3313 (2),	— AUI ethernet NMM w/ RS232 port

m3313M (3),	— AUI enet NMM w/ RS232 port + modem

m3314-ST (4),	— FOIRL ethernet NMM w/ RS232

m3314M-ST (5),	— FOIRL ethernet NMM w/ RS232 + modem

m2310 (6),	— Model 2310

m3512 (7),	— T.R. NMM w/ STP ring in/ring out

m3514 (8),	— T.R. NMM w/ FOIRL ring in/ring out

m332X (9),	— Low Speed Local Bridge

m3356 (10),	— Ethernet Remote Bridge

m332XS (11),	— High Speed Local Bridge

m3910S (12)	— FDDI Super NMM

}

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
	“The 3000 series agent’s Module type.”

::= {s3AgentSw  1 }

s3AgentFwVer OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The agent’s module firmware version 0=Rev A,1=Rev B,etc.”

::= { s3AgentSw 2 }

s3AgentSwMajorVer OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The bridges major software version number (ie.represents major.minor or 3.0).  Object is in decimal.”

::= { s3AgentSw 3 }

s3AgentSwMinorVer OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The bridges minor software version number (ie. represents major.minor or 3.0).  Object is in decimal.”

::= { s3AgentSw 4 }

s3AgentBootProtocol OBJECT-TYPE

SYNTAX	INTEGER {

other(1),	— boot protocol not listed here

bootp-tftp(2),

tftp-only(3),

ieee802-1e(4),

proprietary(5),	— supported proprietary boot protocol

dll-download(6)	— direct async binary download (Retix)

}

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“Boot protocol used to load the module with its software.”

::= {s3AgentSw  5 }

s3AgentBootFile OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(64))

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“The name of the boot file that is sent to the file server during boot time.  The actual boot protocol used to retreive the file is determined by s3AgentBootProtocol. This object shares its data/functionality with ‘downloadFilename {boot 3}.”

::= { s3AgentSw 6 }

s3AgentAuthTrap OBJECT-TYPE

SYNTAX	INTEGER {

disable(1),	— disable authentication traps

enable(2)	— enable authentication traps

}

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“Enable or disable the use of authentication error trap generation. This object shares it data/functionality with :

snmpEnableAuthenTraps {snmp 30}and

authenticationtrapStatus {station 14} “

::= { s3AgentSw 7 }

s3AgentLocation OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(64))

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“This is a informational string that could be used to show the physical location (i.e. area) of the bridge or concentrator. This object shares its functionality/data with ‘sysLocation’ {system 6}”

::= { s3AgentSw  8 }

s3AgentMibLevel OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“Used to determine current MIB release supported by agent.  The encoding of this object is of the following form: xyy, where x is the major version number and yy is the minor version number.  Thus, version 3.0 would be represented as 300,  version 3.01 would be 301 and 3.1 would be 310.  Object is in decimal.”

::= { s3AgentSw  9 }

s3AgentFeatureLevel OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“Used to determine feature level of the agent.  The encoding of this object is of the following form: xyy, where x is the major version number and yy is the minor version number.  Thus, feature level version 1.0 would be represented as 100, version 1.01 would be 101 and 1.1 would be 110. Object is in decimal.”

::= { s3AgentSw  10 }

s3AgentMySlotID OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“Slot where this agent currently resides.  The Remote bridge hardware is unable to determine this and will always return zero.”

::= { s3AgentSw 11 }

s3AgentUnAuthIP OBJECT-TYPE

SYNTAX	IpAddress

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
“This object contains the IP address of the last station that tried to access this agent with an invalid community string.  This object is only used as a VarBind in an Authentication Trap PDU.”

::= { s3AgentSw  12 }

s3AgentUnAuthComm OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(20))

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
“This object contains the community string of the last unauthenticated attempt to access this agent.  This object is only used as a VarBind in an Authentication Trap PDU.”

::= { s3AgentSw  13 }

s3AgentIpAddr OBJECT-TYPE

SYNTAX	IpAddress

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“The bridge’s administrative IP address.  The current operational IP address can be obtained by the ipAdEntAddr entry in the ipAddrTable.”

::= { s3AgentIpProtocol 1 }

s3AgentNetMask OBJECT-TYPE

SYNTAX	IpAddress

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“The bridge’s administrative subnet mask.”

::= { s3AgentIpProtocol 2 }

s3AgentDefaultGateway OBJECT-TYPE

SYNTAX	IpAddress

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“The bridge’s administrative default gateway IP address.”

::= { s3AgentIpProtocol 3 }

s3AgentBootServerAddr OBJECT-TYPE

SYNTAX	IpAddress

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“The IP address to which the requests for boot files are sent. The protocol used to retrieve the boot ffiles is determined by objects3AgentBootProtocol (section 2).This object shares its functionality and data with ‘bootserverIpAddress’ {boot 4}”

::= { s3AgentIpProtocol 4 }

s3AgentTrapReceiverTable OBJECT-TYPE

SYNTAX	SEQUENCE OF S3AgentTrapReceiverEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
“Table containing list of NMS’s that are to receive traps generated by this bridge.”

::= { s3SnmpAgent 3 }

s3AgentTrapReceiverEntry OBJECT-TYPE

SYNTAX	S3AgentTrapReceiverEntry

ACCESS	not-accessible

STATUS	mandatory

DESCRIPTION
“A destination address and community string to a particular trap server, along with a trap filter and entry aging capability.”

INDEX		{ s3AgentTrapRcvrNetAddress }

::= { s3AgentTrapReceiverTable 1 }

S3AgentTrapReceiverEntry ::= SEQUENCE {

s3AgentTrapRcvrStatus

INTEGER,

s3AgentTrapRcvrNetAddress

IpAddress,

s3AgentTrapRcvrComm

OCTET STRING (SIZE(20)),

s3AgentTrapRcvrAgeTime

TimeTicks}

s3AgentTrapRcvrStatus OBJECT-TYPE

SYNTAX	INTEGER {

other (1),	— none of the following

valid (2),	— a valid address

invalid (3)	— an invalid address

}

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“Setting this object to the value invalid(3) has the effect of invalidating the corresponding entry in the s3AgentTrapReceiverTable.  That is, it effectively disassociates the address identified with the entry by removing the entry from the table. On a read the value of valid(2) should be returned if the entry contains a valid record, or invalid(3) should be returned if the entry contains an invalid record.”

::= { s3AgentTrapReceiverEntry 1 }

s3AgentTrapRcvrNetAddress OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(4))

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“IP address for trap server that is to receive the trap.”

::= { s3AgentTrapReceiverEntry  2 }

s3AgentTrapRcvrComm OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(20))

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“Community string used for traps sent to this trap receiver.”

::= { s3AgentTrapReceiverEntry  3 }

s3AgentTrapRcvrAgeTime OBJECT-TYPE

SYNTAX	TimeTicks

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“This is the time interval used to age entries out of the trap receiver table.  The default value if not specified will be 0, or infinite, never to be aged out.”

::= { s3AgentTrapReceiverEntry  4 }

s3AgentStatus OBJECT-TYPE

SYNTAX	INTEGER {

offline(1),	— the agent is offline

online(2)	— the agent is online

}

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“This indicates whether the bridge is online(2) and operating, or offline(1) (ie: not connected to the concentrator backplane). The Remote Bridge hardware is unable to determine this and will always return ‘online’”

::= { s3AgentHw 1 }

s3AgentMdaHwVer OBJECT-TYPE

SYNTAX	INTEGER

ACCESS	read-only

STATUS	mandatory

DESCRIPTION
“The network management module’s MDA hardware version number (0=Rev. A, 1=Rev. B, etc.). The remote bridge hardware is unable to determine this and will always return zero”

::= { s3AgentHw 2 }

s3AgentMode OBJECT-TYPE

SYNTAX	INTEGER {

primary(1),	— in primary mode

secondary(2),	— in secondary mode

other(3)	— the other values do not apply

}

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“This indicates whether the module is operating in primary(1) or secondary(2) mode. This is not relevant to the remote bridge which will always return ‘other’”

::= { s3AgentHw 3 }

s3AgentReset OBJECT-TYPE

SYNTAX	INTEGER {

noReset(1),	— not resetting

reset(2)	— reset

}

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“ Writing a reset(2) to this object will reset the bridge (stop refreshinng the watch dog).”

::= { s3AgentHw 4 }

s3AgentRestart OBJECT-TYPE

SYNTAX	INTEGER {

noRestart(1),	— not restarting

restart(2)	— restart

}

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“Writing a reset(2) to this object will reset the bridge (stop refreshing the watch dog).”

::= { s3AgentHw 5 }

s3AgentBootMode OBJECT-TYPE

SYNTAX	INTEGER {

eeprom(1),	— get boot info from eeprom

other(2)	— or from a boot protocol

}

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“This parameter will be non-functioning, and will always return ‘eeprom’.  Attempting to write any other value will return a ‘bad-value error.”

::= { s3AgentHw 6 }

s3AgentWriteEeprom OBJECT-TYPE

SYNTAX	INTEGER {

noWriteEeprom(1),	— not writing EEPROM

writeEeprom(2)	— write EEPROM

}

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“This object is non-functioning, any attempt to set the value ‘writeEeprom’ will return a ‘bad-value’ error, and it will always be read as ‘noWrtieEeprom’.”

::= { s3AgentHw 7 }

s3AgentBaudRate OBJECT-TYPE

SYNTAX	Gauge

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“Specifies the baud rate in bits per second of the console port of the bridge.”

::= { s3AgentHw 8 }

s3AgentInitString OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(64))

ACCESS	read-write

STATUS	mandatory

DESCRIPTION
“This parameter will be non-functioning, when read will always return a null string, and any attempt to write a non-null string to it will return a ‘gen-err’.”

::= { s3AgentHw 9 }

END

## Trapstc "Traps"§

### generic traps:tc "generic traps\:"§

coldStart TRAP-TYPE

ENTERPRISE  snmp

DESCRIPTION
“A coldStart trap signifies that the sending protocol entity is reinitializing itself such that the agent’s configuration or the protocol entity implementation may be altered.”

::= 0

warmStart TRAP-TYPE

ENTERPRISE  snmp

DESCRIPTION
“A warmStart trap signifies that the sending protocol entity is reinitializing itself such that neither the agent configuration nor the protocol entity implementation is altered.”

::= 1

linkDown TRAP-TYPE

ENTERPRISE  snmp

VARIABLES  { ifIndex }

DESCRIPTION
“A linkDown trap signifies that the sending protocol entity recognizes a failure in one of the communication links represented in the agent’s configuration.”

::= 2

linkUp TRAP-TYPE

ENTERPRISE  snmp

VARIABLES  { ifIndex }

DESCRIPTION
“A linkUp trap signifies that the sending protocol entity recognizes that one of the communication links represented in the agent’s configuration has come up.”

::= 3

authenticationFailure TRAP-TYPE

ENTERPRISE  snmp

DESCRIPTION
“An authenticationFailure trap signifies that the sending protocol entity is the addressee of a protocol message that is not properly authenticated.  While implementations of the SNMP must be capable of generating this trap, they must also be capable of suppressing the emission of such traps via an implementation- specific mechanism.”

::= 4

synoptics  OBJECT IDENTIFIER ::= { enterprises 45 }

topologyChangeTrap TRAP-TYPE

ENTERPRISE  synoptics

DESCRIPTION
“A topologyChangeTrap signifies a topology change has been detected by the bridge.  This is generated by the root bridge of the network following a topology change when the network has stabilised,  thus preventing a flood of traps.”

::= 2